3GPP TSG-RAN WG4 Meeting # 101-e							         R4-2117777

Electronic Meeting, 1st – 12th November 2021

Agenda item:			8.13.2.1

Source:	Samsung

Title:	NR-NTN co-ex assumption and ACIR results

Document for:	Approval

# Introduction

In this meeting, we proposed some clarifications on existing NR-NTN co-ex assumptions, and then proposed our ACIR results and ACLR suggestions for co-ex scenario 3 and scenario 4.

# ACIR model considerations in UL cases

In the output doc of 100-e meeting, it is agreed to use 30+X and 43+X for ACIR analysis in NR-NTN co-ex study. However, we find it only considered the case when aggressor and victim bandwidth are the same, but did not describe asymmetrical bandwidth cases. Hence, we would like to discuss the asymmetrical cases first in this section.

## Aggressor and victim bandwidths are asymmetrical

Consider the scenario of co-ex between NTN UL and TN UL, the interference experienced by UE1 will caused by a mixture of ACIR 30 + X - FACIR and ACIR 43 + X - FACIR. For victim UE2 and UE3, the interference is caused by ACIR 43 + X – FACIR. The effective interference onto UE1 is dependent upon the aggressor and victim bandwidths.

Then for victim UE1 in this case, we need to use ACIR = 39.94 + X – FACIR instead of 30 + X – FACIR, where FACIR = 10 × LOG10(BWvictim/BWAggressor)

Proposal 1: to clarify the UL ACIR model when NTN UE and NR UE are transmitting in asymmetrical bandwidth. For victim UE1 in this scenario 4, NTN UL interfering TN UL, we need to use ACIR = 39.94 + X – FACIR instead of 30 + X – FACIR, where FACIR = 10 × LOG10(BWvictim/BWAggressor). And for scenario 2, use similar principle to derive ACIR for victim UE1.

# Co-ex study initial results

## Scenario 3: NTN DL interfering TN DL

For LEO 600

| Scenarios | NTN | LEO 600 DL |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TN | Urban DL |  |  |  |  |  |  |  |  |  |  |  |
|  | BS Type | AAS |  |  |  |  |  | non AAS |  |  |  |  |  |
| Required ACIR [dB] |  | 5 | 10 | 15 | 20 | 25 | 30 | 5 | 10 | 15 | 20 | 25 | 30 |
| Throughput Loss | Average | 1.55 | 0.54 | 1.18 | 0.06 | 0.02 | 0.01 | 0.14 | 0.05 | 0.01 | 0.00 | 0.00 | 0.00 |
|  | 5%-tile | 0.80 | 0.25 | 0.08 | 0.03 | 0.01 | 0.00 | 0.52 | 0.17 | 0.05 | 0.02 | 0.01 | 0.00 |
| SINR Loss | Average | 15.15 | 6.41 | 2.36 | 0.80 | 0.26 | 0.08 | 0.16 | 0.05 | 0.02 | 0.01 | 0.00 | 0.00 |
|  | 5%-tile | 1.88 | 0.59 | 0.19 | 0.06 | 0.02 | 0.00 | 0.57 | 0.18 | 0.06 | 0.02 | 0.01 | 0.00 |

| Scenarios | NTN | LEO 600 DL |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TN | Rural DL |  |  |  |  |  |  |  |  |  |  |  |
|  | BS Type | AAS |  |  |  |  |  | non AAS |  |  |  |  |  |
| Required ACIR [dB] |  | 5 | 10 | 15 | 20 | 25 | 30 | 5 | 10 | 15 | 20 | 25 | 30 |
| Throughput Loss | Average | 34.10 | 20.05 | 9.52 | 3.72 | 1.28 | 0.42 | 22.74 | 10.38 | 3.94 | 1.34 | 0.44 | 0.14 |
|  | 5%-tile | 73.68 | 47.57 | 22.90 | 8.77 | 2.96 | 0.96 | 100 | 100 | 15.00 | 5.24 | 1.71 | 0.55 |
| SINR Loss | Average | 59.18 | 39.03 | 20.51 | 8.56 | 3.04 | 1.00 | 21.11 | 9.92 | 3.87 | 1.34 | 0.44 | 0.14 |
|  | 5%-tile | 85.02 | 63.90 | 35.84 | 15.03 | 5.28 | 1.74 | 66.82 | 38.03 | 15.92 | 5.60 | 1.84 | 0.59 |

For LEO 1200

| Scenarios | NTN | LEO 1200 DL |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TN | Urban DL |  |  |  |  |  |  |  |  |  |  |  |
|  | BS Type | AAS |  |  |  |  |  | non AAS |  |  |  |  |  |
| Required ACIR [dB] |  | 5 | 10 | 15 | 20 | 25 | 30 | 5 | 10 | 15 | 20 | 25 | 30 |
| Throughput Loss | Average | 1.62 | 0.56 | 0.18 | 0.06 | 0.02 | 0.01 | 0.15 | 0.05 | 0.02 | 0.00 | 0.00 | 0.00 |
|  | 5%-tile | 0.84 | 0.26 | 0.08 | 0.03 | 0.01 | 0.00 | 0.55 | 0.17 | 0.05 | 0.02 | 0.01 | 0.00 |
| SINR Loss | Average | 15.57 | 6.61 | 2.44 | 0.82 | 0.027 | 0.09 | 0.16 | 0.05 | 0.02 | 0.01 | 0.00 | 0.00 |
|  | 5%-tile | 1.97 | 0.61 | 0.20 | 0.06 | 0.02 | 0.01 | 0.59 | 0.19 | 0.06 | 0.02 | 0.01 | 0.00 |

| Scenarios | NTN | LEO 1200 DL |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TN | Rural DL |  |  |  |  |  |  |  |  |  |  |  |
|  | BS Type | AAS |  |  |  |  |  | non AAS |  |  |  |  |  |
| Required ACIR [dB] |  | 5 | 10 | 15 | 20 | 25 | 30 | 5 | 10 | 15 | 20 | 25 | 30 |
| Throughput Loss | Average | 35.52 | 21.20 | 10.20 | 4.01 | 1.34 | 0.45 | 24.11 | 11.15 | 4.26 | 1.45 | 0.47 | 0.15 |
|  | 5%-tile | 75.13 | 49.47 | 24.30 | 9.41 | 3.21 | 1.04 | 100 | 100 | 16.14 | 5.71 | 1.87 | 0.60 |
| SINR Loss | Average | 61.37 | 41.34 | 22.18 | 9.37 | 3.36 | 1.11 | 22.35 | 10.63 | 4.18 | 1.45 | 0.47 | 0.15 |
|  | 5%-tile | 85.99 | 65.68 | 37.67 | 16.06 | 5.71 | 1.88 | 68.61 | 40.09 | 17.10 | 6.09 | 2.00 | 0.64 |

For GEO

| Scenarios | NTN | GEO DL |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TN | Urban DL |  |  |  |  |  |  |  |  |  |  |  |
|  | BS Type | AAS |  |  |  |  |  | non AAS |  |  |  |  |  |
| Required ACIR [dB] |  | 5 | 10 | 15 | 20 | 25 | 30 | 5 | 10 | 15 | 20 | 25 | 30 |
| Throughput Loss | Average | 0.16 | 0.05 | 0.02 | 0.01 | 0.00 | 0.00 | 0.01 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
|  | 5%-tile | 0.07 | 0.02 | 0.01 | 0.00 | 0.00 | 0.00 | 0.05 | 0.02 | 0.01 | 0.00 | 0.00 | 0.00 |
| SINR Loss | Average | 2.15 | 0.73 | 0.23 | 0.07 | 0.02 | 0.01 | 0.01 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
|  | 5%-tile | 0.18 | 0.05 | 0.02 | 0.00 | 0.00 | 0.00 | 0.05 | 0.02 | 0.01 | 0.00 | 0.00 | 0.00 |

| Scenarios | NTN | GEO DL |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TN | Rural DL |  |  |  |  |  |  |  |  |  |  |  |
|  | BS Type | AAS |  |  |  |  |  | non AAS |  |  |  |  |  |
| Required ACIR [dB] |  | 5 | 10 | 15 | 20 | 25 | 30 | 5 | 10 | 15 | 20 | 25 | 30 |
| Throughput Loss | Average | 9.36 | 3.62 | 1.24 | 0.40 | 0.13 | 0.04 | 3.84 | 1.30 | 0.42 | 0.13 | 0.04 | 0.01 |
|  | 5%-tile | 22.27 | 8.49 | 2.88 | 0.93 | 0.30 | 0.09 | 14.64 | 5.11 | 1.67 | 0.54 | 0.17 | 0.05 |
| SINR Loss | Average | 20.59 | 8.51 | 3.01 | 0.99 | 0.32 | 0.10 | 3.78 | 1.30 | 0.42 | 0.14 | 0.04 | 0.01 |
|  | 5%-tile | 34.99 | 14.58 | 5.13 | 1.68 | 0.54 | 0.17 | 15.53 | 5.46 | 1.79 | 0.57 | 0.18 | 0.06 |

## Scenario 4: NTN UL interfering TN UL

For LEO 600

| Scenarios | NTN | 0dBi UE UL |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TN | Urban UL |  |  |  |  |  |  |  |  |  |  |  |
|  | BS Type | AAS |  |  |  |  |  | non AAS |  |  |  |  |  |
| Required ACIR [dB] |  | 5 | 10 | 15 | 20 | 25 | 30 | 5 | 10 | 15 | 20 | 25 | 30 |
| Throughput Loss | Average | 17.88 | 10.71 | 6.09 | 3.31 | 1.73 | 0.87 | 20.20 | 10.72 | 5.65 | 3.07 | 1.62 | 0.77 |
|  | 5%-tile | 71.47 | 51.39 | 31.51 | 16.52 | 8.11 | 4.16 | N/A | N/A | N/A | N/A | N/A | N/A |
| SINR Loss | Average | 32.17 | 20.68 | 12.44 | 7.09 | 3.87 | 2.02 | 20.29 | 10.93 | 5.72 | 3.03 | 1.61 | 0.80 |
|  | 5%-tile | 82.11 | 65.44 | 44.62 | 25.42 | 13.09 | 6.87 | 37.21 | 23.37 | 10.52 | 2.33 | 0.27 | 0.64 |

| Scenarios | NTN | 0dBi UE UL |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TN | Rural UL |  |  |  |  |  |  |  |  |  |  |  |
|  | BS Type | AAS |  |  |  |  |  | non AAS |  |  |  |  |  |
| Required ACIR [dB] |  | 5 | 10 | 15 | 20 | 25 | 30 | 5 | 10 | 15 | 20 | 25 | 30 |
| Throughput Loss | Average | 0.62 | 0.24 | 0.89 | 0.03 | 0.01 | 0.01 | 1.08 | 0.41 | 0.16 | 0.07 | 0.03 | 0.01 |
|  | 5%-tile | 1.91 | 0.69 | 0.26 | 0.10 | 0.04 | 0.02 | N/A | N/A | N/A | N/A | N/A | N/A |
| SINR Loss | Average | 1.37 | 0.55 | 0.21 | 0.08 | 0.03 | 0.01 | 2.32 | 0.86 | 0.33 | 0.13 | 0.05 | 0.02 |
|  | 5%-tile | 2.18 | 0.79 | 0.30 | 0.12 | 0.05 | 0.02 | 0.66 | 0.26 | 0.08 | 0.02 | 0.01 | 0.00 |

For LEO 1200

| Scenarios | NTN | 0dBi UE UL |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TN | Urban UL |  |  |  |  |  |  |  |  |  |  |  |
|  | BS Type | AAS |  |  |  |  |  | non AAS |  |  |  |  |  |
| Required ACIR [dB] |  | 5 | 10 | 15 | 20 | 25 | 30 | 5 | 10 | 15 | 20 | 25 | 30 |
| Throughput Loss | Average | 18.05 | 10.82 | 6.16 | 3.35 | 1.75 | 0.89 | 20.43 | 10.84 | 5.72 | 3.10 | 1.65 | 0.78 |
|  | 5%-tile | 71.83 | 51.80 | 31.84 | 16.72 | 8.21 | 4.22 | N/A | N/A | N/A | N/A | N/A | N/A |
| SINR Loss | Average | 32.44 | 20.88 | 12.57 | 7.17 | 3.92 | 2.05 | 20.51 | 11.06 | 5.78 | 3.07 | 1.63 | 0.81 |
|  | 5%-tile | 82.37 | 65.82 | 45.01 | 25.71 | 13.25 | 6.96 | 37.51 | 23.67 | 10.72 | 2.40 | 0.28 | 0.07 |

| Scenarios | NTN | 0dBi UE UL |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TN | Rural UL |  |  |  |  |  |  |  |  |  |  |  |
|  | BS Type | AAS |  |  |  |  |  | non AAS |  |  |  |  |  |
| Required ACIR [dB] |  | 5 | 10 | 15 | 20 | 25 | 30 | 5 | 10 | 15 | 20 | 25 | 30 |
| Throughput Loss | Average | 0.63 | 0.25 | 0.09 | 0.03 | 0.01 | 0.01 | 1.12 | 0.42 | 0.17 | 0.07 | 0.03 | 0.01 |
|  | 5%-tile | 1.95 | 0.71 | 0.26 | 0.11 | 0.04 | 0.02 | N/A | N/A | N/A | N/A | N/A | N/A |
| SINR Loss | Average | 1.40 | 0.57 | 0.21 | 0.08 | 0.03 | 0.01 | 2.40 | 0.89 | 0.34 | 0.14 | 0.06 | 0.02 |
|  | 5%-tile | 2.24 | 0.81 | 0.30 | 0.12 | 0.05 | 0.02 | 0.68 | 0.26 | 0.08 | 0.02 | 0.01 | 0.00 |

For GEO

| Scenarios | NTN | 0dBi UE UL |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TN | Urban UL |  |  |  |  |  |  |  |  |  |  |  |
|  | BS Type | AAS |  |  |  |  |  | non AAS |  |  |  |  |  |
| Required ACIR [dB] |  | 5 | 10 | 15 | 20 | 25 | 30 | 5 | 10 | 15 | 20 | 25 | 30 |
| Throughput Loss | Average | 18.05 | 10.83 | 6.16 | 3.35 | 1.75 | 0.89 | 20.44 | 10.85 | 5.72 | 3.10 | 1.65 | 0.78 |
|  | 5%-tile | 71.83 | 51.85 | 31.83 | 16.77 | 8.22 | 4.22 | N/A | N/A | N/A | N/A | N/A | N/A |
| SINR Loss | Average | 32.45 | 20.88 | 12.57 | 7.17 | 3.92 | 2.05 | 20.52 | 11.06 | 5.79 | 3.07 | 1.63 | 0.81 |
|  | 5%-tile | 82.37 | 65.88 | 45.01 | 25.78 | 13.26 | 6.97 | 37.56 | 23.74 | 10.78 | 2.41 | 0.28 | 0.07 |

| Scenarios | NTN | 0dBi UE UL |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TN | Rural UL |  |  |  |  |  |  |  |  |  |  |  |
|  | BS Type | AAS |  |  |  |  |  | non AAS |  |  |  |  |  |
| Required ACIR [dB] |  | 5 | 10 | 15 | 20 | 25 | 30 | 5 | 10 | 15 | 20 | 25 | 30 |
| Throughput Loss | Average | 0.62 | 0.24 | 0.09 | 0.03 | 0.01 | 0.01 | 1.08 | 0.41 | 0.16 | 0.07 | 0.03 | 0.01 |
|  | 5%-tile | 1.91 | 0.70 | 0.26 | 0.10 | 0.04 | 0.02 | N/A | N/A | N/A | N/A | N/A | N/A |
| SINR Loss | Average | 1.36 | 0.55 | 0.21 | 0.08 | 0.03 | 0.01 | 2.32 | 0.86 | 0.33 | 0.13 | 0.05 | 0.02 |
|  | 5%-tile | 2.18 | 0.80 | 0.30 | 0.12 | 0.05 | 0.02 | 0.67 | 0.26 | 0.08 | 0.02 | 0.01 | 0.00 |

Observation 1: the TN NR system parameters may need improvement, at least for non-AAS case. The current parameter set results in no throughput @5% for non-AAS BS antenna cases. However, the SINR loss shows the NTN UL interference to non-AAS antenna is similar to the impact results of AAS antenna. So, the overall results can be acceptable.

Proposal 2: to adopt the following table as template to summarize co-ex ACIR results in TR 38.863.

| Required ACIR [dB] |  | 5 | 10 | 15 | 20 | 25 | 30 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Throughput Loss | Average |  |  |  |  |  |  |
|  | 5%-tile |  |  |  |  |  |  |

Proposal 3: to include the above results in NR-NTN co-ex simulation results section of TR 38.863.

## ACIR results and ACLR/ACS analysis

Based on the results above, for scenario 3 NTN DL interfering TN DL case, the required ACIR offset is -6.44, and then the required ACIR is 23.56 dB. Given the TN UE’s ACS is 33 dB, then the required ACLR for NTN BS should be 24.08 dB.

For scenario 4 NTN UL interfering TN UL case, the required ACIR offset is -0.98, then the required ACIR is 29.02 dB. Gvien the TN BS’s ACS is 46 dB, then the required ACLR for NTN UE should be 29.11 dB.

Proposal 4: to suggest 24.08 dB as ACLR for NTN BS, and suggest 29.01 dB as ACLR for NTN UE.

# Conclusion

In this contribution, we submit one observation and three proposals as follows:

Proposal 1: to clarify the UL ACIR model when NTN UE and NR UE are transmitting in asymmetrical bandwidth. For victim UE1 in this scenario 4, NTN UL interfering TN UL, we need to use ACIR = 39.94 + X – FACIR instead of 30 + X – FACIR, where FACIR = 10 × LOG10(BWvictim/BWAggressor). And for scenario 2, use similar principle to derive ACIR for victim UE1.

Proposal 2: to adopt the following table as template to summarize co-ex ACIR results in TR 38.863.

| Required ACIR [dB] |  | 5 | 10 | 15 | 20 | 25 | 30 | … |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Throughput Loss | Average |  |  |  |  |  |  |  |
|  | 5%-tile |  |  |  |  |  |  |  |

Proposal 3: to include the above results in NR-NTN co-ex simulation results section of TR 38.863.

Proposal 4: to suggest 24.08 dB as ACLR for NTN BS, and suggest 29.01 dB as ACLR for NTN UE.

Observation 1: the TN NR system parameters may need improvement, at least for non-AAS case. The current parameter set results in no throughput @5% for non-AAS BS antenna cases. However, the SINR loss shows the NTN UL interference to non-AAS antenna is similar to the impact results of AAS antenna. So, the overall results can be acceptable.

# Reference

[1]. R4-2115750, Simulation assumptions for NTN co-existence study, Moderator.