3GPP TSG-RAN WG4 Meeting #101-e                                     R4-2117818

Electronic Meeting, 1st-12th November, 2021

Source: 	Xiaomi

Title: 	Discussion on measurement procedure for RedCap UE

Agenda Item:	8.20.3.1.5

Document for:	Discussion

# 1 Introduction

In last RAN4#100-e meeting, discussion on RRM requirements for RedCap UE was conducted and a way forward was agreed in [1]. The open issues concerns measurement procedure are captured below:

CSSF design for RedCap UE measurements - Proposal 1: - For measurement outside gap: depends on whether there are frequency layers competing the searcher. - For measurement within gap: RAN4 needs to revisit the design for CSSF within gap to guarantee PCell’s mobility. Gapless measurement capabilities for RedCap UE: ‘inter-frequency without gap’ - Proposal 1 (Ericsson, Apple, Vivo, Nokia): RedCap UE won’t support ‘Inter-frequency without gap’ measurement capability. - Proposal 2 (Huawei, CMCC): Depends on network deployment. - Proposal 3 (Xiaomi, Oppo, MTK): FFS

In this contribution, we provide our consideration on the measurement procedure for RedCap UE.

- Discussion

Inter-frequency without gap

The UE can perform inter-frequency SSB based measurements without measurement gaps if the SSB is completely contained in the active BWP of the UE. As agreed in last meeting to define requirements for 1RX and 2RX capable RedCap UEs based on single searcher assumption, we think it is reasonable for RedCap UE not supporting ‘inter-frequency without gap’ measurement capability.

Proposal 1: RedCap UE does not support inter-frequency without gap measurement in Rel-17.

CSSF design for RedCap UE measurements

For CSSF outside gap, as we supported that RedCap UE won’t support ‘Inter-frequency without gap’ measurement capability, we prefer to define the CSSFoutside_gap,i = 1.

For CSSF within gap, the requirements were specified based on the assumption that UE conducts the measurement of one frequency layer at a time, no matter the layer is intra frequency or inter frequency. We think the existing MG sharing mechanism could be reused.

Proposal 2: The current design for CSSF within gap could be reused for RedCap UE.

- Conclusion

Proposal 1: RedCap UE does not support inter-frequency without gap measurement in Rel-17.

Proposal 2: The current design for CSSF within gap could be reused for RedCap UE.

- Reference

[1] R4-2115358 WF on RedCap RRM requirements, Ericsson