3GPP TSG-RAN WG4 Meeting # 101-e	R4-2117831

# Electronic Meeting, November 01-12, 2021

Source:	LG Electronics

Title:	MPR for NR V2X intra-band con-current operation with Uu

Agenda item:	8.15.4.4

Document for:	Approval

Introduction

For MPR in TR 38.785[2], we provide more MPR simulation results considering a combination of each modulation order(MO). Based on the simulation results, we would like to propose the MPR for NR V2X intra-band con-current operation.

Discussion

In [2], the MPRs for PC3 and PC2 are captured as follows.

Table 5.2.4.1.2-3: Contiguous RB allocation for Power Class 3

| Modulation |  | MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | ≤ [1.0] | ≤ [1.0] |
|  | 16QAM | ≤ [1.0] | ≤ [1.0] |
|  | 64QAM | ≤ [1.0] | ≤ [1.0] |
|  | 256QAM | ≤ [1.5] | ≤ [1.5] |

Table 5.2.4.1.2-4: Non-contiguous RB allocation for Power Class 3

| Modulation |  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | inner | Outer1 | Outer2 |
| CP-OFDM | QPSK | ≤ [1.0] | ≤ [1.5] | ≤ [3.0] |
|  | 16QAM | ≤ [1.0] | ≤ [1.5] | ≤ [3.0] |
|  | 64QAM | ≤ [1.0] | ≤ [1.5] | ≤ [3.0] |
|  | 256QAM | ≤ [1.5] | ≤ [2.0] | ≤ [3.0] |

Table 5.2.4.1.2-5: Contiguous RB allocation for Power Class 2

| Modulation |  | MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | ≤ [1.5] | ≤ [3.5] |
|  | 16QAM | ≤ [2.0] | ≤ [3.5] |
|  | 64QAM | ≤ [3.0] | ≤ [3.5] |
|  | 256QAM | ≤ [5.0] | ≤ [5.0] |

Table 5.2.4.1.2-6: Non-contiguous RB allocation for Power Class 2

| Modulation |  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | inner | Outer1 | Outer2 |
| CP-OFDM | QPSK | ≤ [2.0] | ≤ [4.0] | ≤ [6.0] |
|  | 16QAM | ≤ [2.5] | ≤ [4.0] | ≤ [6.0] |
|  | 64QAM | ≤ [3.5] | ≤ [4.5] | ≤ [6.0] |
|  | 256QAM | ≤ [4.5] | ≤ [5.0] | ≤ [6.0] |

For the MPRs, we did more simulations with different combinations of SL MO and UL MO with the following simulation assumptions.

Considered simulation assumptions.

- Channel BW configurations for class B - 10MHz (NR SL, 30kHz SCS) + 10MHz (NR Uu, 30kHz SCS) - Waveform - CP-OFDM(NR SL) + CP-OFDM(NR Uu) - Maximum total output power - 26dBm : 23dBm 2 PAs - Modulation order - {SL, UL} = {QPSK, QPSK/16QAM/64QAM/256QAM} {16QAM, QPSK/16QAM/64QAM/256QAM} {64QAM, QPSK/16QAM/64QAM/256QAM} {256QAM, QPSK/16QAM/64QAM/256QAM} - RB allocation - Refer table 2-1 & 2-2

The following tables show simulation results of MPR with a maximum total output power of 26dBm assuming 23dBm 2 PAs.

- Table 2.1: MRR for contiguous RB allocation

- Table 2.2: MPR for non-contiguous RB allocation

Table 2.1 : MPR for  contiguous RB allocation in 10MHz(SL) + 10MHz(UL) @ Max.Total.Power = 26dBm

| SL Config. | UL Config. |  |  | MPR |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CBW(10MHz): SCS(30kHz) | CBW(10MHz): SCS(30kHz) |  |  |  |  |  |  |
| (L_CRB,StartRBidx) |  |  |  | UL Modulation Order(MO) |  |  |  |
| SL_RB | UL_RB | Inner/Outer | SL MO | QPSK | 16QAM | 64QAM | 256QAM |
| (10,14) | (1,0) | Inner | QPSK | 0.13 | 0.43 | 1.47 | 3.14 |
| (10,14) | (13,0) | Inner | QPSK | 0.28 | 0.69 | 1.90 | 3.89 |
| (10,14) | (1,0) | Inner | 16QAM | 0.84 | 0.81 | 1.48 | 3.18 |
| (10,14) | (13,0) | Inner | 16QAM | 0.63 | 0.68 | 1.91 | 3.91 |
| (10,14) | (1,0) | Inner | 64QAM | 2.08 | 2.15 | 2.08 | 3.04 |
| (10,14) | (13,0) | Inner | 64QAM | 1.86 | 1.88 | 1.96 | 3.90 |
| (10,14) | (1,0) | Inner | 256QAM | 4.06 | 4.07 | 4.11 | 4.02 |
| (10,14) | (13,0) | Inner | 256QAM | 3.72 | 3.64 | 3.67 | 3.87 |
| (10,14) | (14,0) | Outer | QPSK | 0.40 | 0.77 | 2.00 | 3.91 |
| (10,14) | (24,0) | Outer | QPSK | 2.29 | 2.27 | 2.28 | 4.54 |
| (10,14) | (14,0) | Outer | 16QAM | 0.63 | 0.75 | 1.99 | 3.98 |
| (10,14) | (24,0) | Outer | 16QAM | 2.30 | 2.27 | 2.27 | 4.42 |
| (10,14) | (14,0) | Outer | 64QAM | 1.82 | 1.86 | 2.00 | 3.95 |
| (10,14) | (24,0) | Outer | 64QAM | 2.29 | 2.29 | 2.27 | 4.43 |
| (10,14) | (14,0) | Outer | 256QAM | 3.73 | 3.64 | 3.69 | 3.99 |
| (10,14) | (24,0) | Outer | 256QAM | 3.65 | 3.64 | 3.68 | 4.31 |

Table 2.2 : MPR for  non-contiguous RB allocation in 10MHz(SL) + 10MHz(UL) @ Max.Total.Power = 26dBm

| SL Config. | UL Config. |  |  | MPR |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CBW(10MHz): SCS(30kHz) | CBW(10MHz): SCS(30kHz) |  |  |  |  |  |  |
| (L_CRB,StartRBidx) |  |  |  | UL Modulation Order(MO) |  |  |  |
| SL_RB | UL_RB | Inner/Outer1 /Outer2 | SL MO | QPSK | 16QAM | 64QAM | 256QAM |
| (10,14) | (1,2) | Inner | QPSK | 0.14 | 0.34 | 1.53 | 3.20 |
| (10,14) | (1,2) | Inner | 16QAM | 0.68 | 0.66 | 1.40 | 3.10 |
| (10,14) | (1,2) | Inner | 64QAM | 1.90 | 1.94 | 1.92 | 3.07 |
| (10,14) | (1,2) | Inner | 256QAM | 3.85 | 3.86 | 3.86 | 3.86 |
| (10,14) | (14,3) | Outer1 | QPSK | 1.10 | 1.13 | 1.93 | 3.91 |
| (10,14) | (14,10) | Outer1 | QPSK | 2.46 | 2.47 | 2.44 | 3.70 |
| (10,14) | (14,3) | Outer1 | 16QAM | 1.10 | 1.10 | 1.92 | 3.92 |
| (10,14) | (14,10) | Outer1 | 16QAM | 2.46 | 2.49 | 2.46 | 3.66 |
| (10,14) | (14,3) | Outer1 | 64QAM | 1.81 | 1.81 | 1.93 | 3.97 |
| (10,14) | (14,10) | Outer1 | 64QAM | 2.46 | 2.44 | 2.47 | 3.73 |
| (10,14) | (14,3) | Outer1 | 256QAM | 3.67 | 3.65 | 3.55 | 3.89 |
| (10,14) | (14,10) | Outer1 | 256QAM | 3.64 | 3.58 | 3.64 | 3.68 |
| (10,9) | (1,12) | Outer2 | QPSK | 2.29 | 2.29 | 2.29 | 3.71 |
| (10,9) | (1,12) | Outer2 | 16QAM | 2.29 | 2.32 | 2.32 | 4.30 |
| (10,9) | (1,12) | Outer2 | 64QAM | 2.29 | 2.30 | 2.30 | 3.86 |
| (10,9) | (1,12) | Outer2 | 256QAM | 3.81 | 3.85 | 3.92 | 4.08 |

From Table 2.1 and Table 2.2, we can see different MPRs under the same SL MO according to the combination of UL MO.

Table 2.3 and Table 2.4 summarize maximum MPR among the relative differences to MPR of the combination of {QPSK, QPSK} for all MO combinations in each Inner/Outer for Table 2.3 and in each Inner/Outer1/Outer2 for Table 2.4.

Table 2.3 : Relative maximum difference of other MO combinations from a maximum MPR of {QPSK, QPSK} for contiguous RB allocation in 10MHz(SL) + 10MHz(UL) @ Max.Total.Power = 26dBm

|  |  | Relative MPR from {QPSK, QPSK} |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | UL Modulation Order(MO) |  |  |  |
| Inner/Outer | SL MO | QPSK | 16QAM | 64QAM | 256QAM |
| Inner | QPSK | 0 | 0.41 | 1.62 | 3.61 |
|  | 16QAM | 0.56 | 0.53 | 1.63 | 3.63 |
|  | 64QAM | 1.8 | 1.87 | 1.8 | 3.62 |
|  | 256QAM | 3.78 | 3.79 | 3.83 | 3.74 |
| Outer | QPSK | 0 | -0.02 | -0.01 | 2.25 |
|  | 16QAM | 0.01 | -0.02 | -0.02 | 2.13 |
|  | 64QAM | 0 | 0 | -0.02 | 2.14 |
|  | 256QAM | 1.44 | 1.35 | 1.4 | 2.02 |

Table 2.4 : Relative maximum difference of other MO combinations from a maximum MPR of {QPSK, QPSK}  for  non-contiguous RB allocation in 10MHz(SL) + 10MHz(UL) @ Max.Total.Power = 26dBm

|  |  | MPR |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | UL Modulation Order(MO) |  |  |  |
| Inner/Outer1 /Outer2 | SL MO | QPSK | 16QAM | 64QAM | 256QAM |
| Inner | QPSK | 0 | 0.2 | 1.39 | 3.06 |
|  | 16QAM | 0.54 | 0.52 | 1.26 | 2.96 |
|  | 64QAM | 1.76 | 1.8 | 1.78 | 2.93 |
|  | 256QAM | 3.71 | 3.72 | 3.72 | 3.72 |
| Outer1 | QPSK | 0 | 0.01 | -0.02 | 1.45 |
|  | 16QAM | 0 | 0.03 | 0 | 1.46 |
|  | 64QAM | 0 | -0.02 | 0.01 | 1.51 |
|  | 256QAM | 1.21 | 1.19 | 1.18 | 1.43 |
| Outer2 | QPSK | 0 | 0 | 0 | 1.42 |
|  | 16QAM | 0 | 0.03 | 0.03 | 2.01 |
|  | 64QAM | 0 | 0.01 | 0.01 | 1.57 |
|  | 256QAM | 1.52 | 1.56 | 1.63 | 1.79 |

Based on the difference, there can be 2 options for how to define MPR for intra-band con-current operation in a licensed band.

  - Option1: Define MPR for all combinations of SL MO and UL MO

  - Option 2: Define MPR based on the combination of SL MO and highest UL MO

For Option 1,

- PC2 MPR can be updated as

  - Table 2.5 for contiguous RB allocation taking the difference of Table 2.3 on top of Table 5.2.4.1.2-5 into account

  - Table 2.6 for non-contiguous RB allocation taking the difference of Table 2.4 on top of Table 5.2.4.1.2-6 into account.

- PC3 MPR can be updated as

  - Table 2.7 for contiguous RB allocation taking the difference of Table 2.3 on top of Table 5.2.4.1.2-3 into account

  - Table 2.8 for non-contiguous RB allocation taking the difference of Table 2.4 on top of Table 5.2.4.1.2-4 into account.

For Option 2,

- PC2 MPR can be updated as

  - Table 2.9 for contiguous RB allocation taking the difference of Table 2.3 on top of Table 5.2.4.1.2-5 into account

  - Table 2.10 for non-contiguous RB allocation taking the difference of Table 2.4 on top of Table 5.2.4.1.2-6 into account.

- PC3 MPR can be updated as

  - Table 2.11 for contiguous RB allocation taking the difference of Table 2.3 on top of Table 5.2.4.1.2-3 into account

  - Table 2.12 for non-contiguous RB allocation taking the difference of Table 2.4 on top of Table 5.2.4.1.2-4 into account.

Table 2.5: Contiguous RB allocation for Power Class 2 (Option 1)

|  |  | MPR for bandwidth class B(dB) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner |  |  |  | outer |  |  |  |
| Modulation |  | UL MO |  |  |  | UL MO |  |  |  |
| SL MO |  | QPSK | 16QAM | 64QAM | 256QAM | QPSK | 16QAM | 64QAM | 256QAM |
| CP-OFDM | QPSK | ≤ [1.5] | ≤ [2.0] | ≤ [3.0] | ≤ [5.0] | ≤ [3.5] | ≤ [3.5] | ≤ [3.5] | ≤ [5.5] |
|  | 16QAM | ≤ [2.0] | ≤ [2.0] | ≤ [3.0] | ≤ [5.0] | ≤ [3.5] | ≤ [3.5] | ≤ [3.5] | ≤ [5.5] |
|  | 64QAM | ≤ [3.0] | ≤ [3.0] | ≤ [3.0] | ≤ [5.0] | ≤ [3.5] | ≤ [3.5] | ≤ [3.5] | ≤ [5.5] |
|  | 256QAM | ≤ [5.0] | ≤ [5.0] | ≤ [5.0] | ≤ [5.0] | ≤ [5.0] | ≤ [5.0] | ≤ [5.0] | ≤ [5.5] |

Table 2.6: Non-Contiguous RB allocation for Power Class 2 (Option 1)

|  |  | MPR for bandwidth class B(dB) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner |  |  |  | Outer1 |  |  |  | Outer2 |  |  |  |
| Modulation |  | UL MO |  |  |  | UL MO |  |  |  | UL MO |  |  |  |
| SL MO |  | QPSK | 16QAM | 64QAM | 256QAM | QPSK | 16QAM | 64QAM | 256QAM | QPSK | 16QAM | 64QAM | 256QAM |
| CP-OFDM | QPSK | ≤ [2.0] | ≤ [2.0] | ≤ [3.0] | ≤ [5.0] | ≤ [4.0] | ≤ [4.0] | ≤ [4.0] | ≤ [5.5] | ≤ [6.0] | ≤ [6.0] | ≤ [6.0] | ≤ [6.5] |
|  | 16QAM | ≤ [2.5] | ≤ [2.5] | ≤ [3.0] | ≤ [5.0] | ≤ [4.0] | ≤ [4.0] | ≤ [4.0] | ≤ [5.5] | ≤ [6.0] | ≤ [6.0] | ≤ [6.0] | ≤ [6.5] |
|  | 64QAM | ≤ [3.5] | ≤ [3.5] | ≤ [3.5] | ≤ [5.0] | ≤ [4.5] | ≤ [4.5] | ≤ [4.5] | ≤ [5.5] | ≤ [6.0] | ≤ [6.0] | ≤ [6.0] | ≤ [6.5] |
|  | 256QAM | ≤ [5.5] | ≤ [5.5] | ≤ [5.5] | ≤ [5.5] | ≤ [5.5] | ≤ [5.5] | ≤ [5.5] | ≤ [5.5] | ≤ [6.5] | ≤ [6.5] | ≤ [6.5] | ≤ [6.5] |

Table 2.7: Contiguous RB allocation for Power Class 3 (Option 1)

|  |  | MPR for bandwidth class B(dB) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner |  |  |  | outer |  |  |  |
| Modulation |  | UL MO |  |  |  | UL MO |  |  |  |
| SL MO |  | QPSK | 16QAM | 64QAM | 256QAM | QPSK | 16QAM | 64QAM | 256QAM |
| CP-OFDM | QPSK | ≤ [1.0] | ≤ [1.0] | ≤ [1.0] | ≤ [2.0] | ≤ [1.0] | ≤ [1.0] | ≤ [1.0] | ≤ [2.5] |
|  | 16QAM | ≤ [1.0] | ≤ [1.0] | ≤ [1.0] | ≤ [2.0] | ≤ [1.0] | ≤ [1.0] | ≤ [1.0] | ≤ [2.5] |
|  | 64QAM | ≤ [1.0] | ≤ [1.0] | ≤ [1.0] | ≤ [2.0] | ≤ [1.0] | ≤ [1.0] | ≤ [1.0] | ≤ [2.5] |
|  | 256QAM | ≤ [2.0] | ≤ [2.0] | ≤ [2.0] | ≤ [2.0] | ≤ [2.0] | ≤ [2.0] | ≤ [2.0] | ≤ [2.5] |

Table 2.8: Non-Contiguous RB allocation for Power Class 3 (Option 1)

|  |  | MPR for bandwidth class B(dB) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner |  |  |  | Outer1 |  |  |  | Outer2 |  |  |  |
| Modulation |  | UL MO |  |  |  | UL MO |  |  |  | UL MO |  |  |  |
| SL MO |  | QPSK | 16QAM | 64QAM | 256QAM | QPSK | 16QAM | 64QAM | 256QAM | QPSK | 16QAM | 64QAM | 256QAM |
| CP-OFDM | QPSK | ≤ [1.0] | ≤ [1.0] | ≤ [1.0] | ≤ [2.0] | ≤ [1.5] | ≤ [1.5] | ≤ [1.5] | ≤ [2.5] | ≤ [3.0] | ≤ [3.0] | ≤ [3.0] | ≤ [3.5] |
|  | 16QAM | ≤ [1.0] | ≤ [1.0] | ≤ [1.0] | ≤ [2.0] | ≤ [1.5] | ≤ [1.5] | ≤ [1.5] | ≤ [2.5] | ≤ [3.0] | ≤ [3.0] | ≤ [3.0] | ≤ [3.5] |
|  | 64QAM | ≤ [1.0] | ≤ [1.0] | ≤ [1.0] | ≤ [2.0] | ≤ [1.5] | ≤ [1.5] | ≤ [1.5] | ≤ [2.5] | ≤ [3.0] | ≤ [3.0] | ≤ [3.0] | ≤ [3.5] |
|  | 256QAM | ≤ [2.5] | ≤ [2.5] | ≤ [2.5] | ≤ [2.5] | ≤ [2.5] | ≤ [2.5] | ≤ [2.5] | ≤ [2.5] | ≤ [3.5] | ≤ [3.5] | ≤ [3.5] | ≤ [3.5] |

Table 2.9: Contiguous RB allocation for Power Class 2 (Option 2)

| Modulation |  | MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | ≤ [5.5] | ≤ [5.5] |
|  | 16QAM | ≤ [5.0] | ≤ [5.5] |
|  | 64QAM | ≤ [5.0] | ≤ [5.5] |
|  | 256QAM | ≤ [5.0] | ≤ [5.5] |

Table 2.10: Non-Contiguous RB allocation for Power Class 2 (Option 2)

| Modulation |  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | inner | Outer1 | Outer2 |
| CP-OFDM | QPSK | ≤ [5.0] | ≤ [5.5] | ≤ [6.5] |
|  | 16QAM | ≤ [5.0] | ≤ [5.5] | ≤ [6.5] |
|  | 64QAM | ≤ [5.0] | ≤ [5.5] | ≤ [6.5] |
|  | 256QAM | ≤ [5.5] | ≤ [5.5] | ≤ [6.5] |

Table 2.11: Contiguous RB allocation for Power Class 3 (Option 2)

| Modulation |  | MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | ≤ [2.0] | ≤ [2.5] |
|  | 16QAM | ≤ [2.0] | ≤ [2.5] |
|  | 64QAM | ≤ [2.0] | ≤ [2.5] |
|  | 256QAM | ≤ [2.0] | ≤ [2.5] |

Table 2.12: Non-Contiguous RB allocation for Power Class 3 (Option 2)

| Modulation |  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | inner | Outer1 | Outer2 |
| CP-OFDM | QPSK | ≤ [2.0] | ≤ [2.5] | ≤ [3.5] |
|  | 16QAM | ≤ [2.0] | ≤ [2.5] | ≤ [3.5] |
|  | 64QAM | ≤ [2.0] | ≤ [2.5] | ≤ [3.5] |
|  | 256QAM | ≤ [2.5] | ≤ [2.5] | ≤ [3.5] |

Based on the analysis, we propose as follows,

Proposal 1: Define MPR for intra-band con-current operation in a licensed band taking the combination of SL MO and UL MO.

Proposal 1-1: Define MPR with Table 2.5 and Table 2.6 for Power Class 2.

Proposal 1-2: Define MPR with Table 2.7 and Table 2.8 for Power Class 3.

Based on the proposals, we provide TP related to the MPR.

Proposal 3: Revise MPR with Table 2.5 and Table 2.6 for Power Class 2 and MPR with Table 2.7 and Table 2.8 for Power Class 3.

- Reference

[1] R4-2114983, “WF on MPR for intra-band V2X con-current operation”, LG Electronics

[2] TR 38.785 V0.3.0

R4-2107870, “WF on MPR for intra-band V2X con-current operation”, LG Electronics

< START OF TP >

- 5.2.4.2.2	UE maximum output power reduction

The following assumption can serve as a starting point for MPR simulation assumptions as shown in Table 5.2.4.2.2-1 and Table 5.2.4.2.2-2.

Table 5.2.4.2.2-1: MPR simulation assumption for NR V2X intra-band contiguous con-current operation

| parameter | Assumption |
| --- | --- |
| center frequency | 4.5GHz (n79) |
| Bandwidth | 10/20/30/40MHz for NR SL 10/20/40/60/80MHz for NR Uu |
| Maximum output power | 26 dBm |
| numerology | 15 kHz/30kHz/60kHz |
| Modulation | QPSK/16QAM/64QAM/256QAM |
| Waveform | CP-OFDM for NR SL CP-OFDM or DFT-S-OFDM for NR Uu |
| Carrier leakage | 25dBc |
| IQ image | 25dBc |
| CIM3 | 45dBc or 60dBc |
| PA calibration | PA calibrated to deliver 30dBc ACLR for a fully allocated RBs in 20MHz QPSK DFT- S-OFDM waveform at 1 dB MPR. |

For NR V2X intra-band contiguous con-current operation, simultaneous transmission of PUSCH/PUCCH and NR V2X’s PSCCH and PSSCH in the same sub-frame is supported. The following constraints in Table 5.2.4.2.2-2 can be assumed based on current RAN1’s agreement.

Table 5.2.4.2.2-2: NR V2X intra-band contiguous con-current operation UE’ MPR simulation assumptions

| Items | Assumption |
| --- | --- |
| Allowed sub-channel sizes for NR SL | • Support {10, 12, 15, 20, 25, 50, 75, 100} PRBs for possible sub-channel size. |
| Allowed LCRB allocation for NR SL | 10,12,15,20,24,25,30,36,40,45,48,50,60,70,72,75,80,84,90,96,100,105,108,110,120,130,132,135,140,144,150,156,160,165,168,170,175,180,190,192,195,200,204,210,216. No restriction of LCRB for NR Uu |
| Regarding PSCCH / PSSCH multiplexing for NR SL |  |
| PSCCH size for NR SL | 10RB*3 Symbols |
| PSD offset of X dB between PSCCH and PSSCH for NR SL | 0dB |
| NOTE 1: If there is not indicate the detail parameters, then follow TR38.886 for NR SL operation. Also follow TS38.211/TS38.212/TS38.101-1 for NR Uu operation. |  |

For simultaneous transmission of PSFCH transmission for V2X UE and PUSCH/PUCCH for NR Uu, RAN4 need further discussion how to apply the MPR requirements.

For simultaneous transmission of S-SSB transmission for V2X UE and PUSCH/PUCCH for NR Uu, RAN4 need further discussion how to apply the MPR requirements.

For NR intra-band V2X con-current operation with adjacent channel, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 5.2.4.1.1-1 with contiguous RB allocation is specified in Table 5.2.4.1.2-3 for UE power class 3 with bandwidth class B.

Table 5.2.4.1.2-3: Contiguous RB allocation for Power Class 3

|  |  | MPR for bandwidth class B(dB) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner |  |  |  | outer |  |  |  |
| Modulation |  | UL MO |  |  |  | UL MO |  |  |  |
| SL MO |  | QPSK | 16QAM | 64QAM | 256QAM | QPSK | 16QAM | 64QAM | 256QAM |
| CP-OFDM | QPSK | ≤ [1.0] | ≤ [1.0] | ≤ [1.0] | ≤ [2.0] | ≤ [1.0] | ≤ [1.0] | ≤ [1.0] | ≤ [2.5] |
|  | 16QAM | ≤ [1.0] | ≤ [1.0] | ≤ [1.0] | ≤ [2.0] | ≤ [1.0] | ≤ [1.0] | ≤ [1.0] | ≤ [2.5] |
|  | 64QAM | ≤ [1.0] | ≤ [1.0] | ≤ [1.0] | ≤ [2.0] | ≤ [1.0] | ≤ [1.0] | ≤ [1.0] | ≤ [2.5] |
|  | 256QAM | ≤ [2.0] | ≤ [2.0] | ≤ [2.0] | ≤ [2.0] | ≤ [2.0] | ≤ [2.0] | ≤ [2.0] | ≤ [2.5] |

For bandwidth class B with contiguous RB allocation, the following parameters are defined to specify valid RB allocation ranges for Inner and Outer RB allocations:

An RB allocation is contiguous if LCRB1 = 0 or LCRB2 = 0 or (LCRB1  0 and LCRB2  0 and RBStart1 + LCRB1 = NRB1 and RBStart2 = 0), where RBStart1, LCRB1, and NRB1 are for SL CC1, RBStart2, LCRB2, and NRB2 are for UL CC2. SL CC1 is the component carrier with lower frequency.

In contiguous NR V2X intra-band con-current operation, a contiguous allocation is an inner allocation if

RBStart,Low  ≤  RBStart_SL&UL  ≤  RBStart,High, and NRB_alloc  ≤  ceil(NRB,agg /2),

where

RBStart,Low = max(1, floor(NRB_alloc /2))

RBStart,High = NRB,agg – RBStart,Low – NRB,alloc,

with

NRB_alloc= LCRB1 ∙ 2µ1 + LCRB2 ∙ 2µ2

NRB_alloc= (NRB1 - RBStart1)∙ 2µ1 + (RBStart2 + LCRB2 ) ∙ 2µ2,

NRB,agg=NRB1∙2µ1+ NRB2∙2µ2.

If LCRB1 =0, RBStart_SL&UL = NRB1∙2µ1+ RBStart2∙2µ2,

if LCRB1 > 0, RBStart_SL&UL = RBStart1∙2µ1.

Where, µ1 and µ2 is 0, 1 and 2 for SCS of 15kHz, 30kHz and 60kHz respectively.

A contiguous allocation that is not an Inner contiguous allocation is an Outer contiguous allocation.

For NR intra-band V2X con-current operation with adjacent channel, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 5.2.4.1.1-1 with non-contiguous RB allocation is specified in Table 5.2.4.1.2-4 for UE power class 3 with bandwidth class B.

Table 5.2.4.1.2-4: Non-contiguous RB allocation for Power Class 3

|  |  | MPR for bandwidth class B(dB) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner |  |  |  | Outer1 |  |  |  | Outer2 |  |  |  |
| Modulation |  | UL MO |  |  |  | UL MO |  |  |  | UL MO |  |  |  |
| SL MO |  | QPSK | 16QAM | 64QAM | 256QAM | QPSK | 16QAM | 64QAM | 256QAM | QPSK | 16QAM | 64QAM | 256QAM |
| CP-OFDM | QPSK | ≤ [1.0] | ≤ [1.0] | ≤ [1.0] | ≤ [2.0] | ≤ [1.5] | ≤ [1.5] | ≤ [1.5] | ≤ [2.5] | ≤ [3.0] | ≤ [3.0] | ≤ [3.0] | ≤ [3.5] |
|  | 16QAM | ≤ [1.0] | ≤ [1.0] | ≤ [1.0] | ≤ [2.0] | ≤ [1.5] | ≤ [1.5] | ≤ [1.5] | ≤ [2.5] | ≤ [3.0] | ≤ [3.0] | ≤ [3.0] | ≤ [3.5] |
|  | 64QAM | ≤ [1.0] | ≤ [1.0] | ≤ [1.0] | ≤ [2.0] | ≤ [1.5] | ≤ [1.5] | ≤ [1.5] | ≤ [2.5] | ≤ [3.0] | ≤ [3.0] | ≤ [3.0] | ≤ [3.5] |
|  | 256QAM | ≤ [2.5] | ≤ [2.5] | ≤ [2.5] | ≤ [2.5] | ≤ [2.5] | ≤ [2.5] | ≤ [2.5] | ≤ [2.5] | ≤ [3.5] | ≤ [3.5] | ≤ [3.5] | ≤ [3.5] |

For bandwidth classes B with non-contiguous RB allocation, the following parameters are defined to specify valid RB allocation ranges for Inner, Outer1 and Outer2 RB allocations:

Non-Contiguous RB allocation is defined as RBStart1 + LCRB1 < NRB1, or RBStart2 > 0, when both SL CC and UL CC are activated and allocated with RB(s), where RBStart1, LCRB1, and NRB1 are for SL CC1, RBStart2, LCRB2, and NRB2 are for UL CC2. SL CC1 is the component carrier with lower frequency.

In contiguous NR V2X intra-band con-current operation, a non-contiguous RB allocation is a non-contiguous Inner RB allocation if the following conditions are met:

RBStart,Low  ≤  RBStart_CA  ≤  RBStart,High and NRB_alloc ≤  ceil((BWChannel_SL&UL / 3 – BWgap ) / 0.18MHz),

where

NRB_alloc = (NRB1 - RBStart1)∙ 2µ1 + (RBStart2 + LCRB2 ) ∙ 2µ2, RBStart_SL&UL = RBStart1∙21

RBStart,Low = max(1, floor(NRB_alloc + (BWgap – BWGB,low)/0.18MHz))

RBStart,High = floor((BWChannel_SL&UL – 2 ∙ BWgap – BWGB,low)/0.18MHz – 2 ∙ NRB_alloc)

BWGB,low =Foffset,low – (NRB1∙12+1)∙SCS1/2

BWgap is the bandwidth of the gap between NRB1 and NRB2 possible allocations of SL CC1 and UL CC2 respectively.

In contiguous NR V2X intra-band con-current operation, a non-contiguous RB allocation is a non-contiguous outer 1 RB allocation if the following conditions are met:

RBStart,Low  ≤  RBStart_SL&UL  ≤  RBStart,High and NRB_alloc ≤  ceil((3 BWChannel_SL&UL / 5 – BWgap) / 0.18MHz)

where

RBStart,Low = max(1, 2 ∙ NRB_alloc – floor( (BWChannel_SL&UL – 2 ∙ BWgap + BWGB,low)/0.18MHz)),

RBStart,High = floor((2 ∙ BWChannel_SL&UL – 3 ∙ BWgap – BWGB,low) / 0.18MHz – 3 ∙ NRB_alloc)

NRB_alloc , RBStart_SL&UL , BWgap and BWGB,low are as defined for the Inner region.

In contiguous NR V2X intra-band con-current operation, a non-contiguous allocation is an Outer 2 allocation if it is neither a non-contiguous Inner allocation nor an Outer 1 allocation.

For NR intra-band V2X con-current operation with adjacent channel, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 5.2.4.1.1-1 with contiguous RB allocation is specified in Table 5.2.4.1.2-5 for UE power class 2 with bandwidth class B.

Table 5.2.4.1.2-5: Contiguous RB allocation for Power Class 2

|  |  | MPR for bandwidth class B(dB) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner |  |  |  | outer |  |  |  |
| Modulation |  | UL MO |  |  |  | UL MO |  |  |  |
| SL MO |  | QPSK | 16QAM | 64QAM | 256QAM | QPSK | 16QAM | 64QAM | 256QAM |
| CP-OFDM | QPSK | ≤ [1.5] | ≤ [2.0] | ≤ [3.0] | ≤ [5.0] | ≤ [3.5] | ≤ [3.5] | ≤ [3.5] | ≤ [5.5] |
|  | 16QAM | ≤ [2.0] | ≤ [2.0] | ≤ [3.0] | ≤ [5.0] | ≤ [3.5] | ≤ [3.5] | ≤ [3.5] | ≤ [5.5] |
|  | 64QAM | ≤ [3.0] | ≤ [3.0] | ≤ [3.0] | ≤ [5.0] | ≤ [3.5] | ≤ [3.5] | ≤ [3.5] | ≤ [5.5] |
|  | 256QAM | ≤ [5.0] | ≤ [5.0] | ≤ [5.0] | ≤ [5.0] | ≤ [5.0] | ≤ [5.0] | ≤ [5.0] | ≤ [5.5] |

For NR intra-band V2X con-current operation with adjacent channel, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 5.2.4.1.1-1 with non-contiguous RB allocation is specified in Table 5.2.4.1.2-6 for UE power class 2 with bandwidth class B.

Table 5.2.4.1.2-6: Non-contiguous RB allocation for Power Class 2

|  |  | MPR for bandwidth class B(dB) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner |  |  |  | Outer1 |  |  |  |  | Outer2 |  |  |  |  |
| Modulation |  | UL MO |  |  |  | UL MO |  |  |  |  | UL MO |  |  |  |  |
| SL MO |  | QPSK | 16QAM | 64QAM | 256QAM | QPSK | 16QAM | 64QAM | 256QAM | QPSK |  | 16QAM | 64QAM | 256QAM |
| CP-OFDM | QPSK | ≤ [2.0] | ≤ [2.0] | ≤ [3.0] | ≤ [5.0] | ≤ [4.0] | ≤ [4.0] | ≤ [4.0] | ≤ [5.5] | ≤ [6.0] |  | ≤ [6.0] | ≤ [6.0] | ≤ [6.5] |
|  | 16QAM | ≤ [2.5] | ≤ [2.5] | ≤ [3.0] | ≤ [5.0] | ≤ [4.0] | ≤ [4.0] | ≤ [4.0] | ≤ [5.5] | ≤ [6.0] |  | ≤ [6.0] | ≤ [6.0] | ≤ [6.5] |
|  | 64QAM | ≤ [3.5] | ≤ [3.5] | ≤ [3.5] | ≤ [5.0] | ≤ [4.5] | ≤ [4.5] | ≤ [4.5] | ≤ [5.5] | ≤ [6.0] |  | ≤ [6.0] | ≤ [6.0] | ≤ [6.5] |
|  | 256QAM | ≤ [5.5] | ≤ [5.5] | ≤ [5.5] | ≤ [5.5] | ≤ [5.5] | ≤ [5.5] | ≤ [5.5] | ≤ [5.5] | ≤ [6.5] |  | ≤ [6.5] | ≤ [6.5] | ≤ [6.5] |

< END OF TP >