3GPP TSG-RAN WG4 Meeting # 101-e	R4-217877

# Electronic Meeting, November 1-12, 2021

Source: 	Huawei, HiSilicon

Title: 	Regulatory requirements for full unlicensed band 5925-7125MHz

Agenda Item:	7.2.2

Document for:	Approval

Introduction

At RAN#93 Meeting, the revised WI on introduction of operation in full unlicensed band 5925-7125MHz was approved in [1]. Costa Rica is added in the WID. In this contribution, the discussion on the update is provided.

Discussion

The regulatory requirements of client devices for Costa Rica are compared with FCC 20-51 as below.

Table 2-1: Additional requirements

| Country/NS | Permissible operation | Frequency range (MHz) | Maximum mean EIRP for in-band emissions(dBm) | Maximum mean power density (dBm/MHz) | Maximum mean density for out-of-band emissions |
| --- | --- | --- | --- | --- | --- |
| NS_53 | LPI | 5925 – 7125 | 24 | -1 | -27 dBm/MHz (f ≤ 5925, f ≥ 7125) |
| NS_54 | SP | 5925 – 6425 | 30 | 17 | -27 dBm/MHz (f ≤ 5925, f ≥ 7125) |
|  |  | 6525 – 6875 |  |  |  |
| Costa Rica | LPI | 5925 – 7125 | 24 | none | none |
| Costa Rica | VLP | 5925 – 7125 | 14 | none | none |

For LPI operation, the only limit is the maximum EIRP for in-band emissions which is covered by existing PC5 definition, hence no A-MPR is needed.

For VLP operation, the need of new power class and NS is TBD

Hence the summary of the NS values can be updated as below.

| Country | Mode |  |  |
| --- | --- | --- | --- |
|  | SP | LPI | VLP |
| Canada | NS_54 | NS_x1 (new) | TBD |
| Brazil | N/A | NS_53 | TBD |
| Peru | N/A | [NS_53] Note1 | N/A |
| Chile | N/A | [NS_53] Note1 | N/A |
| South Korea | N/A | NS_x2 (new) | TBD |
| Costa Rica | N/A | No A-MPR | TBD |

NOTE 1: Peru and Chile LPI regulatory parameters are the same as US/Brazil LPI, but there is no -27dBm/MHz out-of-band emission requirement. At the same time, since A-MPR values we are PSD limited, then NS_53 should also apply.

Conclusions

In the contribution, we provide some analysis on the update of regulatory requirements.

References

RP-212302, Revised WID for Introduction of operation in full unlicensed band 5925-7125MHz