3GPP TSG-RAN WG4 Meeting #101-e                                                                      R4-2117933

Online Meeting, 1st -12th November, 2021

Source:	Samsung, KDDI

Title:	TP for TR 38.717-03-02: CA_n1A-n18A-n28A

Agenda item:			7.10.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.717-03-02 to include CA_n1-n18-n28 according to the request in [1].

2. Reference

- R4-2112944 Revised WID on Rel-17 NR Inter-band Carrier Aggregation Dual Connectivity for 3 bands DL with 2 bands UL

- R4-2114090 TR 38.717-03-02 v0.6.0

# 3. Text Proposal

<Start of Text Proposal>

    - 5.1.x	CA_n1-n18-n28

      - 5.1.x.1	Operating bands for CA

Table 5.1.x.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n1-n18-n28 | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

      - 5.1.x.2	Channel bandwidths per operating band for CA

Table 5.1.x.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n1A-n18A-n28A | CA_n1A-n18A CA_n1A-n28A CA_n18A-n28A | n1 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  | 0 |
|  |  | n18 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |

      - 5.1.x.3	UE co-existence studies

For CA combinations with 3 bands DL and 2 bands UL, only IMD issues due to dual Tx operation of two bands falling into the third band Rx need to be considered.

No IMD generated by dual Tx n1 and n18 fall into the third band n28 Rx

IMD5 generated by dual Tx n1 and n28 fall into the third band n18 Rx

IMD5 generated by dual Tx n18 and n28 fall into the third band n1 Rx.

      - 5.1.x.4	REFSENS requirements

Based on co-existence studies on 5.1.x.3, MSD values are defined below.

Table 5.1.x.4-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n18-n28 | n1 | 1965 | 5 | 25 | 2155 | N/A | FDD | N/A |
|  | n28 | 708 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n18 | 822 | 5 | 25 | 867 | 4.6 | FDD | IMD5 |
|  | n18 | 825 | 5 | 25 | 870 | N/A | FDD | N/A |
|  | n28 | 738 | 5 | 25 | 793 | N/A | FDD | N/A |
|  | n1 | 1937 | 5 | 25 | 2127 | 4 | FDD | IMD5 |

<End of Text Proposal>