3GPP RAN WG4 Meeting #101e		R4-2117950

Electronic meeting, 1st – 12th November 2021

Agenda item:	7.2.2

Source:	Apple

Title:	Update of the 6GHz unlicensed band system and regulatory requirements in Region 2 and Region 3 countries

WI/SI:	NR_6GHz_unlic_full

Release:	Rel-17

Document for:	Decision

# 1	Introduction

3GPP Rel-16 NR-U WI [1] specified how the NR technology can be used on the unlicensed spectrum thus offering more resources in frequency bands, such as 5GHz and 6GHz.  5GHz is a well-known band for the unlicensed operation, but 6GHz is a relative new band usage of which was approved recently in different regulatory regions. While the 6GHz band for the US is already part of the Rel-16 core functionality, current 3GPP specifications do not support it for other countries, such as South Korea and Canada, which have finalised their regulatory requirements only recently (refer to the latest regulatory status updated in  [2]). Thus, RAN#92 meeting approved a new WI aim of which is to enable support of the 6GHz unlicensed band for those countries and regions that have finalised recently the corresponding regulatory requirements [3].

After the RAN4#101 meeting existing Region 2 and Region 3 regulatory requirements were analysed and RAN WG4 captured in [4] which NS values can be re-used and for which countries/regions new NS values will be needed. One of the open issues from the RAN4#101 meeting was whether we can re-use NS_53 for Peru and Chile, regulatory parameters of which are almost identical to US and Brazil, but do not have emission requirements of -27dBm/MHz. In addition to that, the latest version of TR 37.890 [3] also captured regulatory parameters for Costa Rica, which was not included into the previous analysis.

# 2	Further updates to the full unlicensed 6GHz band

## 2.1	Costa Rica

The Superintendencia de Telecomunicaciones (SUTEL) in Costa Rica has published its Plan Nacional de Atribución de Frecuencias (PNAF), according to which the 5925-7125MHz frequency range is opened for the license-exempt usage with the following technical conditions:

-	maximum EIRP is 30dBm;

-	maximum output power of the equipment is 24dBm;

-	unlicensed usage is allowed at the following conditions:

-	low power indoor applications (LPI) in accordance with the power limits indicated above;

-	LPI applications are restricted to operation inside buildings for RLAN type systems (wireless local area networks) or similar, which use contention protocols to get access to the environment;

-	very low power (VLP) indoors and outdoors with a maximum EIRP of 14 dBm;

-	VLP applications are for portable or similar type devices, limited in power, allowing connectivity over short distances.

Similar to other Region 2 LAM countries, Costa Rica has opened the full frequency range 5925-7125MHz to the unlicensed operation, for which 3GPP has already defined band n96. Thus, same band n96 can be also used for Costa Rica. As for the exact regulatory parameters, Costa Rica does not put any requirement on PSD or out-of-band emission requirements.

Proposal 1a:	Re-use 3GPP band n96 for Costa Rica.

Proposal 1b:	There is no need to introduce new NS values for Costa Rica.

## 2.2	Peru and Chile

One of the open issues from the RAN4#101 meeting was whether we can re-use NS_53 for Peru and Chile, regulatory parameters of which are identical to Brazil and US in terms of maximum EIRP and PSD, but do not have out-of-band emission requirements of -27dBm/MHz. For this purpose additional A-MPR simulations were run specifically for Peru and Chile results for which can be found in [5]. As a short summary, there are indeed cases when we are not PSD limited and thus A-MPR can be potentially relaxed specifically for these countries. Nevertheless, since the delta is quite marginal, just around 0.5-1dB, we do not anticipate any noticeable performance degradation if we re-use NS_53 for these two countries.

Proposal 2:	Re-use NS_53 for Peru and Chile.

## 2.3	Summary of the required NS values

Table 2.3-1 below presents an updated summary of required NS values. It is based on the endorsed table from [4], where we add Costa Rica and remov "[]" from NS_53 in case of Peru and Chile.

Table 2.3-1: Summary of NS values required for NR-U PC5.

| Country | Mode |  |  |
| --- | --- | --- | --- |
|  | SP | LPI | VLP |
| Region 2 |  |  |  |
| Canada | NS_54 | NS_x1 (new) | TBD |
| Brazil | N/A | NS_53 | TBD |
| Peru | N/A | NS_53 | N/A |
| Chile | N/A | NS_53 | N/A |
| Costa Rica | N/A | NS_01 | TBD |
| Region 3 |  |  |  |
| South Korea | N/A | NS_x2 (new) | TBD |

Proposal 3:	Endorse updated summary of the required NS values.

# 3	Conclusions

In this contribution we have presented further regulatory updates for the full 6GHz unlicensed band and provided further analysis of the A-MPR values for Peru and Chile. For Costa Rica, existing band n96 can be re-used and no new NS values are needed. As for Peru and Chile, even though we can potentially consider slightly optimised and relaxed A-MPR values, we suggest re-using existing NS_53.

Proposal 1a:	Re-use 3GPP band n96 for Costa Rica.

Proposal 1b:	There is no need to introduce new NS values for Costa Rica.

Proposal 2:	Re-use NS_53 for Peru and Chile.

Proposal 3:	Endorse updated summary of the required NS values.

# 4	References

- RP-1912926, "WID on NR-based Access to Unlicensed Spectrum ", Qualcomm Inc.

- 3GPP TR 37.890, "Feasibility Study on 6 GHz for LTE and NR in Licensed and Unlicensed Operations", v0.13.0

- RP-211602, "New WID for Introduction of operation in full unlicensed band 5925-7125MHz", Apple Inc.

- R4-2114884, "WF on introduction of the full unlicensed band", Apple, Skyworks Solutions Inc.

- R4-2117951, "A-MPR for the 6GHz unlicensed band in Region 2 and Region 3 countries", Apple

Annex A: Summary of the regulatory requirements

| Region | Country | Permissible operation (Note 1) | Frequency range | Maximum mean EIRP for in-band emissions | Maximum mean EIRP density for in-band emissions | Maximum mean EIRP density for out-of-band emissions |
| --- | --- | --- | --- | --- | --- | --- |
| Region 1 | EU/CEPT | LPI (see 4.1.1) | 5945 – 6425MHz | 23dBm | 10dBm/MHz | -22 dBm/MHz (below 5935MHz) |
|  |  | VLP (see 4.1.1) |  | 14dBm | 1dBm/MHz 10dBm/MHz (for the narrowband usage) | -45 dBm/MHz (below 5935MHz); |
|  | UK | LPI (see 4.1.3) | 5925 – 6425MHz | 24dBm | 11dBm/MHz | In accordance with directive 2014/53/EC |
|  |  | VLP (see 4.1.3) |  | 14dBm |  |  |
| Region 2 | US | SP (see 4.2.1) | 5925 – 6425MHz 6525 – 6875MHz | 36dBm (AP) 30dBm (CL) | 23dBm/MHz (AP) 17dBm/MHz (CL) | -27 dBm/MHz (outside operational range) |
|  |  | LPI (see 4.2.1) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5 dBm/MHz (AP) -1 dBm/MHz (CL) |  |
|  | Canada | SP (see 4.2.2) | 5925-6875 MHz | 36dBm | 23dBm/MHz |  |
|  |  | LPI (see 4.2.2) | 5925-7125 MHz | 30dBm | 5 dBm/MHz |  |
|  |  | VLP (see 4.2.2) |  | 14dBm | -8dBm/MHz |  |
|  | Brazil | LPI (see 4.2.3) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) | -27 dBm/MHz (outside operational range) |
|  |  | VLP (see 4.2.3) |  | 17 dBm | -5 dBm/MHz |  |
|  | Peru | LPI (see 4.2.4) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) |  |
|  | Chile | LPI (see 4.2.5) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) |  |
|  | Costa Rica | LPI (see 4.2.8) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) |  |  |
|  |  | VLP (see 4.2.8) |  | 14dBm |  |  |
| Region 3 | South Korea | LPI (see 4.3.2) | 5925 – 7125MHz | 24dBm | 2dBm/MHz | -27 dBm/MHz (outside operational range) |
|  |  | VLP (see 4.3.2) | 5925 – 6425MHz | 14dBm | 1dBm/MHz | -34 dBm/MHz (outside operational range of the VLP mode) |