3GPP TSG-RAN WG4 Meeting #101-e		R4-2118010

Electronic Meeting, November 1st – 12th, 2021

Agenda item:	8.9.5.3.2

Source:	Intel Corporation

Title:	Views on FR2 HST UL TA performance requirements

Document for:	Discussion

# Introduction

In RAN4 #100-e meeting WF on HST FR2 demodulation requirements was agreed[1]. Almost all remaining open issues were resolved for UL TA requirements. In this contribution we provide initial set of link-level results and discuss necessity of UL TA channel model revision.

# Discussion

In HST FR2 uni-directional deployments a large propagation delay jump happens during the switching between neighbouring RRHs. This effect causes issues both in DL and in UL. During the RAN4 #100-e the ways to deal with the DL issues were agreed but the uplink timing is still under discussion. The following options were captured in the WF [2] during RAN4 #100-e:

Uplink timing - RAN4 will further study the below options to address uplink timing issues - Option 1: One shot UE autonomous large uplink timing adjustment - Option 2: Other implementation/deployment based solutions

According to the previous discussion most of the companies agree that Option1 is a reasonable approach and RAN4 need to further work on it in more details. In our companion paper we provide our view on a design of this feature [3].

Considering one-shot UE UL timing adjustment as a baseline solution to address large propagation delay jump, there won’t be impact on gNB behaviour. In this case conventional UL TA channel model can be used to efficiently verify that gNB accurately estimate timing offset due to UE movement and properly send TA command. It will be enough to guarantee reliable UL performance from the angle of quick aging of reference timing in HST conditions.

Observation #1: If one-shot UE autonomous uplink timing adjustment will be considered to address timing issues in uni-directional deployments, there is no need to revise UL TA channel model.

In Figure 1 we provide link-level performance evaluations for UL TA scenario considering agreed simulation assumptions that are summarized in Table 1. Table 2 provides obtained performance at 70 of max achievable throughput.

Table 1. Simulation assumptions

| Parameter | Value |
| --- | --- |
| CBW | 50 and 200 MHz |
| PUSCH resource allocation | 50MHz CBW: 16RBs for each UE, Moving UE RBs: 0~15; Stationary UE RBs: 16~31 200MHz CBW: 66RBs for each UE, Moving UE RBs: 0~65; Stationary UE RBs: 66~131 Length of PUSCH data is 10 symbols |
| DMRS configuration | 1 DMRS, 1+1 DMRS, 1+1+1 DMRS |
| MCS | 16 |
| Test Parameters for timing offset | Scenario Y |

| Figure 1. Demodulation performance of moving UE in UL TA |  |
| --- | --- |

Table 2. SNR at 70% of max achievable throughput

|  | 1 DMRS | 1+1 DMRS | 1+1+1 DMRS |
| --- | --- | --- | --- |
| 50 MHz CBW | 5.5 | 5.8 | 5.6 |
| 200 MHz CBW | 6.1 | 5.6 | 5.5 |

# Conclusion

In this contribution we provide initial simulation results for UL TA HST FR2 performance and make the following observation on necessity of channel model revision:

Observation #1: If one-shot UE autonomous uplink timing adjustment will be considered to address timing issues in uni-directional deployments, there is no need to revise UL TA channel model

# References

- R4-2115726 “WF for FR2 HST Demodulation”, Samsung, RAN4 #100-e, August 2021

- R4-2115351 “WF on R17 NR_ext_to_71GHz_RRM_1”, RAN4 #100-e, August 2021

- R4-2118028 “Discussion on uplink timing for HST in FR2”, RAN4 #101-e, November 2021