3GPP TSG RAN WG4 Meeting #101-e	R4-2118077

Electronic meeting, 01st -12th November, 2021

Agenda item:	7.18.2

Source:	LG Electronics

Title:	TP on MSD test results and test configurations for NR DC on LTE xbands DL/1UL(x=1,2,3,4) and NR 2bands DL/1UL band combinations

Document for:	Approval

1	Introduction

In the latest RAN Plenary, some new CA band are included to support PC3 NR DC on LTE xbands DL/1UL(x=1,2,3,4) and NR 2bands DL/1UL band combinations [1].

Based on the self interference analysis results [2], we summary the remaining self-interference problems by dual uplink transmission for NR DC on LTE xbands DL/1UL(x=1,2,3,4) and NR 2bands DL/1UL band combinations as shown in Table 1.

Table 1: summary of the remaining self interference problems in NR DC onLTE xbands DL/1UL(x=1,2,3,4) and NR 2bands DL/1UL band combinations in Rel-17

| Downlink band configuration | Uplink DC Configuration | Harmonic relation issues | intermodulation to own rx band | interference due to small frequency separation | MSD |
| --- | --- | --- | --- | --- | --- |
| DC_8_n28-n79 | DC_8A_n28A | 5th harmonic from B8 into n79 | 5th IMD into n79 | - | - Harmonic problem already covered in DC_8A_n79A - FFS |
|  | DC_8A_n79A |  | 5th IMD into n28 | - | - FFS |
| DC_7_n12-n77 | DC_7A_n12A | - | 3rd & 4th IMDs into n77 | - | - FFS |
| DC_7_n71-n77 | DC_7A_n71A | 5th harmonic from n71 into n77 | 3rd & 4th IMDs into n77 | - | - Harmonic problem already covered in DC_71A_n77A - FFS |
| DC_5_n2-n41 | DC_5A_n2A | 3rd harmonic from B5 into n41 | 2nd IMD into n41 | - | - Harmonic problem already covered in DC_5A_n41A - FFS |
| DC_2-5_n2-n41 | DC_5A_n2A | 3rd harmonics from n2 into n41 | 2nd IMD into n41 |  | - Harmonic issue already covered in DC_2_n41 - FFS |
| DC_3_n28-n75 | DC_3A_n28A | 2nd harmonic from n28 into n75 | 5th IMD into n75 | - | - Harmonic issue already covered in DC_28_n75 - FFS |

Based on the revised WID and above the self interference analysis results, we provided the expected the desense level in own Rx band for the new DC band combinations UE.

2	MSD analysis

2.1 IMD problems by dual uplink DC

In this session, we analyze the required MSD values in the own Rx band by IMD product from dual uplink transmission. In table 2 and Table 3, we show the basic RF simulation assumptions.

Table 2: UE RF Front-end component parameters

| UE ref. architecture Component | Triplexer-Diplexer Architecture w/ single ant. or dual ant. |  |  |  | Cascaded Diplexer Architecture w/ single ant. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DC_8A_n28A-n79A, DC_7A_n12A-n77A, DC_7A_n71A-n77A |  |  |  | DC_5A_n2A-n41A, DC_2A-5A_n2A-n41A, DC_3A_n28A-n75A |  |  |  |
|  | IP2 (dBm) | IP3 (dBm) | IP4 (dBm) | IP5 (dBm) | IP2 (dBm) | IP3 (dBm) | IP4 (dBm) | IP5 (dBm) |
| Ant. Switch | 112 | 68 | 55 | 55 | 112 | 68 | 55 | 55 |
| Triplexer | 110 | 72 | 55 | 52 |  |  |  |  |
| Quadplexer |  |  |  |  | 112 | 72 | 55 | 52 |
| Diplexer | 115 | 87 | 55 | 55 | 115 | 87 | 55 | 55 |
| Duplexer | 100 | 75 | 55 | 53 | 100 | 75 | 55 | 53 |
| PA Forward | 28.0 | 32 | 30 | 28 | 28.0 | 32 | 30 | 28 |
| PA Reversed | 40 | 30.5 | 30 | 30 | 40 | 30.5 | 30 | 30 |
| LNA | 10 | 0 | 0 | -10 | 10 | 0 | 0 | -10 |

Table 3 show the isolation levels according to the RF component.

Table 3: UE RF Front-end component isolation parameters

| Isolation Parameter | Value (dB) | Comment |
| --- | --- | --- |
| Antenna to Antenna | 10 | Main antenna to diversity antenna |
| PA (out) to PA (in) | 60 | PCB isolation (PA forward mixing) |
| Triplexer | 20 | High/low band isolation |
| Diplexer | 25 | High/low band isolation |
| PA (out) to PA (out) | 60 | L-H/H-L cross-band |
| PA (out) to PA (out) | 50 | H-H cross-band |
| LNA (in) to PA (out) | 60 | L-H/H-L cross-band |
| LNA (in) to PA (out) | 50 | H-H cross-band |
| Duplexer | 50 | Tx band rejection at Rx band |

Based on above assumptions and test configuration, we proposed the MSD levels as below.

Table 4: Proposed MSD test configuration and results by IMD problems for NR DC UE

| NR CA inter-bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_8A_n28A-n79A | 8 | IMD5 | ¦fn28 +4*fB8¦ | 912.5 | 5 | 25 | 957.5 | 5 | N/A |
|  | n28 |  |  | 745.5 | 5 | 25 | 800.5 | 5 |  |
|  | n79 |  |  | 4420 | 40 | 216 | 4420 | 40 | 0.0 |
|  | 8 | IMD5 | ¦fn79 -4*fB8¦ | 905 | 5 | 25 | 950 | 5 | N/A |
|  | n79 |  |  | 4420 | 40 | 216 | 4420 | 40 |  |
|  | n28 |  |  | 745 | 5 | 25 | 800 | 5 | 3.9 |
| DC_7A_n12A-n77A | B7 | IMD3 | ¦2*fn12 +fB7¦ | 2520 | 10 | 50 | 2640 | 10 | N/A |
|  | n12 |  |  | 708 | 5 | 25 | 738 | 5 |  |
|  | n77 |  |  | 3936 | 10 | 50 | 3936 | 10 | 15.2 |
| DC_7A_n71A-n77A | B7 | IMD3 | ¦2*fn71 +fB7¦ | 2520 | 10 | 50 | 2640 | 10 | N/A |
|  | n71 |  |  | 693 | 5 | 25 | 647 | 5 |  |
|  | n77 |  |  | 3906 | 10 | 50 | 3906 | 10 | 15.2 |
| DC_5A_n2A-n41A or DC_2A-5A_n2A-n41A | B5 | IMD2 | ¦fB5 +fn2¦ | 829 | 5 | 25 | 874 | 5 | N/A |
|  | n2 |  |  | 1855 | 5 | 25 | 1935 | 5 |  |
|  | n41 |  |  | 2684 | 5 | 25 | 2684 | 5 | 29.7 |
| DC_3A_n28A-n75A | B3 | IMD5 | ¦3*fn28 -2*fB3¦ | 1780 | 5 | 25 | 1875 | 5 | N/A |
|  | n28 |  |  | 708 | 5 | 25 | 763 | 5 |  |
|  | n75 |  |  | - | - | - | 1436 | 5 | 3.3 |

3	Conclusion

Above simulation results, we recommend to merged the other interested companies’ simulation results as average manner to derive MSD levels and propose as follow

Proposal 1: For IMD problem by dual uplink transmission of PC3 NR DC on LTE xbands DL/1UL(x=1,2,3,4) and NR 2bands DL/1UL band combinations, the proposed MSD values in Table 4 shall be considered in TS38.101-3.

4	Proposed text for TR37.717-11-21:

[Unchanged Parts Skipped]

6.130	DC_8_n28-n79

6.130.1	Operating bands for DC

Table 6.130.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8_n28-n79 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

6.130.2	Channel bandwidths per operating band for DC

Table 6.130.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_8A_n28A-n79A | DC_8A_n28A DC_8A_n79A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 140 |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |

6.130.3	Co-existence studies

Based on the co-existence studies of DC_8A-n28A and DC_8A-n79A, 5th order IMD generated by dual uplink of Band 8 + Band n28 may also fall into own Rx of band n79 and 5th order IMD generated by dual uplink of Band 8 + Band n79 may also fall into own Rx of band n28.

6.130.4	∆TIB and ∆RIB values

For DC_8_n28-n79, the TIB,c and RIB,c values are given in the tables below.

Table 6.130.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_n28-n79 | 8 | 0.6 |
|  | n28 | 0.5 |
|  | n79 | 0.8 |

Table 6.130.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8_n28-n79 | 8 | 0.2 |
|  | n28 | 0.2 |
|  | n79 | 0.5 |

6.130.5	MSD

As mentioned in 6.130.3, IMD5 of B8 and n28 to Band n79 Rx and IMD5 of B8 and n79 to Band n28 Rx need to be addressed for REFSENS relaxation. The following values are proposed:

Table 5.1.x.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_8A_n28A-n79A | 8 | 912.5 | 5 | 25 | 957.5 | N/A | FDD | N/A |
|  | n28 | 745.5 | 5 | 25 | 800.5 | N/A | FDD | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | 0.0 | FDD | IMD5 |
| DC_8A_n28A-n79A | 8 | 905 | 5 | 25 | 950 | N/A | FDD | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | TDD | N/A |
|  | n28 | 745 | 5 | 25 | 800 | 3.9 | FDD | IMD5 |

[Unchanged Parts Skipped]

6.W	DC_7_n12-n77

6.W.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there are IMD3 produced by Band 7 and NR band n12 that impact the reference sensitivity of NR band n77. Also IMD4 product impacted to the n77. But the MSD by IMD4 is not specified.

The required MSD level and test configuration are shown in the following Table.

Table 6.W.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| NR CA inter-bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_7A_n12A-n77A | B7 | IMD3 | ¦2*fn12 +fB7¦ | 2520 | 10 | 50 | 2640 | 10 | N/A |
|  | n12 |  |  | 708 | 5 | 25 | 738 | 5 |  |
|  | n77 |  |  | 3936 | 10 | 50 | 3936 | 10 | 15.2 |

6.X	DC_7_n71-n77

6.X.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there are IMD3 produced by Band 7 and NR band n71 that impact the reference sensitivity of NR band n77. Also IMD4 product impacted to the n77. But the MSD by IMD4 is not specified.

The required MSD level and test configuration are shown in the following Table.

Table 6.X.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| NR CA inter-bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_7A_n71A-n77A | B7 | IMD3 | ¦2*fn71 +fB7¦ | 2520 | 10 | 50 | 2640 | 10 | N/A |
|  | n71 |  |  | 693 | 5 | 25 | 647 | 5 |  |
|  | n77 |  |  | 3906 | 10 | 50 | 3906 | 10 | 15.2 |

6.Y	DC_5_n2-n41

6.Y.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD2 produced by Band 5 and NR band n2 that impact the reference sensitivity of NR band n41.

The required MSD levels and test configuration are shown in the following Table.

Table 6.Y.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| NR CA inter-bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_5A_n2A-n41A | B5 | IMD2 | ¦fB5 +fn2¦ | 829 | 5 | 25 | 874 | 5 | N/A |
|  | n2 |  |  | 1855 | 5 | 25 | 1935 | 5 |  |
|  | n41 |  |  | 2684 | 5 | 25 | 2684 | 5 | 29.7 |

6.Z	DC_3_n28-n75

6.Z.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD5 produced by Band 3 and NR band n28 that impact the reference sensitivity of NR band n75.

The required MSD levels and test configuration are shown in the following Table.

Table 6.Z.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| NR CA inter-bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n28A-n75A | B3 | IMD5 | ¦3*fn28 -2*fB3¦ | 1780 | 5 | 25 | 1875 | 5 | N/A |
|  | n28 |  |  | 708 | 5 | 25 | 763 | 5 |  |
|  | n75 |  |  | - | - | - | 1436 | 5 | 3.3 |

[Unchanged Parts Skipped]

7.X	DC_2-5_n2-n41

7.X.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD2 produced by Band 5 and NR band n2 that impact the reference sensitivity of NR band n41.

The required MSD levels and test configuration are shown in the following Table.

Table 7.X.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| NR CA inter-bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A-5A_n2A-n41A | B5 | IMD2 | ¦fB5 +fn2¦ | 829 | 5 | 25 | 874 | 5 | N/A |
|  | n2 |  |  | 1855 | 5 | 25 | 1935 | 5 |  |
|  | n41 |  |  | 2684 | 5 | 25 | 2684 | 5 | 29.7 |

Reference

[1] RP-211750, “Revised WID on Rel-17 DC of LTE x band (x=1,2,3,4) DL_1UL and NR 2 band DL_1UL combination” LG Electronics

[2] TR37.717-11-21 v0.6.0