# 3GPP TSG-RAN WG4 Meeting #101-e	R4-2118274

# Electronic Meeting, 1st – 11th November, 2021

Source: 	vivo

Title: 	Analysis on handheld UE EIRP and spherical coverage requirements for 52.6~71 GHz

Agenda Item:	8.16.3.1

Document for:	Approval

Introduction

In the last meeting, we had an extensive discussion on UE RF requirements for 52.6~71 GHz [1]. Power class related topics drew the most attention during the meeting. For UE EIRP and spherical coverage requirements, more analysis is needed based on antenna element number assumption. In this contribution, we give further analysis on these requirements focusing on handheld UE type.

Discussion

Handheld UE EIRP

For handheld UE minimum peak EIRP, the progress we made in the last meeting [1] is as follows:

Agreement: Have further analysis on the UE EIRP requirements taking both antenna element number and PA performance

- Include multiple antenna element numbers

- Practical form factor should be considered in the analysis

These three aspects, antenna element number, PA performance and practical form factor are the main contributors for UE EIRP requirement.

In FR2-2, the PA performances including power efficiency and saturated power degrade compared to FR2-1. The path loss in the free space is higher as the frequency goes up to FR2-2. In order to compensate the degradation for 52.6~71 GHz, more antenna elements are considered in FR2-2.

In FR2-1, the assumption of antenna element number is 1x4 as a baseline. In the last meeting, a proposal of 2x8 antenna element number [2] was provided for handheld UE type. If the number goes straight up to 2x8, it is four times of the antenna size than that of the original FR2-1. It will increase the complexity of implementing such antenna size into the handheld UE packages. Our preference with moderate two times of antenna element number in FR2-1, that is 1x8 in FR2-2, it will strike a balance between the performance and complexity. We will further analysis handheld UE EIRP and spherical coverage based on 1x8 antenna size assumption.

Proposal 1: For handheld UE, Antenna array 1x8 per polarization is preferred for 52.6~71 GHz.

Based on 1x8 antenna element number assumption, we performed link budget analysis for 52.6~71 GHz as shown in Table 1. We assume these values for the parameters in Table 1 are based on a feasible design for handheld UE.

Table 1. Link budget analysis for FR2-2

| Parameter | Unit | Source |
| --- | --- | --- |
| Frequency range | GHz | 57~71 |
| # ant elements | / | 8 |
| Avg. element gain (per polarization) | dBi | 4 |
| Antenna roll-off loss vs frequency | dB | -1.5 |
| Realized antenna array gain | dBi | 8.5 |
| Polarization gain | dB | 2.8 |
| Total implementation loss (nominal) | dB | -9 |
| Total implementation loss (worst-case) | dB | -13 |
| P1d per PA (nominal) | dBm | 4.5 |
| P1d per PA (minimum) | dBm | 4.5 |
| Peak EIRP (nominal) | dBm | 16.3 |
| Tolerance | dB | -5 |
| Peak EIRP (minimum) | dBm | 11.3 |

We can observe that the nominal peak EIRP is 16.3 dBm and the minimum peak EIRP is 11.3 dBm based on the assumption of antenna element number 1x8.

Observation 1: For handheld UE, the minimum peak EIRP is 11.3dBm with 1x8 antenna element number assumption in FR2-2.

In FR2, for the UEs that support multiple FR2 bands, multiband relaxations are introduced for peak EIRP and EIRP spherical coverage, respectively. In future, UEs are most likely to support both FR2-1 and FR2-2 bands. For these UEs, multiband relaxations should be introduced for peak EIRP and EIRP spherical coverage to support FR2-1 and FR2-2 bands.

Observation 2: For handheld UE supporting both FR2-1 and FR2-2 bands, multiband relaxation factors should be considered on the peak EIRP requirement.

Handheld UE spherical coverage

For handheld UE CDF of coverage, WF on this issue [1] is as follows:

Agreement: Check if the parameters in R4-1801202 could be reused for evaluation of CDF of coverage

In original FR2, we spent a lot of effort to the work for spherical coverage. To decide the spherical coverage requirement, the simulation assumptions in [3] are based on the restrictions of real product UE from all participating companies. We believe the assumption 1~7 in R4-1801202 pretty much covered the possible cases for FR2. These can be a starting point for the evaluation of spherical coverage for handheld UE in FR2-2. If companies do have more extra real product UE assumptions, they can be added in the simulation assumption for evaluation.

Based on the assumption 3 in [3], we give our simulation results for spherical coverage as shown in Figure 1. The CDF curve of normalized antenna gain with one panel and two panels configuration are given in Figure 1a and Figure 1b, respectively. We can observe that the peak to 50% percentile gain drop is 15.54 dB with one panel configuration; 10.03 dB with two panels configuration. Based on the worst-case scenario, we give spherical coverage requirement as 15.54 dB gain drop for the peak to 50% percentile.

| - One panel configuration | - Two panels configuration |
| --- | --- |

Figure 1. Simulation Results for spherical coverage in FR2-2

Proposal 2: The parameters in R4-1801202 can be used as a starting point for evaluating CDF of coverage in FR2-2.

Observation 3: For the handheld UE spherical coverage, the peak to 50% percentile gain drop is 15.54 dB based on one panel configuration.

Conclusion

In this contribution, analyses are conducted on handheld UE EIRP and spherical coverage requirements based on 1x8 antenna element number assumption. The following observations and proposals are made:

Proposal 1: For handheld UE, Antenna array 1x8 per polarization is preferred for 52.6~71 GHz.

Observation 1: For handheld UE, the minimum peak EIRP is 11.3dBm with 1x8 antenna array.

Observation 2: For handheld UE supporting both FR2-1 and FR2-2 bands, multiband relaxation factors should be considered on the peak EIRP requirement.

Proposal 2: The parameters in R4-1801202 can be used as a starting point for evaluating CDF of coverage in FR2-2.

Observation 3: For the handheld UE spherical coverage, the peak to 50% percentile gain drop is 15.54 dB based on one panel configuration.

- References

[1] R4-2114989, WF on 60 GHz UE RF requirements, Qualcomm Inc, RAN4#100-e.

[2] R4-2114480, 60GHz UE TX, Qualcomm, RAN4#100-e.

[3] R4-1801202, WF on EIRP CDF for spherical coverage, Samsung, MediaTek, RAN4#AH-1801.