# 3GPP TSG-RAN WG4 Meeting #101-e	R4-2118278

# Electronic Meeting, 1st – 11th November, 2021

Source: 	vivo

Title: 	TP for SL enhancements

Agenda Item:	8.15.1

Document for:	Approval

- Introduction

In this text proposal, some editorial changes were made on the latest version of TR 38.785[1].

- Reference

[1] TR 38.785 V0.3.0

# Annex: TP for channel arrangement in TR38.785

============================ Start of TP ============================

## 4.3	NR sidelink enhancement operating scenarios

### 4.3.1	General description

The NR Sidelink enhancement operating scenarios will be described to support advanced V2X services, public safety services and other commercial use cases based on agreed WID. RAN4 needs to enhance RF core requirements based on operating scenarios in Rel-16 NR V2X service and the Proximity service (ProSe) in TR 36.877.

Specifically, RAN4 did not complete some open issues in Rel-16 NR V2X, which include:

- TxD for NR V2X

- PC2 HPUE for NR V2X

- Intra-band V2X operation for TDD band, e.g. band n79

- Intra-band con-current V2X operation for TDD band, e.g. band n79

The left over items will be further studied and specified into the related RF requirements in this TR.

From RAN4 RF perspective, it is necessary to study coexistence before introducing the new frequency NR band to support NR sidelink enhancement operating at a frequency adjacent to NR Uu service for advanced V2X services, public safety services and other commercial use cases in licensed spectrum.

In RAN4 #98-e meeting, RAN4 agreed not to introduce a new band for NR sidelink in FR2 since no operator proposed a FR2 NR operating band for NR sidelink.

Hence, RAN4 need to study and specify the RF core requirements for NR sidelink enhancement only in FR1 in Rel-17.

### 4.3.2	Operation Aspects

Operating scenarios for NR sidelink enhancement WI are as follows:

- (Aspect 1) SL services and Operating band perspectives

  - Case 1: Public Safety Service

    - Case 1A: Dedicated public safety licensed band such as NR Band n14

      - Public safety UE only operated in out-of-coverage.

    - Case 1B: Dedicated public safety licensed band in other licensed bands (depends on inputs from operators)

      - Public safety UE operated in both in-coverage and out-of-coverage.

  - Case 2: NR V2X service

    - Case 2A: V2X UE in ITS spectrum (e.g. 5.9GHz (n47))

      - RAN4 already evaluated in ITS spectrum in Rel-16 NR V2X WI

    - Case 2B: V2X UE in FR1 licensed bands (e.g. 4.5GHz(TDD), 2GHz(FDD))

      - TDD coexistence evaluated in Rel-16 NR V2X WI

      - FDD coexistence need to evaluated to protect legacy system (depends on inputs on operating band from operators)

  - Case 3: Other commercial use cases (depends on inputs from operators and other WGs)

    - Basic operation can be performed in existing NR SL operating bands.

    - If specific operating band is requested, need to add a new operating band in suffix E in TS38.101-1.

- (Aspect 2) gNB deployment including network control possibility

  - Case 1: Public safety Service

    - Case 1A: gNB is not deployed  since only public safety UE operated in out-of-coverage in licensed band such as n14

    - Case 1B: gNB is deployed to support both legacy NR UE and public safety UE

      - Public safety UE needs to protect the legacy NR system

  - Case 2: NR V2X service

    - Case 2A: No gNB deployment scenarios based on semi-statically network-configured/pre-configured radio parameters

      - Support UE autonomous resource allocation, at least mode 2.

    - Case 2B: gNB deployment scenarios

      - Providing more UE specific or/and more dynamic resource allocation including Mode 1.

  - Case 3: Other commercial use cases

    - Basic NR sidelink operation can support both in-coverage and out-of-coverage scenarios.

    - If identified, RAN4 need to add new operating scenarios

RAN4 shall consider exact deployment scenarios and operating band planning for public safety service, NR V2X service and other commercial SL operation.

===========================Unchanged parts omitted============================

### 5.1.2	PC2 NR V2X UE RF requirements for single carrier

#### 5.1.2.1	Maximum output power for NR V2X UE

The following V2X UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth. The period of measurement shall be at least one sub frame (1ms).

Table 8.1-1: V2X UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| … |  |  |  |  |  |  |  |  |
| n47 |  |  | 26 | ±2 | 23 | ±2 |  |  |
| NOTE 1: NR Band n47 is used for NR V2X Service. NOTE 2: PPowerClass is the maximum UE power specified without taking into account the tolerance |  |  |  |  |  |  |  |  |

#### 5.1.2.2	UE maximum output power reduction

The following assumption can serve as a starting point for MPR simulation assumptions as shown in Table 5.1.2.2-1 and Table 5.1.2.2-2.

Table 5.1.2.2-1: MPR simulation assumption for PC2 NR V2X

| parameter | Assumption |
| --- | --- |
| center frequency | 5.9GHz |
| Bandwidth | 10/20/30/40MHz |
| Maximum output power | 26 dBm |
| numerology | 15 kHz/30kHz/60kHz |
| Modulation | QPSK/16QAM/64QAM/256QAM |
| Waveform | CP-OFDM |
| Carrier leakage | 25dBc |
| IQ image | 25dBc |
| CIM3 | 45dBc or 60dBc |
| PA calibration | PA calibrated to deliver [31dBc] ACLR for a fully allocated RBs in 20MHz QPSK DFT- S-OFDM waveform at 1 dB MPR. This is based to share PA between LTE V2X and NR V2X at 5.9GHz as worst case. |

For PC2 NR V2X, simultaneous transmission of PSCCH and PSSCH in the same sub-frame is supported. The following constraints in Table 5.1.2.2-2 can be assumed based on current RAN1’s agreement.

Table 5.1.2.2-2: PC2 V2X UE’s MPR simulation assumptions

| Items | Assumption |
| --- | --- |
| Allowed sub-channel sizes | • Support {10, 12, 15, 20, 25, 50, 75, 100} PRBs for possible sub-channel size. |
| Allowed LCRB allocation | 10,12,15,20,24,25,30,36,40,45,48,50,60,70,72,75,80,84,90,96,100,105,108,110,120,130,132,135,140,144,150,156,160,165,168,170,175,180,190,192,195,200,204,210,216 |
| Regarding PSCCH / PSSCH multiplexing |  |
| PSCCH size | 10RB*3 Symbols |
| PSD offset of X dB between PSCCH and PSSCH | 0dB |

For simultaneous transmission of PSFCH transmission for PC2 V2X UE, RAN4 assumed as follow

Table 5.1.2.2-3: PC2 V2X UE’s MPR simulation assumptions for PSFCH transmission

| Items | Assumption |
| --- | --- |
| Modulation for PSSCH | QPSK |
| PSFCH | ZC sequence |
| Structure of Slot | Baseline is follow RAN1 agreements |
| RB allocation | - 1 RB per user - All users have the same power per RB - Total power of all users equals 26dBm for PC2 - Both Non-contiguous PSFCH RB allocation and contiguous PSFCH allocation are allowed - MPR will be derived by non-contiguous PSFCH RB allocation (N>1) - At least, the worst cases with possible RBstart and Ngap need to be checked. ( Ngap = RBend – RBstart ) - For example: The worst case N gap is (106-1 =105*15kHz*12=) 18.9MHz for 20MHz, 15kHz SCS - IMD problem by dual PSFCH in SEM/SE region shall be considered to derive MPR level according to all supporting CBW and SCS. - N (Number of users) is up to 5 and RBs except for RBstart and RBend can be inserted between RBstart and RBend randomly. - Assumption of N in RAN4 is only for MPR simulation purpose, the final number is up to RAN1 decision. |

For S-SSB transmission for PC2 V2X UE, RAN4 assumed as follow

Table 5.1.2.2-4: PC2 V2X UE’s MPR simulation assumptions for S-SSB transmission

| Items | Assumption |
| --- | --- |
| Modulation for PSBCH | QPSK |
| S-PSS | M-sequence |
| S-SSS | Golden-sequence |
| S-SSB structure |  |
| RB allocation | RBstart: All the possible cases LCRB: 11 RB |

##### 5.1.2.2.1	MPR for Power class 2 V2X UE

For contiguous allocation of PSCCH and PSSCH simultaneous transmission, the allowed MPR for the maximum output power for NR V2X physical channels PSCCH and PSSCH shall be specified as inner/outer RB allocations in Table 5.1.2.2.1-1 for power class 2.

Table 5.1.2.2.1-1: Maximum Power Reduction (MPR) for power class 2 V2X (Contiguous PSCCH and PSSCH transmission)

| Modulation |  | Channel bandwidth/MPR (dB) |  |
| --- | --- | --- | --- |
|  |  | Outer RB allocations | Inner RB allocations |
| CP-OFDM | QPSK | ≤ 5.5 | ≤ 2.5 |
|  | 16QAM |  |  |
|  | 64 QAM | ≤ 6.0 | ≤ 4.5 |
|  | 256 QAM | ≤ 7.0 |  |

Where the following parameters are defined to specify valid RB allocation ranges for Outer and Inner RB allocations:

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1 in TS38.101-1.

RBStart,Low = max(1, floor(LCRB/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low ≤ RBStart ≤ RBStart,High, and

LCRB ≤ ceil(NRB/2)

where ceil(x) is the smallest integer greater than or equal to x.

The RB allocation is an Outer RB allocation for all other allocations which are not an Inner RB allocation.

#### 5.1.2.3	UE maximum output power with additional requirements

To comply the EN 302 571 standard emission mask, RAN4 need to derive A-MPR requirements considering with the additional A-SEM and A-SE requirements in EN 302 571 standard as shown in Table 5.1.2.3-1 and Table 5.1.2.3-2.

NR V2X UE shall satisfy the additional SEM and additional SE requirements when NS_33 is configured from pre-configured radio parameters or the cell and the indication from upper layers has indicated was informed.

Table 5.1.2.3-1: Additional SEM requirements for 10MHz channel bandwidth

| Spectrum emission limit (dBm EIRP)/ Channel bandwidth |  |  |
| --- | --- | --- |
| ΔfOOB (MHz) | 10 MHz | Measurement bandwidth |
| 0-0.5 | [-13-12∆fOOBMHz] | 100 kHz |
| 0.5-5 | [-19-169∆fOOBMHz-0.5] | 100 kHz |
| 5-10 | [-27-2∆fOOBMHz-5.0] | 100 kHz |

NOTE 1:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

NOTE 2:	Additional SEM for V2X overrides any other requirements in frequency range 5855-5950MHz.

NOTE 3:	The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex G in TS38.101-1.

Table 5.1.2.3-2: Additional SE requirements

| NR Band | Spurious emission for UE co-existence |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n47 | E-UTRA Band 1, 3, 5, 7, 8, 22, 26, 28, 34, 39, 40, 41, 42, 44, 45, 65, 68, 72, 73 NR band n77, n78 , n79 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | Frequency range |  | 5925 | - | 5950 | -30 EIRP | 1 | 38, 40, 43 |  |
|  | Frequency range |  | 5815 | - | 5855 | -30 EIRP | 1 | 38, 43 |  |
| NOTE 38: Applicable when NS_33 or NS_34 is configured by the pre-configured radio parameters. NOTE 40: In the frequency range x-5950MHz, SE requirement of -30dBm/MHz should be applied; where x = max (5925, fc + 15), where fc is the channel centre frequency. NOTE 43: The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I. |  |  |  |  |  |  |  |  |  |

##### 5.1.2.3.1	A-MPR for NS_33

When NS_33 is indicated by the network or pre-configured radio parameters for NR V2X UE, the additional maximum output power reduction specified as

A-MPR = CEIL {MA, 0.5}

Where MA is defined as follows

MA = A-MPRBase + Gpost connector* A-MPRStep

CEIL{MA, 0.5} means rounding upwards to closest 0.5dB.

A-MPRBase which is specified for PSCCH and PSSCH transmission\S-SSB\PSFCH below is allowed when network signalling value is provided. A-MPRBase is the default A-MPR value when no Gpost connector is declared. The supported post antenna connector gain Gpost connector is declared by the UE following the principle described in 38.101-1.

For the contiguous PSSCH and PSCCH transmission when NS_33 is indicated by the network or pre-configured radio parameters for NR V2X UE, the NR UE allow the follow A-MPR requirements.

Table 5.1.2.3.1-1: A-MPR for PSCCH/PSSCH by NS_33 (at Fc=5860MHz)

| Carrier frequency [MHz] | Resource Block (LCRB) | Start Resource Block | A-MPR(dB) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | QPSK/16QAM | 64QAM | 256QAM |
| 5860 | ≥ 10 and ≤ 15 | 0 and 1 | ≤ 24 |  |  |
|  |  | 2 and 3 | ≤ 22 |  |  |
|  |  | 4 | ≤ 20 |  |  |
|  | ≥ 10 and ≤ 25 | ≥ 5 and ≤ 7 | ≤ 17.5 |  |  |
|  | ≥ 10 and ≤ 30 | 10 | ≤ 16 |  |  |
|  | ≥ 10 | 8 and 9 | ≤ 16 |  |  |
|  |  | ≥ 11 and ≤ 14 | ≤ 14.5 |  |  |
|  |  | ≥ 15 and ≤ 19 | ≤ 13 |  |  |
|  |  | ≥ 20 and ≤ 24 | ≤ 11.5 |  |  |
|  |  | ≥ 25 and ≤ 29 | ≤ 10 |  |  |
|  |  | ≥ 30 | ≤ 8.5 |  |  |
|  | ≥ 20 and ≤ 24 | 1 | ≤ 22 |  |  |
|  | ≥ 20 and ≤ 30 | 0 | ≤ 22 |  |  |
|  |  | 2 and 3 | ≤ 20 |  |  |
|  |  | 4 | ≤ 17.5 |  |  |
|  | ≥ 25 and ≤ 40 | 1 | ≤ 20 |  |  |
|  | ≥ 30 | ≥ 5 and ≤ 7 | ≤ 16 |  |  |
|  | ≥ 36 | 0 | ≤ 20 |  |  |
|  |  | ≥ 2 and ≤ 4 | ≤ 17.5 |  |  |
|  |  | 10 | ≤ 14.5 |  |  |
|  | ≥ 45 | 1 | ≤ 17.5 |  |  |

Table 5.1.2.3.1-2: A-MPR for PSSCH/PSCCH by NS_33 at other carrier frequency

| Carrier frequency [MHz] | RB allocations | A-MPR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | QPSK | 16QAM | 64QAM | 256QAM |
| 5870,5910,5920 | outer | ≤ 8.5 |  |  | ≤ 8.5 |
|  | inner | ≤ 6.0 |  |  |  |
| 5880,5890,5900 | outer | ≤ 6.0 |  |  | ≤ 6.5 |
|  | inner | ≤ 3.5 |  | ≤ 4.5 |  |

Inner and outer RB allocations are defined in section 5.1.2.2.1.

### 5.1.3	PC2 NR V2X UE RF requirements SL-MIMO

### 5.1.4	PC2 NR V2X inter-band con-current UE RF requirements

## 5.2	Intra-band V2X operation in a licensed band

### 5.2.1	Intra-band V2X operation scenarios and basic assumptions

For intra-band V2X operation, both TDM and FDM between Uu and SL are considered. The prioritization of operating scenarios including TDM and FDM and respective basic RF architecture for operating scenarios are defined as follows:

- 1st priority: TDM with same carrier or different carrier (Single RF chain for Tx as baseline)

  - RAN4 allow TDM operation between spectrally partially used PC5 SL and Uu UL/DL operation in a licensed TDD band regardless of adjacent/ non-adjacent carrier

- 2nd priority: FDM with adjacent carrier (Separate RF chain for Tx as baseline)

- 3rd priority: FDM with non-adjacent carrier (Separate RF chain for Tx as baseline)

Other UE RF architecture is not precluded from implementation perspective.

In Rel-17, RAN4 focus on TDD band for intra-band V2X operation and deprioritize FDD band. FDD band can be studied once operators have a request.

For intra-band contiguous V2X con-current operation with FDM operation, simultaneous UL transmission and SL reception in TDD band are not allowed.

### 5.2.2	Coexistence evaluation

#### 5.2.2.1 Coexistence evaluation scenarios

#### 5.2.2.2 Coexistence simulations assumptions

#### 5.2.2.3 Coexistence results

#### 5.2.2.4 Conclusion of Coexistence evaluations

### 5.2.3	Intra-band V2X operation with TDM operation

#### 5.2.3.1 Operating bands for intra-band V2X operation with TDM operation

NR Uu operation is designed to operate with NR sidelink in TDM mode on the operating band combinations listed in Table 5.2.3.1-1.

Table 5.2.3.1-1: Intra-band V2X operating bands with TDM operation

| NR Uu-NR V2X Band Combination | NR Uu or NR V2X Band | Interface |
| --- | --- | --- |
| V2X_n79_n791 | n79 | Uu |
|  | n79 | PC5 |
| NOTE 1: Only single switched sidelink or NR uplink/downlink is supported. |  |  |

For the intra-band V2X service for TDM operation in same carrier or different carriers, the NR Uu operation meet the NR UE RF requirements and NR V2X UE meet the existing TS38.101-1 RF requirements in suffix E, respectively.

#### 5.2.3.2 Tx requirements for intra-band V2X operation with TDM operation

##### 5.2.3.2.1	Additional Tx requirements for TDM operation

To decide the switching position for TDM operation in same/different carrier, RAN4 would be considered as following priority rule in TS38.321

- if both sl-PrioritizationThres and ul-PrioritizationThres are configured, the value of the priority of each NR SL and NR Uu is over the each threshold, then NR Uplink is high priority than NR SL transmission.

- if both sl-PrioritizationThres and ul-PrioritizationThres are configured, the value of the priority of each NR SL and NR Uu is lower the each threshold, NR Uplink is high priority than NR SL transmission.

- if both sl-PrioritizationThres and ul-PrioritizationThres are configured, the value of the priority of NR SL is lower the sl-PrioritizationThres and NR Uu is over the ul-PrioritizationThres, then NR uplink is high priority than NR SL transmission.

- if both sl-PrioritizationThres and ul-PrioritizationThres are configured, the value of the priority of NR SL is over the sl-PrioritizationThres and NR Uu is lower the ul-PrioritizationThres, then NR SL transmisison is high priority than NR uplink.

#### 5.2.3.3 Rx requirements for intra-band V2X operation with TDM operation

### 5.2.4	Intra-band contiguous V2X con-current operation with FDM operation

#### 5.2.4.1 Configuration for intra-band contiguous V2X con-current operation with FDM operation

Bandwidth combination sets and V2X transmission/reception configurations for intra-band contiguous V2X UE are specified in Table 5.2.4.1-1.

Table 5.2.4.1-1: NR  Uu-NR V2X configurations and bandwidth combination sets for intra-band contiguous V2X UE

| V2X configuration | V2X transmisison band | Channel bandwidths for Uu carrier (MHz) | Channel bandwidths for SL carrier (MHz) | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- |
| V2X_n79AA | NR band n79 | 40, 50, 60, 80, 100 | 10,20,30,40 | 140 | 0 |

#### 5.2.4.2 Tx requirements for intra-band contiguous V2X con-current operation with FDM operation

##### 5.2.4.2.1	Maximum output power

The following NR intra-band V2X con-current operating UE’s Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth. The period of measurement shall be at least one sub frame (1ms).

Table 5.2.4.2.1-1: intra-band V2X contiguous con-current V2X UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| … |  |  |  |  |  |  |  |  |
| V2X_n79B |  |  | 26 | ±2 | 23 | ±2 |  |  |
| NOTE 1: The intra-band contiguous con-current NR V2X operating band is used for NR V2X and NR Uu Service. Bandwidth class B is that the aggregated channel bandwidth 20 MHz ≤ BWChannel_SL&UL ≤ 100 MHz. NOTE 2: PPowerClass is the maximum UE power specified without taking into account the tolerance |  |  |  |  |  |  |  |  |

##### 5.2.4.2.2	UE maximum output power reduction

The following assumption can serve as a starting point for MPR simulation assumptions as shown in Table 5.2.4.2.2-1 and Table 5.2.4.2.2-2.

Table 5.2.4.2.2-1: MPR simulation assumption for NR V2X intra-band contiguous con-current operation

| parameter | Assumption |
| --- | --- |
| center frequency | 4.5GHz (n79) |
| Bandwidth | 10/20/30/40MHz for NR SL 10/20/40/60/80MHz for NR Uu |
| Maximum output power | 26 dBm |
| numerology | 15 kHz/30kHz/60kHz |
| Modulation | QPSK/16QAM/64QAM/256QAM |
| Waveform | CP-OFDM for NR SL CP-OFDM or DFT-S-OFDM for NR Uu |
| Carrier leakage | 25dBc |
| IQ image | 25dBc |
| CIM3 | 45dBc or 60dBc |
| PA calibration | PA calibrated to deliver 30dBc ACLR for a fully allocated RBs in 20MHz QPSK DFT- S-OFDM waveform at 1 dB MPR. |

For NR V2X intra-band contiguous con-current operation, simultaneous transmission of PUSCH/PUCCH and NR V2X’s PSCCH and PSSCH in the same sub-frame is supported. The following constraints in Table 5.2.4.2.2-2 can be assumed based on current RAN1’s agreement.

Table 5.2.4.2.2-2: NR V2X intra-band contiguous con-current operation UE’ MPR simulation assumptions

| Items | Assumption |
| --- | --- |
| Allowed sub-channel sizes for NR SL | • Support {10, 12, 15, 20, 25, 50, 75, 100} PRBs for possible sub-channel size. |
| Allowed LCRB allocation for NR SL | 10,12,15,20,24,25,30,36,40,45,48,50,60,70,72,75,80,84,90,96,100,105,108,110,120,130,132,135,140,144,150,156,160,165,168,170,175,180,190,192,195,200,204,210,216. No restriction of LCRB for NR Uu |
| Regarding PSCCH / PSSCH multiplexing for NR SL |  |
| PSCCH size for NR SL | 10RB*3 Symbols |
| PSD offset of X dB between PSCCH and PSSCH for NR SL | 0dB |
| NOTE 1: If there is not indicate the detail parameters, then follow TR38.886 for NR SL operation. Also follow TS38.211/TS38.212/TS38.101-1 for NR Uu operation. |  |

For simultaneous transmission of PSFCH transmission for V2X UE and PUSCH/PUCCH for NR Uu, RAN4 need further discussion how to apply the MPR requirements.

For simultaneous transmission of S-SSB transmission for V2X UE and PUSCH/PUCCH for NR Uu, RAN4 need further discussion how to apply the MPR requirements.

For NR intra-band V2X con-current operation with adjacent channel, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 5.2.4.1.1-1 with contiguous RB allocation is specified in Table 5.2.4.1.2-3 for UE power class 3 with bandwidth class B.

Table 5.2.4.1.2-3: Contiguous RB allocation for Power Class 3

| Modulation |  | MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | ≤ [1.0] | ≤ [1.0] |
|  | 16QAM | ≤ [1.0] | ≤ [1.0] |
|  | 64QAM | ≤ [1.0] | ≤ [1.0] |
|  | 256QAM | ≤ [1.5] | ≤ [1.5] |

For bandwidth class B with contiguous RB allocation, the following parameters are defined to specify valid RB allocation ranges for Inner and Outer RB allocations:

An RB allocation is contiguous if LCRB1 = 0 or LCRB2 = 0 or (LCRB1  0 and LCRB2  0 and RBStart1 + LCRB1 = NRB1 and RBStart2 = 0), where RBStart1, LCRB1, and NRB1 are for SL CC1, RBStart2, LCRB2, and NRB2 are for UL CC2. SL CC1 is the component carrier with lower frequency.

In contiguous NR V2X intra-band con-current operation, a contiguous allocation is an inner allocation if

RBStart,Low  ≤  RBStart_SL&UL  ≤  RBStart,High, and NRB_alloc  ≤  ceil(NRB,agg /2),

where

RBStart,Low = max(1, floor(NRB_alloc /2))

RBStart,High = NRB,agg – RBStart,Low – NRB,alloc,

with

NRB_alloc= LCRB1 ∙ 2µ1 + LCRB2 ∙ 2µ2

NRB_alloc= (NRB1 - RBStart1)∙ 2µ1 + (RBStart2 + LCRB2 ) ∙ 2µ2,

NRB,agg=NRB1∙2µ1+ NRB2∙2µ2.

If LCRB1 =0, RBStart_SL&UL = NRB1∙2µ1+ RBStart2∙2µ2,

if LCRB1 > 0, RBStart_SL&UL = RBStart1∙2µ1.

Where, µ1 and µ2 is 0, 1 and 2 for SCS of 15kHz, 30kHz and 60kHz respectively.

A contiguous allocation that is not an Inner contiguous allocation is an Outer contiguous allocation.

For NR intra-band V2X con-current operation with adjacent channel, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 5.2.4.1.1-1 with non-contiguous RB allocation is specified in Table 5.2.4.1.2-4 for UE power class 3 with bandwidth class B.

Table 5.2.4.1.2-4: Non-contiguous RB allocation for Power Class 3

| Modulation |  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | inner | Outer1 | Outer2 |
| CP-OFDM | QPSK | ≤ [1.0] | ≤ [1.5] | ≤ [3.0] |
|  | 16QAM | ≤ [1.0] | ≤ [1.5] | ≤ [3.0] |
|  | 64QAM | ≤ [1.0] | ≤ [1.5] | ≤ [3.0] |
|  | 256QAM | ≤ [1.5] | ≤ [2.0] | ≤ [3.0] |

For bandwidth classes B with non-contiguous RB allocation, the following parameters are defined to specify valid RB allocation ranges for Inner, Outer1 and Outer2 RB allocations:

Non-Contiguous RB allocation is defined as RBStart1 + LCRB1 < NRB1, or RBStart2 > 0, when both SL CC and UL CC are activated and allocated with RB(s), where RBStart1, LCRB1, and NRB1 are for SL CC1, RBStart2, LCRB2, and NRB2 are for UL CC2. SL CC1 is the component carrier with lower frequency.

In contiguous NR V2X intra-band con-current operation, a non-contiguous RB allocation is a non-contiguous Inner RB allocation if the following conditions are met:

RBStart,Low  ≤  RBStart_CA  ≤  RBStart,High and NRB_alloc ≤  ceil((BWChannel_SL&UL / 3 – BWgap ) / 0.18MHz),

where

NRB_alloc = (NRB1 - RBStart1)∙ 2µ1 + (RBStart2 + LCRB2 ) ∙ 2µ2, RBStart_SL&UL = RBStart1∙21

RBStart,Low = max(1, floor(NRB_alloc + (BWgap – BWGB,low)/0.18MHz))

RBStart,High = floor((BWChannel_SL&UL – 2 ∙ BWgap – BWGB,low)/0.18MHz – 2 ∙ NRB_alloc)

BWGB,low =Foffset,low – (NRB1∙12+1)∙SCS1/2

BWgap is the bandwidth of the gap between NRB1 and NRB2 possible allocations of SL CC1 and UL CC2 respectively.

In contiguous NR V2X intra-band con-current operation, a non-contiguous RB allocation is a non-contiguous outer 1 RB allocation if the following conditions are met:

RBStart,Low  ≤  RBStart_SL&UL  ≤  RBStart,High and NRB_alloc ≤  ceil((3 BWChannel_SL&UL / 5 – BWgap) / 0.18MHz)

where

RBStart,Low = max(1, 2 ∙ NRB_alloc – floor( (BWChannel_SL&UL – 2 ∙ BWgap + BWGB,low)/0.18MHz)),

RBStart,High = floor((2 ∙ BWChannel_SL&UL – 3 ∙ BWgap – BWGB,low) / 0.18MHz – 3 ∙ NRB_alloc)

NRB_alloc , RBStart_SL&UL , BWgap and BWGB,low are as defined for the Inner region.

In contiguous NR V2X intra-band con-current operation, a non-contiguous allocation is an Outer 2 allocation if it is neither a non-contiguous Inner allocation nor an Outer 1 allocation.

For NR intra-band V2X con-current operation with adjacent channel, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 5.2.4.1.1-1 with contiguous RB allocation is specified in Table 5.2.4.1.2-5 for UE power class 2 with bandwidth class B.

Table 5.2.4.1.2-5: Contiguous RB allocation for Power Class 2

| Modulation |  | MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | ≤ [1.5] | ≤ [3.5] |
|  | 16QAM | ≤ [2.0] | ≤ [3.5] |
|  | 64QAM | ≤ [3.0] | ≤ [3.5] |
|  | 256QAM | ≤ [5.0] | ≤ [5.0] |

For NR intra-band V2X con-current operation with adjacent channel, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 5.2.4.1.1-1 with non-contiguous RB allocation is specified in Table 5.2.4.1.2-6 for UE power class 2 with bandwidth class B.

Table 5.2.4.1.2-6: Non-contiguous RB allocation for Power Class 2

| Modulation |  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | inner | Outer1 | Outer2 |
| CP-OFDM | QPSK | ≤ [2.0] | ≤ [4.0] | ≤ [6.0] |
|  | 16QAM | ≤ [2.5] | ≤ [4.0] | ≤ [6.0] |
|  | 64QAM | ≤ [3.5] | ≤ [4.5] | ≤ [6.0] |
|  | 256QAM | ≤ [4.5] | ≤ [5.0] | ≤ [6.0] |

#### 5.2.4.3 Rx requirements for intra-band contiguous V2X con-current operation with FDM operation

### 5.2.5	Intra-band non-contiguous V2X con-current operation with FDM operation

#### 5.2.5.1 Configuration for intra-band non-contiguous V2X con-current operation with FDM operation

Bandwidth combination sets and V2X transmission/reception configurations for intra-band non-contiguous V2X are specified in Table 5.2.5.1-1.

Table 5.2.5.1-1: NR Uu-NR V2X configurations and bandwidth combination sets for intra-band non-contiguous V2X UE

| V2X configuration | V2X transmisison band | Channel bandwidths for Uu carrier (MHz) | Channel bandwidths for NR carrier (MHz) | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- |
| V2X_n79A-n79A | NR band n79 | 40, 50, 60, 80, 100 | 10,20,30,40 | 140 | 0 |

#### 5.2.5.2 Tx NR intra-band V2X con-current operation with non-adjacent channel

#### 5.2.5.3 Rx NR intra-band V2X con-current operation with non-adjacent channel

===========================Unchanged Parts Omitted ===========================

# 6	7	Operating bands and channel arrangement for SL enhancement

## 7.1	Operating bands

### 7.1.1	Operating bands

NR SL enhancement is designed to operate in the operating bands in FR1 defined in Table 7.1.1-1.

Table 7.1.1-1 Operating bands in FR1 for NR SL enhancement

| V2X Operating Band | Sidelink (SL) Transmission operating band |  |  | Sidelink (SL) Reception operating band |  |  | Duplex Mode | Interface |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |  |
| n141 | 788 MHz | - | 798 MHz | 788 MHz | - | 798 MHz | HD | PC5 |
| n792 | 4400 MHz | - | 5000 MHz | 4400 MHz | - | 5000 MHz | HD | PC5 |
| Note 1: When this band is used for public safety service, the NR band is operated with both in-coverage scenarios and out-of-coverage scenarios. Note 2: NR V2X service is partially operated in this band with NR Uu. |  |  |  |  |  |  |  |  |

### 7.1.2	Operating bands for inter-band con-current operation in FR1

## 7.2	Channel bandwidth

### 7.2.1	Channel bandwidth

The operating bands and channel bandwidth for NR SL enhancement in FR1 are shown in Table 7.2.1-1. The same (symmetrical) channel bandwidth is specified for both the TX and RX path. The maximum channel bandwidth for SL operation for NR SL enhancement in licensed band is 40MHz.

Table 7.2.1-1 Channel Bandwidth for NR SL enhancement

| V2X band / SCS/ V2X channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V2X Operating Band | SCS kHz | 5 MHz | 10 MHz | 20 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n14 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |
| n79 | 15 |  | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes | Yes |  |  |  |  |  |

### 7.2.2	Channel bandwidth for inter-band con-current operation

## 7.3	Channel arrangement enhancement

### 7.3.1	Channel raster

#### 7.3.1.1	NR-ARFCN and channel raster

The NR-ARFCN and channel raster defined in subclause 5.4.2.1 in TS38.101-1 are applied to each licensed operating band for NR SL enhancement.

#### 7.3.1.2	Channel raster to resource element mapping

Channel raster to resource element mapping defined in subclause 5.4.2.2 in TS38.101-1 are applied to each licensed operating band for NR SL enhancement.

#### 7.3.1.3	Channel raster entries for each operating band

The channel raster entries defined in subclause 5.4.2.3 in TS38.101-1 are applied to each licensed operating band for NR SL enhancement.

### 7.3.2	Synchronization raster

There is no synchronization raster definition for each licensed operating band for NR SL enhancement.

8	Transmitter/Receiver characteristics for SL enhancement

## 8.1	SL enhancement UE Tx requirements

## 8.2	SL enhancement UE Rx requirements

### 8.2.1 Reference sensitivity power level

The reference sensitivity power level REFSENS is the minimum mean power applied to the UE antenna connector at which the throughput shall meet or exceed 95% of the maximum throughput of the reference measurement channels.

The SL enhancement UE REFSENS is defined by the following equation:

REFSENSV2X=kTB + SNRV2X +10log10(LCRB*SCS*12/RX_BW) +( NFV2X+ IM) – Diversity gain

Where

-	kTB: Thermal noise level is [-174dBm(kT) + 10*log10(RX BW)]dBm.

-	NF: Noise figure. 13 dB is used for LAA and can be reused for NR V2X requirements. Assumed NF is 9dB < 3GHz, NF is 10dB>= 3GHz (e.g B42, n77, n78, n79…) at licensed bands at FR1.

-	IM: 2.5 dB is assumed. When the number of RB size is equal to or less than 24RBs, 0.5dB additional relaxation is allowed.

-	Target SNR: -0.5 dB

-	Diversity gain: 3dB

The REFSENS requirements for NR SL enhancement are specified in Table 8.2.1-1.

Table 8.2.1-1: Reference sensitivity for NR SL enhancement (PC5)

| NR Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| V2X Band | SCS kHz | 5MHz (dBm) | 10MHz (dBm) | 20MHz (dBm) | 30MHz (dBm) | 40MHz (dBm) | Duplex Mode |
| n14 | 15 | TDB | TDB |  |  |  | HD |
|  | 30 |  | TDB |  |  |  |  |
|  | 60 |  |  |  |  |  |  |

Table 8.2.1-2: Sidelink TX configuration for reference sensitivity for NR SL enhancement (PC5)

| NR operating Band / SCS/ Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| V2X Band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 20 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | Duplex Mode |
| n14 | 15 | 25 | 50 |  |  |  | HD |
|  | 30 |  | 24 |  |  |  |  |
|  | 60 |  |  |  |  |  |  |

============================ End of TP ============================