3GPP TSG-RAN WG4 #101-e	R4-2118344

Electronic meeting, 1st – 12th Nov, 2021

Source: 	Ericsson

Title:  	General requirements impacted for HST FR2

Agenda Item:	8.9.4.1

Document for:	Discussion

# Introduction

Work on RRM requirements for train-mounted UE in high speed train scenario in FR2 [1] continued during the RAN4#100e meeting, with outcome in terms of agreements and open issues captured in WFs [2, 3].

In this contribution, we focus on issues general RRM specifications due to HST FR2 deployment.

# Disucssion

According to the approved WF, the following issues related to NW signalling and CPE capabilities for HST FR2 deployments were agreed [1]:

## Network signaling

Identification of different/enhanced RRM requirements

Agreement: Add a flag in a form of cell-specific signalling to indicate UE to use different/enhanced RRM requirements in HST FR2 deployments. Way forward: Following the GtW agreement, FFS on feasibility and methods to differentiate scenarios from UE perspective

We agree with the statement on network signalling e.g. ‘network signalling is needed for indicating to UE that the enhanced NR RRM requirements apply per cell.’ in R4-2114568. However, it is controversial that UE’s capacity needed to be signaled in the LS.

WF R4-2108660 has the following conclusion:

- Dedicated network for roof-mounted CPE:

  - RAN4 assume that in HST FR2 Scenario A and B, only high-speed CPEs installed on the roof of the train can be present in the network.

  - No need to differentiate roof-mounted CPE from other FR2 UEs in HST FR2 scenario.

In this sense, UE doesn’t need flag and to identify capacity to support HST FR2 deployment.

Proposal 1: Support network signaling part in LS R4-2114568. Defer the signaling of UE capacity flag before agreement on issue ‘CPE support for HST FR2 deployment’.

Proposal 2: The definition of UE capacity in RRM hypothetically assumes non-CPE exist or present in HST FR2 scenario. How to handle non-CPE shall be discussed in other session and be reflected into the issue on UE capacity and this issue in turn.

Signaling of uni-/bi-directional operation

Way forward: Discuss further if there is a need to signal uni-bi-directional mode of operation: - Option 1: Network signals type of deployment (uni- or bi-direction) to UE. - Option 2: Signalling of uni-/bi-directional operation is not needed.

Uni- and Bi-directional deployments have been discussed intensively. The rationale behinds NW signaling of Uni- and/or Bi-directional deployment and CPE indication of support of uni-/bi-directional operation is differentiated RRM requirements. Besides of enhancements discussion, it is known that RX beams shall be different is important difference point between uni-/bi-directional deployments. And one-shot timing adjustment bring significant benefit in un-directional deployment.

Proposal 3: Support Option 1: Network signals type of deployment (uni- or bi-direction) to UE.

Proposal 4: With respect to scenario A/B and Dmin, the signaling combined may use the format with 2 dimensions, to cover requirement differentiation among Uni- or Bi-direction are tightly together with [Scenario A/B]:

| Direction | Deployment scenario |  |
| --- | --- | --- |
|  | [Scenario A] ( Distance_railtoRRH<[50]m) | [Scenario B] ( Distance_railtoRRH<[50]m) |
| Unidirection | 00 | 01 |
| Bi-idirection | 10 | 11 |

Signalling of network assistance information

Way forward: FFS signalling of network assistance information: - Option 1: In uni-directional deployment, network signals DL beam w.r.t. UE moving direction to UE. - Option 2: Enable network signaling of SSB index per RRH - Option 3: Network can indicate different SSBs on adjacent RRHs having the same QCL property: signal the mapping between the repeated sets of beams from the adjacent RRHs - Option 4: Scaling factor based on explicit signalling from network to UE or implicit signalling based on UE’s identification of SSB index/TCI-state, position of UE relative to RRHs and so on - Option 5: The following additional beam coverage related information can be signaled to UE - Distance between the projections of adjacent beam peaks on the track - Beam peak direction angle relative to track - The 6 dB beam-width projection on track - In addition, UE can report speed to the network. - The options are not mutually exclusive and other options are not precluded. Companies are encouraged to provide further analysis/details of network assisted signalling addressing potential benefits and drawbacks including but not limited to reducing UE RX beams and L1/L3 measurement delay, differentiating uni- and bi-directional modes, signalling overhead, etc. Companies are recommended to capture their analysis in TR 38.854.

Proposal 5: To our understanding, detailed targets of signaling of network assistance information may comprise:

- Optimized TCI state switching in condition of sudden SNR drop if network can acquire position of UE relative to RRHs(based on any possible direct or indirect calculation) and/or Dmin.

- The issue on RRH position at various side of trail rail can be overcame by explicit signaling from network if network send signaling of known neighbor RRH side position information.

- To correctly signaling UE moving direction to UE in uni-directional deployment, RRH needs to have capacity to identify UE’s moving towards RRH’s direction or backward.  It can be discussed more detailly after agreement on beam sweep number in uni-direcitional deployment.

## UE capabilities

CPE support for HST FR2 deployment

Agreement: HST FR2 CPE has a capacity to support both uni- and bi-directional operation. Way forward: Following the GtW agreement, FFS if different UE capabilities shall be used for Scenario A and B support Continue the discussion of CPE support for HST FR2 deployment: - Option 1: It is not necessary to introduce UE capability to indicate the support of FR2 HST. - Option 2: Define UE capability for FR2 HST enhancement support. FFS the ways to differentiate HST FR2 capable CPEs from any other UEs in the specification: - Option 1: Apply enhanced RRM requirements for FR2 HST based on Power Class corresponding to FR2 UE in TS 38.101-2. - Other options are not precluded. FFS a need for CPE capability to change characteristics, e.g. RX beam sweep number in uni-/bi-directional operation.

The first issue concerns that FFS if different UE capabilities shall be used for Scenario A and B support. To our understanding and based on previous discussion in last meeting, the distance between or UE/train and RRH is various in practice, resulted from region, geography or implementation. It is rational to assume that UE support Scenario A and B both.

Proposal 6: UE shall support Scenario A and B both, no capacity is needed to distinguish them.

The second issue assumes that there is UE which capacity cannot support enhancement in FR2 HST scenario indeed. We have not any agreement whether non-HST UEs can also be camped/operated in HST network.

Proposal 7: Support Option 1: It is not necessary to introduce UE capability to indicate the support of FR2 HST.

Proposal 8:  Regarding UE without enhancement support, if it must be checked, the issue is how to deal with UE without enhancement support in FR2 HST scenario essentially, which shall be discussed in other session.

Proposal 9: Related to differentiating HST FR2 capable CPEs from any other UEs in the specification, we suggest to get agreement on proposal 8 firstly.

Regarding ‘FFS a need for CPE capability to change characteristics, e.g. RX beam sweep number in uni-/bi-directional operation.’  To our understanding, CPE shall have capacity to change beam sweep number in uni-/bi-directional operation from implementation point of view.

Proposal 10: CPE shall have capacity to change beam sweep number in uni-/bi-directional operation from implementation point of view. And if flag is agreed to signal uni-/bi-directional case, UE shall change beam sweep number following the signaling.

# Conclustion

Proposal 1: Support network signaling in LS R4-2114568. Defer the signaling of UE capacity flag before agreement on issue ‘CPE support for HST FR2 deployment’.

Proposal 2: The definition of UE capacity in RRM hypothetically assumes non-CPE exist or present in HST FR2 scenario. How to handle non-CPE shall be discussed in other session and be reflected into the issue on UE capacity and this issue in turn.

Proposal 3: Support Option 1: Network signals type of deployment (uni- or bi-direction) to UE.

Proposal 4: With respect to scenario A/B and Dmin, the signaling combined may use the format with 2 dimensions, to cover requirement differentiation among Uni- or Bi-direction are tightly together with [Scenario A/B]:

| Direction | Deployment scenario |  |
| --- | --- | --- |
|  | [Scenario A] ( Distance_railtoRRH<[50]m) | [Scenario B] ( Distance_railtoRRH<[50]m) |
| Unidirection | 00 | 01 |
| Bi-idirection | 10 | 11 |

Proposal 5: To our understanding, detailed targets of signaling of network assistance information may comprise:

- Optimized TCI state switching in condition of sudden SNR drop if network can acquire position of UE relative to RRHs(based on any possible direct or indirect calculation) and/or Dmin.

- The issue on RRH position at various side of trail rail can be overcame by explicit signaling from network if network send signaling of known neighbor RRH side position information.

- To correctly signaling UE moving direction to UE in uni-directional deployment, RRH needs to have capacity to identify UE’s moving towards RRH’s direction or backward.  It can be discussed more detailly after agreement on beam sweep number in uni-direcitional deployment.

Proposal 6: UE shall support Scenario A and B both, no capacity is needed to distinguish them.

Proposal 7: Support Option 1: It is not necessary to introduce UE capability to indicate the support of FR2 HST.

Proposal 8:  Regarding UE without enhancement support, if it must be checked, the issue is how to deal with UE without enhancement support in FR2 HST scenario essentially, which shall be discussed in other session.

Proposal 9: Related to differentiating HST FR2 capable CPEs from any other UEs in the specification, we suggest to get agreement on proposal 8 firstly.

Proposal 10: CPE shall have capacity to change beam sweep number in uni-/bi-directional operation from implementation point of view. And if flag is agreed to signal uni-/bi-directional case, UE shall change beam sweep number following the signaling.

# References

- RP-202118 “New WID on NR support for high speed train scenario in FR2”, Samsung, Nokia

- R4-2115334 “WF on FR2 HST RRM (part 1)”, Nokia, Nokia Shanghai Bell

- R4-2115335 “WF on FR2 HST RRM (part 2)”, Samsung