3GPP TSG-RAN WG4 #101-e	R4-2118351

Electronic meeting, 1st – 12th Nov, 2021

Source: 	Ericsson

Title:  	Interruption requirements for extending NR operation to 71GHz

Agenda Item:	8.16.7.3

Document for:	Discussion

# Introduction

The WF for extending current NR operation to 71 GHz [1] was approved in RAN#100-e meeting.  In this contribution we present views on Interruption requirements.

# Disucssion

The issues related to Interruption requirements were identified in below:

Issue 3-3-1: Interruptions at SCell addition/release (inter-band CA) Agreements: Update Table 8.2.2.2.1-1 in TS 38.133 with 480/960 kHz subcarrier spacing as below: Interruption length X1 for SCell addition/release for inter-band CA NR Slot length (ms) of victim cell; Interruption length X1 (slots). 0; 1; 1. 1; 0.5; 2. 2; 0.25; Both aggressor cell and victim cell are on FR2; 4. Either aggressor cell or victim cell is on FR1; 5. 3; 0.125; Aggressor cell is on FR2; 8. Aggressor cell is on FR1; 9. 5; 0.03125; Aggressor cell is on FR2-2; 32. Aggressor cell is on FR2-1; FFS. Aggressor cell is on FR1; FFS. 6; 0.015625; Aggressor cell is on FR2-2; 64. Aggressor cell is on FR2-1; FFS. Aggressor cell is on FR1; FFS.

Issue 3-3-3: Interruptions at SCell activation/deactivation (inter-band CA) Agreements: - Update Table 8.2.2.2.2-1 in TS 38.133 with 480/960 kHz subcarrier spacing as below: Interruption length X2 for SCell activation/deactivation for inter-band CA NR Slot length (ms) of victim cell; Interruption length X2 (slots). 0; 1; 1. 1; 0.5; 1. 2; 0.25; Both aggressor cell and victim cell are on FR2; 2. Either aggressor cell or victim cell is on FR1; 3. 3; 0.125; Aggressor cell is on FR2; 4. Aggressor cell is on FR1; 5. 5; 0.03125; Aggressor cell is on FR2-2; 16. Aggressor cell is on FR2-1; FFS. Aggressor cell is on FR1; FFS. 6; 0.015625; Aggressor cell is on FR2-2; 32. Aggressor cell is on FR2-1; FFS. Aggressor cell is on FR1; FFS.

We are concerned that the MRTD number may influence the evaluation and decision of the Interruption needs, therefore it is best to revisit it after the MRTD agreement.

Proposal 1: Interruption requirements need MRTD as input, we prefer to keep the issue open upon MRTD is fixed.

# Conclustion

Proposal 1: Interruption requirements need MRTD as input, we prefer to keep the issue open upon MRTD is fixed.

# References

[1] R4-2115352 - WF on NR extension to 71 GHz – RRM - Part 2, Qualcomm