3GPP TSG-RAN WG4 Meeting # 101-e														R4-2118356

Electronic Meeting, Nov.1-12, 2021

Source: 	OPPO

Title: 	Other RRM requirements for FR2 inter-band DL CA requirements for CBM

Agenda Item:	8.4.6.1.2

Document for:	Discussion

Introduction

A WF on RRM requirements for FR2 Inter-band DL CA and UL CA [1] has been approved in RAN4#100e meeting.

In this contribution, we provide our views on other RRM requirements of FR2 inter-band DL CA RRM requirements.

- Interruption requirements

- Measurement restriction

- SCell activation requirements

Discussion

- Interruption requirements for FR2 inter-band DL CA

- Issue 1-2-2: Interruption requirements

Candidate options:

  - Option 1: The existing Rel16 interruption requirements of intra-band CA shall be applied

  - Option 2: Existing interruption requirements for (non-IBM) inter-band CA in R15/R16 can be reused for CBM type UE in R17

  - Option 3: Interruption requirements which may be impacted by MRTD>X should be FFS

Whether UE uses same or different RF chains to operate in FR2 inter-band CA with CBM will be up to UE implementation. Considering the requirements should be defined so that UE can satisfy the CBM requirements with same RF chain and different RF chains, the worst cases should apply. If UE uses the same RF chain to operate in FR2 inter-band CA with CBM, the existing interruption requirements of intra-band CA can be reused for the UE.

Proposal 1: For CBM based FR2 inter-band CA, the existing interruption requirements of intra-band CA can apply.

Issue 1-2-2: Scheduling restriction - Agreements: - The current scheduling restriction imposed on FR2 intra-band CA should be also applied to CBM-based FR2 inter-band CA. And the MRTD shall be also taken into account in the definition of “the fully or partially overlapped symbols”. - RRM - 9.2.5.3.3 Scheduling availability of UE performing measurements on FR2 - 9.10.2.6.2 Scheduling availability of UE performing CSI-RS based measurements in FR2 - RLM - 8.1.7.3 Scheduling availability of UE performing radio link monitoring on FR2 - Link recovery - 8.5.7.3 Scheduling availability of UE performing beam failure detection on FR2 - 8.5.8.3 Scheduling availability of UE performing L1-RSRP measurement on FR2 - L1 measurement - 9.5.6.3 Scheduling availability of UE performing L1-RSRP measurement on FR2 - 9.8.6.3 Scheduling availability of UE performing L1-SINR measurement on FR2 Issue 1-2-3: Measurement restriction Candidate options: - Option 1: RAN4 not to define additional measurement restrictions for CBM operation in FR2 inter-band CA - Option 2: Measurement restriction requirements need to be defined for CBM capable UE for FR2 inter-band CA scenario.

We agree that the general measurement restriction need to be considered such as UE may not be able to simultaneously measurement two serving CC when MRTD>X. Except that, RAN4 need not to define additional measurement restrictions for CBM operation in FR2 inter-band CA

Proposal 2: Except the general restriction for UE measurement, RAN4 does not need to define additional measurement restrictions due to CBM operation in FR2 inter-band CA.

- SCell activation requirement for CBM UE

Issue 1-2-4: SCell activation delay

Candidate options:

Principles:

  - Option 1: When PCell/PSCell and the target SCell are in a FR2 band pair with CBM and the target SCell is unknown, following components can be reduced/removed from SCell activation requirements:

    - Option 1c: SSB-ID search latency for coarse timing estimation can be skipped if MRTD smaller than CP length is adopted for CBM inter-band CA

    - Option 1d: AGC settling time could be reduced for UE owing to following AGC settling in PCell/PSCell

    - Option 1e: TCI state indication and CSI reporting can be skipped as well for both semi-persistent and periodic CSI reporting.

  - Option 2: The definition of T_SMTC_MAX in SCell activation requirements shall be updated as below.

    - For CBM Inter-band UE, the longer SMTC periodicity between active serving cells and SCell being activated in the bands supported for CBM.

  - Option 3: The target SCell activation delay requirements defined for the scenario where there is at least one active serving cell in the band, can be applied

Text proposals:

- In case of Semi-persistent CSI-RS is used for CSI reporting,

  - Option 1: 3ms + TFirstSSB_MAX + Trs + THARQ +  TFineTiming + 2ms

  - Option 2: 6ms + TFirstSSB_MAX + TSMTC_MAX + Trs + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP).

    - FFS if the definition of “TSMTC_MAX” needs to be updated

- If case of periodic CSI-RS is used for CSI reporting:

  - Option 1: 3ms + TFirstSSB_MAX + Trs + THARQ +  TFineTiming + 2ms

  - Option 2: 3ms + TFirstSSB_MAX + TSMTC_MAX + Trs + max {(THARQ + Tuncertainty_MAC + 5ms + TFineTiming), (Tuncertainty_RRC + TRRC_delay)}.

- FFS on detailed parameters like Tuncertainty_MAC

- FFS if some parameter can be further removed (based on discussion on principle)

- FFS on the impact of MRTD>X

We further discuss the requirements of Scell activation delay of CBM capable UE in this meeting. Since 3us MRTD is agreed for CBM UE, then cell search latency and AGC settling time shall be considered.

For the case if PCell/PSCell and the target SCell are in a FR2 band pair with CBM and the target SCell is unknown, the SCell activation requirements shall be reduced including AGC settling time and cell search time.

Proposal 3: The SCell activation requirements shall be reduced if PCell/PSCell and the target SCell are in a FR2 band pair with CBM and the target SCell is unknown.

Conclusion

In this paper, we provide our views on RRM requirements of FR2 inter-band CA enhancements in Rel17 and have the following proposals:

Proposal 1: For CBM based FR2 inter-band CA, the existing interruption requirements of intra-band CA can apply.

Proposal 2: Except the general restriction for UE measurement, RAN4 does not need to define additional measurement restrictions due to CBM operation in FR2 inter-band CA.

Proposal 3: The SCell activation requirements shall be reduced if PCell/PSCell and the target SCell are in a FR2 band pair with CBM and the target SCell is unknown.

- References

[1] R4-2115332	WF on RRM requirements for FR2 Inter-band DL CA and UL CA, Nokia

[2] R4-2113267, RRM requirements for FR2 inter-band DL CA enhancements, OPPO