3GPP TSG-RAN WG4 Meeting # 101-e	R4-2118358

# Electronic Meeting, Nov. 1-12, 2021

Agenda item:	8.8.2.2

Source:	OPPO

Title:	Inter-frequency measurement requirements for Rel17 FR1 HST

Document for:	Discussion

- Introduction

In last RAN4 meeting, a WF on FR1 HST RRM was approved [1]. In this contribution, we will provide our views on the remaining issues.

- Discussion

As agreements, RAN4 will define the enhancement for inter-frequency measurement in IDLE mode for HST, and support of HST Idle mode inter-frequency measurement enhancements is an optional UE feature without capability signalling. UEs not supporting inter-frequency measurement enhancements shall meet the existing inter-frequency measurement requirements.

Issue 2-2: how to perform the enhancement for inter-frequency measurement in idle mode for HST - Option 1 (QC, CATT, CMCC, Ericsson): using the same requirement as for intra-frequency measurement for HST DRX cycle length [s]; Tdetect,NR_Inter [s] (number of DRX cycles); Tmeasure,NR_Inter [s] (number of DRX cycles); Tevaluate,NR_Inter [s] (number of DRX cycles). 0.32; 2.56 x M2 (8 x M2); 0.32 x M3 (1 x M3); 0.96 x M4 (3 x M4). 0.64; 5.12 (8); 0.64 (1); 1.92 (3). 1.28; 8.96 (7); 1.28 (1); 3.84 (3). 2.56; 58.88 (23); 2.56 (1); 7.68 (3). Note 1: when SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 1.5, M3 = M4 = 2. - Option 2 (OPPO, HW, MTK, vivo, Xiaomi): The R16 enhanced EUTRA-NR inter-RAT measurement requirements in idle mode could be reused for NR inter-frequency measurements DRX cycle length [s]; Tdetect,NR_Inter [s] (number of DRX cycles); Tmeasure,NR_Inter [s] (number of DRX cycles); Tevaluate,NR_Inter [s] (number of DRX cycles). 0.32; 4.16 x M2 (13 x M2)Note 1; 0.64 x M3 (2 x M3)Note 1; 0.96 x M4 (3 x M4) Note 1. 0.64; 7.68 (12); 1.28 (2); 1.92 (3). 1.28; 12.8(10); 1.28 (1); 3.84 (3). 2.56; 58.88 (23); 2.56 (1); 7.68 (3). Note 1: M2=1.5, M3=2 and M4=2 if SMTC periodicity of measured intra-frequency cell > 40 ms; otherwise M2=M2=M3=1..

The UE behavior should be the same for inter-RAT and inter-frequency measurement if target frequencies are the same. Besides, power saving is one of crucial metric for idle mode. Option 2 is relatively relaxed compared with option 1.

Proposal 1: The R16 enhanced EUTRA-NR inter-RAT measurement requirements in idle mode could be reused for NR inter-frequency measurements

| DRX cycle length [s] | Tdetect,NR_Inter [s] (number of DRX cycles) | Tmeasure,NR_Inter [s] (number of DRX cycles) | Tevaluate,NR_Inter [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 4.16 x M2 (13 x M2)Note 1 | 0.64 x M3 (2 x M3)Note 1 | 0.96 x M4 (3 x M4) Note 1 |
| 0.64 | 7.68 (12) | 1.28 (2) | 1.92 (3) |
| 1.28 | 12.8(10) | 1.28 (1) | 3.84 (3) |
| 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |
| Note 1: M2=1.5, M3=2 and M4=2 if SMTC periodicity of measured intra-frequency cell > 40 ms; otherwise M2=M2=M3=1. |  |  |  |

Issue 2-3: FFS on the principle on the requirements if there are both HST inter-frequency layers and non-HST inter-frequency layers to be measured - Option 1 (CATT, CMCC, OPPO, Ericsson, HW, MTK, QC, vivo, Xiaomi): NHST_inter-f carrier * THST_interf + NnonHST_inter-f carrier * TnonHST_interf can be specified if there are HST inter-frequency carriers and non-HST inter-frequency carriers to be measured in idle mode - Option 2: FFS

Both HST and non-HST inter-frequency layers to be measured should be considered. If RAN4 decided to specify inter-frequency measurement enhancement in FR1 HST, the requirements shall follow the principle:

NHST_inter-f carrier * THST_interf + NnonHST_inter-f carrier * TnonHST_interf

Proposal 2: Agree the principle of requirements for nter-frequency measurement enhancement in FR1 HST:

NHST_inter-f carrier * THST_interf + NnonHST_inter-f carrier * TnonHST_interf

Issue 2-4: PSS/SSS detection time requirement for inter-frequency measurement with MG in connected state for HST - Option 1 (QC, Nokia): Condition NOTE1,2; TPSS/SSS_sync_inter. No DRX; Max(600ms, 6 Max(MGRP, SMTC period)) CSSFinter. DRX cycle ≤ 320ms; Max(600ms, Ceil(6*M2 Note 3) Max(MGRP, SMTC period, DRX cycle)) CSSFinter. DRX cycle > 320ms; 6 DRX cycle CSSFinter. NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: When high speed is not configured, M2 = 1.5; When high speed is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1. - Option 2 (CATT, MTK, OPPO, vivo, HW): Condition NOTE1,2; TPSS/SSS_sync_inter. No DRX; Max(600ms, 8 Max(MGRP, SMTC period)) CSSFinter. DRX cycle ≤ 320ms; Max(600ms, Ceil(8*M2 Note 3) Max(MGRP, SMTC period, DRX cycle)) CSSFinter. DRX cycle > 320ms; 8 DRX cycle CSSFinter. NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: When high speed is not configured, M2 = 1.5. When high speed is configured, M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1.. - Option 3 (Ericsson, Nokia): Condition NOTE1,2; TPSS/SSS_sync_inter. No DRX; Max(600ms, N Note 4 Max(MGRP, SMTC period)) CSSFinter. DRX cycle ≤ 320ms; Max(600ms, Ceil(N Note 4 M2 Note 3) Max(MGRP, SMTC period, DRX cycle)) CSSFinter. DRX cycle > 320ms; N Note 4 DRX cycle CSSFinter. NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1. NOTE 4: When highSpeedMeasFlag-r16 is not configured, N = 8; otherwise N = 5.. - Option 4 (Apple): Condition NOTE1,2; TPSS/SSS_sync_inter. No DRX; Max(600ms, Ceil(8*M2/1.5) * Max(MGRP, SMTC period)) * CSSFinter. DRX cycle ≤ 320ms; Max(600ms, Ceil(8*M2 Note 3) * Max(MGRP, SMTC period, DRX cycle)) * CSSFinter. DRX cycle > 320ms; Ceil(8*M2/1.5) * DRX cycle * CSSFinter. NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: When high speed is not configured, M2 = 1.5; When high speed is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1. - Option 5 (CMCC): Condition NOTE1,2; T PSS/SSS_sync_inter. No DRX; Max(600ms, 6 Max(MGRP, SMTC period)) CSSFinter. DRX cycle≤ 320ms; Max(600ms, Ceil(6 M Note 3) Max(MGRP, SMTC period, DRX cycle)) CSSFinter. DRX cycle > 320ms; Y Note 4 DRX cycle CSSFinter. NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: M = 1.5 if SMTC periodicity > 40 ms, otherwise M=1 NOTE 4: Y= 4 when SMTC <= 40ms, Y= 6 when SMTC > 40ms. - Option 6 (QC, Nokia, CMCC, Ericsson, Xiaomi): Condition NOTE1,2; TPSS/SSS_sync_inter for FR1 HST. No DRX; Max(600ms, 6 Max(MGRP, SMTC period)) CSSFinter. DRX cycle ≤ 320ms; Max(600ms, Ceil(6 M2 Note 3) Max(MGRP, SMTC period, DRX cycle)) CSSFinter. DRX cycle > 320ms; 6 DRX cycle CSSFinter. NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1.

Companies’ views are diverse. We propose option 2 to reuse LTE-NR inter-RAT measurement requirements (8 samples are proposed) as baseline, since additional time needs to be allowed for inter-frequency measurement compared with intra-frequency measurement. We can also compromise to option 6 with 6 samples between intra-frequency requirements and LTE-NR inter-RAT measurement requirements.

Proposal 3: About PSS/SSS detection time requirement for inter-frequency measurement with MG in connected state for HST, we can support option 2 or compromise to option 6.

- Conclusion

In this contribution, we provide our views on R17 HST enhancement in FR1.

Proposal 1: The R16 enhanced EUTRA-NR inter-RAT measurement requirements in idle mode could be reused for NR inter-frequency measurements

Proposal 2: Agree the principle of requirements for nter-frequency measurement enhancement in FR1 HST:

NHST_inter-f carrier * THST_interf + NnonHST_inter-f carrier * TnonHST_interf

Proposal 3: About PSS/SSS detection time requirement for inter-frequency measurement with MG in connected state for HST, we can support option 2 or compromise to option 6.

References

- R4-2115439, WF on FR1 HST RRM, CMCC

- R4-2113270, Inter-frequency measurement requirements for Rel17 FR1 HST, OPPO