3GPP TSG-RAN WG4 Meeting # 101-e	R4-2118359

# Electronic Meeting, Nov.1-12, 2021

Agenda item:	8.8.2.3

Source:	OPPO

Title:	L1-SINR requirements for Rel17 FR1 HST

Document for:	Discussion

- Introduction

In last RAN4 meeting, a WF on FR1 HST RRM was approved [1]. In this contribution, we will provide our views on the remaining issues for L1-SINR.

- Discussion

Issue 3-1: whether L1-SINR measurement is applicable to HST - Option 1 (CMCC, vivo): Yes. The configuration of CMR/IMR is up to network configuration, there is no restriction on L1-SINR for HST scenario - Option 2 (MTK, OPPO, Ericsson, HW, Apple, QC, Nokia, Intel): RAN4 needs to confirm whether L1-SINR measurement is applicable for HST or not

RAN4 has agreed on L1-RSRP, SS-SINR, SS-RSRP, SS-RSRQ to reuse the requirements of Rel-16 HST PCells for Rel-17 HST Scells. RAN4 still discussed whether L1-SINR measurement is applicable to HST. As L1-SINR measurement for HST may suffer the same performance degradation due to high speed, we doubt the benefit to introduce such measurement for HST.

Proposal 1: Do not see much benefit to introduce L1-SINR measurement requirement for HST.

Issue 3-2: If L1-SINR measurement is applicable to HST, whether it is necessary to specify upper bound of side condition for L1-SINR measurement - Companies are encouraged to evaluate whether an applicable upper bound for L1-SINR is needed and what is the concrete value of the upper bound, if needed.

Furthermore, for upper bound of side condition for L1-SINR measurement, if network is not allowed to configure CMR/ IMR to enable UE to perform L1-SINR for serving cell management, then this issue is not valid. If L1-SINR measurement is applicable to HST, we prefer legacy L1-SINR accuracy requirements can be reused for high speed train scenario.

Proposal 2: If L1-SINR measurement is applicable to HST, we prefer legacy L1-SINR accuracy requirements can be reused for high speed train scenario.

- Scell link recovery - Option 1 (CATT, CMCC, vivo, Nokia): For SCell link recovery, it depends on network. There is no need to have the limitation on the number of band(s)in the spec - Option 2 (MTK): For SCell link recovery, RAN4 needs to study how many band(s) is supported in R17 HST in FR1 - Option 3 (OPPO, HW, Nokia, vivo): The same limitation on the number of band(s) on which UE is performing beam failure detection for SCell in R16 can be reused in R17 HST

From UE’s perspective, it would be beneficial that the upper bound of the band is aligned between network and UE, for example, UE complexity can be reduced. However, the number of bands should rely on both network configuration and UE capability of performing link recovery on CCs simultaneously. We think the link recovery requirements for R17 FR1 HST can follow the assumption of non-HST case. The same limitation of non-HST scenarios should apply.

Proposal 3: The link recovery requirements for R17 FR1 HST can follow the assumption of non-HST case, where the same limitation of non-HST scenarios should apply.

- CSSF - Option 1(HW, OPPO, QC, CATT, CMCC, Ericsson, Nokia, vivo): For CSSF, it depends on network. There is no need to have the limitation on the number of Scell (s) in the spec - Option 2 (MTK): For CSSF, RAN4 needs to study how many SCell(s) is supported in R17 HST in FR1

Similarly, for CSSF, it is up to network deployment and configuration on the number of Scells. We support the searcher number shall remain unchanged. And the total delay is scaled by the number of SCells.

Proposal 4: CSSF depends on network and no need to have the limitation on the number of Scell (s) in the spec.

- Signaling - FFS whether highSpeedMeasFlag-r16 can be reused for the indication of application of enhanced RRM requirements for HST CA - Option 1 (CMCC, HW, OPPO, MTK, Nokia, Ericsson, CATT, vivo): Yes, the highSpeedMeasFlag-r16 can be reused for the indication of application of enhanced RRM requirements for HST CA - Other option is not precluded - FFS whether NW shall indicate which inter-frequency layers need to be measured more often, for which enhanced inter-frequency measurement requirements shall apply - Option 1 (Apple, vivo): NW shall indicate which inter-frequency layers need to be measured more often, for which enhanced inter-frequency measurement requirements shall apply - Other option is not precluded

We support UE capability should be added for the support of 500km/h in HST CA. No need to add the additional capability for 350km/h in Rel-17 HST. And the highSpeedMeasFlag-r16 can be reused for the indication of application of enhanced RRM requirements for HST CA

Proposal 5: The highSpeedMeasFlag-r16 can be reused for the indication of application of enhanced RRM requirements for HST CA.

- Release independent - Option 1 (CATT, CMCC, Ericsson): Rel-17 NR HST RRM enhancement can be release independent from Rel-15 - Option 2 (vivo, OPPO, Ericsson, vivo): The release independent issue is not discussed until the features discussed in R17 FR1 HST becomes stable

For release independent, we suggest to come back to this issue once any possible UE capabilities for Rel17 FR1 HST were concluded. Maybe some of the features should be discussed case by case.

Proposal 6: FFS release independent issues before the features discussed in R17 FR1 HST becomes stable.

- Conclusion

In this contribution, we provide our views on R17 HST enhancement in FR1.

Proposal 1: Do not see much benefit to introduce L1-SINR measurement requirement for HST.

Proposal 2: If L1-SINR measurement is applicable to HST, we prefer legacy L1-SINR accuracy requirements can be reused for high speed train scenario.

Proposal 3: The link recovery requirements for R17 FR1 HST can follow the assumption of non-HST case, where the same limitation of non-HST scenarios should apply.

Proposal 4: CSSF depends on network and no need to have the limitation on the number of Scell (s) in the spec.

Proposal 5: The highSpeedMeasFlag-r16 can be reused for the indication of application of enhanced RRM requirements for HST CA.

Proposal 6: FFS release independent issues before the features discussed in R17 FR1 HST becomes stable.

References

- R4-2115439, WF on FR1 HST RRM, CMCC

- R4-2113271, RRM requirement for Rel17 FR1 HST, OPPO