3GPP TSG-RAN WG4 Meeting # 101-e	R4-2118370

# Electronic Meeting, Nov.1-12, 2021

Agenda item:	8.20.3.3

Source: 	OPPO

Title: 	RRM measurement relaxations for RedCap UE

Document for:	Discussion and decision

Introduction

A WID of reduced capability NR devices was approved was updated in RAN#91e [1]. A WF [2] have been approved in RAN4#100e meeting. The following agreements are captured in the WF:

Agreements: - Do not postpone the discussion in RAN4 on relaxation methods for IDLE/INACTIVE states - Regarding RAN2’s progress, RAN4 starts discussion from stationary and not-cell-edge relaxation criterion for RRC_IDLE/INACTIVE as high priority - Reuse the same mechanism as Rel-16 RRM measurement relaxation in IDLE/Inactive mode - Method 1: Using scaling factors (value for FFS) - Method 2: Using a period of time (value for FFS) - Criteria for using Method 1 and/or 2 are FFS - Note: EMR is not supported by RedCap UEs and the relaxation mechanisms related to EMR carriers do not apply to RedCap - Wait for RAN2 until more progress on RRM relaxation in CONNECTED mode.

In this paper, we provide our views on the remaining issues of RRM measurement relaxation.

Discussion

In Rel-17, RAN2 has agreed to introduce at least stationary and not-cell-edge criterion for RRC_IDLE/INACTIVE and RRC Connected state. When NW configures Rel-17 RRM relaxation for RRC_IDLE/INACTIVE, Rel-17 stationary criterion is mandatory, and Rel-17 not-at-cell-edge criterion is optional configuration.

RAN4 agreed to start discussion from stationary and not-cell-edge relaxation criterion for RRC_IDLE/INACTIVE as high priority. RRM measurement relaxation and UE behaviour needs to be clarified case by case.

For stationary criterion, RAN2 agreed to reuse R16 low mobility criterion, as part or whole of Rel-17 stationary criterion in RRC_IDLE/INACTIVE. When NW configures both Rel-17 stationary criterion and Rel-16 low mobility criterion, NW configures different Rel-17 thresholds (i.e., SSearchDeltaP_stationary/TSearchDeltaP_stationary) from Rel-16 (SSearchDeltaP / TSearchDeltaP). How to configure the criterion (e.g. more stringent) is left to NW implementation (i.e. no specification impact to RAN2).

Issue 2-1-4: Relaxation when stationary criteria is satisfied Open issues: - Option 1: use scaling factor ( one or multiple and fixed/non-fixed for FFS) (Eric Xiaomi OPPO vivo MTK) - Option 1a: if single criteria (stationarity or not-at-cell-edge) is fulfilled, use K1_RedCap /K2_RedCap to relax the RRM requirement. K1_RedCap /K2_RedCap has the similar applicability condition as K1/K2 in R16 power saving WI, and FFS on the values for K1_RedCap /K2_RedCap (Apple) - Option 1b: When the stationarity criterion is configured, the scaling factor based RRM relaxation principle in Rel-16 should be considered firstly () - Option 1c: The requirements for R16 low mobility criterion can be used as baseline for Rel-17 stationary criterion, with a larger scaling factor (e.g., K2＞3) due to different Rel-17 thresholds. () - Option 1d: For stationary scenario, RRM measurement relaxation with larger scaling factor of measurement interval than R16 low mobility is applied - The scaling factor is fixed () - Option 2: Relax by at least one hour - During Idle/Inactive mode, when a UE is configured with and fulfils the stationarity criterion, then irrespective of other criteria being configured and/or fulfilled, it may relax the neighbour cell measurements by at-least one hour. Exact value is FFS (QC)

The requirements for R16 low mobility criterion can be used as baseline for Rel-17 stationary criterion, but they could be further relaxed with a larger scaling factor (e.g., K2＞3) due to different Rel-17 thresholds. Option 1a, 1c and 1d are fine for UE.

Proposal 1: The requirements for R16 low mobility criterion can be used as baseline for Rel-17 stationary criterion, FFS a larger scaling factor K_RedCap due to different Rel-17 thresholds.

For not-at-cell-edge criterion, RAN2 are still discussing whether the R16 not-at-cell-edge thresholds can be reused or separate R17 not-at-cell-edge thresholds are needed (i.e., SSearchThresholdP / SSearchThresholdQ). For R17 not-at-cell-edge criterion, RAN4 can wait for RAN2 to decide whether to reuse the requirements of R16 not-at-cell-edge.

Issue 2-1-5: Relaxation when either Rel-17 or Rel-16 not-at-cell-edge criteria is satisfied Open issues: - Option 1: same relaxation factor for the Rel-17 stationary criterion is used if a new Rel-17 not-at-cell edge criterion (MTK) - Option 2: For R17 not-at-cell-edge criterion, RAN4 can wait for RAN2 to decide whether to reuse the requirements of R16 not-at-cell-edge. (Eric, Apple, QC Xiaomi OPPO) - Option 3: Rel-17 not-at-cell-edge criterion in RRC_IDLE/INACTIVE reuse Rel-16 not-at-cell-edge criterion with the same thresholds. (Huawei) - Option 4: FFS (vivo) - Option 5: propose to agree on the relaxation method using a scaling factor for this case also ()

Proposal 2: For R17 not-at-cell-edge criterion, RAN4 can wait for RAN2 to decide whether to reuse the requirements of R16 not-at-cell-edge.

In addition, RAN2 also agreed network can configure R17 stationarity criterion/criteria together with a not-at-cell-edge criterion, or configure both R16/R17 relaxation criteria.

Issue 2-1-6: Relaxation when multiple criteria are satisfied Open issues: - Option 1: More relaxed RRM requirements (vivo MTK) - Option 1b: when a UE is configured with and fulfils the stationarity criterion and additionally fulfils Rel 16 and/or Rel-17 (if defined by RAN2) not-at-cell-edge criteria, the measurement relaxation could be greater than the one defined for a UE fulfilling only stationarity criterion. (QC Xiaomi OPPO vivo) - Option 1c: When both R17 stationary and not at cell edge criteria are fulfilled, more relaxed RRM measurement is expected. (Huawei) - Option 2: if both criterion (stationarity and not-at-cell-edge) are configured and fulfilled, use a fixed long measurement period for requirement relaxation, e.g., like 1 hour in R16 power saving WI. (Eric, Apple MTK) - Option 3: FFS ()

RAN4 needs to consider at least the followings：

- whether any new relaxation method will be introduced when both Rel-17 stationarity criterion and Rel-17 not-at-cell-edge criterion are configured and fulfilled，

- minimum requirements at transitions for different criterions

- how to select reselection requirements when UE is configured with R16/R17 relaxation criteria

Issue 2-3-2: Requirements when UE moves between different states Open issues: - Option 1: For RedCap RRM relaxation in RRC connected mode, based on measurement, when UE turns from ‘fulfil criteria’ to ‘not fulfil criteria’, this is also needed to report to network. () - Option 2: RAN4 to consider minimum requirements at transitions for different criterions switching (Apple OPPO); - Option 3: RAN4 to discuss assumptions how to evaluate RRM measurement relaxation. A distinction into RRC states and neighbour cell types may serve well in this context. () - Option 4: FFS (QC MTK Huawei) - Option 5: Option 1 is RAN2 issue () - Option 6: RAN4 to introduce minimum requirements at transition when UE changes between relaxed and normal measurement modes based on same transition principle in release 16 relaxation requirements in IDLE/INACTIE states. Whether to introduce minimum requirements at transition when UE changes between relaxed and normal measurement modes in CONNECTED state is FFS (Eric Apple QC vivo Huawei)

From our side, when NW configures both R16/R17 relaxation criteria and the UE fulfills both, it is up to UE implementation to select either Rel-16 or Rel-17 relaxation operation. At least RAN4 can agree firstly no requirement is defined for multiple transitions of scenarios within one measurement period.

Proposal 3: RAN4 to consider minimum requirements at transitions for different criterions switching;

Proposal 4: RAN4 to consider up to UE implementation to select either Rel-16 or Rel-17 relaxation operation when UE is configured with R16/R17 relaxation criteria

As agreed, the same mechanism as Rel-16 RRM measurement relaxation in IDLE/Inactive mode will be used. For UEs in RRC Connected, RAN2 agreed an RSRP/RSRQ based stationarity criterion can be configured. If the criterion is met, this is reported to the network. NW may reconfigure RRM measurement configuration or enable RRM measurement relaxation based on this reporting, which may have big impact on RRM requirements. RAN4 should consider the principle of trying to reuse the conclusion of RRC idle/inactive for RRC connected if the same criterion rules are defined.

Proposal 5: RAN4 to consider whether the same RRM relaxation mechanism can apply to RRC Connected mode.

Conclusion

In this paper, we provide our views on RRM measurement relaxation.

Proposal 1: The requirements for R16 low mobility criterion can be used as baseline for Rel-17 stationary criterion, FFS a larger scaling factor K_RedCap due to different Rel-17 thresholds.

Proposal 2: For R17 not-at-cell-edge criterion, RAN4 can wait for RAN2 to decide whether to reuse the requirements of R16 not-at-cell-edge.

Proposal 3: RAN4 to consider minimum requirements at transitions for different criterions switching;

Proposal 4: RAN4 to consider up to UE implementation to select either Rel-16 or Rel-17 relaxation operation when UE is configured with R16/R17 relaxation criteria

Proposal 5: RAN4 to consider whether the same RRM relaxation mechanism can apply to RRC Connected mode.

- References

[1] RP-210918, “Revised WID on support of reduced capability NR devices”, Nokia, Ericsson, RAN#91e

[2] R4-2115364, WF on RedCap RRM requirements, RAN4#100e

[3] R4-2113287, RRM measurement relaxations for RedCap UE, OPPO