3GPP TSG-RAN WG4 Meeting #101-e	R4-2118408

Electronic Meeting, 1st Nov. – 12th Nov. 2021

Agenda Item:	8.12.2.1.1

Source:	Ericsson

Title:	Simulation results on PDSCH performance for inter-cell interference

Document for:	Information

# 1	Introduction

In this contribution, the MMSE-IRC for suppressing inter-cells’ interference performance simulation results based will be shown. The detailed simulation assumptions are listed in table below.

| Parameters | Serving cell FDD | Serving cell TDD | Interferer cell(s) |
| --- | --- | --- | --- |
| Network scenario | Sync. |  |  |
| Number of carriers | single carrier |  |  |
| Carrier frequency | [2 GHz] | [4GHz] |  |
| PDSCH allocation in frequency domain | Full PRB |  |  |
| Physical cell ID | 0 | 0 | 1 |
| Active DL BWP index | 1 | 1 | N/A |
| Channel bandwidth | 10MHz | 40MHz |  |
| TDD configuration | N/A | 7DS2U, S=6D+4G+4U | 7DS2U, S=6D+4G+4U |
| SCS | 15kHz | 30kHz |  |
| PDCCH allocation in time domain | symbols #0 and #1 of each slot for PDCCH | symbols #0 and #1 of each slot for PDCCH | symbols #0 and #1 of each slot for PDCCH |
| SSB configuration | SSB index #0 | SSB index #0 | SSB index #0 |
| TRS configuration | k0=0 for CSI-RS resource 1,2,3,4 l0 = 6 for CSI-RS resource 1 and 3 l0 = 10 for CSI-RS resource 2 and 4 Density 3, 1 port FDD: Periodicity 20 slots; Offset 10 for CSI-RS resource 1 and 2; 11 for CSI-RS resource 3 and 4 TDD: Periodicity 40 slots; Offset 20 for CSI-RS resource 1 and 2; 21 for CSI-RS resource 3 and 4 |  | Same as Serving cell |
| PDSCH configuration | Type A, Start symbol 2, Duration 12​ |  | Type A, Start symbol 2, Duration 12​ |
| PRB bundling size for target PDSCH | 2 |  | 2 |
| PDSCH DMRS configuration | Type 1, 1+1 |  | Type 1, 1+1 |
| Scheduling with data multiplexed on the DMRS symbols | One single symbol front loaded and 1 additional DMRS |  | One single symbol front loaded and 1 additional DMRS |
| MCS | MCS 4 (QPSK, CR=0.3) MCS 13 (16QAM, CR=0.5) |  | 16QAM |
| Rank | 1 |  | random rank with 70% and 30% probability for rank 1 and rank 2 transmission in the interfering cell(s) |
| PDSCH precoder | Random precoding with single panel type I codebook per slot and per PRB bundling granularity, with PRB bundling size of 2. |  |  |
| Antenna configuration | 2x2, 2x4 |  |  |
| Antenna Correlation | ULA Low |  |  |
| Propagation condition | TDLA30-10 and TDLC300-100 |  |  |
| Time Offset relative to serving cell | 0 us |  |  |
| Metric | SINR to achieve 70% of maximum throughput |  |  |
| Number of HARQ Processes | 4 | 8 | N/A |
| Maximum HARQ transmission | 4 |  | N/A |

| INR scenario | SNR (dB) for cell 2 (first interferer cell) | SNR (dB) for cell 3 (second interferer cell) | Serving cell SNR (dB) |
| --- | --- | --- | --- |
| Scenario 1 | 5.43 | -1.50 | [-6:26] |
| Scenario 2 | 7.77 | 2.29 | [-6:26] |
| Scenario 3 | 13.91 | 3.34 | [-6:26] |
| Scenario 4 | 11.39 | 5.45 | [-6:26] |

# 2	Simulation Results

The simulation results are summarized as follow.

Table 1. simulation results for FR1 FDD

| Case Group Number | MIMO | MCS | INR scenario | Channel Model | SNR [dB] @ 70% of max T-put |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | MRC | IRC | Gain |
| 1 | 2Tx 2Rx, 2Tx 4Rx | QPSK MCS 4 | 1 | TDL-A 30-10 | 3.3 | 1.5 | 1.8 |
|  |  |  |  |  | -0.2 | -3.8 | 4 |
| 1 | 2Tx 2Rx, 2Tx 4Rx | QPSK MCS 4 | 2 | TDL-A 30-10 | 5.3 | 2.9 | 2.4 |
|  |  |  |  |  | 2 | -3.4 | 5.4 |
| 1 | 2Tx 2Rx, 2Tx 4Rx | QPSK MCS 4 | 3 | TDL-A 30-10 | 9.6 | 4.7 | 4.9 |
|  |  |  |  |  | 6.2 | -1.8 | 8 |
| 1 | 2Tx 2Rx, 2Tx 4Rx | QPSK MCS 4 | 4 | TDL-A 30-10 | 8.4 | 5 | 3.4 |
|  |  |  |  |  | 5.1 | -2.4 | 7.5 |
| 2 | 2Tx 2Rx, 2Tx 4Rx | 16QAM MCS 13 | 1 | TDL-A 30-10 | 10.4 | 8.7 | 1.7 |
|  |  |  |  |  | 6.3 | 2.7 | 3.6 |
| 2 | 2Tx 2Rx, 2Tx 4Rx | 16QAM MCS 13 | 2 | TDL-A 30-10 | 12.5 | 10.2 | 2.3 |
|  |  |  |  |  | 8.3 | 3.4 | 4.9 |
| 2 | 2Tx 2Rx, 2Tx 4Rx | 16QAM MCS 13 | 3 | TDL-A 30-10 | 16.7 | 12 | 4.7 |
|  |  |  |  |  | 12.5 | 3.8 | 8.7 |
| 2 | 2Tx 2Rx, 2Tx 4Rx | 16QAM MCS 13 | 4 | TDL-A 30-10 | 15.3 | 12.3 | 3 |
|  |  |  |  |  | 11.4 | 3.8 | 7.6 |
| 3 | 2Tx 2Rx, 2Tx 4Rx | QPSK MCS 4 | 1 | TDL-C 300-100 | 3.4 | 1.8 | 1.6 |
|  |  |  |  |  | 0.2 | -2.8 | 3 |
| 3 | 2Tx 2Rx, 2Tx 4Rx | QPSK MCS 4 | 2 | TDL-C 300-100 | 5.4 | 3.5 | 1.9 |
|  |  |  |  |  | 2.3 | -1.8 | 4.1 |
| 3 | 2Tx 2Rx, 2Tx 4Rx | QPSK MCS 4 | 3 | TDL-C 300-100 | 9.9 | 6.4 | 3.5 |
|  |  |  |  |  | 7 | 0.5 | 6.5 |
| 3 | 2Tx 2Rx, 2Tx 4Rx | QPSK MCS 4 | 4 | TDL-C 300-100 | 8.5 | 5.9 | 2.6 |
|  |  |  |  |  | -0.1 | 5.4 | 5.5 |
| 4 | 2Tx 2Rx, 2Tx 4Rx | 16QAM MCS 13 | 1 | TDL-C 300-100 | 11.3 | 9.8 | 1.5 |
|  |  |  |  |  | 7.6 | 4.7 | 2.9 |
| 4 | 2Tx 2Rx, 2Tx 4Rx | 16QAM MCS 13 | 2 | TDL-C 300-100 | 13.4 | 11.6 | 1.8 |
|  |  |  |  |  | 9.7 | 5.8 | 3.9 |
| 4 | 2Tx 2Rx, 2Tx 4Rx | 16QAM MCS 13 | 3 | TDL-C 300-100 | 17.9 | 14.7 | 3.2 |
|  |  |  |  |  | 14.1 | 8 | 6.1 |
| 4 | 2Tx 2Rx, 2Tx 4Rx | 16QAM MCS 13 | 4 | TDL-C 300-100 | 16.5 | 14.2 | 2.3 |
|  |  |  |  |  | 13 | 7.7 | 5.3 |

Observation 1: Compared with MRC, IRC receiver can have about 2dB or even more gain in 70% of maximum throughput SNR point.

Observation 2: IRC receiver have gains in both QPSK and 16QAM scenarios.

Observation 3: IRC receiver have gains in both TDLA and TDLC channels.

Observation 4: The higher the inference from the interferer cell(s) the higher the gain with IRC receiver.

Detail simulation results are shown in the appendix.

# References

- R4-2115730, “Way Forward on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC”, Intel Corporation.

# Appendix

- FR1 FDD

TDL-A

TDL-C