3GPP TSG-RAN WG4 Meeting #101-e	R4-211xxxx

Electronic Meeting, 1st Nov. – 12th Nov. 2021

Agenda Item:	8.12.2.1.2

Source:	Ericsson

Title:	Simulation results on CSI reporting for inter-cell interference

Document for:	Information

# 1	Introduction

In this contribution, the MMSE-IRC for suppressing inter-cells’ interference performance simulation results based will be shown. The detailed simulation assumptions are listed in table below.

| Parameter |  |  | Unit | FDD | TDD | Interferer cell |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 10 | 40 |  |
| Duplex Mode |  |  |  | FDD | TDD |  |
| Subcarrier spacing |  |  | kHz | 15 | 30 |  |
| SINR |  |  | dB | [-6: 2 :20] |  | - |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 4 |  | 4 |
|  | CDM Type |  |  | FD-CDM2 |  | FD-CDM2 |
|  | Density (ρ) |  |  | 1 |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5(4) |  | Option 1-1, 1-2: Row 5(4) Option 1-3: Row 5(6) Option 2-1, 2-2: Row 5(6) Option 2-3: Row 5(6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 |  | Option 1-1, 1-2: 9 Option 1-3: 12 Option 2-1, 2-2: 9 Option 2-3: 12 |
|  | CSI-RS periodicity and offset |  | slot | 5/1 | 10/1 | Same as serving cell |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 2 |  | 2 |
|  | CDM Type |  |  | FD-CDM2 |  | FD-CDM2 |
|  | Density (ρ) |  |  | 1 |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | Row 3(6, -) |  | Option 1-1, 2-1: Row 3(6, -) Option 1-2, 1-3, 2-2, 2-3: Row 3(2, -) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 |  | Option 1-1: 13 Option 1-2: 12 Option 1-3: 12 Option 2-1: 13 Option 2-2: 12 Option 2-3: 12 |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 5/1 | 10/1 | Same as serving cell |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  | Periodic |
|  | CSI-IM RE pattern |  |  | 0 |  | 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) |  | Option 1-1, 1-2, 1-3: (4,9) Option 2-1, 2-2, 2-3: (6,9) |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 5/1 | 10/1 | Same as serving cell |
| ReportConfigType |  |  |  | Periodic |  | Not configured |
| CQI-table |  |  |  | Table 2 |  | Table 2 |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  | Not configured |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  | Not configured |
| cqi-FormatIndicator |  |  |  | Wideband |  | Wideband |
| pmi-FormatIndicator |  |  |  | Wideband |  | Wideband |
| Sub-band Size |  |  | RB | 8 | 16 |  |
| Csi-ReportingBand |  |  |  | 1111111 |  | Not configured |
| CSI-Report periodicity and offset |  |  | slot | 5/0 | 10/9 | Not configured |
| aperiodicTriggeringOffset |  |  |  | Not configured |  | Not configured |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  | typeI-SinglePanel |
|  |  | Codebook Mode |  | 1 |  | 1 |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  | Not configured |
|  |  | CodebookSubsetRestriction |  | 010000 |  | 010000 |
|  |  | RI Restriction |  | N/A |  | Not configured |
| Physical channel for CSI report |  |  |  | PUCCH |  | Not configured |
| CQI/RI/PMI delay |  |  | ms | 8 | 9.5 | Not configured |
| Maximum number of HARQ transmission |  |  |  | 1 |  | Not configured |
| Measurement channel |  |  |  | As specified in Table A.4-2, TBS.2-2 |  |  |
| DIP |  |  | dB |  |  | -0.41 |
| Propagation condition |  |  |  | TDLA30-5 |  | AWGN |
| Correlation and antenna configuration |  |  |  | ULA Low |  |  |

| Serving cell configuration | NZP CSI-RS configuration | CSI-IM configuration |
| --- | --- | --- |
| #1 | Overlapping with Interf. Cell NZP CSI-RS | Overlapping with Interf. cell CSI-IM |
| #2 | Overlapping with Interf. Cell PDSCH | Overlapping with Interf. cell CSI-IM |
| #3 | Overlapping with Interf. Cell ZP CSI-RS | Overlapping with Interf. cell CSI-IM |
| #4 | Overlapping with Interf. Cell NZP CSI-RS | Overlapping with Interf. cell PDSCH |
| #5 | Overlapping with Interf. Cell PDSCH | Overlapping with Interf. cell PDSCH |
| #6 | Overlapping with Interf. Cell ZP CSI-RS | Overlapping with Interf. cell PDSCH |

# 2	Simulation Results

The simulation results are summarized as follow.

Table 1. simulation results for FR1 FDD

| Test | SNR [dB] @ 70% of max T-put |  |  | BLER |
| --- | --- | --- | --- | --- |
|  | MRC | IRC | Gain |  |
| #1 | 23.9 | 21.6 | 2.3 | 0.26 |
| #2 | 23.8 | 22.1 | 1.7 | 0.27 |
| #3 | 24.5 | 22.2 | 2.3 | 0.29 |
| #4 | 24 | 21.6 | 2.4 | 0.26 |
| #5 | 23.5 | 22.1 | 1.4 | 0.27 |
| #6 | 24 | 22.2 | 2.8 | 0.27 |

Observation 1: Compared with MRC-based CQI reporting, IRC-based CQI reporting receiver can have obvious throughput gain about 2dB gain in 70% of maximum throughput SNR point.

Observation 2: The BLER is greater than or equal to 10% which implies a reasonable CQI reporting to NW.

Detail simulation results are shown in the appendix.

# References

- R4-2115732, “WF on CSI requirements for inter-cell interference MMSE-IRC”, Ericsson

# Appendix

Test 1

Figure 1 TP and BLER comparison for test 1

Test 2

Figure 2 TP and BLER comparison for Test 2

Test 3

Figure 3 TP and BLER comparison for Test 3

Test 4

Figure 4 TP and BLER comparison for Test 4

Test 5

Figure 5 TP and BLER comparison for Test 5

Test 6

Figure 6 TP and BLER comparison for Test 6