3GPP TSG-RAN WG4 Meeting # 101-e	R4-2118430

Electronic Meeting, November 01-12, 2021

Source:	ZTE Corporation

Title:          Discussion on PDSCH requirements for CA scenarios

Agenda Item:	8.8.3.2

Document for:	Approval

- Introduction

In this contribution, the issues such as applicability rule and release independent are provided.

- Discussion

In RAN4# 100-e meeting, the following issues about applicability rule between SFN and DPS were reached [1]:

- Option 1: - When UE declares the capability demodulationEnhancement-r16, UE need to pass HST-SFN JT for CA. UE can skip HST-DPS for CA - When UE does not declare the capability demodulationEnhancement-r16, UE need to pass HST-DPS for CA. UE can skip HST-SFN for CA. - Option 2: Distribute the HST CA test cases into different SCS combinations. it can be ensured that performance under both HST-SFN CA and HST-DPS CA are verified by this compromise way UE capability; Cases to be tested. demodulationEnhancement-r16; more than 1 SCS configurations; more than 1 CC can track 2 active TCI states; FDD 15 + TDD 30; and/or; TDD 30 + TDD 30. x; x; x; DPS 1a; or; DPS 1a. x; x; √; DPS 1b; or; DPS 1b. x; √; x; DPS 1a; and; DPS 1a. x; √; √; DPS 1b; and; DPS 1b. √; x; x; SFN and DPS 1a; or; SFN and DPS 1a. √; x; √; SFN and DPS 1b; or; SFN and DPS 1b. √; √; x; SFN; and; DPS 1a. √; √; √; SFN; and; DPS 1b.

As pointed in our previous contribution[2], HST-SFN and DPS belong to different transmission scheme, and the receiving algorithm and performance will be different. Whether UE can skip DPS when SFN is tested or UE can skip SFN when DPS is tested was not reached. Option 2 was proposed which is more like not defining applicability rule between SFN and DPS. But from the last 2 row of option 2 above we can find that SFN is tested for one SCS combination and DPS is tested for the other SCS combination. The reason to select SFN or DPS is not clear which is also an applicability rule.

In our opinion, if applicability rule between DPS and SFN is not compromised, the modified option 2 can be considered.

Proposal 1 :To consider option 2a for HST CA test cases.

Option 2a: Distribute the HST CA test cases into different SCS combinations

| UE capability |  |  | Cases to be tested |  |  |
| --- | --- | --- | --- | --- | --- |
| demodulationEnhancement-r16 | more than 1 SCS configurations | more than 1 CC can track 2 active TCI states | FDD 15 + TDD 30 | and/or | TDD 30 + TDD 30 |
| x | x | x | DPS 1a | or | DPS 1a |
| x | x | √ | DPS 1b | or | DPS 1b |
| x | √ | x | DPS 1a | and | DPS 1a |
| x | √ | √ | DPS 1b | and | DPS 1b |
| √ | x | x | SFN and DPS 1a | or | SFN and DPS 1a |
| √ | x | √ | SFN and DPS 1b | or | SFN and DPS 1b |
| √ | √ | x | Declare for SFN and DPS 1a | and | Declare for SFN or DPS 1a |
| √ | √ | √ | Declare for SFN or DPS 1b | and | Declare for SFN or DPS 1b |

Another issue is the UE capability for HST CA.

- Option 1: - Define UE capability for HST-SFN CA, such as demodulationEnhancementCA-r17 (This capability can be release independent from Rel-15), that is different from the capability of demodulationEnhancement-r16 for HST-SFN single carrier - The granularity is per band to allow UE to report the supporting HST-SFN CA in some bands - Other option is not precluded

The fact that UE can support CA and HST separately does not mean that UE can support HST CA. therefore, UE capability can be introduced to indicate whether it support HST CA or not. However, the description of option 1 needs further clarification for The granularity is per band to allow UE to report the supporting HST-SFN CA in some bands needs to updated as The granularity is per band combination to allow UE to report the supporting HST-SFN CA in some bands.

Proposal 2 :To introduce UE capability for HST CA and the granularity is per band combination.

For release independent issue, the following was captured in the WF[1].

- Option 1: Rel-17 FR1 HST PDSCH CA requirements are release independent from Rel-15. - Option 2: Do not define release independent from Rel-15 for HST PDSCH CA requirements - Option 3: - HST-DPS CA requirements are release independent from Rel-15 - HST-SFN CA requirements are applicable from Rel-17 - Option 4: Rel-17 FR1 HST PDSCH CA requirements can be release independent from Rel-15, if Rel-17 RRM requirements for HST are defined release independent from Rel-15.

As the HST and CA is supported for release independent feature, and the supporting of HST CA can improve the user experience. So we support option 1.

Proposal 3 :To support option 1 or option 4 for release independent feature.

- Conclusion

In this contribution, we provide the following observations and proposals for FR1 HST CA:

Proposal 1 :To consider option 2a for HST CA test cases.

Option 2a: Distribute the HST CA test cases into different SCS combinations

| UE capability |  |  | Cases to be tested |  |  |
| --- | --- | --- | --- | --- | --- |
| demodulationEnhancement-r16 | more than 1 SCS configurations | more than 1 CC can track 2 active TCI states | FDD 15 + TDD 30 | and/or | TDD 30 + TDD 30 |
| x | x | x | DPS 1a | or | DPS 1a |
| x | x | √ | DPS 1b | or | DPS 1b |
| x | √ | x | DPS 1a | and | DPS 1a |
| x | √ | √ | DPS 1b | and | DPS 1b |
| √ | x | x | SFN and DPS 1a | or | SFN and DPS 1a |
| √ | x | √ | SFN and DPS 1b | or | SFN and DPS 1b |
| √ | √ | x | Declare for SFN and DPS 1a | and | Declare for SFN or DPS 1a |
| √ | √ | √ | Declare for SFN or DPS 1b | and | Declare for SFN or DPS 1b |

Proposal 2 :To introduce UE capability for HST CA and the granularity is per band combination.

Proposal 3 :To support option 1 or option 4 for release independent feature.

- Reference

R4-2115724 WF on demodulation for FR1 HST_v05_after GTW

R4-2113192 Discussion on PDSCH requirements for CA scenarios