3GPP TSG-RAN WG4 Meeting #101-e	R4-2118435

Electronic Meeting, November 01-12, 2021

Source:	ZTE Corporation

Title:	Link simulation results for PUSCH 256QAM demodulation requirements

Agenda Item:	8.12.3.1

Document for:	Discussion

- Simulation Results

Below are some simulation results of MCS 20 according to the agreed WF in RAN4#100-e [1].

| Mapping Type | Channel | DMRS Config | SCS [kHz] /CBW [MHz] | 1T2R [dB] |  | 1T4R [dB] |  | 1T8R [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Ideal | Impairment | Ideal | Impairment | Ideal | Impairment |
| Type A | TDLA30-10 Low | DMRS 1+1 | 15 /5 | 17.2 | 19.7 | 13.2 | 15.7 | 11.1 | 13.6 |
|  |  |  | 15 /10 | 17.7 | 20.2 | 13.5 | 16 | 11.3 | 13.8 |
|  |  |  | 30/10 | 17.7 | 20.2 | 13.2 | 15.7 | 11.4 | 13.9 |
|  |  |  | 30/40 | 18.4 | 20.9 | 13.4 | 15.9 | 11.4 | 13.9 |

- Reference

R4-2115748 WF for FR1 PUSCH 256QAM performance requirements_final.doc