| 3GPP TR 37.827 V0.3.0 (2021-11) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Networks; Power Class 2 for EN-DC with xLTE band + yNR DL with 1LTE+1(TDD) NR UL band (x= 2, 3, 4, y=1; x=1, 2, y=2). (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	11

1	Scope	13

2	References	13

3	Definitions of terms, symbols and abbreviations	13

3.1	Terms	13

3.2	Symbols	13

3.3	Abbreviations	13

4	Background	13

4.1	TR maintenance	14

5	EN-DC Power Class 2: Specific Band Combination Part	14

5.1	DC_2A-5A_n77A	14

5.1.1	Transmitter Characteristics	14

5.1.1.1	Maximum Output Power	14

5.1.1.2	Configurations for EN-DC	14

5.1.1.3	Co-existence study	14

5.1.2	Receiver Characteristics	14

5.1.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	14

5.1.2.1.1	Power class 2 Case A	14

5.1.2.1.2	Power class 2 Case B	15

5.2	DC_2-13_n77	15

5.2.1	Transmitter Characteristics	15

5.2.1.1	Maximum Output Power	15

5.2.1.2	 Configurations for EN-DC	15

5.2.1.3	Co-existence study	15

5.2.2	Receiver Characteristics	16

5.2.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	16

5.2.2.1.1	Power class 2 Case A	16

5.2.2.1.2	Power class 2 Case B	16

5.3	DC_2-66_n77	16

5.3.1	Transmitter Characteristics	16

5.3.1.1	Maximum Output Power	16

5.3.1.2	Configurations for EN-DC	16

5.3.1.3	Co-existence study	17

5.3.2	Receiver Characteristics	17

5.3.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	17

5.3.2.1.1	Power class 2 Case A	17

5.3.2.1.2	Power class 2 Case B	17

5.4	DC_5-66_n77	18

5.4.1	Transmitter Characteristics	18

5.4.1.1	Maximum Output Power	18

5.4.1.2	Configurations for EN-DC	18

5.4.1.3	Co-existence study	18

5.4.2	Receiver Characteristics	18

5.4.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	18

5.4.2.1.1	Power class 2 Case A	18

5.4.2.1.2	Power class 2 Case B	19

5.5	DC_13-66_n77	19

5.5.1	Transmitter Characteristics	19

5.5.1.1	Maximum Output Power	19

5.5.1.2	Configurations for EN-DC	19

5.5.1.3	Co-existence study	19

5.5.2	Receiver Characteristics	19

5.5.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	19

5.5.2.1.1	Power class 2 Case A	19

5.5.2.1.2	Power class 2 Case B	20

5.6	DC_2_n5-n77	20

5.6.1	Transmitter Characteristics	20

5.6.1.1	Maximum Output Power	20

5.6.1.2	Configurations for EN-DC	20

5.6.1.3	Co-existence study	20

5.6.2	Receiver Characteristics	20

5.6.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	20

5.6.2.1.1	Power class 2 Case A	20

5.6.2.1.2	Power class 2 Case B	21

5.7	DC_66_n2-n77	21

5.7.1	Transmitter Characteristics	21

5.7.1.1	Maximum Output Power	21

5.7.1.2	 Configurations for EN-DC	21

5.7.1.3	Co-existence study	21

5.7.2	Receiver Characteristics	22

5.7.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	22

5.7.2.1.1	Power class 2 Case A	22

5.7.2.1.2	Power class 2 Case B	22

5.8	DC_66_n5-n77	23

5.8.1	Transmitter Characteristics	23

5.8.1.1	Maximum Output Power	23

5.8.1.2	Configurations for EN-DC	23

5.8.1.3	Co-existence study	23

5.8.2	Receiver Characteristics	23

5.8.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	23

5.8.2.1.1	Power class 2 Case A	23

5.8.2.1.2	Power class 2 Case B	24

5.9	DC_2-13_n66-n77	24

5.9.1	Maximum Output Power	24

5.9.3	Co-existence study	24

5.10	DC_2-13-66_n77	25

5.10.1	Maximum Output Power	25

5.10.2	Configuration for EN-DC	25

5.10.3	Co-existence study	25

5.11	DC_13-66_n2-n77	25

5.11.1	Maximum Output Power	25

5.11.2	Co-existence study	25

5.12	DC_2-66_n5-n77	26

5.12.1	Maximum Output Power	26

5.12.3	Co-existence study	26

5.13	DC_2-5-66_n77	26

5.13.1	Maximum Output Power	26

5.13.3	Co-existence study	26

5.14	DC_13_n66-n77	27

5.14.1	Transmitter Characteristics	27

5.14.1.1	Maximum Output Power	27

5.14.1.3	Co-existence study	27

5.14.2	Receiver Characteristics	27

5.14.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	27

5.14.2.1.1	Power class 2 Case A	27

5.14.2.1.2	Power class 2 Case B	28

5.15	DC_13_n2-n77	28

5.15.1	Transmitter Characteristics	28

5.15.1.1	Maximum Output Power	28

5.15.1.3	Co-existence study	28

5.15.2	Receiver Characteristics	28

5.15.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	28

5.15.2.1.1	Power class 2 Case A	28

5.15.2.1.2	Power class 2 Case B	29

5.16	DC_2A-66A_n41A	29

5.16.1	Transmitter Characteristics	29

5.16.1.1	Maximum Output Power	29

5.16.1.2	Co-existence study	29

5.16.2	Receiver Characteristics	29

5.16.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	29

5.16.2.1.1	Power class 2 Case A	29

5.16.2.1.2	Power class 2 Case B	30

5.17	DC_66_n66-n77	30

5.17.1	Transmitter Characteristics	30

5.17.1.1	Maximum Output Power	30

5.17.1.3	Co-existence study	30

5.17.2	Receiver Characteristics	30

5.17.2.1	 MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	30

5.17.2.1.1	Power class 2 Case A	30

5.17.2.1.2	Power class 2 Case B	31

5.18	DC_48-66_n77	31

5.18.1	Transmitter Characteristics	31

5.18.1.1	Maximum Output Power	31

5.18.1.3	Co-existence study	31

5.18.2	Receiver Characteristics	32

5.18.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	32

5.19	DC_13_n5-n77	32

5.19.1	Transmitter Characteristics	32

5.19.1.1	Maximum Output Power	32

5.19.1.3	Co-existence study	32

5.19.2	Receiver Characteristics	33

5.19.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	33

5.19.2.1.1	Power class 2 Case A	33

5.19.2.1.2	Power class 2 Case B	33

5.20	DC_5_n66-n77	33

5.20.1	Transmitter Characteristics	33

5.20.1.1	Maximum Output Power	33

5.20.1.3	Co-existence study	33

5.20.2	Receiver Characteristics	34

5.20.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	34

5.20.2.1.1	Power class 2 Case A	34

5.20.2.1.2	Power class 2 Case B	34

5.21	DC_5A_n5A-n77A	34

5.21.1	Transmitter Characteristics	34

5.21.1.1	Maximum Output Power	34

5.21.1.3	Co-existence study	35

5.21.2	Receiver Characteristics	35

5.21.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	35

5.21.2.1.1	Power class 2 Case A	35

5.21.2.1.2	Power class 2 Case B	35

5.22	DC_5_n2-n77	35

5.22.1	Transmitter Characteristics	35

5.22.1.1	Maximum Output Power	35

5.22.1.3	Co-existence study	36

5.22.2	Receiver Characteristics	36

5.22.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	36

5.22.2.1.1	Power class 2 Case A	36

5.22.2.1.2	Power class 2 Case B	36

5.23	DC_2_n66-n77	37

5.23.1	Transmitter Characteristics	37

5.23.1.1	Maximum Output Power	37

5.23.1.3	Co-existence study	37

5.23.2	Receiver Characteristics	37

5.23.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	37

5.23.2.1.1	Power class 2 Case A	37

5.23.2.1.2	Power class 2 Case B	38

5.24	DC_2-48_n77	38

5.24.1	Transmitter Characteristics	38

5.24.1.1	Maximum Output Power	38

5.24.1.3	Co-existence study	38

5.24.2	Receiver Characteristics	38

5.24.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	38

5.25	DC_2A_n2A-n77A	39

5.25.1	Transmitter Characteristics	39

5.25.1.1	Maximum Output Power	39

5.25.1.3	Co-existence study	39

5.25.2	Receiver Characteristics	39

5.25.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	39

5.25.2.1.1	Power class 2 Case A	39

5.25.2.1.2	Power class 2 Case B	40

5.26	DC_1A-5A_n78A	40

5.26.1	Transmitter Characteristics	40

5.26.1.1	Maximum Output Power	40

5.26.1.2	Co-existence study	40

5.26.2	Receiver Characteristics	40

5.26.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	40

5.26.2.1.1	Power class 2 Case A	40

5.26.2.1.2	Power class 2 Case B	41

5.27	DC_1A-n7A_n78A	41

5.27.1	Transmitter Characteristics	41

5.27.1.1	Maximum Output Power	41

5.27.1.2	Co-existence study	41

5.27.2	Receiver Characteristics	42

5.27.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	42

5.27.2.1.1	Power class 2 Case A	42

5.27.2.1.2	Power class 2 Case B	42

5.28	DC_2-5_n5-n77	43

5.28.1	Maximum Output Power	43

5.28.3	Co-existence study	43

5.29	DC_2-5-48_n77	43

5.29.1	Maximum Output Power	43

5.29.3	Co-existence study	43

5.30	DC_2-5-66_n5-n77	44

5.30.1	Maximum Output Power	44

5.30.3	Co-existence study	44

5.30	DC_2-13_n2-n77	44

5.30.1	Maximum Output Power	44

5.30.3	Co-existence study	45

5.31	DC_2-13_n5-n77	45

5.31.1	Maximum Output Power	45

5.31.3	Co-existence study	45

5.32	DC_2-13-48_n77	45

5.32.1	Maximum Output Power	45

5.32.3	Co-existence study	46

5.33	DC_2-13-66_n2-n77	46

5.33.1	Maximum Output Power	46

5.33.3	Co-existence study	46

5.34	DC_2-13-66_n5-n77	47

5.34.1	Maximum Output Power	47

5.34.3	Co-existence study	47

5.35	DC_2-48-66_n77	47

5.35.1	Maximum Output Power	47

5.35.3	Co-existence study	48

5.36	DC_2-66_n2-n77	48

5.36.1	Maximum Output Power	48

5.36.3	Co-existence study	48

5.37	DC_5-13_n77	48

5.37.1	Transmitter Characteristics	48

5.37.1.1	Maximum Output Power	48

5.37.1.3	Co-existence study	49

5.37.2	Receiver Characteristics	49

5.37.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	49

5.37.2.1.1	Power class 2 Case A	49

5.37.2.1.2	Power class 2 Case B	49

5.38	DC_5-48_n77	50

5.38.1	Transmitter Characteristics	50

5.38.1.1	Maximum Output Power	50

5.38.1.3	Co-existence study	50

5.38.2	Receiver Characteristics	50

5.38.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	50

5.39	DC_5-48-66_n77	50

5.39.1	Maximum Output Power	50

5.39.3	Co-existence study	51

5.40	DC_5-66_n5-n77	51

5.40.1	Maximum Output Power	51

5.40.3	Co-existence study	51

5.41	DC_13-48_n77	52

5.41.1	Transmitter Characteristics	52

5.41.1.1	Maximum Output Power	52

5.41.1.3	Co-existence study	52

5.41.2	Receiver Characteristics	52

5.41.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	52

5.42	DC_13-48-66_n77	52

5.42.1	Maximum Output Power	52

5.42.3	Co-existence study	53

5.43	DC_13-66_n5-n77	53

5.43.1	Maximum Output Power	53

5.43.3	Co-existence study	53

5.44	DC_2-12_n77	54

5.44.1	Transmitter Characteristics	54

5.44.1.1	Maximum Output Power	54

5.44.1.2	Configurations for EN-DC	54

5.44.1.3	Co-existence study	54

5.44.2	Receiver Characteristics	54

5.44.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	54

5.44.2.1.1	Power class 2 Case A	54

5.44.2.1.2	Power class 2 Case B	55

5.45	DC_2-14_n77	55

5.45.1	Transmitter Characteristics	55

5.45.1.1	Maximum Output Power	55

5.45.1.2	Configurations for EN-DC	55

5.45.1.3	Co-existence study	55

5.45.2	Receiver Characteristics	56

5.45.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	56

5.45.2.1.1	Power class 2 Case A	56

5.45.2.1.2	Power class 2 Case B	56

5.46	DC_2-29_n77	56

5.46.1	Transmitter Characteristics	56

5.46.1.1	Maximum Output Power	56

5.46.1.2	Configurations for EN-DC	56

5.46.1.3	Co-existence study	56

5.46.2	Receiver Characteristics	57

5.46.2.1	Reference sensitivity exceptions due to receiver harmonic mixing for PC2 EN-DC in NR FR1	57

5.46.2.1.1	Power class 2 Case A	57

5.46.2.1.2	Power class 2 Case B	57

5.47	DC_2-30_n77	57

5.47.1	Transmitter Characteristics	57

5.47.1.1	Maximum Output Power	57

5.47.1.2	Configurations for EN-DC	58

5.47.1.3	Co-existence study	58

5.47.2	Receiver Characteristics	58

5.47.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	58

5.47.2.1.1	Power class 2 Case A	58

5.47.2.1.2	Power class 2 Case B	58

5.48	DC_5-30_n77	59

5.48.1	Transmitter Characteristics	59

5.48.1.1	Maximum Output Power	59

5.48.1.2	Configurations for EN-DC	59

5.48.1.3	Co-existence study	59

5.48.2	Receiver Characteristics	59

5.48.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	59

5.48.2.1.1	Power class 2 Case A	59

5.48.2.1.2	Power class 2 Case B	60

5.49	DC_12-30_n77	60

5.49.1	Transmitter Characteristics	60

5.49.1.1	Maximum Output Power	60

5.49.1.2	Configurations for EN-DC	60

5.49.1.3	Co-existence study	60

5.49.2	Receiver Characteristics	60

5.49.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	60

5.49.2.1.1	Power class 2 Case A	60

5.49.2.1.2	Power class 2 Case B	61

5.50	DC_12-66_n77	61

5.50.1	Transmitter Characteristics	61

5.50.1.1	Maximum Output Power	61

5.50.1.2	Configurations for EN-DC	61

5.50.1.3	Co-existence study	61

5.50.2	Receiver Characteristics	62

5.50.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	62

5.50.2.1.1	Power class 2 Case A	62

5.50.2.1.2	Power class 2 Case B	62

5.51	DC_14-30_n77	62

5.51.1	Transmitter Characteristics	62

5.51.1.1	Maximum Output Power	62

5.51.1.2	Configurations for EN-DC	62

5.51.1.3	Co-existence study	63

5.51.2	Receiver Characteristics	63

5.51.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	63

5.51.2.1.1	Power class 2 Case A	63

5.51.2.1.2	Power class 2 Case B	63

5.52	DC_14-66_n77	63

5.52.1	Transmitter Characteristics	63

5.52.1.1	Maximum Output Power	63

5.52.1.2	Configurations for EN-DC	64

5.52.1.3	Co-existence study	64

5.52.2	Receiver Characteristics	64

5.52.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	64

5.52.2.1.1	Power class 2 Case A	64

5.52.2.1.2	Power class 2 Case B	64

5.53	DC_29-30_n77	65

5.53.1	Transmitter Characteristics	65

5.53.1.1	Maximum Output Power	65

5.53.1.2	Configurations for EN-DC	65

5.53.1.3	Co-existence study	65

5.53.2	Receiver Characteristics	65

5.53.2.1	Reference sensitivity exceptions due to receiver harmonic mixing for PC2 EN-DC in NR FR1	65

5.53.2.1.1	Power class 2 Case A	65

5.53.2.1.2	Power class 2 Case B	65

5.53.2.2	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	66

5.53.2.2.1	Power class 2 Case A	66

5.53.2.2.2	Power class 2 Case B	66

5.54	DC_29-66_n77	66

5.54.1	Transmitter Characteristics	66

5.54.1.1	Maximum Output Power	66

5.54.1.2	Configurations for EN-DC	67

5.54.1.3	Co-existence study	67

5.54.2	Receiver Characteristics	67

5.54.2.1	Reference sensitivity exceptions due to receiver harmonic mixing for PC2 EN-DC in NR FR1	67

5.54.2.1.1	Power class 2 Case A	67

5.54.2.1.2	Power class 2 Case B	67

5.54.2.2	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	67

5.54.2.2.1	Power class 2 Case A	67

5.54.2.2.2	Power class 2 Case B	68

5.55	DC_30-66_n77	68

5.55.1	Transmitter Characteristics	68

5.55.1.1	Maximum Output Power	68

5.55.1.2	Configurations for EN-DC	68

5.55.1.3	Co-existence study	68

5.55.2	Receiver Characteristics	69

5.55.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands	69

5.55.2.1.1	Power class 2 Case A	69

5.55.2.1.2	Power class 2 Case B	69

6	PC2 MSD analysis for ENDC band combinations	70

6.1	DC_12-30_n77	70

6.2	DC_12-66_n77	70

6.3	DC_14-30_n77	70

6.4	DC_14-66_n77	70

6.5	DC_29-30_n77	71

6.6	DC_29-66_n77	71

6.7	DC_30-66_n77	71

6.8	DC_2-12_n77	71

6.9	DC_2-14_n77	72

6.10	DC_2-30_n77	72

6.11	DC_5-30_n77	72

Annex A - Change history	73

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document is a technical report Power Class 2 for EN-DC with xLTE band + yNR DL with 1LTE+1(TDD) NR UL band (x= 2, 3, 4, y=1; x=1, 2, y=2) under Rel-17 time frame. The purpose is to gather the relevant background information and studies in order to address relevant requirements for the Rel-17 EN-DC band combinations with Power Class 2 requested by proponents and captured in the WID...

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-210816, “New WID on PC2 EN-DC with x LTE band + y NR band (x= 2, 3, 4, y=1; x=1, 2, y=2)”, RAN#91-e

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4	Background

The present document is a technical report for Power Class 2 band combinations under Rel-17 timeframe. The document covers each band combination specific issues (i.e. one sub-clause defined per band combination)

## 4.1	TR maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band combination to ensure that the TPs related to the band combination have been implemented.

# 5	EN-DC Power Class 2: Specific Band Combination Part

## 5.1	DC_2A-5A_n77A

### 5.1.1	Transmitter Characteristics

#### 5.1.1.1	Maximum Output Power

Table 5.1.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_2A_n77A | 266 | +2/-3 |
| DC_5A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

#### 5.1.1.2	Configurations for EN-DC

Table 5.1.1.2-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-5A_n77A DC_2A-2A-5A_n77A DC_2A-5A_n77C DC_2A-2A-5A_n77C | DC_2A_n77A DC_5A_n77A |

#### 5.1.1.3	Co-existence study

According to the PC3 DC_2A-5A_n77A study in 37.717-21-11, the Rx impacts are identified as below,

- 3rd order IMD products generated by DC_5_n77 uplink may fall into own Rx of band 2.

- 5th order IMD products generated by DC_2_n77 uplink may fall into own Rx of band 5.

Thus, additional MSD for IMD 3 and 5 should be considered to mitigate the impact of the interference for PC2 DC_2A-5A_n77A combination.

### 5.1.2	Receiver Characteristics

#### 5.1.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.1.2.1.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 Case A DC_2-5_n77 are defined in table 5.1.2.2.1-1.

Table 5.1.2.1.1-1: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-5A_n77A2 DC_2A-2A-5A_n77A2 DC_2A-5A_n77C2 DC_2A-2A-5A_n77C2 | 2 | 1907.5 | 5 | 25 | 1987.5 | N/A | N/A |
|  | 5 | 842.5 | 5 | 25 | 887.5 | 16.6 | IMD5 |
|  | n77 | 3305 | 5 | 25 | 3305 | N/A | N/A |
|  | 2 | 1907 | 5 | 25 | 1987 | 24.8 | IMD3 |
|  | 5 | 846.5 | 5 | 25 | 891.5 | N/A | N/A |
|  | n77 | 3680 | 5 | 25 | 3680 | N/A | N/A |
| NOTE 2: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction |  |  |  |  |  |  |  |

#### 5.1.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B DC_2-5_n77 are the same as the Case A defined in table 5.1.2.1.1-1.

## 5.2	DC_2-13_n77

### 5.2.1	Transmitter Characteristics

#### 5.2.1.1	Maximum Output Power

Table 5.2.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_2A_n77A | 266 | +2/-3 |
| DC_13A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

#### 5.2.1.2		Configurations for EN-DC

Table 5.32.2-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-13A_n77A DC_2A-2A-13A_n77A DC_2A-13A_n77C DC_2A-2A-13A_n77C | DC_2A_n77A DC_13A_n77A |

#### 5.2.1.3	Co-existence study

According to the PC3 DC_2A-13A_n77A study in 37.717-21-11, the Rx impacts are identified as below,

- No IMD issue for Rx of band 13 with UL DC_2_n77.

- 3rd order IMD products generated by DC_13_n77 uplink may fall into own Rx of band 2

Thus, additional MSD for IMD 3 should be considered to mitigate the impact of the interference for PC2 DC_2A-13A_n77A combination.

### 5.2.2	Receiver Characteristics

#### 5.2.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.2.2.1.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 Case A DC_2-13_n77 are defined in table 5.2.2.2.1-1.

Table 5.2.2.1.1-1: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-13A_n77A DC_2A-2A-13A_n77A DC_2A-13A_n77C DC_2A-2A-13A_n77C | 2 | 1864 | 5 | 25 | 1944 | 24.2 | IMD3 |
|  | 13 | 783 | 5 | 25 | 752 | N/A | N/A |
|  | n77 | 3510 | 5 | 25 | 3510 | N/A | N/A |

#### 5.2.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B DC_2-13_n77 are the same as the Case A defined in table 5.2.2.1.1-1.

## 5.3	DC_2-66_n77

### 5.3.1	Transmitter Characteristics

#### 5.3.1.1	Maximum Output Power

Table 5.3.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_2A_n77A | 266 | +2/-3 |
| DC_66A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

#### 5.3.1.2	Configurations for EN-DC

Table 5.3.1.2-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-66A_n77A DC_2A-2A-66A_n77A DC_2A-66A-66A_n77A DC_2A-2A-66A-66A_n77A DC_2A-66A_n77C DC_2A-2A-66A_n77C DC_2A-66A-66A_n77C DC_2A-2A-66A-66A_n77C | DC_2A_n77A DC_66A_n77A |

#### 5.3.1.3	Co-existence study

According to the PC3 DC_2A-66A_n77A study in 37.717-21-11, the Rx impacts are identified as below,

- 2nd, 4th and 5th order IMD products generated by DC_2_n77 uplink may fall into own Rx of band 66

- 2nd, 4th and 5th order IMD products generated by DC_66_n77 uplink may fall into own Rx of band 2

Thus, additional MSD for IMD 3 and 5 should be considered to mitigate the impact of the interference for PC2 DC_2A-66A_n77A combination.

### 5.3.2	Receiver Characteristics

#### 5.3.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.3.2.1.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 Case A DC_2-66_n77 are defined in table 5.3.2.2.1-1.

Table 5.3.2.1.1-1: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-66A_n77A DC_2A-2A-66A_n77A DC_2A-66A-66A_n77A DC_2A-2A-66A-66A_n77A DC_2A-66A_n77C DC_2A-2A-66A_n77C DC_2A-66A-66A_n77C DC_2A-2A-66A-66A_n77C | 2 | 1855 | 5 | 25 | 1935 | N/A | N/A |
|  | 66 | 1720 | 5 | 25 | 2120 | 34.7 | IMD2 |
|  | n77 | 3975 | 5 | 25 | 3975 | N/A | N/A |
|  | 2 | 1905 | 5 | 25 | 1985 | M/A | N/A |
|  | 66 | 1775 | 5 | 25 | 2175 | 21.1 | IMD44 |
|  | n77 | 3540 | 5 | 25 | 3540 | N/A | N/A |
|  | 2 | 1890 | 5 | 25 | 1970 | 37.6 | IMD2 |
|  | 66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | n77 | 3710 | 5 | 25 | 3710 | N/A | N/A |
|  | 2 | 1860 | 5 | 25 | 1940 | 19.8 | IMD44 |
|  | 66 | 1775 | 5 | 25 | 2195 | N/A | N/A |
|  | n77 | 3385 | 5 | 25 | 3385 | N/A | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction |  |  |  |  |  |  |  |

#### 5.3.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B DC_2-66_n77 are the same as the Case A defined in table 5.3.2.1.1-1.

## 5.4	DC_5-66_n77

### 5.4.1	Transmitter Characteristics

#### 5.4.1.1	Maximum Output Power

Table 5.4.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_5A_n77A | 266 | +2/-3 |
| DC_66A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

#### 5.4.1.2	Configurations for EN-DC

Table 5.4.1.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_5A-66A_n77A DC_5A-66A_n77C DC_5A-66A-66A_n77C | DC_5A_n77A DC_66A_n77A |

#### 5.4.1.3	Co-existence study

According to the PC3 DC_5A-66A_n77A study in 37.717-21-11, the Rx impacts are identified as below,

- The IMD3 of UL DC_5A_n77A may impact to band 66 Rx.

Thus, additional MSD for IMD 3 should be considered to mitigate the impact of the interference for PC2 DC_5A-66A_n77A combination.

### 5.4.2	Receiver Characteristics

#### 5.4.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.4.2.1.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 Case A DC_5-66_n77 are defined in table 5.4.2.2.1-1.

Table 5.4.2.1.1-1: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5A-66A_n77A DC_5A-66A_n77C DC_5A-66A-66A_n77C | 5 | 826.5 | 5 | 25 | 871.5 | N/A | N/A |
|  | 66 | 1742 | 5 | 25 | 2142 | 22.2 | IMD3 |
|  | n77 | 3795 | 10 | 50 | 3795 | N/A | N/A |

#### 5.4.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B DC_5A-66A_n77A are the same as the Case A defined in table 5.4.2.1.1-1.

## 5.5	DC_13-66_n77

### 5.5.1	Transmitter Characteristics

#### 5.5.1.1	Maximum Output Power

Table 5.5.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_13A_n77A | 266 | +2/-3 |
| DC_66A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

#### 5.5.1.2	Configurations for EN-DC

Table 5.21.1.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_13A-66A_n77A DC_13A-66A-66A_n77A DC_13A-66A_n77C DC_13A-66A-66A_n77C | DC_13A_n77A DC_66A_n77A |

#### 5.5.1.3	Co-existence study

According to the PC3 DC_13A-66A_n77A study in 37.717-21-11, the Rx impacts are identified as below,

- 3rd order IMD products generated by DC_13_n77 uplink may fall into own Rx of band 66.

- 3rd order IMD products generated by DC_66_n77 uplink may fall into own Rx of band 13

Thus, additional MSD for IMD 3 should be considered to mitigate the impact of the interference for PC2 DC_13A-66A_n77A combination.

### 5.5.2	Receiver Characteristics

#### 5.5.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.5.2.1.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 Case A DC_13-66_n77 are defined in table 5.5.2.2.1-1.

Table 5.5.2.1.1-1: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_13A-66A_n77A DC_13A-66A-66A_n77A DC_13A-66A_n77C DC_13A-66A-66A_n77C | 13 | 780 | 5 | 25 | 749 | N/A | N/A |
|  | 66 | 1750 | 5 | 25 | 2150 | 25.3 | IMD3 |
|  | n77 | 3710 | 10 | 50 | 3710 | N/A | N/A |
|  | 13 | 781 | 5 | 25 | 750 | 23.4 | IMD32 |
|  | 66 | 1720 | 5 | 25 | 2120 | N/A | N/A |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | N/A |
| NOTE 2: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction |  |  |  |  |  |  |  |

#### 5.5.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B DC_13-66_n77 are the same as the Case A defined in table 5.5.2.1.1-1.

## 5.6	DC_2_n5-n77

### 5.6.1	Transmitter Characteristics

#### 5.6.1.1	Maximum Output Power

Table 5.6.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_2A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

#### 5.6.1.2	Configurations for EN-DC

Table 5.28.1.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A_n5A-n77A DC_2A-2A_n5A-n77A DC_2A_n5A-n77C DC_2A-2A_n5A-n77C | DC_2A_n77A |

#### 5.6.1.3	Co-existence study

According to the PC3 DC_2A_n5A-n77A study in 37.717-11-21, the Rx impacts are identified as below,

- 5th order IMD generated by dual uplink of 2_n77 may fall into own Rx of band n5.

Thus, additional MSD for IMD 3 and 5 should be considered to mitigate the impact of the interference for PC2 DC_2A_n5A-n77A combination.

### 5.6.2	Receiver Characteristics

#### 5.6.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.6.2.1.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 Case A DC_2A_n5A-n77A are defined in table 5.6.2.2.1-1.

Table 5.6.2.1.1-1: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A_n5A-n77A3 | 2 | 1907 | 5 | 25 | 1987 | N/A | N/A |
| DC_2A-2A_n5A-n77A3 | n5 | 844 | 5 | 25 | 889 | 16.6 | IMD5 |
| DC_2A_n5A-n77C3 DC_2A-2A_n5A-n77C3 | n77 | 3305 | 10 | 50 | 3305 | N/A | N/A |
| NOTE 3: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. |  |  |  |  |  |  |  |

#### 5.6.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B DC_2_n5-n77 are the same as the Case A defined in table 5.6.2.1.1-1.

## 5.7	DC_66_n2-n77

### 5.7.1	Transmitter Characteristics

#### 5.7.1.1	Maximum Output Power

Table 5.7.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_66A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

#### 5.7.1.2		Configurations for EN-DC

Table 5.44.2-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_66A_n2A-n77A DC_66A-66A_n2A-n77A DC_66A_n2A-n77C DC_66A-66A_n2A-n77C | DC_66A_n77A |

#### 5.7.1.3	Co-existence study

According to the PC3 DC_66A_n2A-n77A study in 37.717-11-21, the Rx impacts are identified as below,

- 2nd, 4th and 5th order IMD generated by dual uplink of 66_n77 may fall into own Rx of band n2.

Thus, additional MSD for IMD 2, 4 and 5 should be considered to mitigate the impact of the interference for PC2 DC_66A_n2A-n77A combination.

### 5.7.2	Receiver Characteristics

#### 5.7.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.7.2.1.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 Case A DC_66_n2-n77 are defined in table 5.7.2.2.1-1.

Table 5.7.2.1.1-1: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_66A_n2A-n77A DC_66A-66A_n2A-n77A DC_66A_n2A-n77C | n2 | 1900 | 5 | 25 | 1980 | 37.6 | IMD2 |
|  | 66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | 21.1 | IMD42 |
|  | 66 | 1770 | 5 | 25 | 2170 | N/A | N/A |
|  | n77 | 3350 | 10 | 50 | 3350 | N/A | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. |  |  |  |  |  |  |  |

#### 5.7.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B DC_66_n2-n77 are the same as the Case A defined in table 5.7.2.1.1-1.

## 5.8	DC_66_n5-n77

### 5.8.1	Transmitter Characteristics

#### 5.8.1.1	Maximum Output Power

Table 5.8.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_66A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

#### 5.8.1.2	Configurations for EN-DC

Table 5.44.1.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_66A_n5A-n77A DC_66A-66A_n5A-n77A DC_66A_n5A-n77C DC_66A-66A_n5A-n77C | DC_66A_n77A |

#### 5.8.1.3	Co-existence study

According to the PC3 DC_66A_n5A-n77A study in 37.717-11-21, the Rx impacts are identified as below,

- 3rd 4th and 5th order IMD generated by dual uplink of 66_n5 may fall into own Rx of band n77

Thus, additional MSD for IMD 3, 4 and 5 should be considered to mitigate the impact of the interference for PC2 DC_66A_n5A-n77A combination.

### 5.8.2	Receiver Characteristics

#### 5.8.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.8.2.1.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 Case A DC_66A_n5A-n77A are defined in table 5.8.2.2.1-1.

Table 5.8.2.1.1-1: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_66A_n5A-n77A DC_66A-66A_n5A-n77A DC_66A_n5A-n77C DC_66A-66A_n5A-n77C | 66 | 1760 | 5 | 25 | 2160 | N/A | N/A |
|  | n5 | 830 | 5 | 25 | 875 | N/A | N/A |
|  | n77 | 3420 | 10 | 50 | 3420 | 24.9 | IMD3 |
|  | 66 | 1714 | 5 | 25 | 2114 | N/A | N/A |
|  | n5 | 827 | 5 | 25 | 872 | N/A | N/A |
|  | n77 | 4195 | 10 | 50 | 4195 | 24.1 | IMD41 |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

#### 5.8.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B DC_66A_n5A-n77A are the same as the Case A defined in table 5.8.2.1.1-1.

## 5.9	DC_2-13_n66-n77

### 5.9.1	Maximum Output Power

Table 5.9.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_2A_n77A DC_13A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.9.2		Configurations for EN-DC

Table 5.9.2-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-13A_n66A-n77A | DC_2A_n77A DC_13A_n77A |

#### 5.9.3	Co-existence study

MSD have been defined for lower order combinations [2, 3]. No further MSD is needed for both Case A and B.

## 5.10	DC_2-13-66_n77

#### 5.10.1	Maximum Output Power

Table 5.10.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_2A_n77A, DC_13A_n77A, DC_66A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

#### 5.10.2	Configuration for EN-DC

Table 5.10.2-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-13A-66A_n77A DC_2A-13A-66A_n77C DC_2A-2A-13A-66A_n77C DC_2A-13A-66A-66A_n77C | DC_2A_n77A, DC_13A_n77A, DC_66A_n77A |

#### 5.10.3	Co-existence study

MSD have been defined for lower order combinations. No further MSD is needed for both Case A and B.

## 5.11	DC_13-66_n2-n77

### 5.11.1	Maximum Output Power

Table 5.11.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_13A_n77A, DC_66A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.11.2	Configurations for EN-DC

Table 5.11.2-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_13A-66A_n2A-n77A DC_13A-66A-66A_n2A-n77A DC_13A-66A_n2A-n77C | DC_13A_n77A, DC_66A_n77A |

#### 5.11.2	Co-existence study

MSD have been defined for lower order combinations. No further MSD is needed for both Case A and B

## 5.12	DC_2-66_n5-n77

### 5.12.1	Maximum Output Power

Table 5.12.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_66A_n77A, DC_2A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.12.2	Configurations for EN-DC

Table 5.12.2-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-66A_n5A-n77A DC_2A-2A-66A_n5A-n77A DC_2A-66A-66A_n5A-n77A DC_2A-66A_n5A-n77C | DC_66A_n77A, DC_2A_n77A |

#### 5.12.3	Co-existence study

MSD have been defined for lower order combinations. No further MSD is needed for both Case A and B

## 5.13	DC_2-5-66_n77

### 5.13.1	Maximum Output Power

Table 5.13.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_2A_n77A, DC_5A_n77A, DC_66A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.13.2	Configurations for EN-DC

Table 5.13.2-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-5A-66A_n77A DC_2A-5A-66A_n77C DC_2A-2A-5A-66A_n77C DC_2A-5A-66A-66A_n77C | DC_2A_n77A, DC_5A_n77A, DC_66A_n77A |

#### 5.13.3	Co-existence study

MSD have been defined for lower order combinations [2]. No further MSD is needed for both Case A and B.

## 5.14	DC_13_n66-n77

### 5.14.1	Transmitter Characteristics

#### 5.14.1.1	Maximum Output Power

Table 5.14.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_13A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.14.1.2	Configurations for EN-DC

Table 5.14.1.2-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_13A_n66A-n77A DC_13A_n66A-n77C | DC_13A_n77A |

#### 5.14.1.3	Co-existence study

According to the both PC3 DC_13A_n66A-n77A study in 37.717-11-21 and only DC_13A_n77A uplink configuration applied in this band combination, additional MSD due to IMD 3 to the band n66 from the configured uplink EN-DC should be considered to mitigate the impact of the interference to the PC2 DC_13A_n66A-n77A combination.

IMD generated by DC_13A_n77A that falls into the third band, n66

### 5.14.2	Receiver Characteristics

#### 5.14.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.14.2.1.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 Case A of DC_13A_n66A-n77A are defined in table 5.14.2.1.1-1.

Table 5.14.2.1.1-1: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_13A_n66A-n77A DC_13A_n66A-n77C | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n66 | 1756 | 5 | 25 | 2156 | 26.1 | IMD3 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |

#### 5.14.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B of DC_13_n66-n77 are the same as the Case A defined in table 5.14.2.1.1-1.

## 5.15	DC_13_n2-n77

### 5.15.1	Transmitter Characteristics

#### 5.15.1.1	Maximum Output Power

Table 5.15.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_13A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.15.1.2	Configurations for EN-DC

Table 5.15.1.2-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_13A_n2A-n77A DC_13A_n2A-n77C | DC_13A_n77A |

#### 5.15.1.3	Co-existence study

According to the both PC3 DC_13A_n2A-n77A study in 37.717-11-21 and only DC_13A_n77A uplink configuration applied in this band combination, additional MSD due to IMD 3 to the band n2 from the configured uplink EN-DC should be considered to mitigate the impact of the interference to the PC2 DC_13A_n66A-n77A combination.

### 5.15.2	Receiver Characteristics

#### 5.15.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.15.2.1.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 Case A of DC_13_n2-n77 are defined in table 5.15.2.2.1-1.

Table 5.15.2.1.1-1: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_13A_n2A-n77A DC_13A_n2A-n77C | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | 25.0 | IMD3 |
|  | n77 | 3524 | 10 | 50 | 3524 | N/A | N/A |

#### 5.15.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B of DC_13_n2-n77 are the same as the Case A defined in table 5.15.2.1.1-1.

## 5.16	DC_2A-66A_n41A

### 5.16.1	Transmitter Characteristics

#### 5.16.1.1	Maximum Output Power

Table 5.16.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_66A_n41A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

#### 5.16.1.2	Co-existence study

According to the PC3 DC_2A_66A-n41A study in 37.716-21-11, the Rx impacts are identified as below,

- Co-existence analysis for DC_2_n41 shows that there is no impact from DC_2_n41 UL to Band 66 DL.

- Co-existence analysis for DC_66_n41 shows that there is IMD4 impact from DC_66_n41 UL to Band 2 DL.

Thus, additional MSD for IMD 4 should be considered to mitigate the impact of the interference for PC2 DC_2A_66A-n41A combination.

### 5.16.2	Receiver Characteristics

#### 5.16.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.16.2.1.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 Case A DC_2A_66A-n41A are defined in table 5.16.2.2.1-1.

Table 5.16.2.1.1-1: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A-66A_n41A | 2 | 1860 | 5 | 25 | 1940 | 22.6 | IMD4 |
|  | 66 | 1715 | 5 | 25 | 2115 | N/A | N/A |
|  | n41 | 2685 | 5 | 25 | 2685 | N/A | N/A |

#### 5.16.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B DC_2A_66A-n41A are the same as the Case A defined in table 5.16.2.1.1-1.

## 5.17	DC_66_n66-n77

### 5.17.1	Transmitter Characteristics

#### 5.17.1.1	Maximum Output Power

Table 5.17.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_66A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.17.1.2	Configurations for EN-DC

Table 5.17.1.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_66A_n66A-n77A DC_66A_n66A-n77C | DC_66A_n77A |

#### 5.17.1.3	Co-existence study

According to the PC2 coexistence studies performed in the lower order combinations, the Rx impacts are identified as below,

- For UL DC_66A_n77A configuration, IMD2 and IMD5 products fall into the band n66 Rx

Thus additional MSD exception should be considered to mitigate the impact of the interference.

### 5.17.2	Receiver Characteristics

#### 5.17.2.1		MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.17.2.1.1	Power class 2 Case A

Based on co-existence study, additional MSD are specified Table 5.17.2.1.1-1 for this dual connectivity configuration.

Table 5.17.2.1.1-1: MSD test points for SCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_66A_n66A-n77A DC_66A_n66A-n77C | 66 | 1750 | 5 | 25 | 2150 | N/A | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | 37 | IMD2 |
|  | n77 | 3900 | 10 | 50 | 3900 | N/A | N/A |
|  | 66 | 1755 | 5 | 25 | 2175 | N/A | N/A |
|  | n66 | 1755 | 5 | 25 | 2175 | 20 | IMD5 |
|  | n77 | 3750 | 10 | 50 | 3750 | N/A | N/A |

#### 5.17.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B configuration are same as the Case A defined in table 5.17.2.1.1-1.

## 5.18	DC_48-66_n77

### 5.18.1	Transmitter Characteristics

#### 5.18.1.1	Maximum Output Power

Table 5.18.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_66A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.18.1.2	Configurations for EN-DC

Table 5.18.1.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_48A-66A_n77A DC_48A-66A_n77C DC_48C-66A_n77A DC_48C-66A_n77C DC_48D-66A_n77A DC_48D-66A_n77C | DC_66A_n77A |

#### 5.18.1.3	Co-existence study

According to coxeistence studies for the PC3 DC_48A-66A_n77A in TR 37.717-21-11, no harmonics and intermodulation products is present as the duplex mode for both band 48 and n77 is TDD and opertion in fully synchronizing in US.

### 5.18.2	Receiver Characteristics

#### 5.18.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

There is no additional MSD requirement for this PC2 band combination.

## 5.19	DC_13_n5-n77

### 5.19.1	Transmitter Characteristics

#### 5.19.1.1	Maximum Output Power

Table 5.19.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_13A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.19.1.2	Configurations for EN-DC

Table 5.19.1.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_13A_n5A-n77A DC_13A_n5A-n77C | DC_13A_n77A |

#### 5.19.1.3	Co-existence study

According to the PC3 DC_13A_n77A study in TR 37.717-21-11, the Rx impacts are identified as below,

- For UL DC_13A_n77A, IMD5 of the 2UL falls into band n5 Rx

Thus additional MSD should be considered to mitigate the impact of the interference.

### 5.19.2	Receiver Characteristics

#### 5.19.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.19.2.1.1	Power class 2 Case A

Based on co-existence study, additional MSD are specified Table 5.19.2.1.1-1 for this dual connectivity configuration.

Table 5.19.2.1.1-1: MSD test points for SCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_13A_n5A-n77A2 DC_13A_n5A-n77C2 | n5 | 840 | 5 | 25 | 885 | 19.5 | IMD5 |
|  | 13 | 782 | 5 | 20 | 751 | N/A | N/A |
|  | n77 | 4013 | 10 | 50 | 4013 | N/A | N/A |
| NOTE 2: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. |  |  |  |  |  |  |  |

#### 5.19.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B configuratoin are same as the Case A defined in table 5.19.2.1.1-1.

## 5.20	DC_5_n66-n77

### 5.20.1	Transmitter Characteristics

#### 5.20.1.1	Maximum Output Power

Table 5.20.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_5A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.20.1.2	Configurations for EN-DC

Table 5.20.1.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_5A_n66A-n77A DC_5A_n66A-n77C | DC_5A_n77A |

#### 5.20.1.3	Co-existence study

For UL DC_5A_n77A, IMD3 generated by dual uplink of 5_n77 may fall into own Rx of band n66.

Thus additional MSD exception should be considered to mitigate the impact of the interference.

### 5.20.2	Receiver Characteristics

#### 5.20.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.20.2.1.1	Power class 2 Case A

Based on co-existence study, additional MSD are specified Table 5.20.2.1.1-1 for this dual connectivity configuration.

Table 5.20.2.1.1-1: MSD test points for SCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5A_n66A-n77A DC_5A_n66A-n77C | 5 | 826.5 | 5 | 25 | 871.5 | N/A | N/A |
|  | n66 | 1742 | 5 | 25 | 2142 | 22.2 | IMD3 |
|  | n77 | 3795 | 10 | 50 | 3795 | N/A | N/A |

#### 5.20.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B configuration are same as the Case A defined in table 5.20.2.1.1-1.

## 5.21	DC_5A_n5A-n77A

### 5.21.1	Transmitter Characteristics

#### 5.21.1.1	Maximum Output Power

Table 5.21.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_5A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.21.1.2	Configurations for EN-DC

Table 5.21.1.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_5A_n5A-n77A DC_5A_n5A-n77C | DC_5A_n77A |

#### 5.21.1.3	Co-existence study

According to the co-existence studies performed in the lower order combinations, the Rx impacts are identified as below,

- For UL DC_5A_n77A configuration, IMD4 and IMD5 products fall into the band n5 Rx

Thus additional MSD should be considered to mitigate the impact of the interference

### 5.21.2	Receiver Characteristics

#### 5.21.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.21.2.1.1	Power class 2 Case A

Based on co-existence study, additional MSD are specified Table 5.21.2.1.1-1 for this dual connectivity configuration.

Table 5.21.2.1.1-1: MSD test points for SCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR EN-DC Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD for PC2 (dB) | Duplex mode | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_5A_n5A-n77A2 DC_5A_n5A-n77C2 | 5 | 844 | 5 | 25 | 889 | N/A | FDD | N/A |
|  | n5 | 844 | 5 | 25 | 889 | 20.3 | FDD | IMD41 |
|  | n77 | 3421 | 10 | 50 | 3421 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. |  |  |  |  |  |  |  |  |

#### 5.21.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B configuration are same as the Case A defined in table 5.21.2.1.1-1.

## 5.22	DC_5_n2-n77

### 5.22.1	Transmitter Characteristics

#### 5.22.1.1	Maximum Output Power

Table 5.22.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_5A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.22.1.2	Configurations for EN-DC

Table 5.22.1.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_5A_n2A-n77A DC_5A_n2A-n77C | DC_5A_n77A |

#### 5.22.1.3	Co-existence study

According to coexistence studies performed in the PC3 DC_5A_n2A-n77A configuration, the Rx impacts are identified as below

- For UL DC_5A_n77A, IMD3 of 2UL may fall into own Rx of band n2.

Thus additional MSD should be considered to mitigate the impact of the interference for DC_5A_n2A-n77A combination.

### 5.22.2	Receiver Characteristics

#### 5.22.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.22.2.1.1	Power class 2 Case A

Based on co-existence study, additional MSD are specified Table 5.22.2.1.1-1 for this dual connectivity configuration.

Table 5.22.2.1.1-1: MSD test points for SCell due to dual uplink operation for PC2 EN-DC in NR FR1

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5A_n2A-n77A 2 DC_5A_n2A-n77C2 | n2 | 1907 | 5 | 25 | 1987 | 25.5 | IMD3 |
|  | 5 | 846.5 | 5 | 25 | 891.5 | N/A | N/A |
|  | n77 | 3680 | 5 | 25 | 3680 | N/A | N/A |
| NOTE 2: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. |  |  |  |  |  |  |  |

#### 5.22.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B configuratoin are same as the Case A defined in table 5.22.2.1.1-1.

## 5.23	DC_2_n66-n77

### 5.23.1	Transmitter Characteristics

#### 5.23.1.1	Maximum Output Power

Table 5.23.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_2A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.23.1.2	Configurations for EN-DC

Table 5.23.1.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A_n66A-n77A DC_2A-2A_n66A-n77A DC_2A_n66A-n77C DC_2A-2A_n66A-n77C | DC_2A_n77A |

#### 5.23.1.3	Co-existence study

Based on co-existence specified in 38.101-3 for the configuration,

- For UL DC_2A_n77A configuration, IMD2, IMD4 and IMD5 fall into own band n66 Rx.

Thus additional MSD should be considered to mitigate the impact of the interference.

### 5.23.2	Receiver Characteristics

#### 5.23.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.23.2.1.1	Power class 2 Case A

Based on co-existence study, additional MSD are specified Table 5.23.2.1.1-1 for this dual connectivity configuration.

Table 5.23.2.1.1-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A_n66A-n77A DC_2A-2A_n66A-n77A DC_2A_n66A-n77C DC_2A-2A_n66A-n77C | 2 | 1855 | 5 | 25 | 1935 | N/A | N/A |
|  | n66 | 1715 | 5 | 25 | 2115 | 35.2 | IMD2 |
|  | n77 | 3970 | 10 | 50 | 3970 | N/A | N/A |
|  | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 22.3 | IMD43 |
|  | n77 | 3540 | 10 | 50 | 3540 | N/A | N/A |
| NOTE 3: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |

#### 5.23.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B configuratoin are same as the Case A defined in table 5.23.2.1.1-1.

## 5.24	DC_2-48_n77

### 5.24.1	Transmitter Characteristics

#### 5.24.1.1	Maximum Output Power

Table 5.24.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_2A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.24.1.2	Configurations for EN-DC

Table 5.24.1.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-48A_n77A DC_2A-48C_n77A DC_2A-48D_n77A DC_2A-48A_n77C DC_2A-48C_n77C DC_2A-48D_n77C | DC_2A_n77A |

#### 5.24.1.3	Co-existence study

According to the co-existence studies performed, no IMD issue is identified for this configuration.

### 5.24.2	Receiver Characteristics

#### 5.24.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

There is no additional MSD requirement for this PC2 band combination.

## 5.25	DC_2A_n2A-n77A

### 5.25.1	Transmitter Characteristics

#### 5.25.1.1	Maximum Output Power

Table 5.25.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_2A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.25.1.2	Configurations for EN-DC

Table 5.25.1.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A_n2A-n77A DC_2A_n2A-n77C | DC_2A_n77A |

#### 5.25.1.3	Co-existence study

According to the PC2 coexistence studies performed in the lower order combinations, the Rx impacts are identified as below,

- For UL DC_2A_n77A, IMD2, IMD4 and IMD5 products fall into the band n2 Rx

Thus additional MSD should be considered to mitigate the impact of the interference.

### 5.25.2	Receiver Characteristics

#### 5.25.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.25.2.1.1	Power class 2 Case A

Based on co-existence study, additional MSD are specified Table 5.25.2.1.1-1 for this dual connectivity configuration.

Table 5.25.2.1.1-1: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD for PC2 | Duplex mode | Source of IMD |
| Configuration |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| DC_2A_n2A-n77A DC_2A_n2A-n77C | 2 | 1855 | 5 | 25 | 1935 | N/A | FDD | N/A |
|  | n2 | 1855 | 5 | 25 | 1935 | 32.0 | FDD | IMD2 |
|  |  |  |  |  |  | 34.7 4 |  |  |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
|  | 2 | 18 95 | 5 | 25 | 19 75 | N/A | FDD | N/A |
|  | n2 | 18 95 | 5 | 25 | 19 75 | 20.0 | FDD | IMD44 |
|  |  |  |  |  |  | 22.75 |  |  |
|  | n77 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 4: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

#### 5.25.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B configuration are same as the Case A defined in table 5.25.2.1.1-1.

## 5.26	DC_1A-5A_n78A

### 5.26.1	Transmitter Characteristics

#### 5.26.1.1	Maximum Output Power

Table 5.26.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_aA_n78A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

#### 5.26.1.2	Co-existence study

According to the PC3 DC_1A_5A-n78A study in 37.863-02-01, the Rx impacts are identified as below,

- Co-existence analysis for DC_1_n78 shows that there is IMD4 impact from DC_1_n78 UL to Band 1 and IMD5 impact from DC_1_n78 UL to Band 5 DL.

- Co-existence analysis for DC_5_n78 shows that there is IMD4 impact from DC_5_n78 UL to Band 5 DL and IMD3 impact from DC_5_n78 UL to Band 1 DL.

Thus, additional MSD for IMD 4 should be considered to mitigate the impact of the interference for PC2 DC_1A_5A-n78A combination.

### 5.26.2	Receiver Characteristics

#### 5.26.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.26.2.1.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 Case A DC_1A_5A-n78A are defined in table 5.26.2.2.1-1.

Table 5.26.2.1.1-1: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LLRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-5A_n78A | 1 | 1930 | 5 | 25 | 2120 | 19.2 | IMD4 |
|  | 5 | 844 | 5 | 25 | 889 | N/A | N/A |
|  | n78 | 3670 | 10 | 52 | 3670 | N/A | N/A |
| DC_1A-5A_n78A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 5 | 844 | 5 | 25 | 889 | 19.2 | IMD4 |
|  | n78 | 3421 | 10 | 52 | 3421 | N/A | N/A |
| DC_1A-5A_n78A | 1 | 1932 | 5 | 25 | 2122 | 27.0 | IMD3 |
|  | 5 | 829 | 5 | 25 | 874 | N/A | N/A |
|  | n78 | 3780 | 10 | 52 | 3780 | N/A | N/A |
| DC_1A-5A_n78A | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | 5 | 840 | 5 | 25 | 885 | 13.2 | IMD5 |
|  | n78 | 3405 | 10 | 52 | 3405 | N/A | N/A |

#### 5.26.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B DC_1A_5A-n78A are the same as the Case A defined in table 5.26.2.1.1-1.

## 5.27	DC_1A-n7A_n78A

### 5.27.1	Transmitter Characteristics

#### 5.27.1.1	Maximum Output Power

Table 5.27.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_aA_n78A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

#### 5.27.1.2	Co-existence study

According to the PC3 DC_1A_7A-n78A study in 37.863-02-01, the Rx impacts are identified as below,

- Co-existence analysis for DC_1_n78 shows that there is IMD4 impact from DC_1_n78 UL to Band 1 and Band 7 DL.

- Co-existence analysis for DC_7_n78 shows that there is IMD4 impact from DC_7_n78 UL to Band 1 DL.

Thus, additional MSD for IMD 4 should be considered to mitigate the impact of the interference for PC2 DC_1A_7A-n78A combination.

### 5.27.2	Receiver Characteristics

#### 5.27.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.27.2.1.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 Case A DC_1A_7A-n78A are defined in table 5.27.2.2.1-1.

Table 5.27.2.1.1-1: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LLRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-7A_n78A | 1 | 1930 | 5 | 25 | 2120 | 19.2 | IMD4 |
|  | 7 | 2550 | 5 | 25 | 2670 | N/A | N/A |
|  | n78 | 3670 | 10 | 52 | 3670 | N/A | N/A |
| DC_1A-7A_n78A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | 7 | 2507.5 | 5 | 25 | 2627.5 | 20.2 | IMD4 |
|  | n78 | 3305 | 10 | 52 | 3305 | N/A | N/A |
| DC_1A-7A_n78A | 1 | 1950 | 5 | 25 | 2140 | 19.7 | IMD4 |
|  | 7 | 2510 | 10 | 50 | 2630 | N/A | N/A |
|  | n78 | 3580 | 10 | 52 | 3580 | N/A | N/A |

#### 5.27.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B DC_1A_7A-n78A are the same as the Case A defined in table 5.27.2.1.1-1.

## 5.28	DC_2-5_n5-n77

### 5.28.1	Maximum Output Power

Table 5.28.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_2A_n77A | 266 | +2/-3 |
| DC_5A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.28.2	Configurations for EN-DC

Table 5.28.2-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-5A_n5A-n77A DC_2A-5A_n5A-n77C | DC_2A_n77A DC_5A_n77A |

#### 5.28.3	Co-existence study

MSD have been defined for lower order combinations. No further MSD is needed for both Case A and B

## 5.29	DC_2-5-48_n77

### 5.29.1	Maximum Output Power

Table 5.29.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_2A_n77A | 266 | +2/-3 |
| DC_5A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.29.2		Configurations for EN-DC

Table 5.29.2-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-5A-48A_n77A DC_2A-5A-48C_n77A DC_2A-5A-48C_n77C | DC_2A_n77A DC_5A_n77A |

#### 5.29.3	Co-existence study

The related MSD requirements have been studied in the lower order combinations. There is no additional requirement for this PC2 band combination.

## 5.30	DC_2-5-66_n5-n77

### 5.30.1	Maximum Output Power

Table 5.30.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_2A_n77A | 266 | +2/-3 |
| DC_5A_n77A | 266 | +2/-3 |
| DC_66A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.30.2		Configurations for EN-DC

Table 5.30.2-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-5A-66A_n5A-n77A DC_2A-5A-66A-66A_n5A-n77A | DC_2A_n77A DC_5A_n77A DC_66A_n77A |

#### 5.30.3	Co-existence study

The related MSD requirements have been studied in the lower order combinations. There is no additional requirement for this PC2 band combination.

## 5.30	DC_2-13_n2-n77

### 5.30.1	Maximum Output Power

Table 5.30.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_2A_n77A | 266 | +2/-3 |
| DC_13A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.30.2		Configurations for EN-DC

Table 5.30.2-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-13A_n2A-n77A DC_2A-13A_n2A-n77C | DC_2A_n77A DC_13A_n77A |

#### 5.30.3	Co-existence study

The related MSD requirements have been studied in the lower order combinations. There is no additional requirement for this PC2 band combination.

## 5.31	DC_2-13_n5-n77

### 5.31.1	Maximum Output Power

Table 5.31.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_2A_n77A | 266 | +2/-3 |
| DC_13A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.31.2		Configurations for EN-DC

Table 5.31.2-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-13A_n5A-n77A DC_2A-2A-13A_n5A-n77A DC_2A-13A_n5A-n77C | DC_2A_n77A DC_13A_n77A |

#### 5.31.3	Co-existence study

The related MSD requirements have been studied in the lower order combinations. There is no additional requirement for this PC2 band combination.

## 5.32	DC_2-13-48_n77

### 5.32.1	Maximum Output Power

Table 5.32.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_2A_n77A | 266 | +2/-3 |
| DC_13A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.32.2		Configurations for EN-DC

Table 5.32.2-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-13A-48A_n77A DC_2A-13A-48A_n77C DC_2A-13A-48C_n77A DC_2A-13A-48C_n77C | DC_2A_n77A DC_13A_n77A |

#### 5.32.3	Co-existence study

The related MSD requirements have been studied in the lower order combinations. There is no additional requirement for this PC2 band combination.

## 5.33	DC_2-13-66_n2-n77

### 5.33.1	Maximum Output Power

Table 5.33.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_2A_n77A | 266 | +2/-3 |
| DC_13A_n77A | 266 | +2/-3 |
| DC_66A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.33.2		Configurations for EN-DC

Table 5.33.2-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-13A-66A_n2A-n77A DC_2A-13A-66A-66A_n2A-n77A | DC_2A_n77A DC_13A_n77A DC_66A_n77A |

#### 5.33.3	Co-existence study

The related MSD requirements have been studied in the lower order combinations. There is no additional requirement for this PC2 band combination.

## 5.34	DC_2-13-66_n5-n77

### 5.34.1	Maximum Output Power

Table 5.34.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_2A_n77A | 266 | +2/-3 |
| DC_13A_n77A | 266 | +2/-3 |
| DC_66A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.34.2		Configurations for EN-DC

Table 5.34.2-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-13A-66A_n5A-n77A DC_2A-2A-13A-66A_n5A-n77A DC_2A-13A-66A-66A_n5A-n77A | DC_2A_n77A DC_13A_n77A DC_66A_n77A |

#### 5.34.3	Co-existence study

The related MSD requirements have been studied in the lower order combinations. There is no additional requirement for this PC2 band combination.

## 5.35	DC_2-48-66_n77

### 5.35.1	Maximum Output Power

Table 5.35.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_2A_n77A | 266 | +2/-3 |
| DC_66A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.35.2		Configurations for EN-DC

Table 5.35.2-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-48A-66A_n77A DC_2A-48A-66A_n77C DC_2A-48C-66A_n77A DC_2A-48C-66A_n77C | DC_2A_n77A DC_66A_n77A |

#### 5.35.3	Co-existence study

The related MSD requirements have been studied in the lower order combinations. There is no additional requirement for this PC2 band combination.

## 5.36	DC_2-66_n2-n77

### 5.36.1	Maximum Output Power

Table 5.36.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_2A_n77A | 266 | +2/-3 |
| DC_66A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.36.2		Configurations for EN-DC

Table 5.36.2-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-66A_n2A-n77A DC_2A-66A-66A_n2A-n77A DC_2A-66A_n2A-n77C | DC_2A_n77A DC_66A_n77A |

#### 5.36.3	Co-existence study

The related MSD requirements have been studied in the lower order combinations. There is no additional requirement for this PC2 band combination.

## 5.37	DC_5-13_n77

### 5.37.1	Transmitter Characteristics

#### 5.37.1.1	Maximum Output Power

Table 5.37.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_5A_n77A | 266 | +2/-3 |
| DC_13A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.37.1.2	Configurations for EN-DC

Table 5.37.1.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_5A-13A_n77A DC_5A-13A_n77C | DC_5A_n77A DC_13A_n77A |

#### 5.37.1.3	Co-existence study

According to the PC3 DC_5A_n77A and DC_13A_n77A study in TR 37.717-21-11, the Rx impacts are identified as below,

- For UL DC_5A_n77A, IMD5 of the 2UL may fall into band 13 Rx

- For UL DC_13A_n77A, IMD5 of the 2UL may fall into band 5 Rx

Thus additional MSD should be considered to mitigate the impact of the interference.

### 5.37.2	Receiver Characteristics

#### 5.37.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.37.2.1.1	Power class 2 Case A

Based on co-existence study, additional MSD are specified Table 5.37.2.1.1-1 for this dual connectivity configuration.

Table 5.37.2.1.1-1: MSD test points for SCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5A-13A_n77A2 DC_5A-13A_n77C2 | 5 | 840 | 5 | 25 | 885 | N/A | N/A |
|  | 13 | 781 | 5 | 20 | 750 | 19.4 | IMD5 |
|  | n77 | 4110 | 10 | 50 | 4110 | N/A | N/A |
|  | 5 | 840 | 5 | 25 | 885 | 19.5 | IMD5 |
|  | 13 | 782 | 5 | 20 | 751 | N/A | N/A |
|  | n77 | 4013 | 10 | 50 | 4013 | N/A | N/A |
| NOTE 2: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. |  |  |  |  |  |  |  |

#### 5.37.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B configuratoin are same as the Case A defined in table 5.37.2.1.1-1.

## 5.38	DC_5-48_n77

### 5.38.1	Transmitter Characteristics

#### 5.38.1.1	Maximum Output Power

Table 5.38.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_5A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.38.1.2	Configurations for EN-DC

Table 5.38.1.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_5A-48A_n77A DC_5A-48C_n77A DC_5A-48D_n77A DC_5A-48A_n77C DC_5A-48C_n77C DC_5A-48D_n77C | DC_5A_n77A |

#### 5.38.1.3	Co-existence study

According to the co-existence studies performed, no IMD issue is identified for this configuration.

### 5.38.2	Receiver Characteristics

#### 5.38.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

There is no additional MSD requirement for this PC2 band combination

## 5.39	DC_5-48-66_n77

### 5.39.1	Maximum Output Power

Table 5.39.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_5A_n77A | 266 | +2/-3 |
| DC_66A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.39.2		Configurations for EN-DC

Table 5.39.2-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_5A-48A-66A_n77A DC_5A-48C-66A_n77A DC_5A-48C-66A_n77C | DC_5A_n77A DC_66A_n77A |

#### 5.39.3	Co-existence study

The related MSD requirements have been studied in the lower order combinations. There is no additional requirement for this PC2 band combination.

## 5.40	DC_5-66_n5-n77

### 5.40.1	Maximum Output Power

Table 5.40.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_5A_n77A | 266 | +2/-3 |
| DC_66A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.40.2	Configurations for EN-DC

Table 5.40.2-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_5A-66A_n5A-n77A DC_5A-66A-66A_n5A-n77A DC_5A-66A_n5A-n77C | DC_5A_n77A, DC_66A_n77A |

#### 5.40.3	Co-existence study

MSD have been defined for lower order combinations. No further MSD is needed for both Case A and B

## 5.41	DC_13-48_n77

### 5.41.1	Transmitter Characteristics

#### 5.41.1.1	Maximum Output Power

Table 5.41.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_13A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.41.1.2	Configurations for EN-DC

Table 5.41.1.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_13A-48A_n77A DC_13A-48A_n77C DC_13A-48C_n77A DC_13A-48C_n77C DC_13A-48D_n77A DC_13A-48D_n77C | DC_13A_n77A |

#### 5.41.1.3	Co-existence study

According to the co-existence studies performed in TR 37.717-21-11, no IMD issue is identified for this configuration.

### 5.41.2	Receiver Characteristics

#### 5.41.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

There is no additional MSD requirement for this PC2 band combination

## 5.42	DC_13-48-66_n77

### 5.42.1	Maximum Output Power

Table 5.42.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_13A_n77A | 266 | +2/-3 |
| DC_66A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.42.2		Configurations for EN-DC

Table 5.42.2-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_13A-48A-66A_n77A DC_13A-48C-66A_n77A DC_13A-48A-66A_n77C DC_13A-48C-66A_n77C | DC_13A_n77A DC_66A_n77A |

#### 5.42.3	Co-existence study

The related MSD requirements have been studied in the lower order combinations. There is no additional requirement for this PC2 band combination.

## 5.43	DC_13-66_n5-n77

### 5.43.1	Maximum Output Power

Table 5.43.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_13A_n77A | 266 | +2/-3 |
| DC_66A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.43.2		Configurations for EN-DC

Table 5.43.2-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_13A-66A_n5A-n77A DC_13A-66A-66A_n5A-n77A DC_13A-66A_n5A-n77C DC_13A-66A-66A_n5A-n77C | DC_13A_n77A DC_66A_n77A |

#### 5.43.3	Co-existence study

The related MSD requirements have been studied in the lower order combinations. There is no additional requirement for this PC2 band combination.

## 5.44	DC_2-12_n77

### 5.44.1	Transmitter Characteristics

#### 5.44.1.1	Maximum Output Power

Table 5.44.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_2A_n77A | 266 | +2/-31 |
| DC_12A_n77A | 266 | +2/-31 |
| NOTE 1: For the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

#### 5.44.1.2	Configurations for EN-DC

Table 5.44.1.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-12A_n77A | DC_2A_n77A DC_12A_n77A |

#### 5.44.1.3	Co-existence study

Co-existence studies of this 3DL/2UL DC configuration for PC3 are already covered in TR 37.717-21-11. It can be seen that:

-	There are no IMD products produced by Band 2 and n77 that might fall in Rx of band 12.

-	IMD3 and IMD4 products are produced by Band 12 and n77 that might fall in Rx of band 2.

### 5.44.2	Receiver Characteristics

#### 5.44.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.44.2.1.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 Case A DC_2A-12A_n77A are defined in Table 5.44.2.1.1-1.

Table 5.44.2.1.1-1: MSD test points for SCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-12A_n77A | 2 | 1880 | 5 | 25 | 1960 | 24.8 | IMD32,zz |
|  | 12 | 707.5 | 5 | 25 | 737.5 | N/A | N/A |
|  | n77 | 3375 | 10 | 50 | 3375 | N/A | N/A |
| NOTE 2: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. NOTE ZZ: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |

#### 5.44.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B DC_2A-12A_n77A are the same as the Case A defined in Table 5.44.2.1.1-1.

## 5.45	DC_2-14_n77

### 5.45.1	Transmitter Characteristics

#### 5.45.1.1	Maximum Output Power

Table 5.45.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_2A_n77A | 266 | +2/-31 |
| DC_14A_n77A | 266 | +2/-3 |
| NOTE 1: For the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

#### 5.45.1.2	Configurations for EN-DC

Table 5.45.1.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-14A_n77A | DC_2A_n77A DC_14A_n77A |

#### 5.45.1.3	Co-existence study

Co-existence studies of this 3DL/2UL DC configuration for PC3 are already covered in TR 37.717-21-11. It can be seen that:

-	There are no IMD products produced by Band 2 and n77 that might fall in Rx of band 14.

-	IMD3 products are produced by Band 14 and n77 that might fall in Rx of band 2.

### 5.45.2	Receiver Characteristics

#### 5.45.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.45.2.1.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 Case A DC_2A-14A_n77A are defined in Table 5.45.2.1.1-1.

Table 5.45.2.1.1-1: MSD test points for SCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-14A_n77A | 2 | 1874 | 5 | 25 | 1954 | 24.8 | IMD3 |
|  | 14 | 793 | 5 | 25 | 763 | N/A | N/A |
|  | n77 | 3540 | 10 | 50 | 3540 | N/A | N/A |

#### 5.45.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B DC_2A-14A_n77A are the same as the Case A defined in Table 5.45.2.1.1-1.

## 5.46	DC_2-29_n77

### 5.46.1	Transmitter Characteristics

#### 5.46.1.1	Maximum Output Power

Table 5.46.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_2A_n77A | 266 | +2/-31 |
| NOTE 1: For the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

#### 5.46.1.2	Configurations for EN-DC

Table 5.46.1.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-29A_n77A | DC_2A_n77A |

#### 5.46.1.3	Co-existence study

Co-existence studies of this 3DL/2UL DC configuration for PC3 are already covered in TR 37.717-21-11. It can be seen that:

-	There are no IMD products produced by Band 2 and n77 that might fall in Rx of band 29.

Although DC_29_n77 is not defined, 5th order harmonic mixing is produced from band n77 UL that might fall in Rx of band 29.

### 5.46.2	Receiver Characteristics

#### 5.46.2.1	Reference sensitivity exceptions due to receiver harmonic mixing for PC2 EN-DC in NR FR1

#### 5.46.2.1.1	Power class 2 Case A

The MSD due to receiver harmonic mixing for PC2 Case A are same as PC3 DC_29_n77.

#### 5.46.2.1.2	Power class 2 Case B

The additional MSD due to receiver harmonic mixing for PC2 Case B are defined in Table 5.46.2.1.2-1. For harmonic mixing, the n77 power has increased by 3dB which results in 3dB more power down-converted at the 5th RX LO harmonic.

Table 5.46.2.1.2-1: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for PC2 EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n77zz | 291 | 34 | 31 |  |  |  |  |  |  |  |  |  |
| NOTE 1: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE ZZ: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. |  |  |  |  |  |  |  |  |  |  |  |  |

## 5.47	DC_2-30_n77

### 5.47.1	Transmitter Characteristics

#### 5.47.1.1	Maximum Output Power

Table 5.47.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_2A_n77A | 266 | +2/-31 |
| DC_30A_n77A | 266 | +2/-3 |
| NOTE 1: For the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

#### 5.47.1.2	Configurations for EN-DC

Table 5.47.1.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-30A_n77A | DC_2A_n77A DC_30A_n77A |

#### 5.47.1.3	Co-existence study

Co-existence studies of this 3DL/2UL DC configuration for PC3 are already covered in TR 37.717-21-11. It can be seen that:

-	IMD4 and IMD5 products are produced by Band 2 and n77 that might fall in Rx of band 30.

-	IMD4 products are produced by Band 30 and n77 that might fall in Rx of band 2.

### 5.47.2	Receiver Characteristics

#### 5.47.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.47.2.1.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 Case A DC_2A-30A_n77A are defined in Table 5.47.2.1.1-1.

Table 5.47.2.1.1-1: MSD test points for SCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-30A_n77A | 2 | 1906 | 5 | 25 | 1986 | 19.3 | IMD42 |
|  | 30 | 2312 | 5 | 25 | 2357 | N/A | N/A |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | N/A |
|  | 2 | 1905 | 5 | 25 | 1985 | N/A | N/A |
|  | 30 | 2309 | 5 | 25 | 2354 | 22.2 | IMD42 |
|  | n77 | 3361 | 10 | 50 | 3361 | N/A | N/A |
|  | 2 | 1860 | 5 | 25 | 1940 | N/A | N/A |
|  | 30 | 2309 | 5 | 25 | 2354 | 12.9 | IMD5 |
|  | n77 | 3967 | 10 | 50 | 3967 | N/A | N/A |
| NOTE 2: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. |  |  |  |  |  |  |  |

#### 5.47.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B DC_2A-30A_n77A are the same as the Case A defined in Table 5.47.2.1.1-1.

## 5.48	DC_5-30_n77

### 5.48.1	Transmitter Characteristics

#### 5.48.1.1	Maximum Output Power

Table 5.48.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_5A_n77A | 266 | +2/-3 |
| DC_30A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

#### 5.48.1.2	Configurations for EN-DC

Table 5.48.1.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_5A-30A_n77A | DC_5A_n77A DC_30A_n77A |

#### 5.48.1.3	Co-existence study

Co-existence studies of this 3DL/2UL DC configuration for PC3 are already covered in TR 37.717-21-11. It can be seen that:

-	IMD3 products are produced by Band 5 and n77 that might fall in Rx of band 30.

-	IMD3 and IMD5 products are produced by Band 30 and n77 that might fall in Rx of band 5.

### 5.48.2	Receiver Characteristics

#### 5.48.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.48.2.1.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 Case A DC_5A-30A_n77A are defined in Table 5.48.2.1.1-1.

Table 5.48.2.1.1-1: MSD test points for SCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5A-30A_n77A | 5 | 835 | 5 | 25 | 880 | 23.5 | IMD31 |
|  | 30 | 2310 | 5 | 25 | 2355 | N/A | N/A |
|  | n77 | 3740 | 10 | 50 | 3740 | N/A | N/A |
|  | 5 | 835 | 5 | 25 | 880 | N/A | N/A |
|  | 30 | 2310 | 5 | 25 | 2355 | 21.4 | IMD32 |
|  | n77 | 4025 | 10 | 50 | 4025 | N/A | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. |  |  |  |  |  |  |  |

#### 5.48.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B DC_5A-30A_n77A are the same as the Case A defined in Table 5.48.2.1.1-1.

## 5.49	DC_12-30_n77

### 5.49.1	Transmitter Characteristics

#### 5.49.1.1	Maximum Output Power

Table 5.49.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_12A_n77A | 266 | +2/-31 |
| DC_30A_n77A | 266 | +2/-3 |
| NOTE 1: For the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

#### 5.49.1.2	Configurations for EN-DC

Table 5.49.1.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_12A-30A_n77A | DC_12A_n77A DC_30A_n77A |

#### 5.49.1.3	Co-existence study

Co-existence studies of this 3DL/2UL DC configuration for PC3 are already covered in TR 37.717-21-11. It can be seen that:

-	IMD3 products are produced by Band 12 and n77 that might fall in Rx of band 30.

-	IMD3 and IMD5 products are produced by Band 30 and n77 might fall in Rx of band 12.

### 5.49.2	Receiver Characteristics

#### 5.49.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.49.2.1.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 Case A DC_12A-30A_n77A are defined in Table 5.49.2.1.1-1.

Table 5.49.2.1.1-1: MSD test points for SCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_12A-30A_n77A | 12 | 710 | 5 | 25 | 740 | 23.5 | IMD31 |
|  | 30 | 2310 | 5 | 25 | 2355 | N/A | N/A |
|  | n77 | 3880 | 10 | 50 | 3880 | N/A | N/A |
|  | 12 | 707.5 | 5 | 25 | 737.5 | N/A | N/A |
|  | 30 | 2310 | 5 | 25 | 2355 | 21.4 | IMD3 |
|  | n77 | 3770 | 10 | 50 | 3770 | N/A | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

#### 5.49.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B DC_12A-30A_n77A are the same as the Case A defined in Table 5.49.2.1.1-1.

## 5.50	DC_12-66_n77

### 5.50.1	Transmitter Characteristics

#### 5.50.1.1	Maximum Output Power

Table 5.50.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_12A_n77A | 266 | +2/-31 |
| DC_66A_n77A | 266 | +2/-3 |
| NOTE 1: For the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

#### 5.50.1.2	Configurations for EN-DC

Table 5.50.1.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_12A-66A_n77A | DC_12A_n77A DC_66A_n77A |

#### 5.50.1.3	Co-existence study

Co-existence studies of this 3DL/2UL DC configuration for PC3 are already covered in TR 37.717-21-11. It can be seen that:

-	IMD3 products are produced by Band 12 and n77 that might fall in Rx of band 66.

-	IMD3 products are produced by Band 66 and n77 that might fall in Rx of band 12.

### 5.50.2	Receiver Characteristics

#### 5.50.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.50.2.1.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 Case A DC_12A-66A_n77A are defined in Table 5.50.2.1.1-1.

Table 5.50.2.1.1-1: MSD test points for SCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_12A-66A_n77A | 12 | 710 | 5 | 25 | 740 | 23.5 | IMD32 |
|  | 66 | 1720 | 5 | 25 | 2120 | N/A | N/A |
|  | n77 | 4180 | 10 | 50 | 4180 | N/A | N/A |
|  | 12 | 707 | 5 | 25 | 737 | N/A | N/A |
|  | 66 | 1726 | 5 | 25 | 2126 | 21.4 | IMD3 |
|  | n77 | 3540 | 10 | 50 | 3540 | N/A | N/A |
| NOTE 2: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. |  |  |  |  |  |  |  |

#### 5.50.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B DC_12A-66A_n77A are the same as the Case A defined in Table 5.50.2.1.1-1.

## 5.51	DC_14-30_n77

### 5.51.1	Transmitter Characteristics

#### 5.51.1.1	Maximum Output Power

Table 5.51.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_14A_n77A | 266 | +2/-3 |
| DC_30A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

#### 5.51.1.2	Configurations for EN-DC

Table 5.51.1.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_14A-30A_n77A | DC_14A_n77A DC_30A_n77A |

#### 5.51.1.3	Co-existence study

Co-existence studies of this 3DL/2UL DC configuration for PC3 are already covered in TR 37.717-21-11. It can be seen that:

-	IMD3 products are produced by Band 14 and n77 that might fall in Rx of band 30.

-	IMD3 and IMD5 products are produced by Band 30 and n77 that might fall in Rx of band 14.

### 5.51.2	Receiver Characteristics

#### 5.51.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.51.2.1.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 Case A DC_14A-30A_n77A are defined in Table 5.51.2.1.1-1.

Table 5.51.2.1.1-1: MSD test points for SCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_14A-30A_n77A | 14 | 793 | 5 | 25 | 763 | 23.5 | IMD31 |
|  | 30 | 2310 | 5 | 25 | 2355 | N/A | N/A |
|  | n77 | 3857 | 10 | 50 | 3857 | N/A | N/A |
|  | 14 | 793 | 5 | 25 | 763 | N/A | N/A |
|  | 30 | 2310 | 5 | 25 | 2355 | 21.4 | IMD3 |
|  | n77 | 3941 | 10 | 50 | 3941 | N/A | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

#### 5.51.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B DC_14A-30A_n77A are the same as the Case A defined in Table 5.51.2.1.1-1.

## 5.52	DC_14-66_n77

### 5.52.1	Transmitter Characteristics

#### 5.52.1.1	Maximum Output Power

Table 5.52.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_14A_n77A | 266 | +2/-3 |
| DC_66A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

#### 5.52.1.2	Configurations for EN-DC

Table 5.52.1.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_14A-66A_n77A | DC_14A_n77A DC_66A_n77A |

#### 5.52.1.3	Co-existence study

Co-existence studies of this 3DL/2UL DC configuration for PC3 are already covered in TR 37.717-21-11. It can be seen that:

-	IMD3 products are produced by Band 14 and n77 that might fall in Rx of band 66.

-	IMD3 products are produced by Band 66 and n77 that might fall in Rx of band 14.

### 5.52.2	Receiver Characteristics

#### 5.52.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.52.2.1.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 Case A DC_14A-66A_n77A are defined in Table 5.52.2.1.1-1.

Table 5.52.2.1.1-1: MSD test points for SCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_14A-66A_n77A | 14 | 793 | 5 | 25 | 763 | 23.5 | IMD32 |
|  | 66 | 1712.5 | 5 | 25 | 2112.5 | N/A | N/A |
|  | n77 | 4188 | 10 | 50 | 4188 | N/A | N/A |
|  | 14 | 793 | 5 | 25 | 763 | N/A | N/A |
|  | 66 | 1755 | 5 | 25 | 2155 | 21.4 | IMD3 |
|  | n77 | 3741 | 10 | 50 | 3741 | N/A | N/A |
| NOTE 2: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. |  |  |  |  |  |  |  |

#### 5.52.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B DC_14A-66A_n77A are the same as the Case A defined in Table 5.52.2.1.1-1.

## 5.53	DC_29-30_n77

### 5.53.1	Transmitter Characteristics

#### 5.53.1.1	Maximum Output Power

Table 5.53.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_30A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

#### 5.53.1.2	Configurations for EN-DC

Table 5.53.1.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_29A-30A_n77A | DC_30A_n77A |

#### 5.53.1.3	Co-existence study

Co-existence studies of this 3DL/2UL DC configuration for PC3 are already covered in TR 37.717-21-11. It can be seen that:

-	IMD3 and IMD5 products are produced by Band 30 and n77 that might fall in Rx of band 29.

Although DC_29_n77 is not defined, 5th order harmonic mixing is produced from band n77 UL that might fall in Rx of band 29.

### 5.53.2	Receiver Characteristics

#### 5.53.2.1	Reference sensitivity exceptions due to receiver harmonic mixing for PC2 EN-DC in NR FR1

#### 5.53.2.1.1	Power class 2 Case A

The MSD due to receiver harmonic mixing for PC2 Case A are same as PC3 DC_29_n77.

#### 5.53.2.1.2	Power class 2 Case B

The additional MSD due to receiver harmonic mixing for PC2 Case B are defined in Table 5.53.2.1.2-1. For harmonic mixing, the n77 power has increased by 3dB which results in 3dB more power down-converted at the 5th RX LO harmonic.

Table 5.53.2.1.2-1: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for PC2 EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n77zz | 291 | 34 | 31 |  |  |  |  |  |  |  |  |  |
| NOTE 1: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE ZZ: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. |  |  |  |  |  |  |  |  |  |  |  |  |

#### 5.53.2.2	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.53.2.2.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 Case A DC_29A-30A_n77A are defined in Table 5.53.2.2.1-1.

Table 5.53.2.2.1-1: MSD test points for SCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_29A-30A_n77A | 29 | N/A | 5 | N/A | 722 | 23.5 | IMD31 |
|  | 30 | 2310 | 5 | 25 | 2355 | N/A | N/A |
|  | n77 | 3898 | 10 | 50 | 3898 | N/A | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

#### 5.53.2.2.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B DC_29A-30A_n77A are the same as the Case A defined in Table 5.53.2.2.1-1.

## 5.54	DC_29-66_n77

### 5.54.1	Transmitter Characteristics

#### 5.54.1.1	Maximum Output Power

Table 5.54.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_66A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

#### 5.54.1.2	Configurations for EN-DC

Table 5.54.1.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_29A-66A_n77A | DC_66A_n77A |

#### 5.54.1.3	Co-existence study

Co-existence studies of this 3DL/2UL DC configuration for PC3 are already covered in TR 37.717-21-11. It can be seen that:

-	IMD3 products are produced by Band 66 and n77 that might fall in Rx of band 29.

Although DC_29_n77 is not defined, 5th order harmonic mixing is produced from band n77 UL that might fall in Rx of band 29.

### 5.54.2	Receiver Characteristics

#### 5.54.2.1	Reference sensitivity exceptions due to receiver harmonic mixing for PC2 EN-DC in NR FR1

#### 5.54.2.1.1	Power class 2 Case A

The MSD due to receiver harmonic mixing for PC2 Case A are same as PC3 DC_29_n77.

#### 5.54.2.1.2	Power class 2 Case B

The additional MSD due to receiver harmonic mixing for PC2 Case B are defined in Table 5.54.2.1.2-1. For harmonic mixing, the n77 power has increased by 3dB which results in 3dB more power down-converted at the 5th RX LO harmonic.

Table 5.54.2.1.2-1: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for PC2 EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n77zz | 291 | 34 | 31 |  |  |  |  |  |  |  |  |  |
| NOTE 1: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE ZZ: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. |  |  |  |  |  |  |  |  |  |  |  |  |

#### 5.54.2.2	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.54.2.2.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 Case A DC_29A-66A_n77A are defined in Table 5.54.2.2.1-1.

Table 5.54.2.2.1-1: MSD test points for SCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_29A-66A_n77A | 29 | N/A | 5 | N/A | 722 | 23.5 | IMD32 |
|  | 66 | 1734 | 5 | 25 | 2134 | N/A | N/A |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | N/A |
| NOTE 2: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. |  |  |  |  |  |  |  |

#### 5.54.2.2.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B DC_29A-66A_n77A are the same as the Case A defined in Table 5.54.2.2.1-1.

## 5.55	DC_30-66_n77

### 5.55.1	Transmitter Characteristics

#### 5.55.1.1	Maximum Output Power

Table 5.55.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_30A_n77A | 266 | +2/-3 |
| DC_66A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

#### 5.55.1.2	Configurations for EN-DC

Table 5.55.1.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_30A-66A_n77A | DC_30A_n77A DC_66A_n77A |

#### 5.55.1.3	Co-existence study

Co-existence studies of this 3DL/2UL DC configuration for PC3 are already covered in TR 37.717-21-11. It can be seen that:

-	IMD4 products are produced by Band 30 and n77 that might fall in Rx of band 66.

-	IMD2 and IMD5 products are produced by Band 66 and n77 that might fall in Rx of band 30.

### 5.55.2	Receiver Characteristics

#### 5.55.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 5.55.2.1.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 Case A DC_30A-66A_n77A are defined in Table 5.55.2.1.1-1.

Table 5.55.2.1.1-1: MSD test points for SCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_30A-66A_n77A | 30 | 2310 | 5 | 25 | 2355 | 34.2 | IMD22 |
|  | 66 | 1745 | 5 | 25 | 2145 | N/A | N/A |
|  | n77 | 4100 | 10 | 50 | 4100 | N/A | N/A |
|  | 30 | 2310 | 5 | 25 | 2355 | 12.9 | IMD5 |
|  | 66 | 1735 | 5 | 25 | 2135 | N/A | N/A |
|  | n77 | 3780 | 10 | 50 | 3780 | N/A | N/A |
|  | 30 | 2310 | 5 | 25 | 2355 | N/A | N/A |
|  | 66 | 1760 | 5 | 25 | 2160 | 19.2 | IMD42 |
|  | n77 | 3390 | 10 | 50 | 3390 | N/A | N/A |
| NOTE 2: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. |  |  |  |  |  |  |  |

#### 5.55.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B DC_30A-66A_n77A are the same as the Case A defined in Table 5.55.2.1.1-1.

## 5.56	DC_5-66_n2-n77

### 5.56.1	Maximum Output Power

Table 5.56.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_5A_n77A | 266 | +2/-3 |
| DC_66A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

5.56.2		Configurations for EN-DC

Table 5.56.2-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_5A-66A_n2A-n77A DC_5A-66A-66A_n2A-n77A | DC_5A_n77A DC_66A_n77A |

#### 5.56.3	Co-existence study

The related MSD requirements have been studied in the lower order combinations. There is no additional requirement for this PC2 band combination.

# 6	PC2 MSD analysis for ENDC band combinations

## 6.1	DC_12-30_n77

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_12A-30A_n77A | 12 | 710 | 5 | 25 | 740 | 23.5 | IMD31 |
|  | 30 | 2310 | 5 | 25 | 2355 | N/A | N/A |
|  | n77 | 3880 | 10 | 50 | 3880 | N/A | N/A |
|  | 12 | 707.5 | 5 | 25 | 737.5 | N/A | N/A |
|  | 30 | 2310 | 5 | 25 | 2355 | 21.4 | IMD3 |
|  | n77 | 3770 | 10 | 50 | 3770 | N/A | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

## 6.2	DC_12-66_n77

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_12A-66A_n77A | 12 | 710 | 5 | 25 | 740 | 23.5 | IMD32 |
|  | 66 | 1720 | 5 | 25 | 2120 | N/A | N/A |
|  | n77 | 4180 | 10 | 50 | 4180 | N/A | N/A |
|  | 12 | 707 | 5 | 25 | 737 | N/A | N/A |
|  | 66 | 1726 | 5 | 25 | 2126 | 21.4 | IMD3 |
|  | n77 | 3540 | 10 | 50 | 3540 | N/A | N/A |
| NOTE 2: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. |  |  |  |  |  |  |  |

## 6.3	DC_14-30_n77

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_14A-30A_n77A | 14 | 793 | 5 | 25 | 763 | 23.5 | IMD31 |
|  | 30 | 2310 | 5 | 25 | 2355 | N/A | N/A |
|  | n77 | 3857 | 10 | 50 | 3857 | N/A | N/A |
|  | 14 | 793 | 5 | 25 | 763 | N/A | N/A |
|  | 30 | 2310 | 5 | 25 | 2355 | 21.4 | IMD3 |
|  | n77 | 3941 | 10 | 50 | 3941 | N/A | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

## 6.4	DC_14-66_n77

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_14A-66A_n77A | 14 | 793 | 5 | 25 | 763 | 23.5 | IMD32 |
|  | 66 | 1712.5 | 5 | 25 | 2112.5 | N/A | N/A |
|  | n77 | 4188 | 10 | 50 | 4188 | N/A | N/A |
|  | 14 | 793 | 5 | 25 | 763 | N/A | N/A |
|  | 66 | 1755 | 5 | 25 | 2155 | 21.4 | IMD3 |
|  | n77 | 3741 | 10 | 50 | 3741 | N/A | N/A |
| NOTE 2: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. |  |  |  |  |  |  |  |

## 6.5	DC_29-30_n77

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_29A-30A_n77A | 29 | N/A | 5 | N/A | 722 | 23.5 | IMD31 |
|  | 30 | 2310 | 5 | 25 | 2355 | N/A | N/A |
|  | n77 | 3898 | 10 | 50 | 3898 | N/A | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

## 6.6	DC_29-66_n77

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_29A-66A_n77A | 29 | N/A | 5 | N/A | 722 | 23.5 | IMD32 |
|  | 66 | 1734 | 5 | 25 | 2134 | N/A | N/A |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | N/A |
| NOTE 2: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. |  |  |  |  |  |  |  |

## 6.7	DC_30-66_n77

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_30A-66A_n77A | 30 | 2310 | 5 | 25 | 2355 | 34.2 | IMD22 |
|  | 66 | 1745 | 5 | 25 | 2145 | N/A | N/A |
|  | n77 | 4100 | 10 | 50 | 4100 | N/A | N/A |
|  | 30 | 2310 | 5 | 25 | 2355 | 12.9 | IMD5 |
|  | 66 | 1735 | 5 | 25 | 2135 | N/A | N/A |
|  | n77 | 3780 | 10 | 50 | 3780 | N/A | N/A |
|  | 30 | 2310 | 5 | 25 | 2355 | N/A | N/A |
|  | 66 | 1760 | 5 | 25 | 2160 | 19.2 | IMD42 |
|  | n77 | 3390 | 10 | 50 | 3390 | N/A | N/A |
| NOTE 2: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. |  |  |  |  |  |  |  |

## 6.8	DC_2-12_n77

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-12A_n77A | 2 | 1880 | 5 | 25 | 1960 | 24.8 | IMD32,zz |
|  | 12 | 707.5 | 5 | 25 | 737.5 | N/A | N/A |
|  | n77 | 3375 | 10 | 50 | 3375 | N/A | N/A |
| NOTE 2: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. NOTE ZZ: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |

## 6.9	DC_2-14_n77

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-14A_n77A | 2 | 1874 | 5 | 25 | 1954 | 24.8 | IMD3 |
|  | 14 | 793 | 5 | 25 | 763 | N/A | N/A |
|  | n77 | 3540 | 10 | 50 | 3540 | N/A | N/A |

## 6.10	DC_2-30_n77

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-30A_n77A | 2 | 1906 | 5 | 25 | 1986 | 19.3 | IMD42 |
|  | 30 | 2312 | 5 | 25 | 2357 | N/A | N/A |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | N/A |
|  | 2 | 1905 | 5 | 25 | 1985 | N/A | N/A |
|  | 30 | 2309 | 5 | 25 | 2354 | 22.2 | IMD42 |
|  | n77 | 3361 | 10 | 50 | 3361 | N/A | N/A |
|  | 2 | 1860 | 5 | 25 | 1940 | N/A | N/A |
|  | 30 | 2309 | 5 | 25 | 2354 | 12.9 | IMD5 |
|  | n77 | 3967 | 10 | 50 | 3967 | N/A | N/A |
| NOTE 2: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. |  |  |  |  |  |  |  |

## 6.11	DC_5-30_n77

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5A-30A_n77A | 5 | 835 | 5 | 25 | 880 | 23.5 | IMD31 |
|  | 30 | 2310 | 5 | 25 | 2355 | N/A | N/A |
|  | n77 | 3740 | 10 | 50 | 3740 | N/A | N/A |
|  | 5 | 835 | 5 | 25 | 880 | N/A | N/A |
|  | 30 | 2310 | 5 | 25 | 2355 | 21.4 | IMD32 |
|  | n77 | 4025 | 10 | 50 | 4025 | N/A | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. |  |  |  |  |  |  |  |

# Annex A - Change history

| Date | Meeting | TDoc | Subject/Comment | New version |
| --- | --- | --- | --- | --- |
| 2021-04 | 3GPP RAN4#98-bis-e | R4-2105489 | Initial version Implemented TP’s from RAN4 #98-bis-e: R4-2105351, “TP for TR 37.xxx for DC_2-5_n77”, Verizon Denmark R4-2105352, “TP for TR 37.xxx for DC_2-13_n77”, Verizon Denmark R4-2105353, “TP for TR 37.xxx for DC_2-66_n77”, Verizon Denmark R4-2105354, “TP for TR 37.xxx for DC_5-66_n77”, Verizon Denmark R4-2105355, “TP for TR 37.xxx for DC_13-66_n77”, Verizon Denmark R4-2105356, “TP for TR 37.xxx for DC_2_n5-n77”, Verizon Denmark R4-2105357, “TP for TR 37.xxx for DC_66_n2-n77”, Verizon Denmark R4-2105358, “TP for TR 37.xxx for DC_66_n5-n77”, Verizon Denmark | 0.0.1 |
| 2021-08 | 3GPP RAN4#100-e | R4-2113565 | Implemented TP’s from RAN4 #99-e: R4-2108901, TP for TR 37.827 for DC_2-13_n66-n77, Verizon Denmark R4-2108902, TP for TR 37.827 for DC_2-13-66_n77, Verizon Denmark R4-2108903, TP for TR 37.827 for DC_13-66_n2-n77, Verizon Denmark R4-2108904, TP for TR 37.827 for DC_2-66_n5-n77, Verizon Denmark R4-2108905, TP for TR 37.827 for DC_2-5-66_n77, Verizon Denmark R4-2107837, TP for TR 37.827 for DC_13_n66-n77, Verizon Denmark R4-2107838, TP for TR 37.827 for DC_13_n2-n77, Verizon Denmark R4-2110957, TP for PC2 DC_2_66-n41, Huawei Tech.(UK) Co.. Ltd | 0.1.0 |
| 2021-08 | 3GPP RAN4#100-e | R4-2113566 | Implemented TP’s from RAN4 #100-e: R4-2112669, TP for TR 37.827 for DC_66_n66-n77, Verizon, Samsung R4-2112670, TP to TR 37.717-21-11 for DC_48-66_n77, Verizon, Samsung R4-2112672 TP for TR 37.827 for DC_13_n5-n77, Verizon, Samsung R4-2112673, TP for TR 37.827 for DC_5_n66-n77, Verizon, Samsung R4-2114940 TP for TR 37.827 for DC_5_n5-n77, Verizon, Samsung R4-2114941 TP for TR 37.827 for DC_5_n2-n77, Verizon, Samsung R4-2112679, TP for TR 37.827 for DC_2_n66-n77, Verizon, Samsung R4-2112681, TP for TR 37.827 for DC_2-48_n77, Verizon, Samsung R4-2114942, TP for TR 37.827 for DC_2_n2-n77, Verizon, Samsung R4-2114043, Text proposal for TR 37.827 to include DC_1A-5A_n78A, Huawei Tech.(UK) Co.. Ltd R4-2114054, Text proposal for TR 37.827 to include DC_1A-n7A_n78A, Huawei Tech.(UK) Co.. Ltd | 0.2.0 |
| 2021-11 | 3GPP RAN4#101-e | R4-2118497 | Implemented TP’s from RAN4 #101-e: R4-2117107, PC2 MSD ENDC xLTE yNR for TR 37.827, Qualcomm Incorporated R4-2117114, TP for TR 37.827 for DC_2_n2-n77, Verizon Denmark R4-2117115, TP for TR 37.827 for DC_2_n5-n77, Verizon Denmark R4-2117116, TP for TR 37.827 for DC_2_n66-n77, Verizon Denmark R4-2117117, TP for TR 37.827 for DC_2-5_n5-n77, Verizon Denmark R4-2117118, TP for TR 37.827 for DC_2-5_n77, Verizon Denmark R4-2117119, TP for TR 37.827 for DC_2-5-48_n77, Verizon Denmark R4-2117120, TP for TR 37.827 for DC_2-5-66_n5-n77, Verizon Denmark R4-2117121, TP for TR 37.827 for DC_2-5-66_n77, Verizon Denmark R4-2117122, TP for TR 37.827 for DC_2-13_n2-n77, Verizon Denmark R4-2117123, TP for TR 37.827 for DC_2-13_n5-n77, Verizon Denmark R4-2117124, TP for TR 37.827 for DC_2-13_n77, Verizon Denmark R4-2117125, TP for TR 37.827 for DC_2-13-48_n77, Verizon Denmark R4-2117126, TP for TR 37.827 for DC_2-13-66_n2-n77, Verizon Denmark R4-2117127, TP for TR 37.827 for DC_2-13-66_n5-n77, Verizon Denmark R4-2117128, TP for TR 37.827 for DC_2-13-66_n77, Verizon Denmark R4-2117129, TP for TR 37.827 for DC_2-48_n77, Verizon Denmark R4-2117130 TP for TR 37.827 for DC_2-48-66_n77, Verizon Denmark R4-2117131, TP for TR 37.827 for DC_2-66_n2-n77, Verizon Denmark R4-2117132, TP for TR 37.827 for DC_2-66_n5-n77, Verizon Denmark R4-2117133, TP for TR 37.827 for DC_2-66_n77, Verizon Denmark R4-2117134, TP for TR 37.827 for DC_5_n2-n77, Verizon Denmark R4-2117135, TP for TR 37.827 for DC_5_n5-n77, Verizon Denmark R4-2117136, TP for TR 37.827 for DC_5_n66-n77, Verizon Denmark R4-2117137, TP for TR 37.827 for DC_5-13_n77, Verizon Denmark R4-2117138, TP for TR 37.827 for DC_5-48_n77, Verizon Denmark R4-2117139, TP for TR 37.827 for DC_5-48-66_n77, Verizon Denmark R4-2117141, TP for TR 37.827 for DC_5-66_n5-n77, Verizon Denmark R4-2117142, TP for TR 37.827 for DC_5-66_n77, Verizon Denmark R4-2117143, TP for TR 37.827 for DC_13_n2-n77, Verizon Denmark R4-2117144, TP for TR 37.827 for DC_13_n5-n77, Verizon Denmark R4-2117145, TP for TR 37.827 for DC_13_n66-n77, Verizon Denmark R4-2117146, TP for TR 37.827 for DC_13-48_n77, Verizon Denmark R4-2117147, TP for TR 37.827 for DC_13-48-66_n77, Verizon Denmark R4-2117148, TP for TR 37.827 for DC_13-66_n2-n77, Verizon Denmark R4-2117149, TP for TR 37.827 for DC_13-66_n5-n77, Verizon Denmark R4-2117150, TP for TR 37.827 for DC_13-66_n77, Verizon Denmark R4-2117151, TP for TR 37.827 for DC_48-66_n77, Verizon Denmark R4-2117152, TP for TR 37.827 for DC_66_n2-n77, Verizon Denmark R4-2117153, TP for TR 37.827 for DC_66_n5-n77, Verizon Denmark R4-2117154, TP for TR 37.827 for DC_66_n66-n77, Verizon Denmark R4-2117263, TP for TR 37.827 Addition of DC_2-12_n77, AT&T R4-2117264, TP for TR 37.827 Addition of DC_2-14_n77, AT&T R4-2117265, TP for TR 37.827 Addition of DC_2-29_n77, AT&T R4-2117266, TP for TR 37.827 Addition of DC_2-30_n77, AT&T R4-2117267, TP for TR 37.827 Addition of DC_5-30_n77, AT&T R4-2117268, TP for TR 37.827 Addition of DC_12-30_n77, AT&T R4-2117269, TP for TR 37.827 Addition of DC_12-66_n77, AT&T R4-2117270, TP for TR 37.827 Addition of DC_14-30_n77, AT&T R4-2117271, TP for TR 37.827 Addition of DC_14-66_n77, AT&T R4-2117272, TP for TR 37.827 Addition of DC_29-30_n77, AT&T R4-2117273, TP for TR 37.827 Addition of DC_29-66_n77, AT&T R4-2117274, TP for TR 37.827 Addition of DC_30-66_n77, AT&T R4-2117140, TP for TR 37.827 for DC_5-66_n2-n77, Verizon Denmark | 0.3.0 |