3GPP TSG-RAN WG4 Meeting #101-e	R4-2118514

Electronic Meeting, 1 November – 11 November 2021

Source:	Ericsson, T-Mobile US

Title:	TP to add CA_n41(2A), CA_n41(3A) and CA_n41(A-C)

Agenda item:	7.6.2

Document for:	Approval

# Background

This text proposal for TR 38.717-01-01 to add CA_n41(2A), CA_n41(3A) and CA_n41(A-C) as defined in WID [1].

# Text Proposal

---Start of changes---

## 6.x	CA_2DL_n41(2A)_1UL_n41A
CA_2DL_n41(A-C)_1UL_n41A
CA_3DL_n41(3A)_1UL_n41A

### 6.x.1	Channel bandwidths per operating band for CA

Table 6.x.1-1: Supported bandwidth combinations for CA_2DL_n41(xA)_1UL _n41A

|  |  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_n41(2A) | - | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  | 190 | 3 |
| CA_n41(3A) | - | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  | 190 | 0 |
| CA_n41(A-C) |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | See CA_n41C Bandwidth Combination Set 3 in Table 5.5A.1-1 |  |  |  | 190 | 0 |

### 6.x.2	REFSENS

Since n41 is TDD there is no need to define additional REFSENS requirements.

---End of changes---

# Reference

[1]		RP-211757,  Rel-17 revised WID Band combination table for NR Intra band CA for xCC DL_yCC UL, Ericsson