3GPP TSG-RAN WG4 Meeting#101     	R4-2118621

E-meeting, 1st – 12th November 2021

Title:	TP to TR 38.717-04-01 Addition of CA_n1A-n28A-n40-n78A

Source:	Nokia

Agenda item:	7.9.2

Document for:	Approval

# Introduction

This is a TP to TR 38.717-04-01 Addition of CA_n1A-n28A-n40-n78A. Note a TP for 2 band UL have also been submitted as R4-2118622

********************** Start of TP *********************************

## 5.x	CA_n1-n28-n40-n78

#### 5.x.1	Channel bandwidths per operating band for CA

Table 5.x.1-1: Supported bandwidths

| NR CA Configuration | UL Config | NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n28A-n40A-n78A | - | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n40 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 |  | 80 |  |  |  |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n1A-n28A-n40B-n78A | - | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n40 | See CA_n40B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

### 5.x.2	∆TIB,c and ∆RIB,c values

For CA_n1-n28-n40-n78 the TIB,c and RIB,c  values are shown in table 5.30.2-1 and  table 5.30.2-2, respectively. Values are derived from CA_n1-n28-n40, CA_n28-n40-n78 and CA_n1-n40-n78.

Table 5.x.2-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n1-n28-n40-n78 | n1 | 0.3 |
|  | n28 | 0.6 |
|  | n40 | 0.5 |
|  | n78 | 0.8 |

Table 5.x.2-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n1-n28-n40-n78 | n1 | 0 |
|  | n28 | 0.2 |
|  | n40 | 0 |
|  | n78 | 0.5 |

### 5.x.3	REFSENS requirements

MSD requirements are captured in lower order combinations.

********************** End of TP *********************************