3GPP TSG-RAN WG4 Meeting #101-e	R4-2118690

Electronic Meeting, 1 November – 12 November 2021

Agenda Item:	8.6.5.3

Source:	Ericsson

Title:	CQI reporting requirements for DL 1024QAM

Document for:	Discussion

# 1	Introduction

This contribution discusses our view on the scope of CQI reporting requirements for DL 1024QAM according to WID [1].

# 2	Discussion

Under the WI DL 1024-QAM for NR FR1, RAN1 defined a new CQI table support of 1024 QAM for PDSCH [2]. When gNB configures this new CQI table for UE capable of DL 1024QAM table, according to TS38.214 5.2.2.1, UE shall report the highest CQI index where the block error rate of transport block corresponding to the reported CQI index shall be within 0.1. We therefore propose to define a new CQI reporting requirements for the new DL 1024 QAM table.

Proposal 1: RAN4 defines CQI definition tests with new CQI table with 1024QAM entries.

According to the latest TS38.101-4, RAN4 defines two CQI reporting definition tests; one for CQI Table 2 (256QAM table) and CQI Table 3 (Higher reliable CQI table). For 1024QAM CQI reporting test, we propose to reuse the test setup and metric for CQI Table 2 reporting tests.

Proposal 2: For 1024QAM CQI reporting test, RAN4 reuses the test setup and test metrics for 256QAM CQI reporting tests.

# 3	Simulation assumption

Table 1 shows the proposed test parameters for CQI reporting test with new CQI table with 1024QAM entries. Based on the simulation results, we propose to discuss the test points for this CQI definition test.

The exiting CQI reporting test with Table 2 sets two test points: one for lower CQI entry and another for higher CQI entry. We propose to set two SNR test points: one for SNR where UE reports CQI index 14 or 15; another for lower SNR test points e.g., 8/9dB, to verify the UE reports the proper CQI index when new 1024QAM CQI table is configured.

Proposal 3: RAN4 sets one SNR test point to verify the UE reports CQI index 14 or 15 in the CQI table with 1024 entries.

Proposal 4: RAN4 sets one more SNR test (e.g., 8/9dB) to verify the UE reports the proper CQI index when the new CQI table is configured.

Table 1	Test parameters for CQI definition reporting with 1024QAM.

| Parameter |  |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 10 | 40 |
| Duplex Mode |  |  |  | FDD | TDD |
| Subcarrier spacing |  |  | kHz | 15 | 30 |
| SNR |  |  | dB | TBD | TBD |
| Propagation channel |  |  |  | AWGN | AWGN |
| Antenna configuration |  |  |  | Test 1-1: 2×2 with static channel specified in Annex B.1 Test 1-2: 2×4 with static channel specified in Annex B.1 | Test 2-1: 2×2 with static channel specified in Annex B.1 Test 2-2: 2×4 with static channel specified in Annex B.1 |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  | 4 | 4 | 4 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5,4 | Row 5,4 | Row 5,4 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 | 9 |
|  | CSI-RS periodicity and offset |  | slot | 5/1 | 10/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 2 | 2 |
|  | CDM Type |  |  | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | Row 3,(6,-) | Row 3,(6,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 | 13 |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 5/1 | 10/1 |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic | Periodic |
|  | CSI-IM RE pattern |  |  | 0 | 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) | (4, 9) |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 5/1 | 10/1 |
| ReportConfigType |  |  |  | Periodic | Periodic |
| CQI-table |  |  |  | Table 4 | Table 4 |
| reportQuantity |  |  |  | cri-RI-PMI-CQI | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured | Not configured |
| cqi-FormatIndicator |  |  |  | Wideband | Wideband |
| pmi-FormatIndicator |  |  |  | Wideband | Wideband |
| Sub-band Size |  |  | RB | 8 | 16 |
| Csi-ReportingBand |  |  |  | 1111111 | 1111111 |
| CSI-Report periodicity and offset |  |  | slot | 5/0 | 10/9 |
| aperiodicTriggeringOffset |  |  |  | Not configured | Not configured |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel | typeI-SinglePanel |
|  |  | Codebook Mode |  | 1 | 1 |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured | Not configured |
|  |  | CodebookSubsetRestriction |  | 010000 | 010000 |
|  |  | RI Restriction |  | N/A | N/A |
| Physical channel for CSI report |  |  |  | PUCCH | PUCCH |
| CQI/RI/PMI delay |  |  | ms | 8 | 9.5 |
| Maximum number of HARQ transmission |  |  |  | 1 | 1 |
| Measurement channel |  |  |  | TBD | TBD |

Test metric:

a)	The reported CQI value according to the reference channel shall be in the range of ±1 of the reported median more than 90% of the time.

b)	If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, then the BLER using the transport format indicated by the (median CQI+1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, then the BLER using transport format indicated by (median CQI-1) shall be less than or equal to 0.1.

# 4	Summary

Proposal 1: RAN4 defines CQI definition tests with new CQI table with 1024QAM entries.

Proposal 2: For 1024QAM CQI reporting test, RAN4 reuses the test setup and test metrics for 256QAM CQI reporting tests.

Proposal 3: RAN4 sets one SNR test point to verify the UE reports CQI index 14 or 15 in the CQI table with 1024 entries.

Proposal 4: RAN4 sets one more SNR test (e.g., 8/9dB) to verify the UE reports the proper CQI index when the new CQI table is configured.

# References

- RP-202886, “Revised WID on Introduction of DL 1024QAM for NR FR1”, Nokia.

- R1-2102088, “LS on Introduction of DL 1024QAM for NR”, 3GPP RAN WG1.

# Appendix: CQI table with 1024QAM entries

| CQI index | modulation | code rate x 1024 | efficiency |
| --- | --- | --- | --- |
| 0 | Out of range |  |  |
| 1 | QPSK | 78 | 0.1523 |
| 2 | QPSK | 193 | 0.377 |
| 3 | QPSK | 449 | 0.877 |
| 4 | 16QAM | 378 | 1.4766 |
| 5 | 16QAM | 616 | 2.4063 |
| 6 | 64QAM | 567 | 3.3223 |
| 7 | 64QAM | 666 | 3.9023 |
| 8 | 64QAM | 772 | 4.5234 |
| 9 | 64QAM | 873 | 5.1152 |
| 10 | 256QAM | 711 | 5.5547 |
| 11 | 256QAM | 797 | 6.2266 |
| 12 | 256QAM | 885 | 6.9141 |
| 13 | 256QAM | 948 | 7.4063 |
| 14 | 1024QAM | 853 | 8.3301 |
| 15 | 1024QAM | 948 | 9.2578 |