3GPP TSG RAN WG4 Meeting #101-e	R4-2118750

Electronic Meeting, 01 November 2021 – 12 November 2021

Agenda Item:	8.4.6.2

Source: 	Ericsson

Title:	Discussion on RRM requirements of FR2 inter-band UL CA for IBM

Document for:	Discussion

# Introduction

In RAN plenary it was agreed to drop inter-band UL CA for CBM UE in the WID. This implies RAN4 needs to define requirements only for IBM UE for FR2 inter-band UL CA.

In this contribution, we provide our views on RRM requirements of FR2 inter-band UL CA for IBM UE.

# Discussion

In last meeting following issues are discussed and only DL interruption at NR SRS carrier-based switching is open and other issues are resolved or made agreements.

- Interruption due to UL carrier RRC reconfiguration

- Interruption at active BWP switching

- DL interruption at UE Tx switching between two uplink carriers

- DL Interruption at NR SRS carrier-based switching

We provide our views on above open issues in subsequent sections.

## DL Interruption at NR SRS carrier-based switching

Following WF [1] is agreed in last meeting.

- Option 1: To further consider whether interruption due to SRS carrier-based switching can be limited to the band in which SRS carrier-based switching is taking place.

  - The decision shall be subject to confirmation by RF session, and the details are FFS, e.g., whether it depends on the signalling/triggering mechanism of SRS carrier-based switching.

- Option 2: Interruption due to SRS carrier switching in one band will occur on all serving cells within FR if UE supports per FR gaps; otherwise, interruption occurs on all serving cells.

- Option 3: RAN4 need to study whether and/or how to define the interruption requirements for inter-band SRS carrier-based switching in FR2

  - Option 3a: Wait for RF room on conclusion of applicable SRS carrier switching time for inter-band CA in FR2

In Rel-16 DL interruption is defined based on whether UE supports per FR gap. Unless RAN4#RF session agrees on different architecture, we feel same principle can be used. Following same principle, interruption due to SRS carrier switching in one band will occur on all serving cells within FR if UE supports per FR gaps; otherwise, interruption occurs on all serving cells.

Proposal 1:  RAN4 to agree that Interruption due to SRS carrier switching in one band will occur on all serving cells within FR if UE supports per FR gaps; otherwise, interruption occurs on all serving cells.

# Summary and Conclusion

In this contribution we have analysed RRM requirements of FR2 inter-band UL CA for IBM UE and made following proposals and observations.

Proposal 1:  RAN4 to agree that Interruption due to SRS carrier switching in one band will occur on all serving cells within FR if UE supports per FR gaps; otherwise, interruption occurs on all serving cells.

# References

- R4-2115332