3GPP TSG-RAN WG4 Meeting # 101-e 	R4-2118755

# Electronic Meeting, 01 November – 12 November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on RRM requirements for HO with PSCell |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_enh2-Core |  |  |  |  |  | Date: |  |  | 2021-10-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | RAN4 introduced requirements for HO with PSCell feature in Rel-17 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introducing the specification skeleton for HO with PSCell requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | HO with PSCell requirements will be missing in Rel-17 specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change>

### 6.1.5	NR Handover with PSCell

#### 6.1.5.1	Introduction

The purpose of NR handover with PSCell is to change the NR PCell to another NR cell and simultaneously add the PSCell along with handover of PCell. The requirements in this clause are applicable to following scenarios.

- HO with PSCell from NR SA to EN-DC

- HO with PSCell from EN-DC to EN-DC

- HO with PSCell from NE-DC to NE-DC

- HO with PSCell from NR-DC to NR-DC

Further requirements in this clause applies to FR1+FR2 NR-DC and FR1+LTE NE-DC.

#### 6.1.5.2	HO with PSCell from NR SA to EN-DC

The requirements in this clause are applicable to inter-RAT handovers from NR PCell to LTE cell, and the PSCell addition can be on FR1 or FR2.

##### 6.1.5.2.1	Handover delay

When the UE receives a RRC message implying handover with PSCell addition, the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover msec from the end of the last TTI containing the RRC command. Handover delay includes PCell handover and PSCell addition delay.

Where:

Dhandover equals the applicable RRC procedure delay defined in clause 12 in TS 38.331 [2] plus the interruption time stated in clause 6.1.5.2.2.

##### 6.1.5.2.2	Interruption time

The interruption time is the time between end of the last TTI containing the RRC command on the old PDSCH and the time the UE starts transmission of the new PRACH, excluding the RRC procedure delay.

When the HO with PSCell is commanded the interruption time shall be less than Tinterrupt

Tinterrupt = Tsearch + TIU + Tprocessing  + T∆ + Tmargin ms

Where:

Tsearch is FFS.

T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = Trs.

Tprocessing is FFS.

Tmargin is time for SSB post-processing. Tmargin can be up to 2ms.

TIU is FFS.

Trs is FFS.

In the interruption requirement a cell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds otherwise it is unknown. Relevant cell identification requirements are described in Clause 9.2.5 for intra-frequency handover and Clause 9.3.4 for inter-frequency handover.

#### 6.1.5.3	HO with PSCell from EN-DC to EN-DC

The requirements in this clause are applicable to both intra-frequency and inter-frequency handovers from LTE PCell to LTE cell, and the PSCell addition can be on FR1 or FR2.

##### 6.1.5.3.1	Handover delay

When the UE receives a RRC message implying handover with PSCell addition, the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover msec from the end of the last TTI containing the RRC command. Handover delay includes PCell handover and PSCell addition delay.

Where:

Dhandover equals the applicable RRC procedure delay defined in clause 12 in TS 38.331 [2] plus the interruption time stated in clause 6.1.5.2.2.

##### 6.1.5.3.2	Interruption time

The interruption time is the time between end of the last TTI containing the RRC command on the old PDSCH and the time the UE starts transmission of the new PRACH, excluding the RRC procedure delay.

When the HO with PSCell is commanded the interruption time shall be less than Tinterrupt

Tinterrupt = Tsearch + TIU + Tprocessing  + T∆ + Tmargin ms

Where:

Tsearch is FFS.

T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = Trs.

Tprocessing is FFS.

Tmargin is time for SSB post-processing. Tmargin can be up to 2ms.

TIU is FFS.

Trs is FFS.

In the interruption requirement a cell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds otherwise it is unknown. Relevant cell identification requirements are described in Clause 9.2.5 for intra-frequency handover and Clause 9.3.4 for inter-frequency handover.

#### 6.1.5.4	HO with PSCell from NE-DC to NE-DC

The requirements in this clause are applicable to both intra-frequency and inter-frequency handovers from NR PCell to NR cell on FR1 and FR2, and the PSCell addition can be on LTE.

##### 6.1.5.4.1	Handover delay

When the UE receives a RRC message implying handover with PSCell addition, the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover msec from the end of the last TTI containing the RRC command. Handover delay includes PCell handover and PSCell addition delay.

Where:

Dhandover equals the applicable RRC procedure delay defined in clause 12 in TS 38.331 [2] plus the interruption time stated in clause 6.1.5.2.2.

##### 6.1.5.4.2	Interruption time

The interruption time is the time between end of the last TTI containing the RRC command on the old PDSCH and the time the UE starts transmission of the new PRACH, excluding the RRC procedure delay.

When the HO with PSCell is commanded the interruption time shall be less than Tinterrupt

Tinterrupt = Tsearch + TIU + Tprocessing  + T∆ + Tmargin ms

Where:

Tsearch is FFS.

T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = Trs.

Tprocessing is FFS.

Tmargin is time for SSB post-processing. Tmargin can be up to 2ms.

TIU is FFS.

Trs is FFS.

In the interruption requirement a cell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds otherwise it is unknown. Relevant cell identification requirements are described in Clause 9.2.5 for intra-frequency handover and Clause 9.3.4 for inter-frequency handover.

#### 6.1.5.5	HO with PSCell from NR-DC to NR-DC

The requirements in this clause are applicable to both intra-frequency and inter-frequency handovers from NR PCell to NR cell on FR1 and FR2, and the PSCell addition can be on NR.

##### 6.1.5.5.1	Handover delay

When the UE receives a RRC message implying handover with PSCell addition, the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover msec from the end of the last TTI containing the RRC command. Handover delay includes PCell handover and PSCell addition delay.

Where:

Dhandover equals the applicable RRC procedure delay defined in clause 12 in TS 38.331 [2] plus the interruption time stated in clause 6.1.5.2.2.

##### 6.1.5.5.2	Interruption time

The interruption time is the time between end of the last TTI containing the RRC command on the old PDSCH and the time the UE starts transmission of the new PRACH, excluding the RRC procedure delay.

When the HO with PSCell is commanded the interruption time shall be less than Tinterrupt

Tinterrupt = Tsearch + TIU + Tprocessing  + T∆ + Tmargin ms

Where:

Tsearch is FFS.

T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = Trs.

Tprocessing is FFS.

Tmargin is time for SSB post-processing. Tmargin can be up to 2ms.

TIU is FFS.

Trs is FFS.

In the interruption requirement a cell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds otherwise it is unknown. Relevant cell identification requirements are described in Clause 9.2.5 for intra-frequency handover and Clause 9.3.4 for inter-frequency handover.

<End of Change>