3GPP TSG-RAN WG4 Meeting#101                	R4-2118769

E-meeting, November 1st – 12th, 2021

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on Inter-band UL CA for IBM

Agenda item:	8.4.6.2

Document for:	Discussion

Introduction

In last meeting RAN4#100 progress was made on the topic of inter-band UL CA for IBM. A number of agreements were reached [1]:

- Agreements:

  - Existing interruption requirements at UL carrier RRC reconfiguration can be reused in R17.

Issue 2-1-2: Interruption at active BWP switching

- Agreements:

  - Interruption at active BWP switching is the same as the existing interruption requirement

Issue 2-1-3: DL interruption due to UE Tx switching between two UL carriers

- Agreements:

  - Not define DL interruption at UE Tx switching between two uplink carriers for FR2 inter-band CA, unless there is further discussion in RF session first.

Only one open was left, and this is addressed next

Discussion

In the last meeting one open item was left for further discussion:

Issue 2-1-4 DL interruption at NR SRS carrier based switching

Candidate options:

  - Option 1: To further consider whether interruption due to SRS carrier based switching can be limited to the band in which SRS carrier based switching is taking place.

    - The decision shall be subject to confirmation by RF session, and the details are FFS, e.g. whether it depends on the signalling/triggering mechanism of SRS carrier based switching.

  - Option 2: Interruption due to SRS carrier switching in one band will occur on all serving cells within FR if UE supports per FR gaps; otherwise, interruption occurs on all serving cells.

  - Option 3: RAN4 need to study whether and/or how to define the interruption requirements for inter-band SRS carrier based switching in FR2

    - Option 3a: Wait for RF room on conclusion of applicable SRS carrier switching time for inter-band CA in FR2

Our understanding is that NR SRS carrier-based switching is not defined for FR2 in RF. Hence, RRM should not discuss RRM requirements related to the topic before agreement have been made in RF room related to NR SRS carrier-based switching. We suggest waiting progress in RF room.

RRM discontinue the discussion related to NR SRS carrier-based switching and wait RF progress.

Conclusion

In last meeting RAN4#100 progress was made on the topic of inter-band UL CA for IBM. One item was left open for which we propose:

- RRM discontinue the discussion related to NR SRS carrier-based switching and wait RF progress.

References

- R4-2115332, WF on RRM requirements for FR2 Inter-band DL CA and UL CA, Nokia, Nokia Shanghai Bell.