3GPP TSG-RAN WG4 Meeting #101-e	R4-2118819

Electronic Meeting, November 1 – 12, 2021

Title: 	Simulation results for cell detection, L3 measurement and L1 RSRP measurement

Source: 	Huawei, HiSilicon

Agenda item:	8.20.3.1.5

Document for:	Discussion

Introduction

In this contribution, we provide the simulation results of cell detection (including PSS/SSS detection and PBCH detection), L3 measurements and L1-RSRP measurements for R17 RedCap evaluation.

Simulation Results

Simulation Results of PSS/SSS Detection

Based on the simulation assumptions in [1], the simulation results of PSS/SSS detection time with 1Rx and 2Rx are provided in this section.

Table 2.1-1: PSS/SSS detection time (Unit: samples), SCS=15kHz

| Channel | SINR | 1Rx |  |  |  | 2Rx |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Case 1 | Case 2 | Case 3 | Case 4 | Case 1 | Case 2 | Case 3 | Case 4 |
| AWGN | -6dB | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| TDL-B 100ns | -6dB | 3 | 3 | 3 | 3 | 2 | 2 | 2 | 2 |
| TDL-C 300ns | -6dB | 3 | 3 | 3 | 3 | 2 | 2 | 2 | 2 |

Table 2.1-2: PSS/SSS detection time (Unit: samples), SCS=30kHz

| Channel | SINR | 1Rx |  |  |  | 2Rx |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Case 1 | Case 2 | Case 3 | Case 4 | Case 1 | Case 2 | Case 3 | Case 4 |
| AWGN | -6dB | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| TDL-B 100ns | -6dB | 3 | 3 | 3 | 3 | 2 | 2 | 2 | 2 |
| TDL-C 300ns | -6dB | 3 | 3 | 3 | 3 | 2 | 2 | 2 | 2 |

Table 2.1-3: PSS/SSS detection time (Unit: samples), SCS=120kHz

| Channel | SINR | 1Rx |  |  |  | 2Rx |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Case 1 | Case 2 | Case 3 | Case 4 | Case 1 | Case 2 | Case 3 | Case 4 |
| AWGN | -6dB | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| TDL-A 30ns | -6dB | 3 | 3 | 3 | 3 | 2 | 2 | 2 | 2 |

Table 2.1-4: PSS/SSS detection time (Unit: samples), SCS=240kHz

| Channel | SINR | 1Rx |  |  |  | 2Rx |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Case 1 | Case 2 | Case 3 | Case 4 | Case 1 | Case 2 | Case 3 | Case 4 |
| AWGN | -6dB | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| TDL-A 30ns | -6dB | 3 | 3 | 3 | 3 | 2 | 2 | 2 | 2 |

Simulation Results of PBCH detection

Based on the simulation assumptions in [2], the simulation results of PBCH detection time with 1Rx are provided in this section.

DMRS time index reading

Table 2.2.1-1: DMRS time index reading time (Unit: samples), AWGN

| SINR | AWGN |  |  |  |
| --- | --- | --- | --- | --- |
|  | SCS=15kHz | SCS=30kHz | SCS=120kHz | SCS=240kHz |
| -8dB | 3 | 3 | 3 | 3 |
| -7dB | 3 | 3 | 3 | 3 |
| -6dB | 2 | 2 | 2 | 2 |
| -5dB | 1 | 1 | 1 | 1 |
| -4dB | 1 | 1 | 1 | 1 |
| -3dB | 1 | 1 | 1 | 1 |

Table 2.2.1-2: DMRS time index reading time (Unit: samples), TDL-A

| SINR | TDL-A 30ns |  |  |  |
| --- | --- | --- | --- | --- |
|  | SCS=15kHz | SCS=30kHz | SCS=120kHz | SCS=240kHz |
| -6dB | 6 | 6 | 6 | 6 |
| -4dB | 5 | 5 | 5 | 5 |
| -2dB | 4 | 4 | 4 | 4 |
| 0dB | 3 | 3 | 3 | 2 |

Table 2.2.1-3: DMRS time index reading time (Unit: samples), TDL-B/C

| SINR | TDL-B 100ns |  | TDL-B 300ns |  |
| --- | --- | --- | --- | --- |
|  | SCS=15kHz | SCS=30kHz | SCS=15kHz | SCS=30kHz |
| -6dB | 6 | 6 | 6 | 6 |
| -4dB | 5 | 5 | 5 | 5 |
| -2dB | 4 | 4 | 4 | 3 |
| 0dB | 3 | 3 | 3 | 3 |

PBCH reading

Table 2.2.2-1: SI reading time (Unit: samples), AWGN

| SINR | AWGN |  |  |  |
| --- | --- | --- | --- | --- |
|  | SCS=15kHz | SCS=30kHz | SCS=120kHz | SCS=240kHz |
| -8dB | 10 | 10 | 10 | 10 |
| -7dB | 8 | 8 | 8 | 8 |
| -6dB | 3 | 3 | 3 | 3 |
| -5dB | 2 | 2 | 2 | 2 |
| -4dB | 1 | 1 | 1 | 1 |
| -3dB | 1 | 1 | 1 | 1 |

Table 2.2.2-2: SI reading time (Unit: samples), TDL-A

| SINR | TDL-A 30ns |  |  |  |
| --- | --- | --- | --- | --- |
|  | SCS=15kHz | SCS=30kHz | SCS=120kHz | SCS=240kHz |
| -6dB | 9 | 10 | 10 | 10 |
| -4dB | 6 | 6 | 6 | 6 |
| -2dB | 4 | 4 | 4 | 4 |
| 0dB | 3 | 3 | 3 | 2 |

Table 2.2.2-3: SI reading time (Unit: samples), TDL-B/C

| SINR | TDL-B 100ns |  | TDL-B 300ns |  |
| --- | --- | --- | --- | --- |
|  | SCS=15kHz | SCS=30kHz | SCS=15kHz | SCS=30kHz |
| -6dB | 10 | 10 | 10 | 11 |
| -4dB | 6 | 6 | 6 | 6 |
| -2dB | 4 | 4 | 4 | 3 |
| 0dB | 3 | 3 | 3 | 3 |

Simulation Results of SSB based L3 measurements

Based on the simulation assumptions in [3], the simulation results of SS-RSRP, SS-RSRQ and SS-SINR with 1Rx and 2Rx are provided in this section.

SS-RSRP for L3 measurements

Table 2.1.1-1: Delta SS-RSRP for L3 measurements (SCS=15kHz)

| Channel | Sample Number | SINR | 1Rx |  |  | 2Rx |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5% | 50% | 95% | 5% | 50% | 95% |
| AWGN | 5 | -6dB | -1.10 | -0.05 | 0.85 | -0.76 | -0.05 | 0.62 |
|  | 6 | -6dB | -1.01 | -0.04 | 0.81 | -0.70 | -0.05 | 0.56 |
|  | 7 | -6dB | -0.92 | -0.06 | 0.58 | -0.67 | -0.05 | 0.48 |
|  | 8 | -6dB | -0.85 | -0.03 | 0.69 | -0.61 | -0.05 | 0.48 |
| TDL-B 100ns | 5 | -6dB | -1.25 | -0.03 | 0.92 | -0.82 | -0.04 | 0.66 |
|  | 6 | -6dB | -1.14 | -0.05 | 0.89 | -0.77 | -0.05 | 0.58 |
|  | 7 | -6dB | -1.03 | -0.04 | 0.79 | -0.67 | -0.05 | 0.54 |
|  | 8 | -6dB | -0.91 | -0.05 | 0.73 | -0.64 | -0.04 | 0.49 |
| TDL-C 300ns | 5 | -6dB | -1.26 | -0.06 | 0.92 | -0.81 | -0.04 | 0.60 |
|  | 6 | -6dB | -1.09 | -0.04 | 0.85 | -0.72 | -0.03 | 0.58 |
|  | 7 | -6dB | -1.05 | -0.06 | 0.76 | -0.70 | -0.04 | 0.55 |
|  | 8 | -6dB | -0.97 | -0.06 | 0.66 | -0.67 | -0.04 | 0.49 |

Table 2.3.1-2: Delta SS-RSRP for L3 measurements (SCS=30kHz)

| Channel | Sample Number | SINR | 1Rx |  |  | 2Rx |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5% | 50% | 95% | 5% | 50% | 95% |
| AWGN | 5 | -6dB | -1.00 | -0.04 | 0.88 | -0.77 | -0.04 | 0.61 |
|  | 6 | -6dB | -1.03 | -0.04 | 0.73 | -0.66 | -0.05 | 0.54 |
|  | 7 | -6dB | -0.94 | -0.03 | 0.74 | -0.66 | -0.04 | 0.49 |
|  | 8 | -6dB | -0.92 | -0.07 | 0.67 | -0.60 | -0.03 | 0.46 |
| TDL-B 100ns | 5 | -6dB | -1.23 | -0.05 | 0.93 | -0.81 | -0.04 | 0.63 |
|  | 6 | -6dB | -1.08 | -0.02 | 0.86 | -0.73 | -0.03 | 0.57 |
|  | 7 | -6dB | -0.96 | -0.03 | 0.79 | -0.70 | -0.03 | 0.54 |
|  | 8 | -6dB | -0.96 | -0.10 | 0.70 | -0.64 | -0.04 | 0.47 |
| TDL-C 300ns | 5 | -6dB | -1.21 | -0.08 | 0.89 | -0.81 | -0.05 | 0.64 |
|  | 6 | -6dB | -1.02 | -0.04 | 0.82 | -0.74 | -0.05 | 0.58 |
|  | 7 | -6dB | -0.99 | -0.07 | 0.77 | -0.65 | -0.04 | 0.54 |
|  | 8 | -6dB | -0.97 | -0.06 | 0.67 | -0.60 | -0.04 | 0.47 |

Table 2.3.1-3: Delta SS-RSRP for L3 measurements (SCS=120kHz)

| Channel | Sample Number | SINR | 1Rx |  |  | 2Rx |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5% | 50% | 95% | 5% | 50% | 95% |
| AWGN | 5 | -6dB | -1.09 | -0.05 | 0.87 | -0.78 | -0.07 | 0.64 |
|  | 6 | -6dB | -1.00 | -0.03 | 0.76 | -0.68 | -0.03 | 0.56 |
|  | 7 | -6dB | -0.94 | -0.05 | 0.74 | -0.64 | -0.03 | 0.50 |
|  | 8 | -6dB | -0.84 | -0.04 | 0.65 | -0.60 | -0.04 | 0.49 |
| TDL-A 30ns | 5 | -6dB | -1.30 | -0.06 | 0.91 | -0.83 | -0.05 | 0.62 |
|  | 6 | -6dB | -1.10 | -0.06 | 0.88 | -0.74 | -0.06 | 0.56 |
|  | 7 | -6dB | -1.04 | -0.05 | 0.79 | -0.68 | -0.04 | 0.54 |
|  | 8 | -6dB | -1.01 | -0.07 | 0.72 | -0.62 | -0.05 | 0.49 |

Table 2.3.1-4: Delta SS-RSRP for L3 measurements (SCS=240kHz)

| Channel | Sample Number | SINR | 1Rx |  |  | 2Rx |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5% | 50% | 95% | 5% | 50% | 95% |
| AWGN | 5 | -6dB | -1.09 | -0.04 | 0.87 | -0.77 | -0.04 | 0.61 |
|  | 6 | -6dB | -1.00 | -0.04 | 0.82 | -0.69 | -0.05 | 0.56 |
|  | 7 | -6dB | -0.94 | -0.04 | 0.71 | -0.66 | -0.07 | 0.50 |
|  | 8 | -6dB | -0.86 | -0.04 | 0.66 | -0.61 | -0.03 | 0.48 |
| TDL-A 30ns | 5 | -6dB | -1.26 | -0.08 | 0.92 | -0.86 | -0.07 | 0.61 |
|  | 6 | -6dB | -1.07 | -0.05 | 0.83 | -0.75 | -0.05 | 0.55 |
|  | 7 | -6dB | -1.02 | -0.06 | 0.80 | -0.67 | -0.06 | 0.51 |
|  | 8 | -6dB | -0.94 | -0.06 | 0.76 | -0.65 | -0.06 | 0.47 |

SS-RSRQ for L3 measurements

Table 2.3.2-1: Delta SS-RSRQ for L3 measurements (SCS=15kHz)

| Channel | Sample Number | SINR | 1Rx |  |  | 2Rx |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5% | 50% | 95% | 5% | 50% | 95% |
| AWGN | 5 | -6dB | -1.09 | -0.04 | 0.78 | -0.76 | -0.05 | 0.57 |
| TDL-B 100ns | 5 | -6dB | -1.37 | -0.04 | 0.95 | -0.83 | -0.05 | 0.64 |
| TDL-C 300ns | 5 | -6dB | -1.26 | -0.05 | 0.92 | -0.84 | -0.05 | 0.62 |

Table 2.3.2-2: Delta SS-RSRQ for L3 measurements (SCS=30kHz)

| Channel | Sample Number | SINR | 1Rx |  |  | 2Rx |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5% | 50% | 95% | 5% | 50% | 95% |
| AWGN | 5 | -6dB | -1.04 | -0.05 | 0.79 | -0.74 | -0.04 | 0.56 |
| TDL-B 100ns | 5 | -6dB | -1.23 | -0.05 | 0.89 | -0.81 | -0.05 | 0.59 |
| TDL-C 300ns | 5 | -6dB | -1.17 | -0.08 | 0.85 | -0.78 | -0.06 | 0.61 |

Table 2.3.2-3: Delta SS-RSRQ for L3 measurements (SCS=120kHz)

| Channel | Sample Number | SINR | 1Rx |  |  | 2Rx |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5% | 50% | 95% | 5% | 50% | 95% |
| AWGN | 5 | -6dB | -1.08 | -0.06 | 0.84 | -0.77 | -0.03 | 0.58 |
| TDL-A 30ns | 5 | -6dB | -1.20 | -0.06 | 0.89 | -1.82 | -0.05 | 0.61 |

Table 2.3.2-4: Delta SS-RSRQ for L3 measurements (SCS=240kHz)

| Channel | Sample Number | SINR | 1Rx |  |  | 2Rx |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5% | 50% | 95% | 5% | 50% | 95% |
| AWGN | 5 | -6dB | -1.05 | -0.03 | 0.78 | -0.76 | -0.05 | 0.59 |
| TDL-A 30ns | 5 | -6dB | -1.21 | -0.08 | 0.89 | -0.83 | -0.07 | 0.59 |

SS-SINR for L3 measurements

Table 2.3.3-1: Delta SS-SINR for L3 measurements (SCS=15kHz)

| Channel | Sample Number | SINR | 1Rx |  |  | 2Rx |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5% | 50% | 95% | 5% | 50% | 95% |
| AWGN | 5 | -6dB | -1.19 | -0.04 | 1.00 | -0.80 | 0.02 | 0.75 |
| TDL-B 100ns | 5 | -6dB | -1.43 | 0.02 | 1.17 | -0.84 | 0.01 | 0.82 |
| TDL-C 300ns | 5 | -6dB | -1.32 | -0.01 | 1.18 | -0.86 | -0.01 | 0.81 |

Table 2.3.3-2: Delta SS-SINR for L3 measurements (SCS=30kHz)

| Channel | Sample Number | SINR | 1Rx |  |  | 2Rx |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5% | 50% | 95% | 5% | 50% | 95% |
| AWGN | 5 | -6dB | -1.12 | -0.01 | 1.04 | -0.78 | 0.02 | 0.73 |
| TDL-B 100ns | 5 | -6dB | -1.32 | 0.01 | 1.11 | -0.88 | 0.02 | 0.80 |
| TDL-C 300ns | 5 | -6dB | -1.28 | -0.04 | 1.10 | -0.84 | -0.01 | 0.80 |

Table 2.3.3-3: Delta SS-SINR for L3 measurements (SCS=120kHz)

| Channel | Sample Number | SINR | 1Rx |  |  | 2Rx |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5% | 50% | 95% | 5% | 50% | 95% |
| AWGN | 5 | -6dB | -1.18 | -0.01 | 1.05 | -0.81 | 0.02 | 0.75 |
| TDL-A 30ns | 5 | -6dB | -1.27 | -0.01 | 1.10 | -0.85 | 0.00 | 0.81 |

Table 2.3.3-4: Delta SS-SINR for L3 measurements (SCS=240kHz)

| Channel | Sample Number | SINR | 1Rx |  |  | 2Rx |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5% | 50% | 95% | 5% | 50% | 95% |
| AWGN | 5 | -6dB | -1.15 | 0.05 | 1.00 | -0.83 | 0.01 | 0.76 |
| TDL-A 30ns | 5 | -6dB | -1.27 | -0.04 | 1.11 | -0.87 | -0.02 | 0.77 |

Simulation Results of L1-RSRP measurements

Based on the simulation assumptions in [4], the simulation results of SSB based and CSI-RS based L1-RSRP measurements with 1Rx and 2Rx are provided in this section.

SSB based L1-RSRP

Table 2.4.1-1: Delta L1-RSRP measured on SSB (SCS=15kHz)

| Channel | Sample Number | SINR | 1Rx |  |  | 2Rx |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5% | 50% | 95% | 5% | 50% | 95% |
| AWGN | 1 | -3dB | -1.66 | -0.10 | 1.23 | -1.12 | -0.05 | 0.86 |
|  |  | -2dB | -1.44 | -0.03 | 1.09 | -1.00 | -0.05 | 0.80 |
|  |  | -1dB | -1.26 | -0.05 | 0.97 | -0.82 | -0.03 | 0.72 |
|  |  | 0dB | -1.06 | -0.02 | 0.89 | -0.73 | -0.02 | 0.60 |
|  | 3 | -3dB | -0.87 | -0.04 | 0.71 | -0.65 | -0.06 | 0.51 |
|  |  | -2dB | -0.79 | -0.03 | 0.65 | -0.56 | -0.04 | 0.46 |
|  |  | -1dB | -0.68 | -0.05 | 0.58 | -0.50 | -0.05 | 0.39 |
|  |  | 0dB | -0.60 | -0.05 | 0.50 | -0.43 | -0.04 | 0.33 |
| TDL-B 100ns | 1 | -3dB | -3.22 | -0.08 | 1.96 | -1.58 | -0.05 | 1.10 |
|  |  | -2dB | -2.35 | -0.03 | 1.63 | -1.31 | -0.04 | 1.03 |
|  |  | -1dB | -2.03 | -0.08 | 1.36 | -1.18 | -0.04 | 0.87 |
|  |  | 0dB | -1.83 | -0.06 | 1.24 | -0.94 | -0.05 | 0.77 |
|  | 3 | -3dB | -1.05 | -0.05 | 0.89 | -0.73 | -0.04 | 0.54 |
|  |  | -2dB | -0.98 | -0.07 | 0.72 | -0.61 | -0.04 | 0.49 |
|  |  | -1dB | -0.81 | -0.05 | 0.69 | -0.52 | -0.04 | 0.41 |
|  |  | 0dB | -0.75 | -0.06 | 0.55 | -0.47 | -0.04 | 0.37 |
| TDL-C 300ns | 1 | -3dB | -2.73 | -0.08 | 1.68 | -1.44 | -0.04 | 1.15 |
|  |  | -2dB | -2.17 | -0.04 | 1.52 | -1.31 | -0.06 | 0.99 |
|  |  | -1dB | -1.83 | -0.07 | 1.39 | -1.05 | -0.05 | 0.85 |
|  |  | 0dB | -1.65 | -0.07 | 1.11 | -0.95 | -0.08 | 0.65 |
|  | 3 | -3dB | -0.65 | -0.06 | 0.51 | -0.77 | -0.07 | 0.56 |
|  |  | -2dB | -0.56 | -0.04 | 0.46 | -0.61 | -0.05 | 0.44 |
|  |  | -1dB | -0.50 | -0.05 | 0.39 | -0.54 | -0.04 | 0.41 |
|  |  | 0dB | -0.43 | -0.04 | 0.33 | -0.48 | -0.05 | 0.36 |

Table 2.4.1-2: Delta L1-RSRP measured on SSB (SCS=30kHz)

| Channel | Sample Number | SINR | 1Rx |  |  | 2Rx |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5% | 50% | 95% | 5% | 50% | 95% |
| AWGN | 1 | -3dB | -1.64 | -0.10 | 1.23 | -1.20 | -0.10 | 0.86 |
|  |  | -2dB | -1.47 | -0.04 | 1.09 | -1.00 | -0.04 | 0.77 |
|  |  | -1dB | -1.21 | -0.06 | 0.93 | -0.84 | -0.04 | 0.69 |
|  |  | 0dB | -1.10 | -0.07 | 0.85 | -0.78 | -0.07 | 0.61 |
|  | 3 | -3dB | -0.92 | -0.06 | 0.67 | -0.60 | -0.03 | 0.49 |
|  |  | -2dB | -0.77 | -0.04 | 0.65 | -0.56 | -0.05 | 0.44 |
|  |  | -1dB | -0.68 | -0.04 | 0.58 | -0.48 | -0.04 | 0.38 |
|  |  | 0dB | -0.60 | -0.04 | 0.52 | -0.45 | -0.04 | 0.34 |
| TDL-B 100ns | 1 | -3dB | -2.73 | -0.08 | 1.73 | -1.42 | -0.06 | 1.00 |
|  |  | -2dB | -2.05 | -0.07 | 1.46 | -1.20 | -0.07 | 0.87 |
|  |  | -1dB | -1.86 | -0.03 | 1.37 | -1.13 | -0.06 | 0.81 |
|  |  | 0dB | -1.64 | -0.07 | 1.14 | -0.89 | -0.02 | 0.75 |
|  | 3 | -3dB | -1.11 | -0.07 | 0.78 | -0.67 | -0.04 | 0.54 |
|  |  | -2dB | -0.90 | -0.06 | 0.66 | -0.61 | -0.04 | 0.47 |
|  |  | -1dB | -0.85 | -0.05 | 0.62 | -0.54 | -0.04 | 0.40 |
|  |  | 0dB | -0.67 | -0.04 | 0.54 | -0.47 | -0.05 | 0.36 |
| TDL-C 300ns | 1 | -3dB | -2.26 | -0.11 | 1.50 | -1.34 | -0.05 | 0.98 |
|  |  | -2dB | -1.07 | -0.07 | 1.39 | -1.19 | -0.07 | 0.86 |
|  |  | -1dB | -1.60 | -0.05 | 1.24 | -0.96 | -0.06 | 0.73 |
|  |  | 0dB | -1.47 | -0.07 | 1.05 | -0.83 | -0.06 | 0.67 |
|  | 3 | -3dB | -1.04 | -0.04 | 0.49 | -0.67 | -0.07 | 0.52 |
|  |  | -2dB | -0.89 | -0.07 | 0.44 | -0.60 | -0.06 | 0.45 |
|  |  | -1dB | -0.83 | -0.06 | 0.38 | -0.55 | -0.05 | 0.39 |
|  |  | 0dB | -0.67 | -0.06 | 0.34 | -0.46 | -0.04 | 0.35 |

Table 2.4.1-3: Delta L1-RSRP measured on SSB (SCS=120kHz)

| Channel | Sample Number | SINR | 1Rx |  |  | 2Rx |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5% | 50% | 95% | 5% | 50% | 95% |
| AWGN | 1 | -3dB | -1.67 | -0.03 | 1.22 | -1.20 | -0.05 | 0.89 |
|  |  | -2dB | -1.42 | -0.05 | 1.10 | -0.99 | -0.03 | 0.82 |
|  |  | -1dB | -1.22 | -0.08 | 0.96 | -0.81 | -0.02 | 0.67 |
|  |  | 0dB | -1.05 | -0.05 | 0.84 | -0.77 | -0.03 | 0.58 |
|  | 3 | -3dB | -0.92 | -0.04 | 0.74 | -0.65 | -0.04 | 0.55 |
|  |  | -2dB | -0.78 | -0.04 | 0.66 | -0.56 | -0.03 | 0.47 |
|  |  | -1dB | -0.69 | -0.05 | 0.54 | -0.53 | -0.03 | 0.41 |
|  |  | 0dB | -0.57 | -0.03 | 0.48 | -0.41 | -0.02 | 0.34 |
| TDL-A 30ns | 1 | -3dB | -2.75 | -0.09 | 1.82 | -1.48 | -0.05 | 1.05 |
|  |  | -2dB | -2.27 | -0.06 | 1.52 | -1.37 | -0.06 | 0.94 |
|  |  | -1dB | -1.89 | -0.05 | 1.36 | -0.97 | -0.05 | 0.87 |
|  |  | 0dB | -1.72 | -0.02 | 1.31 | -0.90 | -0.05 | 0.70 |
|  | 3 | -3dB | -1.08 | -0.08 | 0.78 | -0.68 | -0.04 | 0.53 |
|  |  | -2dB | -0.93 | -0.05 | 0.73 | -0.65 | -0.05 | 0.45 |
|  |  | -1dB | -0.82 | -0.06 | 0.65 | -0.54 | -0.03 | 0.42 |
|  |  | 0dB | -0.70 | -0.02 | 0.55 | -0.48 | -0.04 | 0.36 |

Table 2.4.1-4: Delta L1-RSRP measured on SSB (SCS=240kHz)

| Channel | Sample Number | SINR | 1Rx |  |  | 2Rx |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5% | 50% | 95% | 5% | 50% | 95% |
| AWGN | 1 | -3dB | -1.76 | -0.07 | 1.22 | -1.11 | -0.05 | 0.89 |
|  |  | -2dB | -1.47 | -0.09 | 1.04 | -1.03 | -0.05 | 0.74 |
|  |  | -1dB | -1.24 | -0.05 | 0.95 | -0.88 | -0.03 | 0.67 |
|  |  | 0dB | -1.07 | -0.07 | 0.85 | -0.75 | -0.03 | 0.61 |
|  | 3 | -3dB | -0.92 | -0.04 | 0.73 | -0.65 | -0.04 | 0.52 |
|  |  | -2dB | -0.77 | -0.05 | 0.63 | -0.56 | -0.05 | 0.45 |
|  |  | -1dB | -0.70 | -0.05 | 0.55 | -0.48 | -0.03 | 0.39 |
|  |  | 0dB | -0.61 | -0.05 | 0.48 | -0.43 | -0.03 | 0.35 |
| TDL-A 30ns | 1 | -3dB | -2.46 | -0.12 | 1.63 | -1.45 | -0.08 | 1.04 |
|  |  | -2dB | -2.05 | -0.07 | 1.41 | -1.25 | -0.04 | 0.92 |
|  |  | -1dB | -1.90 | -0.07 | 1.24 | -1.05 | -0.09 | 0.76 |
|  |  | 0dB | -1.46 | -0.06 | 1.15 | -0.88 | -0.04 | 0.71 |
|  | 3 | -3dB | -1.01 | -0.06 | 0.82 | -0.70 | -0.05 | 0.52 |
|  |  | -2dB | -0.90 | -0.05 | 0.74 | -0.59 | -0.06 | 0.46 |
|  |  | -1dB | -0.81 | -0.05 | 0.64 | -0.55 | -0.06 | 0.38 |
|  |  | 0dB | -0.67 | -0.05 | 0.54 | -0.50 | -0.06 | 0.36 |

CSI-RS based L1-RSRP

L1-RSRP based on 24RB with different Rx

Table 2.4.2-1: Delta L1-RSRP measured on CSI-RS (SCS=15kHz)

| Channel | Sample Number | SINR | 1Rx |  |  | 2Rx |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5% | 50% | 95% | 5% | 50% | 95% |
| AWGN | 1 | -3dB | -2.36 | -0.14 | 1.53 | -1.59 | -0.12 | 1.12 |
|  |  | -2dB | -2.04 | -0.14 | 1.29 | -1.39 | -0.10 | 0.99 |
|  |  | -1dB | -1.67 | -0.10 | 1.19 | -1.24 | -0.11 | 0.85 |
|  |  | 0dB | -1.50 | -0.13 | 1.07 | -1.02 | -0.10 | 0.76 |
|  | 3 | -3dB | -1.34 | -0.11 | 0.95 | -0.92 | -0.10 | 0.64 |
|  |  | -2dB | -1.11 | -0.08 | 0.76 | -0.78 | -0.08 | 0.55 |
|  |  | -1dB | -1.02 | -0.10 | 0.68 | -0.72 | -0.09 | 0.47 |
|  |  | 0dB | -0.84 | -0.11 | 0.65 | -0.63 | -0.09 | 0.42 |
| TDL-B 100ns | 1 | -3dB | -3.44 | -0.10 | 2.22 | -2.10 | -0.14 | 1.31 |
|  |  | -2dB | -2.95 | -0.15 | 1.78 | -1.73 | -0.12 | 1.17 |
|  |  | -1dB | -2.77 | -0.14 | 1.60 | -1.47 | -0.11 | 0.98 |
|  |  | 0dB | -2.45 | -0.13 | 1.60 | -1.17 | -0.08 | 0.92 |
|  | 3 | -3dB | -1.53 | -0.09 | 1.00 | -0.97 | -0.10 | 0.70 |
|  |  | -2dB | -1.35 | -0.09 | 0.89 | -0.86 | -0.08 | 0.57 |
|  |  | -1dB | -1.07 | -0.10 | 0.82 | -0.77 | -0.09 | 0.51 |
|  |  | 0dB | -0.97 | -0.09 | 0.68 | -0.66 | -0.09 | 0.44 |
| TDL-C 300ns | 1 | -3dB | -3.18 | -0.16 | 1.82 | -1.91 | -0.15 | 1.19 |
|  |  | -2dB | -2.76 | -0.15 | 1.58 | -1.60 | -0.10 | 1.04 |
|  |  | -1dB | -2.40 | -0.16 | 1.43 | -1.39 | -0.13 | 0.95 |
|  |  | 0dB | -1.98 | -0.15 | 1.19 | -1.28 | -0.13 | 0.78 |
|  | 3 | -3dB | -1.58 | -0.17 | 0.93 | -0.92 | -0.11 | 0.66 |
|  |  | -2dB | -1.23 | -0.12 | 0.86 | -0.82 | -0.10 | 0.58 |
|  |  | -1dB | -1.16 | -0.12 | 0.67 | -0.78 | -0.10 | 0.48 |
|  |  | 0dB | -0.91 | -0.12 | 0.64 | -0.69 | -0.12 | 0.41 |

Table 2.4.2-2: Delta L1-RSRP measured on CSI-RS (SCS=30kHz)

| Channel | Sample Number | SINR | 1Rx |  |  | 2Rx |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5% | 50% | 95% | 5% | 50% | 95% |
| AWGN | 1 | -3dB | -2.30 | -0.14 | 1.79 | -1.59 | -0.06 | 1.06 |
|  |  | -2dB | -2.07 | -0.06 | 1.57 | -1.46 | -0.10 | 1.01 |
|  |  | -1dB | -1.71 | -0.10 | 1.21 | -1.19 | -0.06 | 0.86 |
|  |  | 0dB | -1.56 | -0.13 | 1.02 | -1.03 | -0.10 | 0.79 |
|  | 3 | -3dB | -1.26 | -0.11 | 0.94 | -0.93 | -0.07 | 0.65 |
|  |  | -2dB | -1.11 | -0.10 | 0.83 | -0.81 | -0.11 | 0.53 |
|  |  | -1dB | -0.95 | -0.10 | 0.71 | -0.73 | -0.10 | 0.48 |
|  |  | 0dB | -0.91 | -0.09 | 0.61 | -0.61 | -0.07 | 0.40 |
| TDL-B 100ns | 1 | -3dB | -2.98 | -0.09 | 1.93 | -1.67 | -0.11 | 1.20 |
|  |  | -2dB | -2.45 | -0.14 | 1.68 | -1.61 | -0.09 | 1.13 |
|  |  | -1dB | -2.21 | -0.13 | 1.43 | -1.34 | -0.07 | 0.97 |
|  |  | 0dB | -1.90 | -0.11 | 1.40 | -1.16 | -0.12 | 0.80 |
|  | 3 | -3dB | -1.40 | -0.10 | 0.98 | -0.94 | -0.10 | 0.64 |
|  |  | -2dB | -1.18 | -0.09 | 0.86 | -0.86 | -0.11 | 0.60 |
|  |  | -1dB | -1.10 | -0.08 | 0.80 | -0.73 | -0.10 | 0.47 |
|  |  | 0dB | -0.92 | -0.07 | 0.65 | -0.61 | -0.09 | 0.41 |
| TDL-C 300ns | 1 | -3dB | -3.58 | -0.38 | 1.48 | -2.16 | -0.31 | 1.02 |
|  |  | -2dB | -2.79 | -0.27 | 1.37 | -1.84 | -0.30 | 0.83 |
|  |  | -1dB | -2.58 | -0.32 | 1.13 | -1.70 | -0.29 | 0.76 |
|  |  | 0dB | -2.22 | -0.31 | 1.07 | -1.37 | -0.26 | 0.69 |
|  | 3 | -3dB | -1.73 | -0.32 | 0.72 | -1.16 | -0.28 | 0.48 |
|  |  | -2dB | -1.48 | -0.27 | 0.67 | -1.04 | -0.27 | 0.42 |
|  |  | -1dB | -1.33 | -0.28 | 0.58 | -0.94 | -0.28 | 0.33 |
|  |  | 0dB | -1.23 | -0.28 | 0.43 | -0.88 | -0.27 | 0.25 |

Table 2.4.2-3: Delta L1-RSRP measured on CSI-RS (SCS=60kHz)

| Channel | Sample Number | SINR | 1Rx |  |  | 2Rx |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5% | 50% | 95% | 5% | 50% | 95% |
| AWGN | 1 | -3dB | -2.35 | -0.12 | 1.55 | -1.61 | -0.11 | 1.06 |
|  |  | -2dB | -2.12 | -0.16 | 1.38 | -1.35 | -0.13 | 1.23 |
|  |  | -1dB | -1.66 | -0.15 | 1.18 | -1.19 | -0.10 | 1.10 |
|  |  | 0dB | -1.56 | -0.10 | 1.08 | -1.08 | -0.09 | 0.98 |
|  | 3 | -3dB | -1.26 | -0.13 | 0.87 | -0.87 | -0.09 | 0.68 |
|  |  | -2dB | -1.06 | -0.12 | 0.79 | -0.81 | -0.08 | 0.54 |
|  |  | -1dB | -0.95 | -0.11 | 0.68 | -0.68 | -0.07 | 0.47 |
|  |  | 0dB | -0.88 | -0.09 | 0.56 | -0.62 | -0.08 | 0.41 |
| TDL-A 30ns | 1 | -3dB | -3.50 | -0.13 | 1.91 | -1.96 | -0.11 | 1.21 |
|  |  | -2dB | -2.88 | -0.20 | 1.82 | -1.66 | -0.10 | 1.08 |
|  |  | -1dB | -2.35 | -0.14 | 1.45 | -1.37 | -0.09 | 0.97 |
|  |  | 0dB | -2.07 | -0.15 | 1.35 | -1.26 | -0.11 | 0.81 |
|  | 3 | -3dB | -1.45 | -0.12 | 0.97 | -0.97 | -0.12 | 0.67 |
|  |  | -2dB | -1.23 | -0.06 | 0.89 | -0.82 | -0.09 | 0.58 |
|  |  | -1dB | -1.09 | -0.10 | 0.78 | -0.75 | -0.11 | 0.49 |
|  |  | 0dB | -0.98 | -0.11 | 0.68 | -0.63 | -0.09 | 0.45 |

Table 2.4.2-4: Delta L1-RSRP measured on CSI-RS (SCS=120kHz)

| Channel | Sample Number | SINR | 1Rx |  |  | 2Rx |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5% | 50% | 95% | 5% | 50% | 95% |
| AWGN | 1 | -3dB | -2.44 | -0.18 | 1.56 | -1.53 | -0.15 | 1.06 |
|  |  | -2dB | -1.94 | -0.13 | 1.34 | -1.36 | -0.12 | 1.00 |
|  |  | -1dB | -1.77 | -0.11 | 1.15 | -1.24 | -0.10 | 0.85 |
|  |  | 0dB | -1.48 | -0.11 | 1.05 | -1.00 | -0.09 | 0.75 |
|  | 3 | -3dB | -1.32 | -0.12 | 0.89 | -0.90 | -0.10 | 0.62 |
|  |  | -2dB | -1.18 | -0.11 | 0.76 | -0.80 | -0.10 | 0.59 |
|  |  | -1dB | -0.98 | -0.12 | 0.69 | -0.70 | -0.08 | 0.47 |
|  |  | 0dB | -0.86 | -0.10 | 0.60 | -0.62 | -0.10 | 0.42 |
| TDL-A 30ns | 1 | -3dB | -3.34 | -0.07 | 2.26 | -2.06 | -0.11 | 1.31 |
|  |  | -2dB | -3.13 | -0.15 | 1.81 | -1.78 | -0.09 | 1.09 |
|  |  | -1dB | -2.65 | -0.12 | 1.56 | -1.45 | -0.09 | 0.99 |
|  |  | 0dB | -2.18 | -0.10 | 1.47 | -1.28 | -0.09 | 0.90 |
|  | 3 | -3dB | -1.61 | -0.11 | 0.98 | -0.97 | -0.12 | 0.67 |
|  |  | -2dB | -1.21 | -0.10 | 0.86 | -0.84 | -0.08 | 0.58 |
|  |  | -1dB | -1.09 | -0.10 | 0.83 | -0.73 | -0.10 | 0.51 |
|  |  | 0dB | -0.99 | -0.10 | 0.70 | -0.63 | -0.10 | 0.44 |

L1-RSRP based 2Rx with different RBs

Table 2.4.2-5: Delta L1-RSRP measured on CSI-RS (SCS=15kHz)

| Channel | Sample Number | SINR | 24RB |  |  | 48RB |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5% | 50% | 95% | 5% | 50% | 95% |
| AWGN | 1 | -3dB | -1.59 | -0.12 | 1.12 | -1.11 | -0.07 | 0.85 |
|  |  | -2dB | -1.39 | -0.10 | 0.99 | -0.89 | -0.05 | 0.73 |
|  |  | -1dB | -1.24 | -0.11 | 0.85 | -0.82 | -0.04 | 0.66 |
|  |  | 0dB | -1.02 | -0.10 | 0.76 | -0.71 | -0.05 | 0.55 |
| TDL-B 100ns | 1 | -3dB | -2.10 | -0.14 | 1.31 | -1.26 | -0.08 | 0.91 |
|  |  | -2dB | -1.73 | -0.12 | 1.17 | -1.04 | -0.07 | 0.79 |
|  |  | -1dB | -1.47 | -0.11 | 0.98 | -0.89 | -0.06 | 0.63 |
|  |  | 0dB | -1.17 | -0.08 | 0.92 | -0.85 | -0.05 | 0.60 |
| TDL-C 300ns | 1 | -3dB | -1.91 | -0.15 | 1.19 | -1.22 | -0.08 | 0.82 |
|  |  | -2dB | -1.60 | -0.10 | 1.04 | -1.05 | -0.06 | 0.77 |
|  |  | -1dB | -1.39 | -0.13 | 0.95 | -0.89 | -0.05 | 0.67 |
|  |  | 0dB | -1.28 | -0.13 | 0.78 | -0.82 | -0.07 | 0.58 |

Table 2.4.2-6: Delta L1-RSRP measured on CSI-RS (SCS=30kHz)

| Channel | Sample Number | SINR | 1Rx |  |  | 2Rx |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5% | 50% | 95% | 5% | 50% | 95% |
| AWGN | 1 | -3dB | -1.59 | -0.06 | 1.06 | -1.02 | -0.08 | 0.81 |
|  |  | -2dB | -1.46 | -0.10 | 1.01 | -0.88 | -0.04 | 0.73 |
|  |  | -1dB | -1.19 | -0.06 | 0.86 | -0.76 | -0.05 | 0.64 |
|  |  | 0dB | -1.03 | -0.10 | 0.79 | -0.72 | -0.06 | 0.57 |
| TDL-B 100ns | 1 | -3dB | -1.67 | -0.11 | 1.20 | -1.21 | -0.06 | 0.88 |
|  |  | -2dB | -1.61 | -0.09 | 1.13 | -0.97 | -0.08 | 0.80 |
|  |  | -1dB | -1.34 | -0.07 | 0.97 | -0.92 | -0.07 | 0.70 |
|  |  | 0dB | -1.16 | -0.12 | 0.80 | -0.80 | -0.03 | 0.60 |
| TDL-C 300ns | 1 | -3dB | -2.16 | -0.31 | 1.02 | -1.51 | -0.24 | 0.75 |
|  |  | -2dB | -1.84 | -0.30 | 0.83 | -1.30 | -0.22 | 0.59 |
|  |  | -1dB | -1.70 | -0.29 | 0.76 | -1.23 | -0.25 | 0.49 |
|  |  | 0dB | -1.37 | -0.26 | 0.69 | -1.05 | -0.26 | 0.41 |

Table 2.4.2-7: Delta L1-RSRP measured on CSI-RS (SCS=60kHz)

| Channel | Sample Number | SINR | 1Rx |  |  | 2Rx |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5% | 50% | 95% | 5% | 50% | 95% |
| AWGN | 1 | -3dB | -1.61 | -0.11 | 1.06 | -1.10 | -0.05 | 0.81 |
|  |  | -2dB | -1.35 | -0.13 | 1.23 | -0.93 | -0.07 | 0.70 |
|  |  | -1dB | -1.19 | -0.10 | 1.10 | -0.77 | -0.07 | 0.65 |
|  |  | 0dB | -1.08 | -0.09 | 0.98 | -0.71 | -0.05 | 0.54 |
| TDL-A 30ns | 1 | -3dB | -1.96 | -0.11 | 1.21 | -1.36 | -0.08 | 1.01 |
|  |  | -2dB | -1.66 | -0.10 | 1.08 | -1.10 | -0.05 | 0.88 |
|  |  | -1dB | -1.37 | -0.09 | 0.97 | -1.01 | -0.05 | 0.72 |
|  |  | 0dB | -1.26 | -0.11 | 0.81 | -0.80 | -0.05 | 0.65 |

Table 2.4.2-8: Delta L1-RSRP measured on CSI-RS (SCS=120kHz)

| Channel | Sample Number | SINR | 1Rx |  |  | 2Rx |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5% | 50% | 95% | 5% | 50% | 95% |
| AWGN | 1 | -3dB | -1.53 | -0.15 | 1.06 | -1.06 | -0.06 | 0.84 |
|  |  | -2dB | -1.36 | -0.12 | 1.00 | -0.93 | -0.06 | 0.70 |
|  |  | -1dB | -1.24 | -0.10 | 0.85 | -0.78 | -0.03 | 0.63 |
|  |  | 0dB | -1.00 | -0.09 | 0.75 | -0.73 | -0.07 | 0.52 |
| TDL-A 30ns | 1 | -3dB | -2.06 | -0.11 | 1.31 | -1.15 | -0.07 | 0.90 |
|  |  | -2dB | -1.78 | -0.09 | 1.09 | -1.08 | -0.07 | 0.81 |
|  |  | -1dB | -1.45 | -0.09 | 0.99 | -0.85 | -0.05 | 0.70 |
|  |  | 0dB | -1.28 | -0.09 | 0.90 | -0.77 | -0.07 | 0.58 |

Conclusions

This contribution provides the simulation results of cell detection and L3/L1 measurements for NR RedCap UE evaluation. It is suggested to consider the above results when investigating RRM measurements and L1-RSRP measurements requirements for NR RedCap UE.

Reference

- R4-2115359, “Simulation assumptions for RedCap cell detection performance”, vivo

- R4-2115363, “Simulation assumptions for RedCap PBCH detection”, vivo

- R4-2115360, “Simulation assumptions for RedCap SSB based RRM measurement performance”, Huawei, HiSilicon

- R4-2115362, “Simulation assumptions for RedCap L1 RSRP measurement performance”, Ericsson