3GPP TSG-RAN WG4 Meeting # 101-e									R4-2118842

Electronic Meeting, November 01-12, 2021

Agenda item:	8.10.2.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM requirements for SRS antenna port switching

Document for:	Discussion

# Introduction

The requirements of SRS antenna switching were discussed in the last RAN4 meeting, and the agreements and open issues are captured in the WF [1]. There are still some open issues left unsettled, in terms of scheduling restriction, impact to other requirements, and framework of interruption requirements, etc. In this paper, we further provide our views on these left issues.

# Discussion

### 2.1 Scope of SRS antenna switching requirements

Whether to have scheduling restrictions on the carrier where SRS AS switching occurs was discussed with the following agreements:

- Agreements: - Do not define the scheduling restriction on symbols before and after SRS transmission for the cell with SRS antenna port switching and on SRS transmit symbols in Rel-17 - Performance degradation on these symbols can be expected - FFS whether and how to capture this in TS 38.133

The discussion about whether to have scheduling restriction is about UE performing the antenna port switching before and after SRS AS transmission, and the relation to the transient period defined in RF spec. However, by looking at the agreement, it is ambiguous that SRS transmit symbols are also involved where the antenna will not switched on these symbols. And it is also strange to say the scheduling restriction on SRS transmit symbols as RAN1 has defined the rules when PUCCCH/PUSCH and SRS symbols are overlapped. Thus, it should be clarified at very beginning that the performance degradation are intended for symbols before and after SRS transmission to perform antenna switch instead of the SRS transmit symbols.

Observation 1: Performance degradation are intended for symbols before and after SRS transmission to perform antenna switch instead of the SRS transmit symbols.

Companies have consistent understanding that the performance on symbols before and after SRS transmission may be impact due to antenna port switching. As companies pointed out in the last meeting, there are different cases on how SRS resources are allocated within a slot. As elaborated in Fig.1, in case 1, there is only one SRS AS resource allocated within the slot. Then the performance on symbol #10 and symbol #13 is impacted. For case 2, there are two SRS AS resource allocated within the slot, and according to RAN1 spec TS 38.214, there is guard period between these two resources whether any transmission is not expected. Then the performance degradation on symbol#7 and symbol#12 can be expected, and the restriction on guard symbols should not be changed. For cases 3, the SRS AS resource is allocated at the end of current slot which is followed by a DL slot. For this case, the performance on symbol#11 and symbol#0 in the next slot could be expected.

Fig.1 SRS AS resource allocation within a slot

Based on the analysis above, it could happen that the symbols before or after the SRS transmission are overlapped with the guard period defined in RAN1 spec. From our understanding, it is reasonable not to change the behavior/restrictions of the guard period, where UE does not transmit any other signals. On these symbols, there is no need to clarify in the RAN4 spec about the performance degradation as there is no transmission scheduled.

Observation 2: There is no need to clarify the performance degradation on symbols overlapped with guard period as there is no transmission scheduled.

The remaining point is whether and how to capture the potential performance degradation in TS 38.133. Some companies suggested that there is already transient period in 38.101-1 on symbols before and after SRS resource. From our understanding, it is preferred to have clear clarification on which symbols the performance can be impacted. Based on the analysis above, it is suggested to capture in TS 38.133 that the performance degradation can be expected on 1 OFDM symbol before and after each SRS resource configured for antenna switching which is not overlapped with the guard period defined in TS 38.214 on the carrier where SRS antenna switching occurs.

Proposal 1: It is suggested to capture in TS 38.133 that the performance degradation can be expected on 1 OFDM symbol before and after each SRS resource configured for antenna switching which is not overlapped with the guard period defined in TS 38.214 on the carrier where SRS antenna switching occurs.

Regarding the scope of different SRS resource configuration, the status are summarized as follows:

Issue 1-1-2: RAN4 requirement scope with different SRS resource configuration - FFS - Option 1 (CATT): - Only defining the requirements for the scenario that the SRS resources of a set are transmitted in the same slot with consecutive SRS transmission. - Option 2 (Nokia, Ericsson): - RAN4 shall define the requirements for the following scenarios in Rel17 where - The SRS resources of a set are transmitted NOT in the same slot, or - The SRS resources of a set are transmitted in the same slot with consecutive SRS transmission - RAN4 do not define the requirements if the SRS resources of a set are transmitted in the same slot with non-consecutive SRS transmission, before the guard period in this scenario gets clarified in RAN1. - Option 3 (Apple, Xiaomi, Huawei, QC, vivo, CATT): - RAN4 to define a generic requirement regardless of SRS resource configuration. - Option 3a: define interruption requirements for a limited set of SRS configurations.

Companies have concerns on the case when SRS resource of a set are transmitted in the same slot with “non-consecutive” SRS transmission. It is the case that the symbols between two SRS resource of an SRS set within a slot is larger the minimum guard period. For instance, with SCS of 15 KHz, the minimum guard period is 1 symbols as defined in TS38.214. It is possible that two SRS resources are spaced by more than two OFDM symbols. Some companies argued that for this case, which is the guard period that UE performs antenna switching is not clear and it needs clarification in RAN1. However, from our understanding, RAN1 spec only defines the minimum guard period between two SRS resources, there is no need to specify which symbol that UE will use for antenna switching. From requirement perspective, the interruption on victim cells could be defined in a generic manner。

Proposal 3: RAN4 to define a generic requirement regardless of SRS resource configuration.

### 2.2 Impact of SRS antenna port switching to other requirements

The agreements and status issues are summarized as follows:

Issue 1-2-1: Impact of SRS antenna port switching to RRM requirements in NR-SA - Agreement in 2nd round: - For NR-SA, UE is not required to perform SRS antenna port switching when SRS resource and the NR measurement, i.e., L3 measurement and RLM/BFD/CBD, are scheduled in the same OFDM symbol. - FFS when SRS resource and L1-RSRP/L1-SINR measurement are scheduled in the same OFDM symbol. Issue 1-2-2: Impact of SRS antenna port switching to RRM requirements in EN-DC or NE-DC - Agreements in 2nd round - For EN-DC, UE is not required to perform SRS antenna port switching when SRS resource and the NR measurement, i.e., L3 measurement and RLM/BFD/CBD, are scheduled in the same OFDM symbol in the SCG. - For NE-DC, UE is not required to perform SRS antenna port switching when SRS resource and the NR measurement, i.e., L3 measurement and RLM/BFD/CBD, are scheduled in the same OFDM symbol in the MCG. - FFS on L1-RSRP/L1-SINR (follow the conclusion from issue 1-2-1) Issue 1-2-3: Impact of SRS antenna port switching to RRM requirements in NR-DC - Agreements in 2nd round - For NR-DC, UE is not required to perform SRS antenna port switching when SRS resource and the NR measurement, i.e., L3 measurement and RLM/BFD/CBD, are scheduled in the same OFDM symbol in the same CG. - FFS on L1-RSRP/L1-SINR (follow the conclusion from issue 1-2-1)

The only remaining issue whether SRS or L1-RSRP/L1-SINR measurement to be prioritized when SRS collide with DL RS for L1-RSRP/L1-SINR. First, we would like to elaborate that it happens when CC1 is configured for SRS AS transmission which collides with the DL RS on another CC for L1-RSRP/L1-SINR. Some companies referred to the RAN1 spec in TS 38.214 clause 6.2.1 about the collision between SRS resource and PUCCH carrying L1 report. From our understanding, it is for the case when PUCCH and SRS are on the same carrier as shown below. But here we are discussing collision between SRS AS on one carrier and DL RS for L1-RSRP/L1-SINR on another carrier which will be impacted by SRS AS indicated by txSwitchImpactToRx.

Observation 3: The impact on L1-RSRP/L1-SINR measurement is for the case when collision happens between SRS AS on one carrier and DL RS for L1-RSRP/L1-SINR on another carrier which will be impacted by SRS AS indicated by txSwitchImpactToRx.

TS 38.214 6.2.1 For PUCCH and SRS on the same carrier, a UE shall not transmit SRS when semi-persistent or periodic SRS is configured in the same symbol(s) with PUCCH carrying only CSI report(s), or only L1-RSRP report(s), or only L1-SINR report(s). A UE shall not transmit SRS when semi-persistent or periodic SRS is configured or aperiodic SRS is triggered to be transmitted in the same symbol(s) with PUCCH carrying HARQ-ACK, link recovery request (as defined in clause 9.2.4 of [6, 38.213]) and/or SR. In the case that SRS is not transmitted due to overlap with PUCCH, only the SRS symbol(s) that overlap with PUCCH symbol(s) are dropped. PUCCH shall not be transmitted when aperiodic SRS is triggered to be transmitted to overlap in the same symbol with PUCCH carrying semi-persistent/periodic CSI report(s) or semi-persistent/periodic L1-RSRP report(s) only, or only L1-SINR report(s). TS 38.214 6.2.1.3 For a carrier of a serving cell with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, the UE shall not transmit SRS whenever SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell and PUSCH/PUCCH transmission carrying HARQ-ACK/positive SR/RI/CRI/SSBRI and/or PRACH happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability included in [13, TS 38.306]. For a carrier of a serving cell with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, the UE shall not transmit a periodic/semi-persistent SRS whenever periodic/semi-persistent SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell and PUSCH transmission carrying aperiodic CSI happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability included in [13, TS 38.306]. For a carrier of a serving cell with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, the UE shall drop PUCCH/PUSCH transmission carrying periodic/semi-persistent CSI comprising only CQI/PMI/L1-RSRP/L1-SINR, and/or SRS transmission on another serving cell configured for PUSCH/PUCCH transmission whenever the transmission and SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the serving cell happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability included in [13, TS 38.306]. For a carrier of a serving cell with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, the UE shall drop PUSCH transmission carrying aperiodic CSI comprising only CQI/PMI/L1-RSRP/L1-SINR whenever the transmission and aperiodic SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133]) as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability included in [13, TS 38.306].

Then during the discussion in the last meeting, companies proposed to follow the same principle of SRS carrier switching defined in TS 38.214 clause 6.2.1.3, which is for the case when SRS collides with UL transmission among carriers. The logic is if the report carrier L1-RSRP/L1-SINR is dropped, then there is no need to prioritize the measurement procedure. From our understanding, it is reasonable to following the existing rules defined in RAN1 spec though it is not for SRS AS specifically. According to the rules defined in TS38.214 clause 6.2.1.3, if CSI report carries SSBRI or CRI, SRS are always deprioritized. If CSI report only carries L1-RSRP or L1-SINR, only aperiodic CSI report are prioritized when overlapped with P/SP SRS. From our understanding, in most cases, the SSBRI/CRI and L1-RSRP/L1-SINR are reported together to NW, otherwise NW is not able to tell on which resource the L1 results are measure. Thus, it is proposed that NR measurement are always prioritized including L3 measurement, RLM/BFD/CBD and L1-RSRP/L1-SINR measurement.

Proposal 4: NR measurement are always prioritized including L3 measurement, RLM/BFD/CBD and L1-RSRP/L1-SINR measurement.

There was also discussion on impact to other RRM specific requirements are prioritization rules. The agreements and status are summarized as follows:

Issue 1-2-4: Impact of SRS antenna port switching to other specific RRM requirements - Agreements in 2nd round - RAN4 to clarify that other specific RRM requirements except for the NR measurements only applies when no SRS antenna port switching occurs during those RRM activities. - RAN4 not to define prioritizing rules of SRS antenna switching and other specific RRM requirements except for the NR measurements. - FFS whether spec clarification/note is needed to reflect the above agreements in the corresponding requirement section. Issue 1-2-5: Impact of SRS antenna port switching to CSF and other RS - FFS - Option 1 (CATT): If it is necessary to check, RAN4 can make requirements and test for the collision cases of SRS and PUSCH/PUCCH transmission carrying HARQ-ACK/positive SR/RI/CRI/SSBRI and/or PRACH according to the rules defined in clause 6.2.1 in RAN1specification. - Option 2 (Apple, QC, OPPO, vivo): Scheduling of SRS antenna switching should avoid collision to all reference signals including CSI-IM except DMRS and UCI containing CSF report. If collision happens, it is considered as an error case and no UE requirement is imposed. - Option 3 (vivo): Send LS to RAN1 to check the prioritization rule for SRS antenna switching, especially for the case in CA/DC operation. - Option 3a (Ericsson, Nokia): Prioritization between scheduling of SRS antenna switching and transmission of certain signals and channels is to be handled by RAN1. If anything is unclear, RAN4 shall send LS to RAN1 and ask for clarification. - Option 4 (Huawei, Ericsson, Nokia, CATT): Not to define the rules to avoid collisions except what has been define in RAN1 and NR measurement. Issue 1-5-2: Interruption requirement related with prioritization rule in RAN1 - FFS - Option 1 (MTK, Apple, OPPO, vivo): - For SRS antenna port switching, define the interruption requirement for the case not covered by TS 38.214, e.g., add a note “if the case has no priority rule defined in TS 38.214, UE is allowed to cause the interruption on the other CC(s)”. - Option 2 (Nokia, Ericsson) : - The SRS transmission will cause UL interruption on the other carriers if simultaneous SRS and PUCCH/PUSCH transmissions are not supported by the UE. - Option 3 (Huawei, Ericsson, CATT): - Not to define the rules to avoid collisions except what has been define in RAN1 and NR measurement. Issue 1-5-3: Two SRS colliding on same symbol - FFS - For SRS antenna port switching (FR1 only), when two SRS resources having the same time domain behavior, i.e., both SRS resources are periodic, semi-persistent or aperiodic, are scheduled on the same OFDM symbol: - Option1 (Apple): Two different SRS resources having the same time domain behavior to be scheduled on the same OFDM symbol are not expected. - Option2: Up to UE implementation. - Option3 (Nokia): This is up to RAN1 discussion.

Regarding the impact to other specific RRM requirements, it was agreed that RRM requirements except for the NR measurements only applies when no SRS antenna port switching occurs during those RRM activities and not to define prioritization rules in RAN4 spec. The remaining issue is whether and how to capture the principles in RAN4 spec. From our understanding, there is no need to further capture this in the spec.

Proposal 5: No specification impact of impact of SRS antenna port switching to other specific RRM requirements except for NR measurement.

For other issues regarding the prioritization rules between SRS with different time domain behavior or between SRS resource with other RS, it is within RAN1 scope and it is not related to the RRM requirements of SRS AS and the impact to existing RRM requirements.

Observation 4: The rules of scheduling of SRS and other RS/transmission is within RAN1 scope, and no conclusion shall be made in RAN4, and it is not related to the RRM requirements of SRS AS and the impact to existing RRM requirements.

Proposal 6: 	Not to define the rules to avoid collisions except what has been define in RAN1 and NR measurement.

### 2.3 Interruption requirement design

Regarding the details of interruption requirements, the most controversial issue is whether to define interruption requirements in slot level or symbol level. Different from the analysis above about the scheduling restriction on aggressor CCs, it is hard to allocate the precise impacted symbols among victim CCs due to MRTD and TA. Though the actual duration of antenna switching is much less than the length of a slot, as there is no reliable test method to verify the symbol level interruption, the promising way is to define the interruption requirements based on slot level. And the composites of interruption requirement for SRS antenna port switching in FR1 include SRS transmission time (6 symbols as minimum requirements) and antenna switching time before and after SRS transmission (2*15us).

Proposal 7: The components within interruption time of SRS antenna port switching in FR1 include SRS transmission time (6 symbols as minimum requirements) and antenna switching time before and after SRS transmission (2*15us).

It was agreed in the last meeting to first define the interruption requirements for async case first. Based on the above proposal, the interruption in number of slots for asyc case is proposed in the following table.

Table I Interruption requirement in number of slots

|  | Aggressor CC SCS(kHz) |  |  |
| --- | --- | --- | --- |
| Victim CC SCS (kHz) | 15 | 30 | 60 |
| 15 | 2 | 2 | 2 |
| 30 | 2 | 2 | 2 |
| 60 | 3 | 2 | 2 |
| 120 | 5 | 3 | 3 |

Proposal 8: Define the interruption requirements for asyc case as Table I above.

# Conclusions

Observation 1: Performance degradation are intended for symbols before and after SRS transmission to perform antenna switch instead of the SRS transmit symbols.

Observation 2: There is no need to clarify the performance degradation on symbols overlapped with guard period as there is no transmission scheduled.

Proposal 1: It is suggested to capture in TS 38.133 that the performance degradation can be expected on 1 OFDM symbol before and after each SRS resource configured for antenna switching which is not overlapped with the guard period defined in TS 38.214 on the carrier where SRS antenna switching occurs.

Proposal 3: RAN4 to define a generic requirement regardless of SRS resource configuration.

Observation 3: The impact on L1-RSRP/L1-SINR measurement is for the case when collision happens between SRS AS on one carrier and DL RS for L1-RSRP/L1-SINR on another carrier which will be impacted by SRS AS indicated by txSwitchImpactToRx.

Proposal 4: NR measurement are always prioritized including L3 measurement, RLM/BFD/CBD and L1-RSRP/L1-SINR measurement.

Proposal 5: No specification impact of impact of SRS antenna port switching to other specific RRM requirements except for NR measurement.

Observation 4: The rules of scheduling of SRS and other RS/transmission is within RAN1 scope, and no conclusion shall be made in RAN4, and it is not related to the RRM requirements of SRS AS and the impact to existing RRM requirements.

Proposal 6: 	Not to define the rules to avoid collisions except what has been define in RAN1 and NR measurement.

Proposal 7: The components within interruption time of SRS antenna port switching in FR1 include SRS transmission time (6 symbols as minimum requirements) and antenna switching time before and after SRS transmission (2*15us).

Proposal 8: Define the interruption requirements for asyc case as following table。

|  | Aggressor CC SCS(kHz) |  |  |
| --- | --- | --- | --- |
| Victim CC SCS (kHz) | 15 | 30 | 60 |
| 15 | 2 | 2 | 2 |
| 30 | 2 | 2 | 2 |
| 60 | 3 | 2 | 2 |
| 120 | 5 | 3 | 3 |

# References

[1] R4-2115336 WF on further RRM enhancement for NR and MR-DC - SRS antenna port switching, Apple