3GPP TSG-RAN WG4 Meeting #101-e	R4-2118862

Electronic Meeting, November 1-12, 2021

Source:	China Telecom

Title:	On CSI requirements for UE MMSE-IRC receiver for inter-cell interference suppression

Agenda Item:	8.12.2.1.2

Document for:	Discussion

- Introduction

In the RAN4#100-e meeting, we discussed the test parameters for CQI reporting requirements for UEs supporting MMSE-IRC receiver for suppressing inter-cell interference, and the WF was agreed in [1].

In this paper, we give our views on the remaining open issues.

- Discussion

General Issues

Status in [1]:

Issue 1-2: Whether to define PMI reporting requirements with inter-cell interference scenario?

  - Option 1: Yes

  - Option 2: Need further discussion

  - Option 3: No

For PMI reporting, in the last meeting, there were companies analysis show that UE PMI calculation will not be obviously impacted under ICI scenario with or without MMSE-IRC processing. Therefore, currently it is of less meaningful to additionally define PMI reporting requirements with inter-cell interference scenario. For this meeting, we suggest to go with option 3 unless some PMI calculation difference can be shown.

Proposal 1: For this meeting, make decision not to define PMI reporting requirements with inter-cell interference scenario unless some PMI calculation difference can be shown.

Interference Model

Status in [1]:

Interference signal with serving cell’s NZP CSI-RS and CSI-IM REs

- CSI-IM on target cell

  - CSI-IM overlaps with PDSCH from interference is the baseline scenario for defining CSI reporting requirement.

  - Other scenarios are FFS:

- Option 1: Only consider overlapping with PDSCH from interference

- Option 2: Further include overlapping with NZP CSI-RS from interference

- Option 3: Further include overlapping with Interf. cell CSI-IM

- Option 4: Make further down selection based on simulation results and also take practical network configuration into account.

- NZP CSI-RS on target cell

  - Option 1: Overlaps with PDSCH from interference

  - Option 2: Overlaps with NZP CSI-RS from interference

  - Option 3: Make further down selection based on simulation results and also take practical network configuration into account.

As for the interference model, since we have decided to use 1Tx with static channel model for the interference cell, there will be no difference between neighbor cell’s PDSCH and NZP CEI-RS in terms of interference level. Therefore, similar performance can be expected with target CSI-IM overlapped with neighbor PDSCH or NZP CSI-RS, and with target NZP CSI-RS overlapped with neighbor PDSCH or NZP CSI-RS.

As a result, for target CSI-IM, we propose only to consider overlapping with PDSCH from interference, and for NZP CSI-RS on target cell, we slightly prefer overlaps with NZP CSI-RS from interference since it is more practical.

Observation 1: Since we have decided to use 1Tx with static channel model for the interference cell, there will be no difference between neighbor cell’s PDSCH and NZP CEI-RS in terms of interference level.

Proposal 2: For target CSI-IM, we propose only to consider overlapping with PDSCH from interference, and for NZP CSI-RS on target cell, we slightly prefer overlaps with NZP CSI-RS from interference since it is more practical.

- Conclusions

The following proposals were given for CQI reporting requirements:

Proposal 1: For this meeting, make decision not to define PMI reporting requirements with inter-cell interference scenario unless some PMI calculation difference can be shown.

Observation 1: Since we have decided to use 1Tx with static channel model for the interference cell, there will be no difference between neighbor cell’s PDSCH and NZP CEI-RS in terms of interference level.

Proposal 2: For target CSI-IM, we propose only to consider overlapping with PDSCH from interference, and for NZP CSI-RS on target cell, we slightly prefer overlaps with NZP CSI-RS from interference since it is more practical.

- References

R4-2115732, WF on CQI reporting requirements for inter-cell interference MMSE-IRC, Ericsson, RAN4 #100-e, Aug 2021.