3GPP TSG-RAN WG4 Meeting #101-e	R4-2118891

Electronic Meeting, 1-12 Nov, 2021

Title:	R17 FR1 Redcap UE power class

Source:	OPPO

Agenda item:	8.20.2.1.1

Document for:	Approval

- Introduction

In RAN4#101e, it was agreed that for RedCap UE power class PC3 should be specified for Redcap in FR1, and PC2 for RedCap UE depending on operator request, and No PC1 and PC1.5 for RedCap UE in FR1. Then in the following RAN#93e, the discussion of supporting more bands happened, it was agreed in [1] as below table. This paper further looks into the power class that Redcap UE can support.

Moderator’s proposal for discussion in Friday GTW: • In Rel-17, there will be no work on any RedCap specific specification update for any of the following: • RedCap UEs also supporting V2X/PC5 on n47 • RedCap UEs operating in unlicensed bands • RedCap UEs supporting SUL • The specification will not contain any explicit restriction to prevent implementation of RedCap UEs with these features. • Note: The consequence of this agreement would be: • If any spec change/addition is found necessary in order to enable one of the options above then it will not happen in Rel-17.

- Discussion

In 38.101-1, there is only PC5 (20dBm) defined for unlicensed bands n46 and n96. Therefore, if Redcap UE operating in unlicensed bands is allowed, then at least PC5 needs to be allowed.

Observation 1:    In 38.101-1 only power class 5 (20dBm) is supported for unlicensed bands.

Proposal 1:         PC5 is defined for Redcap UE when it works under unlicensed bands.

It was agreed in [2] that PC2 is based on operator requests, and as far as we know, in some countries the NWs are deployed according to PC2 UE power class. For these countries, most likely the PC2 is needed for Redcap UE. If it is the case, then one questions is whether 2Tx should be defined for Redcap UE.

Observation 2:    Some NWs are deployed according to PC2, in this case PC2 might need to be supported by Redcap UE.

In the WID [3], there is no explicit description about the Tx part and only mentions about the Rx branches and DL MIMO as below. The straight forward interpretation is that both 1Tx and 2Tx can be supported by Redcap UE, more specifically, Redcap UEs could be 1T1R, 1T2R, 2T2R. And for Redcap UE with 2Tx and power class 2, does TxD is applied? If it is, then the handling of normal UE from UL MIMO fallback to single antenna port (1T or 2T+TxD) should also apply.

Observation 3:    In the WID, there is no explicit description about the Tx part and only mentions about the Rx branches and DL MIMO which makes the Tx capability is ambiguous.

Observation 4:    Redcap UEs could be 1T1R, 1T2R, or 2T2R.

Observation 5:    Redcap UEs with 2Tx and PC2, TxD should be applied as normal UE.

Proposal 2:         It is proposed that Redcap UE architectures could be 1T1R, 1T2R, or 2T2R.

Proposal 3:         It is proposed that TxD could be used for Redcap UEs to achieve PC2 if supported.

- References

[1] RP-212634, summary for discussion [93e-16-RedCap-WI], RAN#93e

[2] R4-2115096, WF on the RedCap RF, RAN4#100e

[3] RP-210918, Revised WID on support of reduced capability NR devices, RAN#91e