3GPP TSG-RAN WG4 Meeting #101-e		R4-2118903
Electronic Meeting, 1st Nov. – 12th Nov., 2021

Agenda item:	8.1.3.3

Source:	OPPO

Title:	The reference curves of channel model validation

Document for:	Approval

1	Introduction

In the last meeting of RAN4 #100-e, the channel model reference curves are aligned between CE vendors as the WF agreement [1].

This paper presents our further considerations and proposals for FR1 channel model validation.

2	Discussion

The approach of beam simultaneous (beam combined) is proposed by Keysight and CMCC in RAN4 #98-e [2]. And as the paper presented, the approach is for validation of spatial correlation, but not PDP validation.

And in the RAN4 #99-e meeting, the WF agreement on gNB beam usage criteria for channel model validation is that keep both beam specific and beam simultaneous in the TR at this stage.

After checked with the endorsed reference channel model curves in RAN4 #100-e meeting [3], for UMa CDL-C, it is found that the beam simultaneous curves are proposed, but beam specific curves for PDP are not in the paper. So as the previous WF agreement, the proposal is that the reference channel model curves with beam specific approach should be introduced.

Proposal 1: The reference channel model curves for UMa CDL-C with beam specific approach should be introduced for channel model validation.

And the beam usage criteria for each items of channel model validation are proposed as follow.

|  | Beam specific | Beam simultaneous |
| --- | --- | --- |
| Power Delay Profile (PDP) | √ |  |
| Temporal Correlation Function (TCF) | √ |  |
| Spatial Correlation Function (SCF) | √ | √ |
| V/H Ratio | √ |  |

Proposal 2: adopt the above beam usage criteria for channel model validation.

3	Conclusion

Proposal 1: The reference channel model curves for UMa CDL-C with beam specific approach should be introduced for channel model validation.

Proposal 2: adopt the following beam usage criteria for channel model validation.

|  | Beam specific | Beam simultaneous |
| --- | --- | --- |
| Power Delay Profile (PDP) | √ |  |
| Temporal Correlation Function (TCF) | √ |  |
| Spatial Correlation Function (SCF) | √ | √ |
| V/H Ratio | √ |  |

References

[1] R4-2115756 WF on NR MIMO OTA

[2] R4-2102613 gNB Beam Usage_v3

[3] R4-2115759 Reference Curves