3GPP TSG-RAN WG4 Meeting #101-e	R4-2118948

# Electronic,  , 1st - 12th November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DRAFT | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of CCA model wifh DRX for NR-U perf requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-Perf |  |  |  |  |  | Date: |  |  | 2021-10-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | LCCA limit of CCA failures have to be adapted when DRX is configured in a test case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Distinction on how unavailable samples are considered depending on whether DRX is configured or not. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If LCCA is used for avoiding to reach Lmax on TCs with CCA, there is a high probability that all the CCA failures would fall in DRX innactive periods. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.26 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Submited to AI 5.1.1.5.1 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <Start of Change 1>

## A.3.26	CCA model

### A.3.26.1	Introduction

The CCA model is used in some RRM test cases with at least one cell on a carrier frequency with CCA. The intention with the CCA model is to emulate in the test equipment the behaviour of a gNB or UE which performs channel measurement to check that the channel is clear prior to performing one or more downlink or uplink transmissions.

### A.3.26.2	CCA model for operation on a carrier frequency with CCA in FR1

#### A.3.26.2.1	DL CCA model

The same DL CCA model is applicable regardless of whether DRX is used or not.

Prior to each DBT window, the test equipment shall determine whether the CCA attempt is successful (i.e., the corresponding signals have to be transmitted), based on probability PCCA_DL of successful CCA configured in the corresponding test case.

If the CCA attempt is successful for a transmission, then the test equipment shall transmit also other remaining transmissions, according to the configuration, within the same DBT window.

If the CCA attempt is not successful for a transmission within the DBT window, the test equipment shall determine whether the CCA attempt is successful for the next configured transmission, based on probability PCCA_DL.

The probability can be different in different time intervals Ti during a test case. One probability value (per cell) applies at any time point during a test; one or more probability values can be configured in the entire test, one value PCCA_DL per time interval Ti where i≥1, and the multiple time intervals (when i>1) do not overlap (e.g., PCCA_DL=1.0 in T1 and PCCA_DL=0.75 in T2)..

For semi-static channel access configuration, a single value PCCA_DL is used to configure the probability of CCA success in different time intervals Ti during a test realization.  An additional limit LCCA_DL is used to determine the maximum number of unavailable DBT samples within an evaluation window WCCA_DL. If the number of unavailable DBT samples on the last WCCA_DL DBT samples is larger or equal to LCCA_DL, the CCA attempt is considered successful for transmission.

For dynamic channel access configuration, the parameters PCCA_DL_1 and PCCA_DL_2 are used to configure the probability of CCA success on the first and second SSB candidate positions, respectively, in different time intervals Ti during a test realization. An additional limit LCCA_DL is used to determine the maximum number of unavailable DBT samples within an evaluation window WCCA_DL. If the number of unavailable DBT samples on the last WCCA_DL DBT samples is larger or equal to LCCA_DL, the CCA attempt is considered successful for transmission.

A counter lCCA is used o monitor the number of unavailable DBT samples withing an evaluation window WCCA_DL. If DRX is not configured, the counter lCCA is incremented for every unavailable DBT sample due to CCA failure, and discard samples that are outside of the evaluation window WCCA. If DRX is configured, the counter lCCA is incremented for every DRX cycle with at least one unavailable DBT sample due to CCA failure, and the samples that are outside of the evaluation window WCCA are discarded

For semi-static channel access configuration or for dynamic channel access configuration where one candidate SSB position is modeled, prior to each discovery burst transmission window within a time interval Ti of the test, the test equipment shall:

1 - Generate a uniform random variable p1 from the range [0, 1] for the first candidate position.

2 - Transmit the discovery burst based on p1 in the first candidate position. If p1 ≤ PCCA_DL, the discovery burst is transmitted at the first candidate SSB location; else if lCCA within a window WCCA_DL is larger or equal to LCCA_DL,  the discovery burst is transmitted at the first candidate SSB location, otherwise the discovery burst is muted.

For dynamic channel access configuration where two candidate SSB positions are modelled, prior to each discovery burst transmission window within a time interval Ti of the test, the test equipment shall:

1 - Generate a uniform random variable p1 from the range [0, 1] for the first candidate position.

2 - Transmit the discovery burst based on p1 in the first candidate position: if p1 ≤ PCCA_DL1, the discovery burst is transmitted at first candidate SSB location, else the test equipment shall:

a - Generate a uniform random variable p2 from the range [0, 1] for the second candidate SSB position.

b - Transmit the discovery burst based on p2 in the second candidate position.  If p2 ≤ PCCA_DL2, the discovery burst is transmitted at the second candidate SSB location; else if lCCA within a window WCCA_DL is larger or equal to LCCA_DL,  the discovery burst is transmitted at the second candidate SSB location, otherwise the discovery burst is muted.

The above steps are repeated for each discovery burst transmission window in each time interval Ti of the test. The limit LCCA_DL and window WCCA_DL is a configuration parameter for each test case.

#### A.3.26.2.2	UL CCA model

For UL CCA, the modelling approach is based on probability PCCA_UL of successful CCA. Probability PCCA_UL is configured in the corresponding test case, based on a set SCCA_UL of possible values including 75 % and  87% as typical values for dynamic and semi-static channel access configurations, 0% to model consistent UL CCA failures, and 100% to model no UL CCA failures.

Consistent UL CCA failures are modelled by configuring a low value for PCCA_UL, e.g., PCCA_UL = 0%.

In the same time interval Ti during the same test case, PCCA_UL can be different from PCCA_DL.

The probability can be different in different time intervals Ti during a test case. One probability value applies at any time point during a test; one or more probability values can be configured in the entire test, one value PCCA_UL per time interval Ti where I ≥ 1, and the multiple time intervals (when I > 1) do not overlap (e.g., PCCA_UL = 1.0 in T1 and PCCA_UL = 0.75 in T2).

TCCA µs prior to each UL transmission burst in the test, the test equipment (TE) shall generate a uniform random variable p from the range [0, 1]. If p<PCCA_UL, the TE transmits an OCNG noise pattern with an energy level X within the UE BW scheduled/configured for the UL transmission for at-least TCCA µs. Where TCCA µs is energy detection time for accessing the uplink channel as defined in section 5.1.1 of TS 37.106 [36]. Where:

- X is 3 dB above the energy detection threshold defined in section 5.1.1 of TS 37.106 [36].

- TCCA is the channel sensing period depending on CCA category for the next UL transmission.

The TE shall count the number of UL CCA failures, and no further UL CCA failures are modeled if the number of failures exceeds the limit LCCA_UL within a window WCCA_UL. For each UL CCA failure generated by the model, the TE shall monitor the corresponding UL resource for the desired UL signal, and based on when and/or whether the TE received the desired UL signal, it deems the test case to pass or fail.

In many cases, the requirement under a test depends on the number of configured signal occasions which are not available during the test, so the test equipment shall track how many such signal occasions are not transmitted in UL during the test period.

<End of Change 1>