3GPP TSG-RAN WG4 Meeting # 101-e                	R4-2118958

Electronic Meeting, 1st - 12th November 2021

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Discussion on RRM requirements for extension to 71 GHz

Agenda item: 	8.16.7.1

Document for: 	Discussion

Introduction

This paper presents Nokia’s view on RRM aspects related to the operation between 52.6 GHz and 71 GHz. This paper discusses the impact of operation above 52.6 GHz on requirements with CCA.

Discussion

FR2-2 operation with CCA

One important aspect of FR2-2 operation is the operation with CCA on unlicensed bands. Requirements with CCA already exist in the 38.133 specification, as they were introduced during the work of NR_unlic for FR1. Since NR_unlic was specified for FR1, all the new requirements with CCA introduced in Rel 16 were not specified for FR2. Some of the requirements with CCA may be understood as band agnostic, and many of those need to be reviewed for FR2-2 since there is no baseline FR2 that can be reused.

Most of the requirements with CCA are only applicable for FR1. For these requirements, adaptation for FR2-2 needs to be considered.

RAN4 to consider Table 1 for the discussion of the impact of FR2-2 for requirements with CCA.

Table 1 Analysis of the specification impact of the operation on FR2-2 for requirements with CCA

| Requirements |  | Impact for FR2-2 | Comments |
| --- | --- | --- | --- |
| Cell reselection | Measurement of serving cell | Yes | FR2 scaling factor N1 needs to be defined for requirements with CCA |
|  | Intra-frequency | Yes | FR2 scaling factor N1 needs to be defined for requirements with CCA FFS number of SSB candidate positions |
|  | Inter-frequency | Yes | FR2 scaling factor N1 needs to be defined for requirements with CCA FFS number of SSB candidate positions |
|  | Inter-RAT | No | Reuse requirements without CCA as for NR-U |
| Handover |  | yes | Requirements with CCA need to include - FR2-FR2 - FR2-FR1 - FR1-FR2 FFS number of SSB candidate positions |
| RRC Connection Mobility Control | RRC re-establishment | Yes | Time to identify target NR cell needs to be defined for FR2 requirements with CCA FFS number of SSB candidate positions |
|  | RA | No | Existing requirements with CCA are also applicable for FR2-2 |
|  | RRC release with redirection | Yes | Time to identify target NR cell needs to be defined for FR2 requirements with CCA |
| UE timing | UE transmit timing | FFS | FFS number of SSB candidate positions |
|  | UE maximum receive timing difference | No | Reuse FR2 or FR2-2 requirements |
|  | UE maximum transmission timing difference | No | Reuse FR2 or FR2-2 requirements |
|  | TA | No | Reuse FR2 or FR2-2 requirements |
|  | UE timer accuracy | No | Reuse FR2 or FR2-2 requirements |
| Signalling characteristics | SCell activation and deactivation delay | Yes | FR2-2 specific requirements need to be defined, like the Tactivation_time FFS number of SSB candidate positions |
|  | Interruption | FFS | FFS if impact on interruptions due to Active BWP switching Requirement |
|  | PSCell addition and release delay | No |  |
|  | Active TCI state switching delay | Yes | Requirements with CCA should include FR2-2 specific timing TL1-RSRP for MAC-CE based TCI state switch delay FFS number of SSB candidate positions |
|  | Active BWP switching delay | No | Impact is generic and covered with FR2-2 requirements without CCA |
|  | PSCell change | No |  |
|  | Conditional PSCell change | No |  |
|  | Radio link monitoring | Yes | The following needs to be defined for FR2-2 with CCA: - Maximum number of RLM resources, - Evaluation period TEvaluate_out_SSB,CCA and TEvaluate_in_SSB,CCA - Measurement restrictions - Scheduling availability of UE during radio link monitoring fir FR2-2 |
|  | Link recovery procedures | Yes | Define TEvaluate_BFD_SSB_CCA, TEvaluate_CBD_SSB_CCA for FR2-2 Define requirements for cheduling availability of UE performing beam failure detection on FR2-2 with CCA |
|  | Uplink spatial relation switch delay | No |  |
|  | UE specific CBW change | No |  |
|  | Pathloss reference signal switch delay | No |  |
| Measurement requirements | Measurement gap | No |  |
|  | UE measurement capability | No |  |
|  | Intra-frequency | Yes | Switching time for FR2 SS-RSRP, SS-RSRQ, SS-SINR side conditions for FR2 Number of Cell and SSBs for FR2 and others. FFS number of SSB candidate positions |
|  | Inter-frequency | Yes | Switching time for FR2 SS-RSRP, SS-RSRQ, SS-SINR side conditions for FR2 Number of Cell and SSBs for FR2 and others. FFS number of SSB candidate positions |
|  | Inter-RAT | No |  |
|  | L1-RSRP measurements for reporting | Yes | Side conditions nrofReportedRS must be defined for FR2 The measurement period of TL1-RSRP_Measurement_Period_SSB_CCA must be defined for FR2-2 FFS number of SSB candidate positions |
|  | L1-SINR measurements for reporting | No |  |
|  | Cross link interference measurements | No |  |
|  | CSI-RS based measurement | No |  |

Another point that was discussed during the NR_unlic work was the assumptions on how many SSB candidate positions that need to be monitored by a UE. RAN1 has specified as part of the NR-U work a feature to make the SSBs more robust against CCA failures. When using this feature, each SSB index may be transmitted in different candidate positions, depending on whether there is an LBT failure or not. That means that if the gNB misses the opportunity to transmit SSB due to LBT failure, it may attempt transmitting It again in the next candidate position. Form the RAN4 discussion, it was decided to use an working assumption where the UE is requirement to monitor 2 SSB candidate positions of the same SSM index. When FR2-2 is considered, this feature is also considered and few issues remain open in RAN1. Considering that in FR2-2 the use of narrow beams will be more prominent, the probability of LBT failures will be smaller in comparison to the probability in FR1. Therefore, there is no pressing need to increase the number of candidate positions that the UE is required to monitor, and we can use the working assumption, which can be revisited once RAN1 reaches conclusive agreements on that matter.

In Rel 16 NR unlicensed the number of SSB candidate positions used for RRM core requirements as 2.

At 60 GHz the narrow beams are expected, which results in a CCA failure probability which is smaller than what is experienced at 5 GHz band.

RAN4 to maintain the working assumption on the number of SSB successive candidate positions of the same SSB index from requirements with CCA in FR1 for the operation in FR2-2 before RAN1 reaches conclusive agreement on that topic.

Conclusion

This paper has presented Nokia’s views on RRM requirements for the extension to 71 GHz. From this discussion we have derived the following observations and proposals:

- Most of the requirements with CCA are only applicable for FR1. For these requirements, adaptation for FR2-2 needs to be considered.

- RAN4 to consider Table 1 for the discussion of the impact of FR2-2 for requirements with CCA.

In Rel 16 NR unlicensed the number of SSB candidate positions used for RRM core requirements as 2.

At 60 GHz the narrow beams are expected, which results in a CCA failure probability which is smaller than what is experienced at 5 GHz band.

RAN4 to maintain the working assumption on the number of SSB successive candidate positions of the same SSB index from requirements with CCA in FR1 for the operation in FR2-2 before RAN1 reaches conclusive agreement on that topic.

References

- R4-2108353, Work plan for NR extension to 71 GHz - RRM, Qualcomm

- R4-2108354, WF on NR extension to 71 GHz - RRM, Qualcomm

- R4-2115351, WF on NR extension to 71 GHz – RRM - Part 1, Qualcomm

- R4-2115352, WF on NR extension to 71 GHz – RRM - Part 2, Intel

- 3GPP TS 38.133: “NR; Requirements for support of radio resource management”

- TS 38.213, NR; Physical layer procedures for control

- TS 38.211, NR; Physical channels and modulation

- 3GPP TR 38.808 v17.0.0, Study on supporting NR from 52.6 GHz to 71 GHz