3GPP TSG-RAN WG4 Meeting # 101-e                	R4-2118959

Electronic Meeting, 1st - 12th November 2021

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Discussion on RRM timing requirements for extension to 71 GHz

Agenda item: 	8.16.7.2

Document for: 	Discussion

Introduction

This paper presents Nokia’s view on RRM aspects related to the operation between 52.6 GHz and 71 GHz. This paper discusses the impact of operation above 52.6 GHz on timing aspects. Among the discussion points are timing aspects including TA, UE transmit timing, and UE timer accuracy for operation above 52.6 GHz, and the impact on RRM core requirements.

Discussion

UE transmit timing error

During the last RAN4 meeting, one important point that was discussed was the approach for determining the UE transmit timing error requirements. As part of this discussion, the following agreement was reached in order to guide the study for further proposals during this meeting:

Issue 2-1-1: General principles in defining the Te requirements GTW agreements: - Further study percentage of UL CP length Te can occupy without impacting UL system performance? - Option 1: Keep the Te within the same percentage of the CP length as existing SCS - Option 2: 30% - Option 3: 50% - Further study achievable Te from UE perspective - Study different combinations of SSB SCS and UL signal SCS for FR2-2

From the base station perspective, the UL transmit timing is of major importance, since it may have a direct impact on the demodulation performance of UL signals. If the UL receive timing at the BS does not match the expected receive timing, the BS will not be able to have a correct placement of the FFT window, resulting in inter-symbol interference.

Figure 1 shows a diagram showing how the error builds up in time from a gNB perspective. During initial access PRACH is transmitted without timing advance correction, so the timing of PRACH is the UE timing offset, which is two times the propagation delay between UE and gNB, and a transmit error specified by Te. After the timing advance command (TAC), the UL timing error will be a composition of the quantization error TAC,Q, and the TAC adjustment accuracy TAC,e, as specified in 7.3.2.2 of 38.133 [5]. The TAC quantization resolution can be calculated as (38.213 [6])

TAC,Q=16∙64∙Tc/2μ

where μ indicates the used numerology and Tc is the basic timing unit defined in TS 38.211 [6].

Figure 1 UL receiving timing error during and shortly after initial access

After application of the TAC, the UE should monitor if its timing offset in relation to the downlink frame is changing, and apply autonomous gradual timing adjustments. After this point the maximum composed UL receive timing error is

TCOMP=TAC,Q2+Te

There are several challenges when TCOMP is large. The most obvious one is that if TCOMP is large in comparison to the CP length, the gNB cannot adjust the timing of the UE with enough accuracy to guarantee that there is no inter-symbol interference. In order to certify that FFT placement can be done such that it doesn’t happen, there must be enough time margin TMAR, which can be computed as

TMAR=TCP-2⋅TCOMP-TCH

where TCH is the channel delay spread.

Figure 2 shows a time diagram displaying how an OFDM FFT window can be placed considering the ideal UL receive timing, and the maximum composed error when TMAR > 0. In this situation, the gNB is able to place the FFT window without inter-symbol interference even if the UE UL transmit timing uncertainty reaches the maximum expected value ±TCOMP, and TMAR provides some extra margin for uncertainty on gNB timing estimation of the UE, UE movement, and others.

Figure 2 Example of FFT window placement when the composed timing error allows for a safety margin.

Figure 3 shows one example of FFT window placement when TMAR < 0. In that situation, there is no possible solution for the FFT placement for UL demodulation that prevents inter symbol interference. In this particular example, the FFT window is placed such that there is inter-symbol interference both when the UE is ahead or behind the ideal UL transmit timing.

Figure 3 Example of FFT window placement when the composed timing error has a negative safety margin.

The composed UL transmit timing error including TAC command quantization TAC,Q, TAC adjustment accuracy TAC,e, and Te including the channel delay spread TCH influence whether inter-symbol interference can be avoided using TA command.

The UL TX timing accuracy has to be designed in accordance to the CP length TCP such that TMAR = TCP - TCH  - 2 ( Te + TAC,Q /2 + TAC,e ) > 0.

RAN4 to choose Te such that the condition TCP - TCH  - 2 ( Te + TAC,Q /2 + TAC,e ) > 0 holds.

In Session 5.2 we present PUSCH simulation results that show the impact of UL timing error on UL demodulation performance. This result shows that the inaccurate UL timing may have a direct impact on the overall system performance, since the timing offsets larger than 25% of the CP length can cause a degradation of several dBs in demodulation performance.

UL timing errors larger than 25% of the CP length can cause UL demodulation performance degradation of 14 dB for 960 kHz SCS.

One possible source of UL timing error is the accuracy in which a UE can estimate DL timing. In fact, the Te requirements are defined based on the SSB subcarrier spacing, since it has an impact on how the UE can estimate the DL timing. The UE may estimate timing from the PSS and SSS signals of the SSB as well as from TRS signals. For this reason we have presented link level simulations with the accuracy of the timing estimation based on TRS on Session 5.1.

For the TRS simulation results, we observed that the timing estimation accuracy was relatively close for the different SCSs, while the accuracy had large variation depending on the channel model. In any case, the accuracy for TRS configured with SCS of 120 kHz, 480 kHz, and 960 kHz have an accuracy of about 12.7 ns for TDLA-20 and 6.7 ns for TDLA-10.

DL timing estimation based on TRS can achieve accuracy of 6.7 for TDLA-10 and 12.7 ns for TDLA-20 with SCSs 120 kHz, 480 kHz, and 960 kHz.

In Table 1 we present an analysis of the timing requirements considering the analysis above for the existing SCSs and proposed Te values for 480 kHz and 960 kHz. In this analysis the timing margin is estimated based on assumed delay spread shown in the table with values normalized by the CP length for each numerology. The worst-case scenario among existing numerologies is the case of 120 kHz SCS where a margin TMAR=43.2% is allowed. From this analysis and considering the PUSCH simulations we believe that Te should not exceed 20% of the CP to avoid degradation on UL demodulation performance. This is also in line with existing Te requirements, since none of the existing requirements exceed 20% of the CP length.

Table 1 Analysis of the composed UL receive timing error for different sub-carrier spacings with the existing RAN4 requirements and proposed ones for 480 kHz and 960 kHz SCS

| SCS (kHz) | 15 | 15 | 15 | 30 | 30 | 30 | 120 | 120 | 480 | 960 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SSB SCS (kHz) | 15 | 30 | 60 | 15 | 30 | 60 | 60 | 120 | 120, 480, 960 | 120, 480, 960 |
| CP length (ns) | 4688 | 4688 | 4688 | 2344 | 2344 | 2344 | 586 | 586 | 146 | 73 |
| TAC,e (ns) | 130 | 130 | 130 | 130 | 130 | 130 | 16.3 | 16.3 | 4.07 | 2.03 |
| TAC,e (%) | 2.8 | 2.8 | 2.8 | 5.6 | 5.6 | 5.6 | 2.8 | 2.8 | 2.8 | 2.8 |
| TAC,Q (ns) | 521 | 521 | 521 | 260 | 260 | 260 | 65.1 | 65.1 | 16.28 | 8.14 |
| TAC,Q (%) | 11.1 | 11.1 | 11.1 | 11.1 | 11.1 | 11.1 | 11.1 | 11.1 | 11.1 | 11.1 |
| Te/(64*Tc) | 12 | 10 | 10 | 8 | 8 | 7 | 3.5 | 3.5 | 0.9 | 0.45 |
| Te | 390.6 | 325.5 | 325.5 | 260.4 | 260.4 | 227.9 | 113.9 | 113.9 | 29.3 | 14.6 |
| Te (%) | 8.3 | 6.9 | 6.9 | 11.1 | 11.1 | 9.7 | 19.4 | 19.4 | 20.0 | 20.0 |
| TCH | 300 | 300 | 300 | 300 | 300 | 300 | 40 | 40 | 40 | 30 |
| TMAR (ns) | 3085.4 | 3215.6 | 3215.6 | 1262.5 | 1262.5 | 1327.6 | 253.0 | 253.0 | 31.6 | 5.8 |
| TMAR % | 65.8 | 68.6 | 68.6 | 53.9 | 53.9 | 56.6 | 43.2 | 43.2 | 21.6 | 7.9 |

TR 38.808 considers channel models with delay spread up to 50 ns.

Error floor is reported in TR 38.808 for 960 kHz SCS with delay spread above 40 ns and high MCS (64 QAM).

When defining the margin for the Te calculation, consider a maximum channel delay spread of 40 ns for 480 kHz SCS, and 30 ns for 960 kHz SCS.

Define Te values that do not exceed 20% of the CP length for UL transmission using 480 kHz and 960 kHz SCS.

Define Te = 0.9*64*Tc for 480 kHz SCS, and Te=0.45*64*Tc for 960 kHz SCS.

Part of the UL timing accuracy depends on the DL timing estimation, which can be done based on both SSB and TRS timing estimation. The existing Te requirements depend on the SSB SCS, since the larger SCS provides better timing accuracy. However, FR2-2 introduces a scenario where the SSB SCS may be much smaller than the UL SCS, with 120 kHz SCS used for SSB and 960 kHz used for data and UL transmission. In that scenario, the achievable timing accuracy will be lower in comparison to the one achieved when 960 kHz SCS is used for SSB, which would indicate that TRS may be used for improved accuracy. Our simulations show that it is accurate even when 120 kHz SCS is configured for TRS.

The existing RRM core timing requirements do not include TRS, only SSB, as part of the specification. This is partially because Te requirements are generic and cover also initial access from RRC_Idle with PRACH. However, the existing RRM performance requirements in RRC_Connected mode do include TRS configuration for FR2 requirements in A.5.4.1.1 and A.7.4.1.1 of 38.133. Additionally, initial access with PRACH transmission is only allowed with 120 kHz and 480 kHz SCS from RAN1 agreements. Therefore, if it is identified that DL timing estimation accuracy based on a 120 kHz SCS SSB is not enough for 480/960 kHz SCS UL transmission, we propose to configure TRS as part of the RRM core requirements for 480 and 960 kHz SCS.

Existing FR2-1 core requirements have no mention of TRS, but performance requirements in RRC_Connected include TRS in their configuration.

TRS can be used for improving timing estimation of the UE even when SSB uses a small SCS in comparison to the SCS used for UL transmission.

If it is identified that 120 kHz SCS is not enough for Te using 480/960 kHz SCS, UE UL transmit requirements for 480 and 960 kHz for non-initial access may be developed considering that TRS is configured.

MRTD requirements

MRTD was discussed during the last RAN4 meeting #100-e, where the following agreements were reached:

Issue 2-4-1: MTTD/MRTD Agreements: - Consider FR2-1 requirements for 120kHz SCS for FR2-2 as baseline. - RAN4 to further discuss whether new MRTD/MTTD requirements are needed for SCS of 480/960 kHz based on the agreed deployment scenarios - Identify other parameters that needs to be considered for the discussion - Consider using the current FR2-1 MRTD requirements and rules for FR2-2

This agreement implies that the MRTD requirements would be using TAE + propagation delay for synchronous cases, and half of the TTI for the asynchronous cases. A summary on the existing MRTD requirements and interpolation for FR2-2 is shown in Table 2. The highlighted values of this table have the FR2-2 interpolated values, where few points can be observed. Firstly, by keeping the existing rules, MRTD requirements for synchronous case in NR-DC would be 8 us if TAE is kept as 3 us, and 5 us is used for 1500 m propagation. That would mean that the MRTD requirement in that case would be larger than half of the TTI and larger than the requirement used for the asynchronous cases.

Reusing existing MRTD derivation rules with TAE=3 us and propagation delay for 1500 m results in requirements larger than half of the slot length for 960 kHz SCS.

TAE requirements are still under discussion for FR2-2.

Wait for the definition of TAE requirements in the BS RF discussion before concluding on the MRTD requirements for FR2-2.

If needed, revise the cell size based on the propagation delay of 1500 m used in FR2-1 when calculating MRTD requirements for FR2-2 with 960 kHz SCS, e.g. use 1000 m that results in 3.33 us propagation delay.

Table 2 Summary of existing MRTD requirements in 38.133 and interpolation of MRTD for FR2-2 considering existing rules

|  |  |  |  |  | MRTD (us) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Intra/inter band | Scenario | FR | Sync/assync | TAE (us) | 15 | 30 | 60 | 120 | 480 | 960 |
| Inter-band | EN-DC | - | synchronous |  | 33 |  |  |  |  |  |
|  |  | - | assynchronous |  | 500 | 250 | 125 | 62.5 | 15.625 | 7.8125 |
|  | NE-DC | - | synchronous |  | 33 |  |  |  |  |  |
|  |  | - | assynchronous |  | 500 | 250 | 125 | 62.5 | 15.625 | 7.8125 |
|  | NR-DC | FR1 - FR1 | synchronous |  | 33 |  |  |  |  |  |
|  |  | FR2-1 - FR2-1 | synchronous | 3 | 8 |  |  |  |  |  |
|  |  | FR1 - FR2-1 | synchronous |  | 33 |  |  |  |  |  |
|  |  | FR2-2 - FR2-2 | synchronous | 3 Note 1 | 8 for TAE=3 and propagation delay 5 us (1500 m) 6.3 for TAE=3 and propagation delay 3.3 us (1000 m) |  |  |  |  |  |
|  |  |  | assynchronous |  | 500 | 250 | 125 | 62.5 | 15.625 | 7.8125 |
|  | CA | FR1 |  | 3 | 33 |  |  |  |  |  |
|  |  | FR2-1 - FR2-1 |  | 3 | 8 |  |  |  |  |  |
|  |  | FR1 - FR2-1 |  |  | 25 |  |  |  |  |  |
|  |  | FR2-2 - FR2-2 |  | 3 Note 1 | 8 for TAE=3 and propagation delay 5 us (1500 m) 6.3 for TAE=3 and propagation delay 3.3 us (1000 m) |  |  |  |  |  |
|  |  | FR1 - FR2-2 |  |  | 25 |  |  |  |  |  |
| Intra-band | EN-DC |  | synchronous |  | 3 |  |  |  |  |  |
|  |  |  | assynchronous |  | 500 | 250 | 125 | 62.5 | NA | NA |
|  | non-contiguous CA | FR1 |  | 3 | 3 |  |  |  |  |  |
|  |  | FR2-1 |  | 0.26 | 0.26 |  |  |  |  |  |
|  |  | FR2-2 |  | 0.26 Note 1 | 0.26 |  |  |  |  |  |
| NOTE 1: TAE is still under discussion for FR2-2. NOTE 2: FR2-2 values are interpolated considering the existing rules for the definition of MRTD requirements. |  |  |  |  |  |  |  |  |  |  |

Conclusion

This paper has presented Nokia’s views on RRM requirements for the extension to 71 GHz. From this discussion we have derived the following observations and proposals:

Observation 1: The composed UL transmit timing error including TAC command quantization TAC,Q, TAC adjustment accuracy TAC,e, and Te including the channel delay spread TCH influence whether inter-symbol interference can be avoided using TA command.

Observation 2: The UL TX timing accuracy has to be designed in accordance to the CP length TCP such that TMAR = TCP - TCH  - 2 ( Te + TAC,Q /2 + TAC,e ) > 0.

Proposal 1: RAN4 to choose Te such that the condition TCP - TCH  - 2 ( Te + TAC,Q /2 + TAC,e ) > 0 holds.

Observation 3: UL timing errors larger than 25% of the CP length can cause UL demodulation performance degradation of 14 dB for 960 kHz SCS.

Observation 4: DL timing estimation based on TRS can achieve accuracy of 6.7 for TDLA-10 and 12.7 ns for TDLA-20 with SCSs 120 kHz, 480 kHz, and 960 kHz.

Observation 5: TR 38.808 considers channel models with delay spread up to 50 ns.

Observation 6: Error floor is reported in TR 38.808 for 960 kHz SCS with delay spread above 40 ns and high MCS (64 QAM).

Proposal 2: When defining the margin for the Te calculation, consider a maximum channel delay spread of 40 ns for 480 kHz SCS, and 30 ns for 960 kHz SCS.

Proposal 3: Define Te values that do not exceed 20% of the CP length for UL transmission using 480 kHz and 960 kHz SCS.

Proposal 4: Define Te = 0.9*64*Tc for 480 kHz SCS, and Te=0.45*64*Tc for 960 kHz SCS.

Observation 7: Existing FR2-1 core requirements have no mention of TRS, but performance requirements in RRC_Connected include TRS in their configuration.

Observation 8: TRS can be used for improving timing estimation of the UE even when SSB uses a small SCS in comparison to the SCS used for UL transmission.

Proposal 5: If it is identified that 120 kHz SCS is not enough for Te using 480/960 kHz SCS, UE UL transmit requirements for 480 and 960 kHz for non-initial access may be developed considering that TRS is configured.

Observation 9: Reusing existing MRTD derivation rules with TAE=3 us and propagation delay for 1500 m results in requirements larger than half of the slot length for 960 kHz SCS.

Proposal 6: Wait for the definition of TAE requirements in the BS RF discussion before concluding on the MRTD requirements for FR2-2.

Proposal 7: If needed, revise the cell size based on the propagation delay of 1500 m used in FR2-1 when calculating MRTD requirements for FR2-2 with 960 kHz SCS, e.g. use 1000 m that results in 3.33 us propagation delay.

References

- R4-2108353, Work plan for NR extension to 71 GHz - RRM, Qualcomm

- R4-2108354, WF on NR extension to 71 GHz - RRM, Qualcomm

- R4-2115351, WF on NR extension to 71 GHz – RRM - Part 1, Qualcomm

- R4-2115352, WF on NR extension to 71 GHz – RRM - Part 2, Intel

- 3GPP TS 38.133: “NR; Requirements for support of radio resource management”

- TS 38.213, NR; Physical layer procedures for control

- TS 38.211, NR; Physical channels and modulation

- 3GPP TR 38.808 v17.0.0, Study on supporting NR from 52.6 GHz to 71 GHz

Appendix: Link level results

TRS timing detection accuracy

| Parameter | Value |
| --- | --- |
| Carrier frequency | 60 GHz |
| SSB SCS | 120, 480, 960 kHz |
| Bandwidth | 400 MHz: 264/64/32 PRB for 120/480/960 kHz |
| Channel model | TDL-A 10 TDL-A 20 |
| Mobility | 3 km/h |
| TRS periodicity | 10 slots |
| Metric | TO error @ 95th percentile |

Figure 4 Cummulative distribution function of the estimated timing error based on TRS signals

Table 3 95th percentile of the timing estimation error based on TRS

| TRS SCS | Propagation condition | TO error (ns) |
| --- | --- | --- |
| 120 | TDLA-10 | 6.35 |
| 120 | TDLA-20 | 12.72 |
| 480 | TDLA-10 | 6.67 |
| 480 | TDLA-20 | 11.95 |
| 960 | TDLA-10 | 6.70 |
| 960 | TDLA-20 | 11.83 |

Note: IFFT is performed to the raw channel estimates and the time offset estimation is based on the correlation peak of the channel impulse response.

PUSCH performance degradation with Te

| Parameter | Value |
| --- | --- |
| Carrier frequency | 60 GHz |
| Subcarrier spacing | 480 kHz, 960 kHz |
| Antenna configuration | 2x2 (FD-OCC single layer) |
| DMRS configuration | 2 DMRS symbols at (2,11) symbol index Note: no data multiplexing is assumed in DMRS symbols |
| MCS | MCS16 |
| BW | 400 MHz: 64/32 PRB for 480/960 kHz |
| Number of OFDM symbols | 14 |
| Channel estimation | Practical |
| Channel | TDL-A 10ns |
| Time offset | TO:0, 10, 15, 20, 25,50, 75, 100% of CP |
| Mobility | 3 km/h |
| Test metric | SNR @ 10% BLER |

| a) | b) |
| --- | --- |

Figure 5 Time offset impact in the BLER performance of the data channel (PUSCH) with a) 480 kHz SCS and b) 960 kHz SCS

Table 4 SNR requirements at 10% BLER for 480 kHz SCS

| TO | SNR (dB) @ 10% BLER |
| --- | --- |
| 0 us | 11.81 |
| 10% CP | 12.30 |
| 15% CP | 12.82 |
| 20% CP | 13.67 |
| 25% CP | 16.33 |
| 50% CP | 10% BLER not achievable |

Table 5 SNR requirements at 10% BLER for 960 kHz SCS

| TO | SNR (dB) @ 10% BLER |
| --- | --- |
| 0 us | 12.44 |
| 10% CP | 13.49 |
| 15% CP | 14.99 |
| 20% CP | 17.66 |
| 25% CP | 27.0 |
| 50% CP | 10% BLER not achievable |