3GPP TSG-RAN WG4 Meeting #101-e	R4-2119019

# Electronic Meeting, 1st - 12th Nov, 2021

Title: 	Discussion on PRACH demodulation requirements for FR2 HST

Source: 	Huawei, HiSilicon

Agenda item:	8.9.5.3.3

Document for:	Discussion

Background

During RAN#100-e meeting, WF [1] on FR2 HST demodulation was approved. In this contribution, we share our views about PRACH demodulation requirements for FR2 HST.

Discussion

Timing offset configuration

- Time offset configuration - Option 1: Reuse Rel-15 timing offset configuration for PRACH, i.e., 0.8 us - Value of Timing offset start: 0 - Value of Timing offset start: 0.1us - Option 2: Configure the maximum timing offset (i.e. the end of the tested range) in HST FR2 testing setup equal to 4.6us - Value of Timing offset start: 0 - Value of Timing offset start: 0.46us - Note: Scenario; Cell radius (m); Maximum timing offset (us) (RTT); Coverage of format C2; Maximum timing offset for C2 (us) (RTT). Scenario A (Ds =700m, Dmin =10m); 700; 4.6us; 1.15km; 7.6. Scenario B (Ds=700m, Dmin=150m); 716; 4.78us; 1.15km; 7.6. - Option 1 can be regarded as baseline. Companies are encouraged to bring the simulation results to compare the performance between option 1 and option 2 in the next meeting.

For Option 1, the Rel-15 FR2 timing offset configuration is reused, i.e. maximum 0.8us. For Option 2, the maximum timing offset can be about 4.6us corresponding to the maximum cell radius derived from Scenario A. As per the simulation results in section 3, there is very small difference between two options, and we think both options are feasible to verify the baseband demodulation function since the algorithm is same. However, in Rel-16 FR1 HST, 0.8 us is reused from the non-HST test setup although the maximum cell radius can be about 716m that is same as HST FR2 Scenario B, so we prefer to reuse Rel-15 FR2 timing offset configuration for PRACH, i.e., 0.8us.

Reuse Rel-15 FR2 timing offset configuration for PRACH, i.e., [0,0.1...0.8]us.

Simulations

Here we provide the simulation results for FR2 HST PRACH for alignment.

Table 3-1 Ideal simulation results for FR2 HST PRACH

| Case number | Timing offset | SNR (dB) |
| --- | --- | --- |
| 1 | [0,0.1...0.8] | -12.30 |
| 2 | [0,0.46...4.6] | -12.36 |

Figure 3-1 Ideal simulation results for FR2 HST PRACH

Proposals

In this contribution, we discuss on PRACH performance requirements for NR HST FR2. Our observations and proposals are:

- Reuse Rel-15 FR2 timing offset configuration for PRACH, i.e., [0,0.1...0.8]us.

Reference

R4-2115726, WF on FR2 HST demodulation, RAN4#100-e, Samsung