3GPP TSG-RAN WG4 Meeting #101-e	R4-2119024

# Electronic Meeting, 1st - 12th Nov, 2021

Title: 	Discussion on the PDSCH performance requirements for NR DL 1024QAM for FR1

Source: 	Huawei, HiSilicon

Agenda item:	8.6.5.2

Document for:	Discussion

Background

As per WID [1], the objective of Performance part WI is extracted as following.

The objective is to specify the necessary UE performance requirements for DL 1024QAM for PDSCH in FR1 and BS RF conformance tests in FR1. The main objectives are: - UE demodulation performance requirements for PDSCH. - CQI reporting requirements. - Conformance tests for BS RF. - UE demodulation requirements, CQI requirements and BS RF conformance tests are specified for stationary wireless scenarios with up to 2 layer DL MIMO. - The cell size(s) and type of stationary wireless scenarios for which UE demodulation requirements, CQI requirements and BS RF conformance tests are defined will be studied and decided by RAN4.

In this contribution, we share our views on PDSCH demodulation requirements for NR DL 1024QAM for FR1.

Discussion

  - PDSCH requirements

The principle of determining setup for simulation assumption is to select typical values to satisfy the proper scenario and reduce the testing cost as well.

Channel bandwidth

For 1024QAM demodulation requirements definition, we prefer to use 10MHz bandwidth with 15kHz SCS for FDD and 40MHz bandwidth with 30kHz SCS for TDD.

Tx EVM

At the BS side, there is still under discussion for the value of the EVM as per the WF [2]. We prefer to use 2.8% for the demodulation requirement assumption to consider the worst case from all options.

RAN4 will make decision on Nov 2021 RAN4 meeting with following options: - Option 1: 2.5% for all classes - Option 2: 2.5% for LA and MR classes; 2.8% for WA class - Option 3: 2.8% for all BS classes

For UE side, the Tx EVM impact should be calculated into impairment results as other demodulation requirements did.

FRC

The maximum DL MIMO layer is 2 as per the WID. There is 4 MCS defined for 1024QAM as following table shows.

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| 23 | 10 | 805.5 | 7.8662 |
| 24 | 10 | 853 | 8.3301 |
| 25 | 10 | 900.5 | 8.7939 |
| 26 | 10 | 948 | 9.2578 |

The selection of MCS and rank should be based on the simulation results of the different combinations. We propose to perform the simulation and decide the MCS and rank based on the simulation results.

Propagation condition

Considering the stationary wireless scenario with sufficiently high DL SINR and better channel conditions (e.g., LOS or LOS-like channels), with no mobility or very low mobility environment, we propose to use TDLD30-5 as the propagation condition for 1024QAM requirements. Notice that simplified TDLD30 model has been defined and used for Rel-16 256QAM FR2 WI, it is reasonable to reuse this propagation condition and there is no additional simplified work needed.

Other parameters

For other parameters, typical values can be selected to reduce the test cost.

Based on above discussion, we can get the following proposal:

Define new performance requirements for NR DL 1024QAM for FR1 based on the evaluations by using the following simulation assumptions:

| Parameters | Value |
| --- | --- |
| Channel bandwidth | 10MHz for FDD 15kHz, 40MHz for TDD 30kHz |
| Tx EVM | 2.8% |
| FRC | Rank 1/2, MCS 23/24/25/26 |
| Propagation condition | TDLD30-5 |
| Antenna configuration | 2x2 ULA low, 2x4 ULA low |
| SSB configuration | Periodicity 20 ms, Allocated in first slot within 20ms |
| PDSCH configuration | Type A, Start symbol 2, Duration 12 |
| DMRS configuration | Type 1, 1 additional DMRS, Single symbol |
| TRS configuration | 20 ms periodicity, 2 slots, offset 10 ms |
| TDD UL-DL pattern | 7D1S2U with S=6:4:4 for TDD 30kHz |
| HARQ process number | 4 for FDD 15kHz, 8 for TDD 30kHz |
| RV | {0,2,3,1} |
| Allocated RBs | Full BWP |
| PRB bundling size | 2 |
| Precoding model | Random Precoding, per slot, PRB bundling size granularity (codebook configuration Single panel Type 1) |
| Receiver type | MMSE-IRC |
| Test metric | 70% of max TP |

  - SDR requirements

When the new MCS for 1024QAM is introduced, the Sustained Data Rate (SDR) requirements should be defined to verify that the Layer 1 and Layer 2 correctly process in a sustained manner for UEs that support 1024QAM. However, we also notice that the SDR requirements are not included in the WID currently, maybe the WID should be updated if it is agreed to define SDR requirements.

For SDR requirements, the MCS indexes for indicated UE capabilities (i.e. MCS1) and the maximum achievable MCS indexes derived from simulation results (i.e. MCS2) are shown as following table by using the new MCS table 4. The tested MCS should be min{MCS1, MCS2}.

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS1 | MCS2 |
| --- | --- | --- | --- | --- |
| 1 | 10 | 1 | 26 | FFS |
| 1 | 10 | 0.8 | 22 |  |
| 1 | 10 | 0.75 | 20 |  |
| 1 | 10 | 0.4 | 9 |  |
| 2 | 10 | 1 | 26 | FFS |
| 2 | 10 | 0.8 | 22 |  |
| 2 | 10 | 0.75 | 20 |  |
| 2 | 10 | 0.4 | 9 |  |

Define SDR requirements for FR1 1024QAM. Perform simulation to find the maximum achievable MCS indexes for rank 1 and rank 2 separately.

Proposal

In this contribution, we discuss on the PDSCH performance requirements for NR DL 1024QAM for FR1. Our observations and proposals are:

- Define new performance requirements for NR DL 1024QAM for FR1 based on the evaluations by using the following simulation assumptions:

| Parameters | Value |
| --- | --- |
| Channel bandwidth | 10MHz for FDD 15kHz, 40MHz for TDD 30kHz |
| EVM | 2.8% TX EVM |
| FRC | Rank 1/2, MCS 23/24/25/26 |
| Propagation condition | TDLD30-5 |
| Antenna configuration | 2x2 ULA low, 2x4 ULA low |
| SSB configuration | Periodicity 20 ms, Allocated in first slot within 20ms |
| PDSCH configuration | Type A, Start symbol 2, Duration 12 |
| DMRS configuration | Type 1, 1 additional DMRS, Single symbol |
| TRS configuration | 20 ms periodicity, 2 slots, Offset 10 ms |
| TDD UL-DL pattern | 7D1S2U with S=6:4:4 for TDD 30kHz |
| HARQ process number | 4 for FDD 15kHz, 8 for TDD 30kHz |
| Transform precoding | CP-OFDM |
| Allocated RBs | Full BWP |
| PRB bundling | 2 |
| Precoding model | Random Precoding, per slot, WB granularity (codebook configuration Single panel Type 1) |
| Receiver type | MMSE-IRC |
| Test metric | 70% of max TP |

Define SDR requirements for FR1 1024QAM. Perform simulation to find the maximum achievable MCS indexes for rank 1 and rank 2 separately.

Reference

RP-202886, Revised WID on Introduction of DL 1024QAM for NR FR1,RAN#90-e, Nokia

R4-2115638, WF on 1024QAM requirements, RAN4#100-e, Ericsson