3GPP TSG-RAN WG4 Meeting # 101-e	R4-2119044

# Electronic Meeting, November 01-12, 2021

Title: 	Simulation results for PDSCH demodulation requirements for inter-cell MMSE-IRC receiver

Source: 	Huawei, HiSilicon

Agenda item:	8.12.2.1

Document for:	Discussion

Background

As per WF [1], we provide the simulation results for inter-cell interference demodulation requirements in this paper.

Discussions

The corresponding simulation results as per WF [1] are captured in Table 2-1:

Table 2-1: Simulation results for MMSE-IRC receiver

| SINR(dB)70% of max TP |  | TDLA30-10 |  |  |  | TDLC300-100 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MCS4 |  | MCS13 |  | MCS4 |  | MCS13 |  |
|  |  | 2RX | 4RX | 2RX | 4RX | 2RX | 4RX | 2RX | 4RX |
| Two interference cells with INR1= 5.43dB and INR2=-1.5dB Total INR: 6.23dB | MMSE | -3.5 | -6 | 4.1 | 1.4 | -3.1 | -5.6 | 4.9 | 2 |
|  | MMSE-IRC | -5.4 | -9.9 | 2.7 | -0.9 | -4.8 | -9.1 | 3.6 | -0.2 |
|  | Gain | 1.9 | 3.9 | 1.4 | 2.3 | 1.7 | 3.5 | 1.3 | 2.2 |
|  | Target SNR for MMSE-IRC | 1.76 | -2.74 | 9.86 | 6.26 | 2.36 | -1.94 | 10.76 | 6.96 |
| Two interference cells with INR1= 7.77dB and INR2=2.29dB Total INR: 8.85dB | MMSE | -3.66 | -5.78 | 3.88 | 1.36 | -3.30 | -5.50 | 4.70 | 2.0 |
|  | MMSE-IRC | -7.21 | -12.11 | 1.18 | -2.23 | -6.51 | -11.23 | 2.4 | -1.5 |
|  | Gain | 3.55 | 6.33 | 2.70 | 3.59 | 3.21 | 5.73 | 2.3 | 3.5 |
|  | Target SNR for MMSE-IRC | 2.17 | -2.73 | 10.56 | 7.15 | 2.87 | -1.74 | 11.78 | 7.88 |
| Two interference cells with INR1= 13.91dB and INR2=3.34dB Total INR: 14.28dB | MMSE | -3.75 | -5.65 | 3.67 | 1.37 | -3.37 | -5.33 | 4.59 | 1.97 |
|  | MMSE-IRC | -10.79 | -17.51 | -1.11 | -4.85 | -9.25 | -15.39 | 0.65 | -3.49 |
|  | Gain | 7.04 | 11.86 | 4.78 | 6.22 | 5.88 | 10.06 | 3.94 | 5.46 |
|  | Target SNR for MMSE-IRC | 3.64 | -3.08 | 13.32 | 9.58 | 5.18 | -0.96 | 15.08 | 10.94 |
| One interference cell with INR=3.1dB | MMSE | -3.5 | -5.9 | 4.1 | 1.4 | -3.1 | -5.6 | 4.9 | 2 |
|  | MMSE-IRC | -5.1 | -9.0 | 2.8 | -0.4 | -4.6 | -8.4 | 3.8 | 0.3 |
|  | Gain | 1.6 | 3.1 | 1.3 | 1.8 | 1.5 | 2.8 | 1.1 | 1.7 |

The simulation assumptions are captured in Table 2-2:

Table 2-2: Simulation assumptions for MMSE-IRC receiver

| Parameter |  | Unit | Serving cell | Interference cell 1 | Interference cell 2 |
| --- | --- | --- | --- | --- | --- |
| Cell ID |  |  | 0 | 1 | 2 |
| MIMO layers |  |  | 1 | 70% for 1 layer and 30% for 2 layers | 70% for 1 layer and 30% for 2 layers |
| MCS |  |  | 4,13 | 16QAM random symbols | 16QAM random symbols |
| INR |  |  | - Option 1: INR1=5.43,INR2=-1.5 - Option 2: INR1=7.77,INR2=2.29 - Option 3: INR1=13.91,INR2=3.34 - Option 4: INR1=3.1 |  |  |
| Duplex mode |  |  | FDD |  |  |
| BWP and SCS |  | MHz/kHz | 10/15 |  |  |
| Propagation conditions |  |  | TDLA30-10,TDLC300-100 |  |  |
| MIMO configurations |  |  | 2T2R Low, 2T4R Low |  |  |
| PDSCH configuration | Mapping type |  | Type A |  |  |
|  | Starting symbol (S) |  | 2 |  |  |
|  | Length (L) |  | 12 |  |  |
|  | PRB bundling type |  | Static |  |  |
|  | PRB bundling size |  | 2 |  |  |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |  |  |
|  | Number of additional DMRS |  | 1 |  |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |  |
| Number of HARQ Processes |  |  | 4 |  |  |

Conclusion

In this paper, we provide our simulation results for inter-cell interference MMSE-IRC receiver.

Reference

- R4-2115730, WF on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC. Intel Corporation