3GPP TSG-RAN WG4 Meeting #101-e 	R4-2119054

Electronic Meeting,  Nov 1  12, 2021

Agenda Item:	8.20.3.3

Source: 	Ericsson

Title:	Discussions on RRM measurement relaxations

Document for:	Discussion

# Introduction

RRM measurement relaxation for RedCap UEs in IDLE/INACTIVE and CONNECTED states was discussed at last meeting, and some high-level agreements were reached in [1]. In this contribution we discuss and provide our view on the open issues that were identified in [1] and we provide additional views on Rel-17 relaxed measurement requirements.

# Discussion

## IDLE/INACTIVE state relaxation

At last meeting, RAN4 discussed measurement relaxation for stationary and not-at-cell edge criterion in RRC IDLE/INACTIVE states and a high-level agreement to reuse the relaxation mechanism from Rel-16 RRM relaxation work was [1]. In addition, different ways of introducing the relaxation requirements were identified as follows [1]:

Issue 2-1-3: Mechanism for R17 RedCap UE in IDLE/Inactive mode - Agreements: - Reuse the same mechanism as Rel-16 RRM measurement relaxation in IDLE/Inactive mode - Method 1: Using scaling factors (value for FFS) - Method 2: Using a period of time (value for FFS) - Criteria for using Method 1 and/or 2 are FFS - Note: EMR is not supported by RedCap UEs and the relaxation mechanisms related to EMR carriers do not apply to RedCap

Rel-16 relaxed neighbour cell requirements

Relaxed neighbour cell measurement requirements in Rel-16 were introduced by scaling the legacy requirements with a measurement scaling factor (K1). The UE configured with the lowMobilityEvaluation field and also meets the low mobility criterion as defined in [2] is required to meet the relaxed neighbour cell measurement requirements. Similarly, the UE configured with the cellEdgeEvaluation field and also meets the not at cell edge criterion as defined in [2] is required to meet the relaxed neighbour cell measurement requirements. The UE configured with the combineRelaxedMeasCondition field that also meets the low mobility and not at cell edge criteria as defined above is required to meet the requirements at least with a periodicity of 1 hour under some additional conditions.

Rel-17 relaxation criteria

In RAN2#115-e it was agreed to introduce two new relaxation criteria in Rel-17 which are Rel-17 not-at-cell-edge criterion and Rel-17 stationary criterion. It is noted that the Rel-17 not-at-cell-edge criterion is only associated with Rel-17 stationary criterion. Thus, there are two new operating scenarios based on only Rel-17 criteria for which RAN4 requirements are needed:

- Rel-17 stationary criterion

- Rel-17 stationary + Rel-17 not-at-cell edge criterion

For a), our view is to follow the methods from Rel-16 low mobility criterion where the legacy requirements were extended by factor 3 (K1=3). The main difference is that if Rel-17 stationary criterion is fulfilled, then the UE is expected to be stationary compared to UE fulfilling Rel-16 criterion in which case UE is expected to have limited mobility. Therefore, a larger scaling factor can be used for the relaxed requirements when fulfilling the Rel-17 stationary criterion, e.g. K1=4.

- Proposal #1: For UE fulfilling Rel-17 stationary criterion, the relaxed requirements are defined by scaling the legacy requirements by factor K1’, where K1’=4.

For b), the main difference between UE fulfilling Rel-16 low mobility criterion + Rel-16 not-at-cell-edge criterion and Rel-17 stationary + Rel-17 not-at-cell-edge criterion is that in the latter case the UE mobility is further limited or at least temporarly stationary. Therefore the relaxation can be further relaxed compared to the Rel-16 scenario. It is recalled that in Rel-16 the UE is required to meet the requirements at least with a periodicity of 1 hour when UE is configured with both criteria and UE has fulfilled both criteria, and less than 1 hour has passed since cell reselection measurements were performed.

- Proposal #2: For UE fulfilling both Rel-17 stationary criterion + Rel-17 not-at-cell edge criterion, relaxed requirements are defined using fixed long time period (T) in which the UE can stop its measurements for cell reselection, where T > 1 hour.

Relaxation when multiple criteria are fulfilled

The RedCap UE can be configured with both Rel-16 relaxation criteria and Rel-17 relaxation criteria at the same time. Till now, RAN4 has discussed requirements for those criteria separately, i.e. corresponding to rows 1-3 and rows 4 – 5 in Table 1 for Rel-16 and Rel-17 respectively. But RAN4 also needs to discuss the requirements when the UE is configured with both Rel-16 and Rel-17 criteria at the same time and has fulfilled both corresponding to rows 6 – 11 in Table 1. This results in following operating scenarios for RRM relaxation for RedCap UE:

| No | Rel-16 relaxation criterion | Rel-17 relaxation criterion |
| --- | --- | --- |
| 1 | Rel-16 low mobility | N/A |
| 2 | Rel-16 not-at-cell-edge | N/A |
| 3 | Rel-16 low mobility & Rel-16 not-at-cell edge | N/A |
| 4 | N/A | Rel-17 stationary |
| 5 | N/A | Rel-17 stationary & Rel-17 not-at-cell-edge |
| 6 | Rel-16 low mobility | Rel-17 stationary |
| 7 | Rel-16 not-at-cell-edge | Rel-17 stationary |
| 8 | Rel-16 low mobility & Rel-16 not-at-cell-edge | Rel-17 stationary |
| 9 | Rel-16 low-mobility | Rel-17 stationary & Rel-17 not-at-cell-edge |
| 10 | Rel-16 not-at-cell-edge | Rel-17 stationary & Rel-17 not-at-cell-edge |
| 11 | Rel-16 low mobility & Rel-16 not-at-cell-edge | Rel-17 stationary & Rel-17 not-at-cell-edge |

Table 1 List of possible scenarios for applying relaxed measurements for RedCap UE

- Proposal #3: RAN4 to specify relaxed measurement requirements for the case when UE is configured with both Rel-16 and Rel-17 relaxation criteria and UE has fulfill both.

Our view is that RAN4 should avoid introducing separate relaxed requirements for each unique scenario in Table 1 which would result in complex and redundant requirements. RAN4 should instead follow a simpler approach. For instance, for the case when UE is configured with both Rel-16 and Rel-17 relaxation criteria and has fulfilled both criteria, then UE can be allowed to fulfill the requirements that are relaxed out of Rel-16 and Rel-17 requirements.

- Proposal #4: For the case when UE is configured with both Rel-16 and Rel-17 relaxation criteria and UE has fulfilled both criteria, UE is allowed to meet the requirements that are most relaxed out of Rel-16 and Rel-17 requirements.

RRM measurement relaxation and eDRX

At last RAN4 meeting, a high level agreement was reached regarding requirements for UE fulfilling the RRC IDLE/INACTIVE relaxation criteria and is also configured with eDRX as follows [1]:

### Sub-topic 1-5 eDRX requirements relaxation Issue 1-5-1: Whether to consider relaxed eDRX for low mobility and not-at-cell edge criteria - Agreement: - Agree with option 3. Open issues: - Option 1: Support the design of new relaxed eDRX for low mobility and not-at-cell edge criteria for low eDRX cycles (MTK) - Option 2: FFS () - Option 3: study the RRM relaxation without eDRX first. (Eric Apple QC OPPO vivo Huawei)

Our understanding of the above agreement is that the scenario where the UE is configured with both measurement relaxation criteria and eDRX is not excluded for in Rel-17 for RedCap, but will be discussed at later stage. Based on this understanding we provide our view on the requirements for this scenario. Combination of eDRX and RRM relaxation results in following scenarios:

| No | eDRX | Rel-16/17 relaxation criteria |
| --- | --- | --- |
| 1 | eDRX configured | Not met |
| 2 | eDRX configured | Met |
| 3 | eDRX not configured | Not met |
| 4 | eDRX not configured | Met |

Table 2 List of possible scenarios for combination of eDRX and relaxed measurements for RedCap UE

In all cases except case no 2, the UE requirements obvious since UE meets one of the eDRX or relaxed requirements, or UE is not configured with eDRX nor relaxation criteria. Case #2, where UE is configured with eDRX and UE has fulfilled the relaxation criteria, needs more discussions. In our view, there is already large power saving achieved through long eDRX cycles and use of PTW. In this case, applying additional relaxation by applying a scaling factor within PTW should be avoided since all measurement activities take place within the PTW. However, power saving can be achieved for the for shorter eDRX cycles (e.g. 2.56 sec – 10.24 sec) for which there is no PTW by relaxing the measurements by applying a relaxation factor. The maximum eDRX cycles with PTW up to which UE is allowed to apply relaxed measurements using a scaling factor needs further RAN4 discussions.

- Proposal #5:  Rel-16/17 relaxed measurement requirements can be applied with eDRX cycles up to 10.24 seconds, without any PTW.

- Proposal #6: The maximum eDRX cycles with PTW up to which UE is allowed to apply Rel-16/17 relaxed measurement requirements is X ms, where value of X is FFS.

## CONNECTED state relaxation

RAN2 has agreed to not introduce nor reuse not-at-cell edge threshold for CONNECTED state. Instead RAN2 has agreed to introduce a new criterion corresponding to stationary UE operation in RRC CONNECTED state. The criterion which is based on RSRP/RSRQ measurements is configured by network and UE evaluates and reports to the network when criterion is fulfilled. The network may use the report to reconfigure the RRM measurement configuration using the release 15 configuration. RAN2 is currently discussing the details of using stationary condition in RRC CONNECTED, e.g. how to configure the UE with the stationary thresholds, how the reporting is done when criteria is fulfilled or not fulfilled, when to report and what to report etc.

The discussions on methods for measurement relaxation in RRC CONNECTED state is ongoing in RAN2 and for this reason RAN4 agreed at last meeting to wait for more RAN2 progress [1]. The type of relaxation methods which are under discussions are twofold:

- Based on a new RRC measurement mode which is enabled by the NW based on reported information

- Based on network configuration, e.g. network can deconfigure/reconfigure RRM measurements

If a) is agreed, then RAN4 may need to specify requirements for relaxed measurements similar to RRC IDLE/INACTIVE relaxation. However, if b) is agreed then RAN4 may not have to defined relaxed requirements. However RAN4 may still have to discuss requirements related to the evaluation, e.g.

- what is the evaluation period, e.g. can current measurement period be applied?

- how frequent is the evaluation done, e.g. every DRX cycle, or every Nth DRX cycle.

In order to discuss the type of RRM requirements that RAN4 may have to introduce for relaxed measurements, RAN4 should wait for conclusion on the relaxation method from RAN2.

- Proposal 7: RAN4 discussions on relaxed requirements should be postponed until RAN2 has agreed on the relaxation methods for measurements in RRC CONNECTED state.

Below we provide our views on other open issues that were identified at last meeting.

When to report [1]:

Following option was discussed without any agreement at last meeting:

- Option 1: For RedCap RRM relaxation in RRC connected mode, based on measurement, when UE turns from ‘fulfil criteria’ to ‘not fulfil criteria’, this is also needed to report to network. (Apple)

When the UE shall report information related to fulfillment or non-fulfillment of relaxation criteria is under RAN2 scope and RAN4 should not discuss the topic in parallel.

- Proposal #8: RAN4 shall wait and follow the conclusion from RAN2 conclusion regarding reporting rules for the stationary criterion.

## Transition requirements:

The need for introducing transition requirements when UE operates in relaxed mode was brought up without any agreement as follows [1]:

- Option 2: RAN4 to consider minimum requirements at transitions for different criterions switching (Apple CMCC Oppo); - Option 6: RAN4 to introduce minimum requirements at transition when UE changes between relaxed and normal measurement modes based on same transition principle in release 16 relaxation requirements in IDLE/INACTIE states. Whether to introduce minimum requirements at transition when UE changes between relaxed and normal measurement modes in CONNECTED state is FFS (Ericsson vivo QC)

We understand the motivation to introduce transition requirements following the approach from Rel-16 power saving requirements in IDLE/INACTIVE states. For the cases when UE is only configured with one of Rel-16 and Rel-17 relaxation criteria, the current transition requirements defined in 4.2.2.8 in TS 38.133 can be reused. In addition, new transition requirements need to be introduced for the cases when UE is configured with multiple relaxation criteria (both Rel-16 and Rel-17 relaxation criteria) which are not covered by current requirements. These correspond to rows 6 – 11 in Table 1. For these cases, the principle from Rel-16 transition requirements can be reused which is as follows [3]:

- When switching from a less relaxed state to more relaxed state, UE shall fulfill the requirements corresponding to the less relaxed state over a measurement period (Trelaxed)

- When switching from more relaxed state to less relaxed state, the UE shall fulfill the requirements corresponding to the less relaxed state upon fulling the criteria

- When switching from normal mode to any relaxed mode, the UE shall fulfill the requirements corresponding to the normal mode over a measurement period (Tnormal)

- When switching from any relaxed mode to normal mode, UE shall fulfill the requirements corresponding to the normal mode upon fulfilling the switching criteria.

However, our current view is that it is premature for RAN4 to discuss transition requirements at this point since RAN2 has not yet agreed on the relaxation methods. As explained in earlier, what type of requirements RAN4 needs to introduce will depend on the relaxation methods.

- Proposal #9: The transition requirements for RedCap UE configured with both Rel-16 and Rel-17 relaxation criteria is defined based on principle used in Rel-16 transition requirements (4.2.2.8 in TS 38.133) as follows:

  - When switching from a less relaxed state (Rel-16 or Rel-17) to more relaxed state (Rel-16 or Rel-17), UE shall fulfill the requirements corresponding to the less relaxed state over a measurement period (Trelaxed)

  - When switching from more relaxed state (Rel-16 or Rel-17) to less relaxed state (Rel-16 or Rel-17), the UE shall fulfill the requirements corresponding to the less relaxed state upon fulling the criteria

  - When switching from normal mode to any relaxed mode (Rel-16 or Rel-17), the UE shall fulfill the requirements corresponding to the normal mode over a measurement period (Tnormal)

  - When switching from any relaxed mode (Rel-16 or Rel-17) to normal mode, UE shall fulfill the requirements corresponding to the normal mode upon fulfilling the switching criteria.

- Proposal #10: Discussions on transition requirements for RRC CONNECTED state shall be postponed until relaxation method is agreed in RAN2.

# Summary

In this contribution we have discussed relaxed RRM measurement requirements for RedCap UE in release 17. Based on the discussions, we have made following proposals:

- Proposal #1: For UE fulfilling Rel-17 stationary criterion, the relaxed requirements are defined by scaling the legacy requirements by factor K1’, where K1’=4.

- Proposal #2: For UE fulfilling both Rel-17 stationary criterion + Rel-17 not-at-cell edge criterion, relaxed requirements are defined using fixed long time period (T) in which the UE can stop its measurements for cell reselection, where T > 1 hour.

- Proposal #3: RAN4 to specify relaxed measurement requirements for the case when UE is configured with both Rel-16 and Rel-17 relaxation criteria and UE has fulfill both.

- Proposal #4: For the case when UE is configured with both Rel-16 and Rel-17 relaxation criteria and UE has fulfilled both criteria, UE is allowed to meet the requirements that are most relaxed out of Rel-16 and Rel-17 requirements.

- Proposal #5:  Rel-16/17 relaxed measurement requirements can be applied with eDRX cycles up to 10.24 seconds, without any PTW.

- Proposal #6: The maximum eDRX cycles with PTW up to which UE is allowed to apply Rel-16/17 relaxed measurement requirements is X ms, where value of X is FFS.

- Proposal 7: RAN4 discussions on relaxed requirements should be postponed until RAN2 has agreed on the relaxation methods for measurements in RRC CONNECTED state.

- Proposal #8: RAN4 shall wait and follow the conclusion from RAN2 conclusion regarding reporting rules for the stationary criterion.

- Proposal #9: The transition requirements for RedCap UE configured with both Rel-16 and Rel-17 relaxation criteria is defined based on principle used in Rel-16 transition requirements (4.2.2.8 in TS 38.133) as follows:

  - When switching from a less relaxed state (Rel-16 or Rel-17) to more relaxed state (Rel-16 or Rel-17), UE shall fulfill the requirements corresponding to the less relaxed state over a measurement period (Trelaxed)

  - When switching from more relaxed state (Rel-16 or Rel-17) to less relaxed state (Rel-16 or Rel-17), the UE shall fulfill the requirements corresponding to the less relaxed state upon fulling the criteria

  - When switching from normal mode to any relaxed mode (Rel-16 or Rel-17), the UE shall fulfill the requirements corresponding to the normal mode over a measurement period (Tnormal)

  - When switching from any relaxed mode (Rel-16 or Rel-17) to normal mode, UE shall fulfill the requirements corresponding to the normal mode upon fulfilling the switching criteria.

- Proposal #10: Discussions on transition requirements for RRC CONNECTED state shall be postponed until relaxation method is agreed in RAN2.

# References

- R4-2115364, “WF on R17 Redcap eDRX enhancements and RRM measurement relaxations”, Ericsson

- TS 38.304 V16.6.0

- TS 38.133 V17.3.0