3GPP TSG-RAN WG4 Meeting # 101-e 	    R4-2119063

Electronic Meeting, 1st November – 12th November 2021

Agenda Item:	5.1.6.3.1

Source: 	Ericsson

Title:	Reply LS on Rel-16 UE power saving requirements

Document for:	Approval

# Introduction

RAN4 has received a reply LS in [1] asksing RAN4 for clarification about UE requirements for the scenario when UE fulfills both low mobility and not-at-cell edge criterion. This clarification is discussed in this contribution and a reply LS is provided in Appendix.

# Discussion

In [1], clarification is asked regarding the UE requirements for the scenario when UE fulfills both lowMobilityEvalutation and not-at-cell edge criterion and these requirements are defined in sections 4.2.2.10.4 (inter-frequency) and 4.2.2.11.4 (inter-RAT) in [3]. We consider the inter-frequency case as an example for further discussions below.

##### 4.2.2.10.4 Measurements for UE fulfilling low mobility and not-at-cell edge criterion This clause contains requirements for measurements on inter-frequency NR cells provided that: - T331 timer is not running for EMR measurements on inter-frequency NR carrier, and - UE is configured with both lowMobilityEvalutation [2] criterion and cellEdgeEvaluation [2] criterion, and - Has also fulfilled both criteria, and - less than 1 hour have passed since measurements for cell reselection were last performed In this case the UE is not required to meet Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter as defined in Table 4.2.2.4-1.

Provided that the above conditions are fulfilled, the UE is not required to fulfill the legacy measurement requirements, i.e. the UE is allowed to not measure up to 1 hour. As highlighted in [1], in this scenario the the frequency layers of higher priority are less relaxed compared to the scenario when lowMobilityEvalutation criterion is fulfilled and highPriorityMeasRelax is configured. RAN2 has asked the question whether 1 hour shall be changed to 1 hour * Nlayers. RAN4 has spent many meeting and concluded on these requirements. Our view is that this part of the current requirement is not a typo and RAN4 shall therefore maintain the current requirements.

Also given that the UE is allowed to completely skip all types of meaurements for 1 hour in this scenario compared to the other scenario (lowMobilityEvalutation criterion is fulfilled and highPriorityMeasRelax is configured) where the UE is still measuring on the normal normal frequcy layers with higher measurement interval, we propose to to keep the current requirements.

Nevertheless, it shall be noted that the parameter highPriorityMeasRelax introduced by RAN2 is used to enable the NW to control whether the UE is allowed to relaxed RRM measurements for higher priority frequency for all use cases. The relaxation in the scenario when UE has fulfilled both low mobility criterion and not-at-cell-edge criterion applies to all carriers including higher priority carriers. It should be clarified that relaxation of higher priority carriers in this scenario is allowed only when highPriorityMeasRelax is configured.

# Conclusion

In this contribution, we have discussed the incoming RAN2 reply LS in [1], and based on the discussions we make following proposal:

- Proposal #1: RAN4 shall maintain existing requirements defined in clauses 4.2.2.10.4 and 4.2.2.11.4 in TS 38.133.

- Proposal #2: RAN4 to clarify that the relaxation of higher priority carriers in scenario when UE fulfills both lowMobilityEvalutation and not-at-cell edge criterion is allowed only when highPriorityMeasRelax is configured.

# References

[1]	R2-2108877, “Reply LS on RRM relaxation in power saving”, RAN2

[2]	R4-2108230, “LS on RRM relaxation in power saving”, RAN WG4

[3]	TS 38.133 V16.9.0

# Appendix

3GPP TSG-RAN WG4 Meeting # 101-e    				 									 R4-210xxxx

Electronic Meeting, 1st November – 12th November 2021

Title:	Reply LS on RRM relaxation in power saving

Response to:	R2-2108877

Release:	Rel-16

Work Item:	NR_UE_pow_sav-Core

Source:	RAN WG4

To:	RAN WG2

Cc:

Contact Person:

#### Name:	Santhan Thangarasa

#### E-mail Address:	santhan.thangarasa@ericsson.com

Attachments:

1. Overall Description:

RAN4 has discussed the measurement requirements for the case when both lowMobilityEvalutation and cellEdgeEvaluation criteria are fulfilled, referring to clauses 4.2.2.10.4 and 4.2.2.11.4 in TS 38.133. Based on the discussions, RAN4 has reached the conclusion to maintain the current requirements and to not make any changes in the measurement conditions.

2. To RAN WG2 group.

ACTION: 	RAN4 respectfully asks RAN2 to align their specifications with the above RAN4 requirements.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN4 Meeting #101-bis-e										17 - 25 January 2022, Electronic Meeting

TSG-RAN4 Meeting #102-e							    21 February – 3 March 2022, Electronic Meeting