3GPP TSG-RAN WG4 Meeting#101-e                	R4-2119104

Electronic Meeting, 1 – 12 November 2021

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on inter-frequency measurements for FR1 HST CA

Agenda item:	8.8.2.2

Document for:	Discussion

Introduction

During the RAN4#100-e meeting, a way forward on NR RRM enhancements for FR1 HST [1] was approved. Referring to [1], enhancements on inter-frequency PSS/SSS detection time in connected mode remain an open issue. Further, inter-frequency measurements in idle mode are also open issues.

In this contribution, we discuss the above open issues and present our views.

Discussion

From [1], a number of options on how to enhance inter-frequency PSS/SSS detection time requirements with measurement gaps:

- Inter-frequency measurement with MG, connected state

  - PSS/SSS detection time requirement for inter-frequency measurement with MG in connected state for HST

    - Option 1 (QC, Nokia):

| Condition NOTE1,2 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600ms, 6 Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(600ms, Ceil(6*M2 Note 3) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | 6 DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: When high speed is not configured, M2 = 1.5; When high speed is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1 |  |

    - Option 2 (CATT, MTK, OPPO, vivo, HW)

| Condition NOTE1,2 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600ms, 8 Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(600ms, Ceil(8*M2 Note 3) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | 8 DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: When high speed is not configured, M2 = 1.5. When high speed is configured, M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1. |  |

    - Option 3 (Ericsson, Nokia)

| Condition NOTE1,2 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600ms, N Note 4 Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(600ms, Ceil(N Note 4 M2 Note 3) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | N Note 4 DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1. NOTE 4: When highSpeedMeasFlag-r16 is not configured, N = 8; otherwise N = 5. |  |

    - Option 4 (Apple)

| Condition NOTE1,2 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600ms, Ceil(8*M2/1.5) * Max(MGRP, SMTC period)) * CSSFinter |
| DRX cycle ≤ 320ms | Max(600ms, Ceil(8*M2 Note 3) * Max(MGRP, SMTC period, DRX cycle)) * CSSFinter |
| DRX cycle > 320ms | Ceil(8*M2/1.5) * DRX cycle * CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: When high speed is not configured, M2 = 1.5; When high speed is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1 |  |

    - Option 5 (CMCC)

| Condition NOTE1,2 | T PSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600ms, 6 Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle≤ 320ms | Max(600ms, Ceil(6 M Note 3) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | Y Note 4 DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: M = 1.5 if SMTC periodicity > 40 ms, otherwise M=1 NOTE 4: Y= 4 when SMTC <= 40ms, Y= 6 when SMTC > 40ms |  |

    - Option 6 (QC, Nokia, CMCC, Ericsson, Xiaomi)

| Condition NOTE1,2 | TPSS/SSS_sync_inter for FR1 HST |
| --- | --- |
| No DRX | Max(600ms, 6 Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(600ms, Ceil(6 M2 Note 3) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | 6 DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1 |  |

Following the same technical argument as provided in our previous contribution, the same methodology for Rel-16 FR1 HST can be used to derive the proposed enhancement, which results in Option 3. Even though we prefer Option 3 but can compromise on Option 6. Option 1 seems similar to Option 6.

Proposal 1: For enhancements of inter-frequency PSS/SSS detection time requirements with measurement gaps, we can compromise on Option 6.

From [1], several options for enhancements on inter-frequency measurement requirements in idle state are

- Option 1 (QC, CATT, CMCC, Ericsson): using the same requirement as for intra-frequency measurement for HST

| DRX cycle length [s] | Tdetect,NR_Inter [s] (number of DRX cycles) | Tmeasure,NR_Inter [s] (number of DRX cycles) | Tevaluate,NR_Inter [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 2.56 x M2 (8 x M2) | 0.32 x M3 (1 x M3) | 0.96 x M4 (3 x M4) |  |
| 0.64 | 5.12 (8) | 0.64 (1) | 1.92 (3) |  |
| 1.28 | 8.96 (7) | 1.28 (1) | 3.84 (3) |  |
| 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |  |
| Note 1: when SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 1.5, M3 = M4 = 2 |  |  |  |  |

- Option 2 (OPPO, HW, MTK, vivo, Xiaomi): The R16 enhanced EUTRA-NR inter-RAT measurement requirements in idle mode could be reused for NR inter-frequency measurements

| DRX cycle length [s] | Tdetect,NR_Inter [s] (number of DRX cycles) | Tmeasure,NR_Inter [s] (number of DRX cycles) | Tevaluate,NR_Inter [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 4.16 x M2 (13 x M2)Note 1 | 0.64 x M3 (2 x M3)Note 1 | 0.96 x M4 (3 x M4) Note 1 |
| 0.64 | 7.68 (12) | 1.28 (2) | 1.92 (3) |
| 1.28 | 12.8(10) | 1.28 (1) | 3.84 (3) |
| 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |
| Note 1: M2=1.5, M3=2 and M4=2 if SMTC periodicity of measured intra-frequency cell > 40 ms; otherwise M2=M2=M3=1. |  |  |  |

Based on the same technical argument as for the inter-frequency measurements in connected, the same methodology for Rel-16 FR1 HST can be adopted to derive the requirement.

Proposal 2: For enhancements of inter-frequency measurements in idle mode for FR1 NR HST, the same requirement as for intra-frequency measurements for FR1 HST is used – Option 1.

Conclusion

The document has discussed and presented our views on enhancements inter-frequency measurements with measurement gaps in connected mode and inter-frequency measurements in idle mode for FR1 HST CA.

Proposal 1: For enhancements of inter-frequency PSS/SSS detection time requirements with measurement gaps, we can compromise on Option 6.

Proposal 2: For enhancements of inter-frequency measurements in idle mode for FR1 NR HST, the same requirement as for intra-frequency measurements for FR1 HST is used – Option 1.

References

[1]    R4-2115439, WF on RRM for FR1 HST, CMCC