3GPP TSG-RAN WG4 Meeting#101-e                	R4-2119107

Electronic Meeting, 1 – 12 November 2021

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on L1-SINR measurements for Rel-17 FR1 HST CA

Agenda item:	8.8.2.3

Document for:	Discussion

Introduction

During the RAN4#100-e meeting, a way forward on NR RRM enhancements for FR1 HST [1] was approved. One remaining open issue concerns the applicability of Rel-16 L1-SINR measurement accuracy requirements to Rel-17 FR1 HST.

To deal with the above open issue, we provide our simulation results and present our views to go forward.

Discussion

From [1], the simulation assumptions are as follows:

| Simulation parameters | Comments/values |  |
| --- | --- | --- |
| Carrier frequency for Cell 1 | 3.6 GHz |  |
| channel | AWGN |  |
| BS transmit antennas for CSI-RS | 1 tx |  |
| UE receive antennas | 2 rx |  |
| Data channel subcarrier spacing | The same as CSI-RS subcarrier spacing |  |
| Measurement period (in number of measurement samples) | 1 sample |  |
| Subcarrier spacing | 15 kHz; 30 kHz |  |
| CSI-RS periodicity | 5 ms |  |
| Carrier frequency offset | Option 1: 972Hz for 15kHz; 1667Hz for 30kHz | Option 2: 1944Hz for 15kHz; 3334Hz for 30kHz |
| Propagation condition | High speed train condition specified in Table G.3 in TS 38.104 |  |
| Number of PRBs | 48 |  |
| Density | 3 |  |
| Side condition (SNR) on CMR | -3 dB |  |
| Frequency tracking | TRS |  |

As the aim of the simulation studies is to confirm whether the Rel-16 L1-SINR measurement accuracy requirements are applicable to FR1 HST, it is sufficient to consider only the largest carrier frequency offset. Referring to the above table, Option 2: carrier frequency offset = 1944 Hz (15 kHz SCS) and 3334 Hz (30 kHz SCS) are larger than the carrier frequency offset in Option 1 and in Table G.3, TS 38.104 under high speed condition for UE velocity 500 km/h. It is worth mentioning that the conclusion reached for the SS-SINR accuracy requirement for FR1 HST was based on AWGN channel conditions. Thus, a similar channel should be adopted for the simulation studies instead of other channel conditions.

In the simulation, TRS was configured which is used to track Doppler shifts. The standard configuration of TRS consists of four, density-3 CSI-RS across two consecutive slots. The CSI-RS within a specific slot is always separated by four symbols, which sets the limit on the maximum carrier frequency offset that can be successfully tracked. As such, the maximum theoretical carrier frequency offset that can be tracked is 1750 Hz and 3500 Hz for 15 kHz SCS and 30 kHz SCS, respectively. This is confirmed by our simulation results for L1-SINR accuracy as shown in Figure 1.

The simulation results are derived from the CDF plots in Figure 2. Note, the simulation results are raw data with no additional implementation margins included. The SNR region that is of interest is SNR ≥ -3 dB. Within this region, the L1-SINR accuracy is within the average L1-SINR accuracy (without additional implementation margins) for the Rel-16 eMIMO. Therefore, we propose to adopt the Rel-16 L1-SINR accuracy for CSI-RS based CMR for FR1 HST.

Observation 1: Using TRS for tracking Doppler shift, the achieved L1-SINR accuracy under FR1 HST scenarios is within that of Rel-16 L1-SINR accuracy for CSI-RS based CMR only.

Proposal 1: Rel-16 L1-SINR accuracy requirements for CSI-RS based CMR are applicable to FR1 HST.

Figure 1: L1-SINR accuracy vs SNR with Doppler shift compensation using TRS

Figure 2: CDF plots for L1-SINR accuracy with Doppler shift compensation using TRS

Conclusion

The contribution has presented the simulation results of L1-SINR accuracy for CSI-RS based SMR only under FR1 HST scenarios. Based on our simulation results, we have made the following observation and proposal:

Observation 1: Using TRS for tracking Doppler shift, the achieved L1-SINR accuracy under FR1 HST scenarios is within that of Rel-16 L1-SINR accuracy for CSI-RS based CMR only.

Proposal 1: Rel-16 L1-SINR accuracy requirements for CSI-RS based CMR are applicable to FR1 HST.

References

[1]    R4-2115439, WF on RRM for FR1 HST, CMCC