3GPP TSG-RAN WG4 Meeting #101-e	R4-2119110

Electronic Meeting, 1st - 12th Nov, 2021

Source:	ZTE Corporation

Title:	Discussion on RRC Idle/Inactive and Connected state mobility requirements for HST FR2

Agenda Item:	8.9.4.3

Document for:	Approval

- Introduction

In the last meeting, a way forward on RRM NR FR2 HST was approved in [1]:

- CONNECTED state mobility： - DRX upper bound Way forward: FFS the DRX upper bound for enhanced RRM HST FR2 requirements: - Option 1: Up to [256] ms - Option 2: [80] ms - Option 3: [60] ms - Other options are not precluded - Requirements for long DRX Agreement: Apply existing R16 requirements for long DRX cycles, i.e. above the upper bound of DRX cycle. - SMTC periodicity Agreement: HST FR2 CPE has a capacity to support both uni- and bi-directional operation. Enhancements for short DRX Way forward: Further consider the possibility of requirement enhancement for SMTC periodicity <= [40] ms - Option 1: M2 = 1.5 if SMTC periodicity > [40] ms, otherwise M2=1. - Option 2: Keep the 1.5 relaxation factor. Connection Re-establishment to known cell Way forward: FFS time to identify target NR cell for RRC connection re-establishment to known NR intra-frequency cell: - Option 1: MAX (TBD ms, 5 x N1 x TSMTC). - Option 3: Do not introduce any enhancements. - Other options are not precluded Connection Re-establishment to unknown cell Way forward: FFS time to identify target NR cell for RRC connection re-establishment to unknown NR intra-frequency cell: - Option 1: Keep current requirements with N1 scaling factor corresponding the number of RX beams: MAX (1000 ms, 10 x N1 x TSMTC) - Option 2: Do not introduce any enhancements. - Other options are not precluded - Handover： Agreement: Criteria of known cell for FR2: The target FR2 cell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds; otherwise, it is unknown. Way forward: FFS the following issues: - The scaling factor in HO requirement corresponding the number of RX beam sweep. - A feasibility of HO to unknow cell in HST FR2 deployments. - IDEL/INACTIVE state mobility： Agreements: - Use cell selection criterion S and number of DRX cycles as a baseline for cell selection requirement. - Use table below as a background for further discussion of HST FR2 IDLE mode requirement: Tdtect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra for UE configured with RRM enhancements for high speed (Frequency range FR2) DRX cycle length [s]; Tdetect,NR_Intra [s] (number of DRX cycles); Tmeasure,NR_Intra [s] (number of DRX cycles); Tevaluate,NR_Intra [s] (number of DRX cycles). 0.32; 2.56 x N1 x M2 (8 x N1 x M2); 0.32 x N1 x M3 (1 x N1 x M3); 0.96 x N1 x M4 (3 x M4). 0.64; 5.12 x N1 (8 x N1); 0.64 x N1 (1 x N1); 1.92 x N1 (3 x N1). 1.28; 8.96 x N1 (7 x N1); 1.28 x N1 (1 x N1); 3.84 x N1 (3 x N1). 2.56; 58.88 x N1 (23 x N1); 2.56 x N1 (1 x N1); 7.68 x N1 (3 x N1). Note 1: when SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 1.5, M3 = M4 = 2. Way forward: FFS the following issues: - The upper bound of DRX cycle with enhanced requirements: - Option 1: Use 320 ms DRX cycle as baseline for following analysis and requirement definition - Other options are not precluded - The values of scaling factors: N1, M2, and SMTC: - Option 1: N1 is not necessarily the agreed Rx numbers for each scenario, N1 = 3 - Option 2: N1 refers to agreed Rx numbers - Other options are not precluded.

Based on the conclusion in 100 meeting, for the mobility requirements of CONNECTED state and IDLE/INACTIVE state, some issues are still open, we provide our analysis for the following issues:

  - For CONNECTED state

- Enhancements for short DRX

- Connection Re-establishment to known cell and unknown cell

- Handover requirement

  - For IDLE/INACTIVE state

- The upper bound of DRX cycle

- The values of scaling factors

- Discussion

2.1 CONNECTED state

2.1.1 Enhancements for short DRX

It has been agreed that for long DRX cycle, reusing existing Rel-16 requirements, But the upper bound of DRX cycle is still open, we have on strong preference for all candidates. Based on the final upper bound of DRX cycle, for the short DRX case, the requirement enhancements should be further discussed. According to the discussion during 100 meeting, the following two options are suggested:

Further consider the possibility of requirement enhancement for SMTC periodicity <= [40] ms - Option 1: M2 = 1.5 if SMTC periodicity > [40] ms, otherwise M2=1. - Option 2: Keep the 1.5 relaxation factor.

Option 1 is actually reusing the enhancement in Rel-16 FR1 HST. Replace the scaling factor 1.5 with M2, and M2 = 1.5 if SMTC periodicity > [40] ms, otherwise M2=1, we believe such solution is applicable to both FR1 HST and FR2 HST, so we support using Option 1, which aligned with R16 FR1 HST enhancement.

Proposal 1: Reusing Rel-16 FR1 HST enhancement is enough, so we support M2 = 1.5 if SMTC periodicity > [40] ms, otherwise M2=1.

2.1.2 Connection Re-establishment to known cell and unknown cell

According to chapter 6.2.1 in [2], there isn’t any definition of known FR2 cell. So we believe it is necessary to define criteria of known cell for FR2, including intra-frequency and inter-frequency.

Table 6.2.1.2.1-1: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell

| Serving cell | FR of target NR | Tidentify_intra_NR [ms] |  |
| --- | --- | --- | --- |
| SSB Ês/Iot (dB) | cell | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (200 ms, 5 x TSMTC) | MAX (800 ms, 10 x TSMTC) |
| ≥ -8 | FR2 | N/A | MAX (1000 ms, 80 x TSMTC)) |
| < -8 | FR1 | N/A | 800Note1 |
| < -8 | FR2 | N/A | 3520Note1 |
| Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB. |  |  |  |

Table 6.2.1.2.1-2: Time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell

| Serving cell SSB Ês/Iot (dB) | FR of target NR cell | Tidentify_inter_NR, i [ms] |  |
| --- | --- | --- | --- |
|  |  | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (200 ms, 6 x TSMTC, i) | MAX (800 ms, 13 x TSMTC, i) |
| ≥ -8 | FR2 | N/A | MAX (1000 ms, 104 x TSMTC, i)) |
| < -8 | FR1 | N/A | 800Note1 |
| < -8 | FR2 | N/A | 4000Note1 |
| Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB. |  |  |  |

For FR2 HST scenario, focus on intra-frequency cell identify, according to all proposed solution, actually two aspects enhancements are possible:

- Aspect 1: Add the identify latency for known cell in FR2

- Aspect 2: Modify the identify latency for unknown cell in FR2 based on the RX beam number conclusion

For Aspect 1, we support adding MAX(200ms, 5 x N1 x TSMTC) for known cell in FR2, where N1 depends on the number of RX beam.

For Aspect 2, replace MAX (1000 ms, 80 x TSMTC)) with MAX (1000 ms, 10 x N1 x TSMTC)), where N1 also depends on the number of RX beam.

Overall, we suggest using the following enhancements for FR2 HST:

| Serving cell | FR of target NR | Tidentify_intra_NR [ms] |  |
| --- | --- | --- | --- |
| SSB Ês/Iot (dB) | cell | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (200 ms, 5 x TSMTC) | MAX (800 ms, 10 x TSMTC) |
| ≥ -8 | FR2 | MAX(200ms, 5 x N1 x TSMTC) | MAX (1000 ms, 10 x N1 x TSMTC)) |
| < -8 | FR1 | N/A | 800 Note1 |
| < -8 | FR2 | N/A | 3520 Note1 |
| Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB. |  |  |  |

Proposal 2: For the enhancement of connection re-establishment, enhanced as the following table.

| Serving cell | FR of target NR | Tidentify_intra_NR [ms] |  |
| --- | --- | --- | --- |
| SSB Ês/Iot (dB) | cell | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (200 ms, 5 x TSMTC) | MAX (800 ms, 10 x TSMTC) |
| ≥ -8 | FR2 | MAX(200ms, 5 x N1 x TSMTC) | MAX (1000 ms, 10 x N1 x TSMTC)) |
| < -8 | FR1 | N/A | 800 Note1 |
| < -8 | FR2 | N/A | 3520 Note1 |
| Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB. |  |  |  |

2.1.3 Handover requirement

According to the WF attached in 100 meeting, the feasibility of HO to unknown cell is still FFS. From our perspective, in HST scenario, since the railway track is fixed, the possibility of handover to an unknown cell is very slim, so we do not need to consider handover to unknown cell.

Proposal 3: Do not need to consider handover to unknown cell.

Considering for the handover requirement enhancement, as defined in [2], Tinterrupt = Tsearch + TIU + Tprocessing  + T∆ + Tmargin ms. For handover to known cell, then Tsearch = 0 ms. Trs is TSMTC,  Tprocessing can be up to 20ms, and Tmargin can be up to 2ms. T∆ = Trs for both known and unknown target cell. So Tinterrupt = TIU + TSMTC + 22  ms.

For TIU, which is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. According to Table 8.1-1 in [2], the candidate values of TIU are {10+10, 20+10, 40+10, 80+10, 160+10} ms. Even for the worst case, i.e. TIU is 170ms, Tinterrupt = TSMTC + 192  ms, the CPE can only move 18 meters during 192 ms, so the limitation of SMTC periodicity can be relaxed.

Proposal 4: For handover to known cell case, the limitation of SMTC periodicity can be relaxed.

2.2 IDLE/INACTIVE state

2.2.1 The upper bound of DRX cycle

Actually FR2 HST CPE is not common user, but a dedicated equipment, so the power consumption is not a bottleneck. The motivation for longer DRX cycle in IDLE mode is not appealing. Similar as in CONNECTED mode, we can determine an upper bound of DRX cycle for IDLE mode, so that the requirement enhancements only focus on the short DRX cycle case.

For simplicity, the same upper bound of DRX cycle in Rel-16 HST can be reused, i.e. 320 ms. Based on the conclusion of RX beam number, the values of scaling factors should be determined.

Proposal 5: Use 320 ms DRX cycle as baseline for following analysis and requirement definition.

2.2.2 The values of scaling factors

The following table was agreed during 100 meeting, so only N1 is still FFS.

Tdtect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra for UE configured with RRM

enhancements for high speed (Frequency range FR2)

| DRX cycle length [s] | Tdetect,NR_Intra [s] (number of DRX cycles) | Tmeasure,NR_Intra [s] (number of DRX cycles) | Tevaluate,NR_Intra [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 2.56 x N1 x M2 (8 x N1 x M2) | 0.32 x N1 x M3 (1 x N1 x M3) | 0.96 x N1 x M4 (3 x M4) |  |
| 0.64 | 5.12 x N1 (8 x N1) | 0.64 x N1 (1 x N1) | 1.92 x N1 (3 x N1) |  |
| 1.28 | 8.96 x N1 (7 x N1) | 1.28 x N1 (1 x N1) | 3.84 x N1 (3 x N1) |  |
| 2.56 | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |  |
| Note 1: when SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 1.5, M3 = M4 = 2 |  |  |  |  |

This table is almost similar with Rel-16 HST enhancement, just except for N1. In our opinion here N1 means RX beam number, which we support N1 refers to agreed RX beam number.

Proposal 6: The value of N1 refers to agreed RX beam number.

- Conclusion

In this contribution, we have the following proposals for CONNECTED and IDLE/INACTIVE state requirements enhancement:

Proposal 1: Reusing Rel-16 FR1 HST enhancement is enough, so we support M2 = 1.5 if SMTC periodicity > [40] ms, otherwise M2=1.

Proposal 2: For the enhancement of connection re-establishment, enhanced as the following table.

| Serving cell | FR of target NR | Tidentify_intra_NR [ms] |  |
| --- | --- | --- | --- |
| SSB Ês/Iot (dB) | cell | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (200 ms, 5 x TSMTC) | MAX (800 ms, 10 x TSMTC) |
| ≥ -8 | FR2 | MAX(200ms, 5 x N1 x TSMTC) | MAX (1000 ms, 10 x N1 x TSMTC)) |
| < -8 | FR1 | N/A | 800 Note1 |
| < -8 | FR2 | N/A | 3520 Note1 |
| Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB. |  |  |  |

Proposal 3: Do not need to consider handover to unknown cell.

Proposal 4: For handover to known cell case, the limitation of SMTC periodicity can be relaxed.

Proposal 5: Use 320 ms DRX cycle as baseline for following analysis and requirement definition.

Proposal 6: The value of N1 refers to agreed RX beam number.

- Reference

R4-2115334, “WF on FR2 HST RRM (part 1)”, Nokia, Nokia Shanghai Belll.

TS 38.133-h20