3GPP TSG-RAN WG4 Meeting#101-e                               R4-2119190

E-meeting,1st Nov– 12th Nov, 2021

Agenda item:	8.16.4.1

Source: 	ZTE Corporation

Title: 	Discussion on BS Tx requirements for 52.6-71GHz

Document for:	Approval

Introduction

In the last RAN4 meeting, there were lots of progress for BS Tx requirements for 52.6-71GHz [2]. In this contribution, we want to share some further considerations on the remaining open issues for BS TX requirements.

Discussion

## 2.1.TAE requirements for MIMO, CA case

As discussed in 60GHz SI phase, it was recognized that TAE 65ns for MIMO transmission might be not valid anymore especially considering the CP length for 960kHz SCS as shown in Table 1. TAE 65ns is almost close to CP length of 960kHz which would result in serious performance degradation at UE receiver side, we provided some initial simulation results are provided in Figure 1 and Figure 2 to show the impacts of different TAE values on downlink performance. Detailed simulation assumptions are listed in Annex 5.

Table 1. Summary for CP and data length for different SCS

| u | SCS | CP[ns] | data[ns] | Sampling Rate [MHz] |
| --- | --- | --- | --- | --- |
| 2 | 60 | 1171.875 | 16666.6 | 245.76 |
| 3 | 120 | 585.9375 | 8333.3 | 491.52 |
| 4 | 240 | 292.96875 | 4166.6 | 983.04 |
| 5 | 480 | 146.484375 | 2083.3 | 1966.08 |
| 6 | 960 | 73.2421875 | 1041.6 | 3932.16 |

Figure 1. the impacts of TAE on downlink link performance [960kHz]

Figure 2. the impacts of TAE on downlink link performance [480kHz]

Based on the initial simulation results in Figure 1 and Figure 2, it could be easily found that 65ns TAE would cause severe performance degradation especially for 960kHz.

For 960kHz SCS, the acceptable TAE requirement should be around 10-20ns for 960kHz and 10-40ns for 480kHz,

Proposal 1: the acceptable TAE requirement should be around 10-20ns for 960kHz and 10-40ns for 480kHz;

## 2.2. EVM requirements for 52.6-71GHz

Regarding EVM window length for 52.6-71GHz, since this requirement is mainly related with waveform used implemented to meet the out of band emissions and in-band EVM requirement, however at the current phase, out of band emission requirement and ACLR requirement is still pending, therefore we would like to postpone the discussion into conformance testing phase.

Proposal 2: to postpone EVM window requirement into conformance testing phase due to unclear out of band emission requirements;

## 2.4. Unwanted emission requirements

In the last RAN4 meeting, there were some discussions to further evaluate coexistence performance for 52.6-71GHz or coexistence work in TR 38.803 can be used as reference. Based on our initial simulation results in the companion contribution [7], for indoor scenario, more stringent DL ACIR and UL ACIR requirement as summarized in Table 2 and Table 3 are expected due to different power assumption and antenna configuration assumed for 52.6-71GHz compared with assumption in TR38.803, otherwise the coexistence requirement with 5% performance loss cannot been ensured, therefore we propose to follow the outcome of coexistence study in another agenda for ACLR and unwanted emission requirements.

Proposal 3: to propose to follow the outcome of coexistence study of 52.6-71GHz for ACLR and out of band emission requirement.

Table 2. summary of required ACIR requirements for indoor DL case

| Case | Metric | Required ACIR |
| --- | --- | --- |
| 60GHz 100MHz | Average throughput loss | 13.87 |
|  | Cell edge through loss | 23.83 |
| 70GHz 400MHz | Average throughput loss | 14.06 |
|  | Cell edge through loss | 19.98 |

Table 3. summary of required ACIR requirements for indoor UL case

| Case | Metric | Required ACIR |
| --- | --- | --- |
| 60GHz 100MHz | Average throughput loss | 8.10 |
|  | Cell edge through loss | 22.57 |
| 70GHz 400MHz | Average throughput loss | 8.12 |
|  | Cell edge through loss | 16.68 |

Conclusions

In this contribution, we want to further considerations on each BS Tx requirements for 60GHz and some proposals and observations are made as following:

Proposal 1: the acceptable TAE requirement should be around 10-20ns for 960kHz and 10-40ns for 480kHz;

Proposal 2: to postpone EVM window requirement into conformance testing phase due to unclear out of band emission requirements;

Proposal 3: to propose to follow the outcome of coexistence study of 52.6-71GHz for ACLR and out of band emission requirement.

References

- R4-2106113,WF on BS RF TX requirements for 52.6 – 71 GHz, approved.

- R4-2108638,WF on BS TX RF requirements for 52.6 – 71 GHz, approved.

- R4-210xxx, initial coexistence simulation results for 52.6-71GHz, ZTE

- R4- 1713558, Link Level Simulation Results for EVM FR2, Ericsson

- R4-2115643,WF on BS Tx RF requirement, approved.

- R4-2115643,WF on BS Tx RF requirement, approved.

- R4-211xxx, Coexistence simulation results for 52.6-71GHz, ZTE

Annex: simulation assumption

Table A1-1 Link level simulation assumption

| Parameters | Values or assumptions |
| --- | --- |
| Carrier Frequency | 60GHz |
| Waveform | CP-OFDM |
| System Bandwidth | 400MHz |
| Subcarrier spacing | 480kHz/960kHz |
| CP type | Normal CP |
| Channel Model | TDL-A 5ns |
| PN model | 3GPP TR38.803 example 2 |
| DMRS Configuration | 2 DMRS symbols per slot at (2,11) |
| PTRS Configuration | (K = 2, L = 1) |
| SLIV | (S=0, L=14) |
| Channel Estimation | Realistic |
| PN Estimation | Realistic |
| PN compensation | CPE & ICI |
| UE receiver | MMSE-IRC |
| BS antenna Array configuration | 2 |
| UE antenna Array configuration | 2 |