3GPP TSG-RAN WG4 Meeting#101-e                               R4-2119193

E-meeting,1st Nov– 12th Nov, 2021

Agenda item:	8.18.2.2

Source: 	ZTE Corporation

Title: 	Discussion on phase discontinuity and power inconsistency tolerance across different repetitions

Document for:	Approval

Introduction

In the last RAN4 meeting, there were extensive discussions on phase discontinuity for NR coverage enhancement and reply LS [2] has been sent back to RAN1. However there are also some remaining open issues as listed in WF [5] left for further discussions, therefore in this contribution, we want to share some further considerations.

Discussion

## 2.1. Required tolerance of the phase continuity and amplitude consistency

In the previous RAN4 meetings, the majority of interested companies support to have some LLS to evaluate the acceptable tolerance from network performance perspective. Therefore in this contribution, the following scenarios are evaluated to check the impacts of phase discontinuity and power inconsistency on the gNB performance and detailed simulation assumptions are listed in Annex 5.

Table 1. evaluation cases for phase continuity and power consistency

| Scenario | Configurations | Note |
| --- | --- | --- |
| Case 1: Phase discontinuity across different repetitions | 700MHz/4GHz, 2 /8repetition, Baseline: 0 Option 2: uniform distribution | Relative phase discontinuity is added on top of 1st transmission |

Figure 1. The performance impact of phase error on joint channel estimation for PUSCH for 700MHz

Figure 2. The performance impact of phase error on joint channel estimation for PUSCH for 4GHz

Based on initial simulation results in Figure 1 and Figure 2, the acceptable phase error between different repetitions should be within 10o-20o.

Proposal 1: propose the phase error between different repetitions within 10o-20o.

Table 2. evaluation cases for the power consistency

| Scenario | Configurations | Note |
| --- | --- | --- |
| Case 2: Power inconsistency across different repetitions | 700MHz/4GHz, 2/8 repetition, Baseline : 0 Option 2: uniform distribution | Relative power inconsistency is added on top of 1st transmission repetition |

Figure 3. The performance impact of amplitude error on gNB receiver performance for 700MHz

Figure 4. The performance impact of amplitude error on gNB receiver performance for 4GHz

Based on initial simulation results in Figure 3/4, it can be found that 1dB amplitude error will also cause significant performance degradation (e.g. at least 3.97dB deterioration @10% BLER for UMA and 3.81dB deterioration @10% BLER for RMA ), the acceptable amplitude error between different repetitions should be less than 0.5dB.

Proposal 2: propose the amplitude error between different repetitions less than 0.5dB.

Conclusion

- In this contribution, we shared some further analysis on the remaining open issues listed in the approved WF [1] and some proposals and observations are made as following:

Proposal 1: to propose the phase error between different repetitions within 10o-20o;

Proposal 2: to propose the amplitude error between different repetitions less than 0.5dB.

References

- R4-2107881,WF on phase continuity and power consistency for PUCCH and PUSCH repetition, approved

- R4-2107880,Reply LS on PUCCH and PUSCH repetition, approved.

- R4-2111706, LS on joint channel estimation for PUSCH and PUCCH, approved.

- R4-2105477,Email discussion summary for [98-bis-e][140] NR_cov_enh

- R4-2114992, WF on phase continuity and power consistency for PUCCH and PUSCH transmissions, approved.

Annex: Simulation assumptions

- Table A-1 Simulation assumption for different DMRS patterns in the time domain

| Parameter | Value |
| --- | --- |
| - Carrier frequency | - 700MHz |
| - Channel model | - TDL-C (delay spread: 300ns) as in 38.901 for Rural scenario |
| - UE speed | - 3 km/h |
| - BS antenna configuration | - 4Rx |
| - UE antenna configuration | - 1 Tx |
| - System bandwidth | - 20 MHz |
| - Sub-carrier spacing | - 15 kHz |
| - Occupied RB | - 4 |
| - MCS | - 0/1 |
| - Waveform | - DFT-s-OFDM |
| - Number of OS per repetition | - 14 (PUSCH mapping type A) |
| - DMRS overhead | - 1/2 DMRS symbols |
| - Number of repetitions | - 8 |
| - Frequency hopping | - Disabled |
| - Channel estimation | - Practical |
| - Receiver type | - MMSE |

- Table A-2 Simulation assumption for different DMRS patterns in the time domain

| Parameters | Values |  |
| --- | --- | --- |
| - Scenario | - Urban , O2I | - Rural, O2I |
| - Duplexing scheme and frequency | - 4GHz TDD | - 4GHz TDD |
| - Frame structure for TDD | - DDSUU (S: 10D:2G:2U) for 4GHz |  |
| - Transmission bit rate for data channel (bit/s) | - 1 Mbps for eMBB | - 100 kbps for eMBB |
| - Subcarrier spacing | - 30kHz | - 30kHz for TDD |
| - Channel model for link-level simulation | - TDL-C | - TDL-C |
| - Pathloss model (select from LoS or NLoS) | - NLos | - NLos |
| - Delay Spread | - 300ns | - 300ns |
| - UE velocity | - 3 km/h | - 3 km/h for O2I |
| - Number of RF chains for BS | - 4 | - 4 |
| - Number of UE antennas | - 1 | - 1 |
| - Number of RF chains for UE | - 1 | - 1 |
| - Number of occupied RBs | - 30 | - 4 |
| - MCS | - MCS4 | - MCS2 |
| - DMRS overhead | - Type I, 2 DMRS symbol (one front- loaded and one additional), no multiplexing data |  |
| - Frequency hopping | - Disabled |  |
| - Channel estimation | - Practical |  |
| - Receiver type | - MMSE |  |
| - BLER Target | - 10% iBLER for eMBB |  |
| - Waveform | - DFT-s-OFDM |  |
| - Number of repetitions | - 8 repetitions with JCE |  |