3GPP TSG-RAN WG4 Meeting #101-e                   	R4-2119221

Electronic Meeting, 1st – 12th Nov., 2021

Agenda item:		4.1.6

Source:	MediaTek Inc.

Title:	Discussion on applicable DRX cycle in NE-DC and NR-DC mode

Document for:		Discussion

1	Introduction

In RAN4 #100-e meeting, a CR [1] was agreed to clarify the applicable DRX cycle for the infra-frequency measurement requirement in NE-DC and NR-DC mode. However, the applicable principles for inter-frequency measurement requirement are not yet been discussed. In this paper, we provide our views on applicable DRX cycle for inter-frequency measurement requirement.

2	Discussion

In RAN4#89 meeting, the agreed applicable DRX cycle for inter-frequency measurement requirement in EN-DC mode is [2]:

- Inter-frequency and inter-RAT measurement requirements shall follow the below principle when different DRX is configured in MCG and SCG for EN-DC: Rel-15 ENDC; Applicable DRX. Measurement objects configured by MN; Inter-frequency LTE; Follow MCG DRX configuration and state. Inter-RAT NR (36.133 8.17.4); Follow SCG DRX configuration and state. Inter-RAT UTRA/ GSM; Follow MCG DRX configuration and state. Measurement objects configured by SN; Inter-frequency NR (38.133 9.3); Follow SCG DRX configuration and state.

The conclusions were made based on the principles that “which DRX cycle is required to be followed must consider the rules that the measurement objects will be measured by NR device or non-NR device” However, at that time, RAN4 did not discuss related rules in NR-DC and NE-DC mode. For the sake of spec completeness, it is necessary to clarify UE DRX cycle in use for NR-DC mode and NE-DC mode.

Proposal 1: RAN4 to specify the applicable DRX cycle for the infer-frequency measurement requirement in NR-DC and NE-DC mode

If we further explore the UE architecture in NR-DC mode, what make impact is “the measurement objects will be measured by MCG's or SCG's baseband and RF.” Originally we think that RAN4 could define applicable DRX cycle for the infer-frequency measurement requirement based on the CG with the same FR. However, considering that it is a rel-15 requirement and each company already has its own design, it seems to be more appropriate to allow all deployment in the spec than applying a very tight requirement. So we suggest that

Proposal 2: In NR-DC mode, the applicable DRX cycle for the infer-frequency measurement requirement follows the maximum of configured MCG DRX cycle and SCG DRX cycle

For NE-DC, now there exist 4 difference scenarios to be considered: FR1 only CA, FR2 only intra band CA, FR2 only inter band CA, and FR1 +FR2 CA (FR1 PCell). Similar principles can be leveraged, and we have following proposals.

Proposal 3: In NE-DC mode, the applicable DRX cycle for the infer-frequency measurement requirement follows the maximum of configured MCG DRX cycle and SCG DRX cycle

3	Summary

In this contribution, we propose

Proposal 1: RAN4 to specify the applicable DRX cycle for the infer-frequency measurement requirement in NR-DC and NE-DC mode

Proposal 2: In NR-DC mode, the applicable DRX cycle for the infer-frequency measurement requirement follows the maximum of configured MCG DRX cycle and SCG DRX cycle

Proposal 3: In NE-DC mode, the applicable DRX cycle for the infer-frequency measurement requirement follows the maximum of configured MCG DRX cycle and SCG DRX cycle

Reference

- R4-2115237, “CR on TS38.133 for applicable DRX cycle in EN-DC, NR SA, NE-DC, and NR-DC,” MediaTek Inc., Aug., 2021.

- R4-1816109, “Corrections to DRX cycle for EN-DC operation in sections  9.3.x,” Ericsson, Feb., 2018