| 3GPP TR 36.717-04-01 V0.7.0 (2021-11) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Networks; LTE inter-band Carrier Aggregation for x bands DL (x=4, 5) with 1 band UL (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	9

1	Scope	11

2	References	13

3	Definitions of terms, symbols and abbreviations	14

3.1	Terms	14

3.2	Symbols	14

3.3	Abbreviations	14

4	Background	14

4.1	TR maintenance	14

5	4 Band Carrier Aggregation with Single UL: Specific Band Combination Part	15

5.1	CA_2-5-7-66 / CA_2-5-7-66-66	15

5.1.1	Channel bandwidths per operating band for CA	15

5.1.2	∆TIB and ∆RIB values	15

5.1.3 	REFSENS requirements	15

5.2	CA_2-7-28-66	16

5.2.1	Channel bandwidths per operating band for CA	16

5.2.2	∆TIB and ∆RIB values	16

5.2.3 	REFSENS requirements	16

5.3	CA_1-3-20-38	17

5.3.1	Channel bandwidths per operating band for CA	17

5.3.2	∆TIB and ∆RIB values	17

5.3.3	REFSENS requirements	17

5.4	CA_1-3-8-41	20

5.4.1	Channel bandwidths per operating band for CA	20

5.4.2	∆TIB and ∆RIB values	20

5.4.3	REFSENS requirements	21

5.5	CA_1-7-8-38	23

5.5.1	Channel bandwidths per operating band for CA	23

5.5.2	∆TIB and ∆RIB values	23

5.5.3 	REFSENS requirements	23

5.6	CA_1-8-20-38	24

5.6.1	Channel bandwidths per operating band for CA	24

5.6.2	∆TIB and ∆RIB values	25

5.6.3 	REFSENS requirements	25

5.7	CA_3-8-20-38	26

5.7.1	Channel bandwidths per operating band for CA	26

5.7.2	∆TIB and ∆RIB values	26

5.7.3 	REFSENS requirements	26

5.8	CA_1-3-8-38	27

5.8.1	Channel bandwidths per operating band for CA	27

5.8.2	∆TIB and ∆RIB values	27

5.8.3 	REFSENS requirements	28

5.9	CA_1-3-8-20	30

5.9.1	Channel bandwidths per operating band for CA	30

5.9.2	∆TIB and ∆RIB values	30

5.9.3 	REFSENS requirements	30

5.10	CA_1-3-7-20	31

5.10.1	Channel bandwidths per operating band for CA	31

5.10.2	∆TIB and ∆RIB values	31

5.10.3	REFSENS requirements	32

5.11	CA_1-3-40-41	33

5.11.1	Channel bandwidths per operating band for CA	33

5.11.2	∆TIB and ∆RIB values	33

5.11.3	REFSENS requirements	34

5.12	CA_1A-7A-8A-28A	35

5.12.1	Channel bandwidths per operating band for CA	35

5.12.2	∆TIB and ∆RIB values	36

5.12.3	REFSENS requirements	36

5.13	CA_1-7-8-32	37

5.13.1	Channel bandwidths per operating band for CA	37

5.13.2	∆TIB and ∆RIB values	37

5.13.3	REFSENS requirements	37

5.14	CA_1-7-28-32	38

5.14.1	Channel bandwidths per operating band for CA	38

5.14.2	∆TIB and ∆RIB values	38

5.14.3	REFSENS requirements	39

5.15	CA_1-8-20-32	40

5.15.1	Channel bandwidths per operating band for CA	40

5.15.2	∆TIB and ∆RIB values	40

5.15.3	REFSENS requirements	41

5.16	CA_1-8-28-32	41

5.16.1	Channel bandwidths per operating band for CA	41

5.16.2	∆TIB and ∆RIB values	41

5.16.3	REFSENS requirements	41

5.17	CA_1-20-28-32	42

5.17.1	Channel bandwidths per operating band for CA	42

5.17.2	∆TIB and ∆RIB values	42

5.17.3	REFSENS requirements	43

5.18	CA_3A-7A-8A-28A	44

5.18.1	Channel bandwidths per operating band for CA	44

5.18.2	∆TIB and ∆RIB values	44

5.18.3	REFSENS requirements	45

5.19	CA_3-8-40-41	45

5.19.1	Channel bandwidths per operating band for CA	45

5.19.2	∆TIB and ∆RIB values	45

5.19.3	REFSENS requirements	46

5.20	CA_7A-8A-20A-28A	47

5.20.1	Channel bandwidths per operating band for CA	47

5.20.2	∆TIB and ∆RIB values	47

5.20.3	REFSENS requirements	48

5.21	CA_7-8-20-32	48

5.21.1	Channel bandwidths per operating band for CA	48

5.21.2	∆TIB and ∆RIB values	48

5.21.3	REFSENS requirements	49

5.22	CA_7-8-28-32	50

5.22.1	Channel bandwidths per operating band for CA	50

5.22.2	∆TIB and ∆RIB values	50

5.22.3	REFSENS requirements	50

5.23	CA_7-20-28-32	52

5.23.1	Channel bandwidths per operating band for CA	52

5.23.2	∆TIB and ∆RIB values	52

5.23.3	REFSENS requirements	52

5.24	CA_8-20-28-32	54

5.24.1	Channel bandwidths per operating band for CA	54

5.24.2	∆TIB and ∆RIB values	54

5.24.3	REFSENS requirements	54

5.25	 CA_1-7-20-38	56

5.25.1	Channel bandwidths per operating band for CA	56

5.25.2	∆TIB and ∆RIB values	56

5.25.3 	REFSENS requirements	56

5.26	CA_1A-3A-3A-8A-38A	56

5.26.1	Channel bandwidths per operating band for CA	56

5.26.2	∆TIB and ∆RIB values	57

5.26.3	REFSENS requirements	57

5.27	CA_1A-3A-28A-38A	57

5.27.1	Channel bandwidths per operating band for CA	57

5.27.2	∆TIB and ∆RIB values	57

5.27.3	REFSENS requirements	58

5.28	CA_1A-7A-20A-38A	58

5.28.1	Channel bandwidths per operating band for CA	58

5.28.2	∆TIB and ∆RIB values	58

5.28.3	REFSENS requirements	58

5.29	CA_1A-7A-28A-38A	59

5.29.1	Channel bandwidths per operating band for CA	59

5.29.2	∆TIB and ∆RIB values	59

5.29.3	REFSENS requirements	59

5.30	CA_1A-7A-32A-38A	60

5.30.1	Channel bandwidths per operating band for CA	60

5.30.2	∆TIB and ∆RIB values	60

5.30.3	REFSENS requirements	60

5.31	CA_1A-8A-32A-38A	61

5.31.1	Channel bandwidths per operating band for CA	61

5.31.2	∆TIB and ∆RIB values	61

5.31.3	REFSENS requirements	61

5.32	CA_1A-20A-28A-38A	62

5.32.1	Channel bandwidths per operating band for CA	62

5.32.2	∆TIB and ∆RIB values	62

5.32.3	REFSENS requirements	62

5.33	CA_1A-20A-32A-38A	63

5.33.1	Channel bandwidths per operating band for CA	63

5.33.2	∆TIB and ∆RIB values	63

5.33.3	REFSENS requirements	63

5.34	CA_3A-7A-20A-38A	64

5.34.1	Channel bandwidths per operating band for CA	64

5.34.2	∆TIB and ∆RIB values	64

5.34.3	REFSENS requirements	64

5.35	CA_3A-20A-28A-38A	65

5.35.1	Channel bandwidths per operating band for CA	65

5.35.2	∆TIB and ∆RIB values	65

5.35.3	REFSENS requirements	65

5.36	CA_7A-8A-20A-38A	66

5.36.1	Channel bandwidths per operating band for CA	66

5.36.2	∆TIB and ∆RIB values	66

5.36.3	REFSENS requirements	66

5.37	CA_7A-8A-32A-38A	67

5.37.1	Channel bandwidths per operating band for CA	67

5.37.2	∆TIB and ∆RIB values	67

5.37.3	REFSENS requirements	67

5.38	CA_7A-20A-28A-38A	68

5.38.1	Channel bandwidths per operating band for CA	68

5.38.2	∆TIB and ∆RIB values	68

5.38.3	REFSENS requirements	68

5.39	CA_7A-20A-32A-38A	69

5.39.1	Channel bandwidths per operating band for CA	69

5.39.2	∆TIB and ∆RIB values	69

5.39.3	REFSENS requirements	69

5.40	CA_8A-20A-32A-38A	70

5.40.1	Channel bandwidths per operating band for CA	70

5.40.2	∆TIB and ∆RIB values	70

5.40.3	REFSENS requirements	70

5.41	CA_1-3-28-32	70

5.41.1	Channel bandwidths per operating band for CA	70

5.41.2	∆TIB and ∆RIB values	71

5.41.3 	REFSENS requirements	71

6	5 Band Carrier Aggregation with Single UL: Specific Band Combination Part	71

6.1	CA_1-3-7-8-40	71

6.1.1	Channel bandwidths per operating band for CA	71

6.1.2	∆TIB and ∆RIB values	71

6.1.3 	REFSENS requirements	72

6.2	CA_1A-3A-7A-8A-28A	75

6.2.1	Channel bandwidths per operating band for CA	75

6.2.2	∆TIB and ∆RIB values	75

6.2.3	REFSENS requirements	75

6.3	CA_1A-3A-8A-20A-28A	77

6.3.1	Channel bandwidths per operating band for CA	77

6.3.2	∆TIB and ∆RIB values	77

6.3.3	REFSENS requirements	78

6.4	CA_1A-7A-8A-20A-28A	79

6.4.1	Channel bandwidths per operating band for CA	79

6.4.2	∆TIB and ∆RIB values	79

6.4.3	REFSENS requirements	80

6.5	CA_1-7-8-20-32	80

6.5.1	Channel bandwidths per operating band for CA	80

6.5.2	∆TIB and ∆RIB values	80

6.5.3	REFSENS requirements	81

6.6	CA_1-7-8-28-32	82

6.6.1	Channel bandwidths per operating band for CA	82

6.6.2	∆TIB and ∆RIB values	82

6.6.3	REFSENS requirements	83

6.7	CA_1-7-20-28-32	84

6.7.1	Channel bandwidths per operating band for CA	84

6.7.2	∆TIB and ∆RIB values	84

6.7.3	REFSENS requirements	84

6.8	CA_7-8-20-28-32	86

6.8.1	Channel bandwidths per operating band for CA	86

6.8.2	∆TIB and ∆RIB values	86

6.8.3	REFSENS requirements	86

6.9	CA_1-3-8-20-38	88

6.9.1	Channel bandwidths per operating band for CA	88

6.9.2	∆TIB and ∆RIB values	88

6.9.3 	REFSENS requirements	88

6.10	CA_1-3-7-8-38	89

6.10.1	Channel bandwidths per operating band for CA	89

6.10.2	∆TIB and ∆RIB values	89

6.10.3 	REFSENS requirements	89

6.11	CA_3A-7A-8A-20A-28A	90

6.11.1	Channel bandwidths per operating band for CA	90

6.11.2	∆TIB and ∆RIB values	90

6.11.3	REFSENS requirements	90

6.12	CA_1A-3A-7A-20A-38A	91

6.12.1	Channel bandwidths per operating band for CA	91

6.12.2	∆TIB and ∆RIB values	91

6.12.3	REFSENS requirements	92

6.13	CA_1A-3A-7A-28A-38A	92

6.13.1	Channel bandwidths per operating band for CA	92

6.13.2	∆TIB and ∆RIB values	92

6.13.3	REFSENS requirements	93

6.14	CA_1A-3A-20A-28A-38A	93

6.14.1	Channel bandwidths per operating band for CA	93

6.14.2	∆TIB and ∆RIB values	93

6.14.3	REFSENS requirements	94

6.15	CA_1A-7A-8A-20A-38A	94

6.15.1	Channel bandwidths per operating band for CA	94

6.15.2	∆TIB and ∆RIB values	94

6.15.3	REFSENS requirements	94

6.16	CA_1A-7A-8A-32A-38A	95

6.16.1	Channel bandwidths per operating band for CA	95

6.16.2	∆TIB and ∆RIB values	95

6.16.3	REFSENS requirements	95

6.17	CA_1A-7A-20A-28A-38A	96

6.17.1	Channel bandwidths per operating band for CA	96

6.17.2	∆TIB and ∆RIB values	96

6.17.3	REFSENS requirements	96

6.18	CA_1A-7A-20A-32A-38A	97

6.18.1	Channel bandwidths per operating band for CA	97

6.18.2	∆TIB and ∆RIB values	97

6.18.3	REFSENS requirements	97

6.19	CA_1A-8A-20A-32A-38A	98

6.19.1	Channel bandwidths per operating band for CA	98

6.19.2	∆TIB and ∆RIB values	98

6.19.3	REFSENS requirements	98

6.20	CA_3A-7A-8A-20A-38A	99

6.20.1	Channel bandwidths per operating band for CA	99

6.20.2	∆TIB and ∆RIB values	99

6.20.3	REFSENS requirements	99

6.21	CA_3A-7A-20A-28A-38A	100

6.21.1	Channel bandwidths per operating band for CA	100

6.21.2	∆TIB and ∆RIB values	100

6.21.3	REFSENS requirements	100

6.22	CA_7A-8A-20A-32A-38A	101

6.22.1	Channel bandwidths per operating band for CA	101

6.22.2	∆TIB and ∆RIB values	101

6.22.3	REFSENS requirements	101

7	6 Band Carrier Aggregation with Single UL: Specific Band Combination Part	102

7.1	CA_1A-3A-7A-8A-20A-28A	102

7.1.1	Channel bandwidths per operating band for CA	102

7.1.2	∆TIB and ∆RIB values	102

7.1.3	REFSENS requirements	102

7.2	CA_1-7-8-20-28-32	104

7.2.1	Channel bandwidths per operating band for CA	104

7.2.2	∆TIB and ∆RIB values	104

7.2.3	REFSENS requirements	105

7.3	CA_1A-3A-7A-20A-28A-38A	106

7.3.1	Channel bandwidths per operating band for CA	106

7.3.2	∆TIB and ∆RIB values	106

7.3.3	REFSENS requirements	107

7.4	CA_1A-3A-7C-20A-28A-38A	107

7.4.1	Channel bandwidths per operating band for CA	107

7.4.2	∆TIB and ∆RIB values	107

7.4.3	REFSENS requirements	107

7.5	CA_1A-7A-8A-20A-32A-38A	108

7.5.1	Channel bandwidths per operating band for CA	108

7.5.2	∆TIB and ∆RIB values	108

7.5.3	REFSENS requirements	108

Annex A: Change history	109

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document is a technical report on inter-band CA for x bands DL (x=4, 5) with 1 band UL under Rel-17 timeframe. The purpose is to gather the relevant background information and studies in order to address 4 or 5 bands DL/1 band UL Inter-band Carrier Aggregation requirements for the Rel-17 band combinations in Table 1-1 and 1-2.

Table 1-1: Release 17 4 bands DL/1 band UL inter-band carrier aggregation combinations

| CA configuration | Uplink configuration | BCS |
| --- | --- | --- |
| CA_1A-3C-7A-8A | CA_3C | 0 |
| CA_1A-3C-7C-20A | CA_7C | 0 |
| CA_1A-3C-7A-32A | - | 0 |
| CA_1A-3C-8A-20A | - | 0 |
| CA_1A-3C-8A-20A | CA_3C | 0 |
| CA_1A-3C-8A-38A | - | 0 |
| CA_1A-3C-8A-38A | CA_3C | 0 |
| CA_1A-3A-3A-8A-38A | - | 0 |
| CA_1A-3A-8A-40C | - | 0 |
| CA_1A-3A-8A-41A | - | 0 |
| CA_1A-3C-20A-32A | - | 0 |
| CA_1A-3A-20A-38A | - | 0 |
| CA_1A-3C-20A-38A | - | 0 |
| CA_1A-3C-20A-38A | CA_3C | 0 |
| CA_1A-3A-28A-32A | - | 0 |
| CA_1A-3A-28A-38A | - | 0 |
| CA_1A-3A-40A-41A | - | 0 |
| CA_1A-7A-8A-28A | - | 0 |
| CA_1A-7A-8A-32A | - | 0 |
| CA_1A-7A-8A-38A | - | 0 |
| CA_1A-7A-20A-28A | - | 0 |
| CA_1A-7A-20A-38A | - | 0 |
| CA_1A-7A-28A-32A | - | 0 |
| CA_1A-7A-28A-38A | - | 0 |
| CA_1A-7A-32A-38A | - | 0 |
| CA_1A-8A-20A-32A | - | 0 |
| CA_1A-8A-20A-38A | - | 0 |
| CA_1A-8A-28A-32A | - | 0 |
| CA_1A-8A-32A-38A | - | 0 |
| CA_1A-20A-28A-32A | - | 0 |
| CA_1A-20A-28A-38A | - | 0 |
| CA_1A-20A-32A-38A | - | 0 |
| CA_2A-2A-5A-7A-66A | - | 0 |
| CA_2A-5A-7A-66A | - | 0 |
| CA_2A-5A-7A-7A-66A | - | 0 |
| CA_2A-5A-7A-66A-66A | - | 0 |
| CA_2A-5A-7C-66A | - | 0 |
| CA_2A-7A-12A-66A-66A | - | 0 |
| CA_2A-7C-13A-66A | - | 0 |
| CA_2A-7A-7A-13A-66A | - | 0 |
| CA_2A-7A-28A-66A | - | 0 |
| CA_2A-7C-28A-66A | - | 0 |
| CA_3A-7A-8A-28A | - | 0 |
| CA_3A-7A-20A-38A | - | 0 |
| CA_3A-8A-20A-38A | - | 0 |
| CA_3A-8A-40A-41A | - | 0 |
| CA_3A-20A-28A-38A | - | 0 |
| CA_7A-8A-20A-28A | - | 0 |
| CA_7A-8A-20A-32A | - | 0 |
| CA_7A-8A-20A-38A | - | 0 |
| CA_7A-8A-28A-32A | - | 0 |
| CA_7A-8A-32A-38A | - | 0 |
| CA_7A-20A-28A-32A | - | 0 |
| CA_7A-20A-28A-38A | - | 0 |
| CA_7A-20A-32A-38A | - | 0 |
| CA_8A-20A-28A-32A | - | 0 |
| CA_8A-20A-32A-38A | - | 0 |

Table 1-2: Release 17 5 bands DL/1 band UL inter-band carrier aggregation combinations

| CA configuration | Uplink configuration | BCS |
| --- | --- | --- |
| CA_1A-3A-7A-8A-28A | - | 0 |
| CA_1A-3A-7A-8A-38A | - | 0 |
| CA_1A-3A-7A-8A-40A | - | 0 |
| CA_1A-3A-7A-8A-40C | - | 0 |
| CA_1A-3A-7A-20A-38A | - | 0 |
| CA_1A-3A-7A-28A-38A | - | 0 |
| CA_1A-3A-8A-20A-28A | - | 0 |
| CA_1A-3A-8A-20A-38A | - | 0 |
| CA_1A-3A-20A-28A-38A | - | 0 |
| CA_1A-7A-8A-20A-28A | - | 0 |
| CA_1A-7A-8A-20A-32A | - | 0 |
| CA_1A-7A-8A-20A-38A | - | 0 |
| CA_1A-7A-8A-28A-32A | - | 0 |
| CA_1A-7A-8A-32A-38A | - | 0 |
| CA_1A-7A-20A-28A-32A | - | 0 |
| CA_1A-7A-20A-28A-38A | - | 0 |
| CA_1A-7A-20A-32A-38A | - | 0 |
| CA_1A-8A-20A-32A-38A | - | 0 |
| CA_3A-7A-8A-20A-28A | - | 0 |
| CA_3A-7A-8A-20A-38A | - | 0 |
| CA_3A-7A-20A-28A-38A | - | 0 |
| CA_7A-8A-20A-28A-32A | - | 0 |
| CA_7A-8A-20A-32A-38A | - | 0 |

Table 1-3: Release 17 6 bands DL/1 band UL inter-band carrier aggregation combinations

| CA configuration | Uplink configuration | BCS |
| --- | --- | --- |
| CA_1A-3A-7A-8A-20A-28A | - | 0 |
| CA_1A-3A-7A-20A-28A-38A | - | 0 |
| CA_1A-3A-7C-20A-28A-38A | - | 0 |
| CA_1A-7A-8A-20A-28A-32A | - | 0 |
| CA_1A-7A-8A-20A-32A-38A | - | 0 |

This TR contains a band specific combination part. The actual requirements are added to the corresponding technical specifications.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]		RP-201245, “LTE Advanced inter-band CA Rel-17 for x bands DL (x=4, 5) with 1 band UL”, RAN#80.

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4	Background

The present document is a technical report for 4 or 5 bands DL/1 band UL Inter-band Carrier Aggregation under Rel-17 timeframe. The document covers each band combination specific issues (i.e. one sub-clause defined per band combination)

## 4.1	TR maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band combination to ensure that the TPs related to the band combination have been implemented.

# 5	4 Band Carrier Aggregation with Single UL: Specific Band Combination Part

## 5.1	CA_2-5-7-66 / CA_2-5-7-66-66

### 5.1.1	Channel bandwidths per operating band for CA

Table 5.1.1-2: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_2A-5A-7A-66A | - | 2 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_2A-5A-7C-66A | - | 2 |  |  | Yes | Yes | Yes | Yes | 90 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 7 | See CA_7C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_2A-5A-7A-66A-66A | - | 2 |  |  | Yes | Yes | Yes | Yes | 90 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |

### 5.1.2	∆TIB and ∆RIB values

Table 5.1.2-1: IB,c

| CA_2-5-7-66 CA_2-5-7-66-66 | 2 | 0.5 |
| --- | --- | --- |
|  | 5 | 0.3 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |

Table 5.1.2-2: R IB,c

| CA_2-5-7-66 CA_2-5-7-66-66 | 2 | 0.3 |
| --- | --- | --- |
|  | 5 | 0 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |

### 5.1.3 	REFSENS requirements

There is no MSD requirements for CA_2A-5A-7A-66A / CA_2A-5A-7C-66A / CA_2A-5A-7A-66A-66A.

## 5.2	CA_2-7-28-66

### 5.2.1	Channel bandwidths per operating band for CA

Table 5.2.1-2: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_2A-7A-28A-66A | - | 2 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_2A-7C-28A-66A | - | 2 |  |  | Yes | Yes | Yes | Yes | 100 | 0 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.2.2	∆TIB and ∆RIB values

Table 5.2.2-1: IB,c

| CA_2-7-28-66 | 2 | 0.5 |
| --- | --- | --- |
|  | 7 | 0.5 |
|  | 28 | 0.6 |
|  | 66 | 0.5 |

Table 5.2.2-2: R IB,c

| CA_2-7-28-66 | 2 | 0.3 |
| --- | --- | --- |
|  | 7 | 0.5 |
|  | 28 | 0.2 |
|  | 66 | 0.5 |

### 5.2.3 	REFSENS requirements

Table 5.2.3-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_2A-7A-28A-66A5,6 CA_2A-7C-28A-66A5,6 | 66 |  |  | -89,5 | -88,9 | -88,5 | -88,2 | FDD |
| NOTE 5: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 6: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. |  |  |  |  |  |  |  |  |

Table 5.2.3-2: Uplink configuration for the low band (exceptions due to harmonic issue)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_2A-7A-28A-66A CA_2A-7C-28A-66A | 28 |  |  | 8 | 16 | 25 | 25 | FDD |

## 5.3	CA_1-3-20-38

### 5.3.1	Channel bandwidths per operating band for CA

Table 5.3.1-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_1A-3A-20A-38A | - | 1 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 3 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-3C-20A-38A | CA_3C | 1 |  |  | Yes | Yes | Yes | Yes | 100 | 0 |
|  |  | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.3.2	∆TIB and ∆RIB values

For CA_1A-3A-20A-38A, the TIB,c and RIB,c values are shown in table 5.3.2-1 and table 5.3.2-2, respectively.

Table 5.3.2-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1A-3A-20A-38A | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 20 | 0.3 |
|  | 38 | 0.3 |

Table 5.3.2-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1A-3A-20A-38A | 1 | 0 |
|  | 3 | 0 |
|  | 20 | 0 |
|  | 38 | 0 |

### 5.3.3	REFSENS requirements

REFSENS requirements are defined in table 5.3.3-1 for inclusion in TS36.101 table 7.3.1A-0a.

Table 5.3.3-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A-20A-38A8 CA_1A-3C-20A-38A8 | 38 |  |  | N/A | N/A | N/A | N/A | TDD |
| NOTE 8: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity is only verified when this is not the case (the requirements specified in clause 7.3.1 apply). |  |  |  |  |  |  |  |  |

Table 5.3.3-1a: Uplink configuration for the low band (exceptions due to harmonic issues in the combinations of intra-band and inter-band CA)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-3A-20A-38A CA_1A-3C-20A-38A | 20 |  |  | 8 | 16 | 25 | 25 | FDD |

REFSENS requirements are defined in table 5.3.3-2 for inclusion in TS36.101 table 7.3.1A-0bD1.

Table 5.3.3-2: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions for four bands due to close proximity of UL to DL channel)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A-20A-38A CA_1A-3C-20A-38A | 34,9 |  |  | -93.8 | -91.3 | -89.8 | -88.8 | FDD |
|  | 35 |  |  | -96.8 | -93.8 | -92 | -90.8 |  |
| NOTE 4: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in the bands other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 5: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in the bands other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 9: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

Table 5.3.3-2a: Uplink configuration for the low band (exceptions for four bands due to close proximity of UL to DL channel)

| E-UTRA Band / Channel bandwidth of the affected DL band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-3A-20A-38A CA_1A-3C-20A-38A | 11,2 |  |  | 25 | 25 | 25 | 25 | FDD |
|  | 11,3 |  |  | 25 | 45 | 45 | 45 |  |
| NOTE 1: refers to the UL resource blocks shall be located as close as possible to the downlink channel in Band 3 but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1) in the uplink channel in Band 1. NOTE 2: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz NOTE 3: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. |  |  |  |  |  |  |  |  |

REFSENS requirements are defined in table 5.3.3-3 for inclusion in TS36.101 table 7.3.1A-0bE.

Table 5.3.3-3: Void

Table 5.3.3-4: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to cross band isolation issues of TDD and FDD bands)

| EUTRA CA Configuration | EUTRA band | Channel bandwidth |  |  |  |  |  | Duplex mode | Applicable active UL band |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |  |  |
| CA_1A-3A-20A-38A X CA_1A-3C-20A-38A X | 319 |  |  | -94 | -91.5 | -90 | -89 | FDD | 1 |
|  | 38 |  |  | -97.1 | -94.4 | -92.8 | -91.7 | TDD |  |
|  | 38 |  |  | -97.1 | -94.4 | -92.8 | -91.7 | TDD | 3 |
|  | 119 |  |  | -98.1 | -95.1 | -93.3 | -92.1 | FDD | 38 |
|  | 319 |  |  | -95.1 | -92.1 | -90.3 | -89.1 |  |  |
| CA_1A-3A-20A-38AY CA_1A-3C-20A-38AY | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 1 |
|  | 38 |  |  | -97.1 | -94.4 | -92.8 | -91.7 | TDD |  |
|  | 38 |  |  | -97.1 | -94.4 | -92.8 | -91.7 | TDD | 3 |
|  | 119 |  |  | -98.1 | -95.1 | -93.3 | -92.1 | FDD | 38 |
|  | 319 |  |  | -95.1 | -92.1 | -90.3 | -89.1 |  |  |
| NOTE X: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in Band 3 and Band 38, the requirement applies regardless of channel bandwidth in Band 1. NOTE Y: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in Band 3 and Band 38, the requirement applies regardless of channel bandwidth in Band 1. NOTE 19: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |  |

Table 5.3.3-5: Uplink configuration for reference sensitivity (exceptions due to cross band isolation issues of TDD and FDD bands)

| E-UTRA Band / Channel bandwidth of the affected DL band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| CA_1A-3A-20A-38A CA_1A-3C-20A-38A | 11,3 |  |  | 25 | 25 | 25 | 25 | FDD |
|  | 11,4 |  |  | 25 | 45 | 45 | 45 | FDD |
|  | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 38 |  |  | 25 | 50 | 75 | 100 | TDD |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1). NOTE 2: the UL configuration applies regardless of the channel bandwidth of the low band unless the UL resource blocks exceed that specified in Table 7.3.1-2 for the uplink bandwidth in which case the allocation according to Table 7.3.1-2 applies. NOTE 3: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. NOTE 4: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. |  |  |  |  |  |  |  |  |

## 5.4	CA_1-3-8-41

### 5.4.1	Channel bandwidths per operating band for CA

Table 5.4.1-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_1A-3A-8A-41A | - | 1 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 3 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | 8 | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 41 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.4.2	∆TIB and ∆RIB values

For CA_1A-3A-8A-41A, the TIB,c and RIB,c values are shown in table 5.4.2-1 and table 5.4.2-2, respectively.

Table 5.4.2-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1A-3A-8A-41A | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 8 | 0.3 |
|  | 41 | 0.35 |
|  |  | 0.86 |
| NOTE 5: The requirement is specified for the frequency range of 2545-2690MHz. NOTE 6: The requirement is specified for the frequency range of 2496-2545MHz. |  |  |

Table 5.4.2-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1A-3A-8A-41A | 1 | 0 |
|  | 3 | 0 |
|  | 8 | 0 |
|  | 41 | 05 |
|  |  | 0.56 |
| NOTE 5: The requirement is specified for the frequency range of 2545-2690MHz. NOTE 6: The requirement is specified for the frequency range of 2496-2545MHz. |  |  |

### 5.4.3	REFSENS requirements

Table 5.4.3-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A-8A-41A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| NOTE 4: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity for all active downlink component carriers is only verified when this is not the case (the requirements specified in clause 7.3.1 apply unless otherwise specified). |  |  |  |  |  |  |  |  |

Table 5.4.3-2: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions for three bands due to close proximity of UL to DL channel)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A-8A-41A4 | 312 |  |  | -94 | -91.5 | -90 | -89 | FDD |
| CA_1A-3A-8A-41A5 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
| NOTE 4: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in the Bands other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 5: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in the Bands other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 12: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

Table 5.4.3-3: Uplink configuration for the uplink band (exceptions for three bands due to close proximity of UL to DL channel)

| E-UTRA Band / Channel bandwidth of the affected DL band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-3A-8A-41A1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-3A-8A-41A1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| NOTE 1: refers to the UL resource blocks shall be located as close as possible to the downlink channel in Band 3 but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1) in the uplink channel in Band 1. NOTE 2: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz NOTE 3: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. |  |  |  |  |  |  |  |  |

Table 5.4.3-4: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to cross band isolation issues of TDD and FDD bands)

| EUTRA CA Configuration | EUTRA band | Channel bandwidth |  |  |  |  |  | Duplex mode | Applicable active UL band |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |  |  |
| CA_1A-3A-8A-41A12,14 | 319 |  |  | -94 | -91.5 | -90 | -89 | FDD | 1 |
|  | 4119 |  |  | -93.3 | -90.7 | -89.2 | -88.1 | TDD |  |
|  | 4119 |  |  | -93.3 | -90.7 | -89.2 | -88.1 |  | 3 |
| CA_1A-3A-8A-41A13,14 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 1 |
|  | 4119 |  |  | -93.3 | -90.7 | -89.2 | -88.1 | TDD |  |
|  | 4119 |  |  | -93.3 | -90.7 | -89.2 | -88.1 |  | 3 |
| NOTE 12: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in Band 3 and Band 41, the requirement applies regardless of channel bandwidth in Band 1. NOTE 13: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in Band 3 and Band 41, the requirement applies regardless of channel bandwidth in Band 1. NOTE 14: The B41 requirements also apply to the supported CA_1A-41A. NOTE 19: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |  |

Table 5.4.3-5: Uplink configuration for reference sensitivity (exceptions due to cross band isolation issues of TDD and FDD bands)

| E-UTRA Band / Channel bandwidth of the affected DL band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| CA_1A-3A-8A-41A | 1 |  |  | 253 | 251,3 | 251,3 | 251,3 | FDD |
|  |  |  |  | 254 | 451,4 | 451,4 | 451,4 | FDD |
|  | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1). NOTE 3: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. NOTE 4: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. |  |  |  |  |  |  |  |  |

## 5.5	CA_1-7-8-38

### 5.5.1	Channel bandwidths per operating band for CA

Table 5.5.1-1: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-7A-8A-38Ax | - | 1 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |
| NOTE x: UL carrier shall be supported in Band 1 or 8 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within [6dB]. |  |  |  |  |  |  |  |  |  |  |

### 5.5.2	∆TIB and ∆RIB values

Table 5.5.2-1: IB,c

| CA_1-7-8-38 | 1 | 0.5 |
| --- | --- | --- |
|  | 8 | 0.5 |

Table 5.5.2-2: R IB,c

| CA_1-7-8-38 | 1 | 0 |
| --- | --- | --- |
|  | 7 | 0 |
|  | 8 | 0 |
|  | 38 | 0.2 |

### 5.5.3 	REFSENS requirements

The MSD requriements for CA_1A-7A-8A-38A are shown below.

Table 5.5.3-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to cross band isolation issues of TDD and FDD bands)

| EUTRA CA Configuration | EUTRA band | Channel bandwidth |  |  |  |  |  | Duplex mode | Applicable active UL band |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |  |  |
| CA_1A-7A-8A-38A | 719 |  |  | -93.3 | -90.7 | -89.2 | -88.1 | FDD | 1 |
|  | 38 |  |  | -93.3 | -90.7 | -89.2 | -88.1 | TDD |  |
| NOTE 19: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |  |

Table 5.5.3-2: Uplink configuration for reference sensitivity (exceptions due to cross band isolation issues of TDD and FDD bands)

| E-UTRA Band / Channel bandwidth of the affected DL band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| CA_1A-7A-8A-38A | 1 |  |  | 25 | 45 | 451 | 451 | FDD |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1). |  |  |  |  |  |  |  |  |

Table 5.5.3-3: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-7A-8A-38A 5,6 | 733 |  |  |  | -87.1 | -86.7 | -86.4 | FDD |
| NOTE 5: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 6: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 33: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

Table 5.5.3-4: Uplink configuration for the low band (exceptions due to harmonic issue)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-7A-8A-40A | 8 |  |  | 8 | 16 | 25 | 25 | FDD |

## 5.6	CA_1-8-20-38

### 5.6.1	Channel bandwidths per operating band for CA

Table 5.6.1-1: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-8A-20A-38A | - | 1 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.6.2	∆TIB and ∆RIB values

Table 5.6.2-1: IB,c

| CA_1-8-20-38 | 1 | 0.5 |
| --- | --- | --- |
|  | 8 | 0.6 |
|  | 20 | 0.5 |
|  | 38 | 0.5 |

Table 5.6.2-2: R IB,c

| CA_1-8-20-38 | 1 | 0 |
| --- | --- | --- |
|  | 8 | 0 |
|  | 20 | 0 |
|  | 38 | 0 |

### 5.6.3 	REFSENS requirements

MSD due to harmonic interference between band 20 and 38 can is similar to CA_20A-38A.

Table 5.6.3-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issues in the combinations of intra-band and inter-band CA)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-8A-20A-38A8 | 38 |  |  | N/A | N/A | N/A | N/A | TDD |
| NOTE 4: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity for all active downlink component carriers is only verified when this is not the case (the requirements specified in clause 7.3.1 apply unless otherwise specified). NOTE 8: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity is only verified when this is not the case (the requirements specified in clause 7.3.1 apply). |  |  |  |  |  |  |  |  |

Table 5.6.3-2: Uplink configuration for the low band (exceptions due to harmonic issues in the combinations of intra-band and inter-band CA)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-8A-20A-38A | 20 |  |  | 8 | 16 | 25 | 25 | FDD |

## 5.7	CA_3-8-20-38

### 5.7.1	Channel bandwidths per operating band for CA

Table 5.7.1-1: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_3A-8A-20A-38A | - | 3 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.7.2	∆TIB and ∆RIB values

Table 5.7.2-1: IB,c

| CA_3-8-20-38 | 3 | 0.5 |
| --- | --- | --- |
|  | 8 | 0.6 |
|  | 20 | 0.5 |
|  | 38 | 0.5 |

Table 5.7.2-2: R IB,c

| CA_3-8-20-38 | 3 | 0 |
| --- | --- | --- |
|  | 8 | 0 |
|  | 20 | 0 |
|  | 38 | 0 |

### 5.7.3 	REFSENS requirements

MSD due to harmonic interference between band 20 and 38 can is similar to CA_20A-38A.

MSD due to harmonic interference between band 3 and 8 can is similar to CA_3A-8A.

Table 5.7.3-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issues in the combinations of intra-band and inter-band CA)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_3A-8A-20A-38A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_3A-8A-20A-38A8 | 38 |  |  | N/A | N/A | N/A | N/A | TDD |
| NOTE 4: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity for all active downlink component carriers is only verified when this is not the case (the requirements specified in clause 7.3.1 apply unless otherwise specified). NOTE 8: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity is only verified when this is not the case (the requirements specified in clause 7.3.1 apply). |  |  |  |  |  |  |  |  |

Table 5.7.3-2: Uplink configuration for the low band (exceptions due to harmonic issues in the combinations of intra-band and inter-band CA)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_3A-8A-20A-38A | 8 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_3A-8A-20A-38A | 20 |  |  | 8 | 16 | 25 | 25 | FDD |

## 5.8	CA_1-3-8-38

### 5.8.1	Channel bandwidths per operating band for CA

Table 5.8.1-1: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3C-8A-38A | CA_3C | 1 |  |  | Yes | Yes | Yes | Yes | 90 | 0 |
|  |  | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.8.2	∆TIB and ∆RIB values

Table 5.8.2-1: IB,c

| CA_1-3-8-38 | 1 | 0.5 |
| --- | --- | --- |
|  | 3 | 0.5 |
|  | 8 | 0.3 |
|  | 38 | 0.5 |

Table 5.8.2-2: R IB,c

| CA_1-3-8-38 | 1 | 0 |
| --- | --- | --- |
|  | 3 | 0 |
|  | 8 | 0 |
|  | 38 | 0 |

### 5.8.3 	REFSENS requirements

Table 5.8.3-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions for four bands due to close proximity of UL to DL channel)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3C-8A-38A | 34,9 |  |  | -94 | -91.5 | -90 | -89 | FDD |
|  | 35 |  |  | -97 | -94 | -92.2 | -91 |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: The signal power is specified per port NOTE 4: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in the bands other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 5: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in the bands other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 6: Void NOTE 7: The Band 41 requirements are modified by -0.5dB when carrier frequency of the assigned E-UTRA channel bandwidth is within 2545-2690MHz. NOTE 8: The Band 41 requirements also apply to the supported CA_1A-41A. NOTE 9: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

Table 5.8.3-2: Uplink configuration for the low band (exceptions for four bands due to close proximity of UL to DL channel)

| E-UTRA Band / Channel bandwidth of the affected DL band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-3C-8A-38A | 11,2 |  |  | 25 | 25 | 25 | 25 | FDD |
|  | 11,3 |  |  | 25 | 45 | 45 | 45 |  |
| NOTE 1: refers to the UL resource blocks shall be located as close as possible to the downlink channel in Band 3 but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1) in the uplink channel in Band 1. NOTE 2: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz NOTE 3: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. |  |  |  |  |  |  |  |  |

MSD due to harmonic interference between band 3 and 8 can is similar to CA_3A-8A.

Table 5.8.3-3: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issues in the combinations of intra-band and inter-band CA)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3C-8A-38A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| NOTE 4: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity for all active downlink component carriers is only verified when this is not the case (the requirements specified in clause 7.3.1 apply unless otherwise specified). |  |  |  |  |  |  |  |  |

Table 5.8.3-4: Uplink configuration for the low band (exceptions due to harmonic issues in the combinations of intra-band and inter-band CA)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-3C-8A-38A | 8 |  |  | 8 | 16 | 25 | 25 | FDD |

Table 5.8.3-5: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to cross band isolation issues of TDD and FDD bands)

| EUTRA CA Configuration | EUTRA band | Channel bandwidth |  |  |  |  |  | Duplex mode | Applicable active UL band |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |  |  |
| CA_1A-3C-8A-38AX | 319 |  |  | -94 | -91.5 | -90 | -89 | FDD | 1 |
|  | 38 |  |  | -97.1 | -94.4 | -92.8 | -91.7 | TDD |  |
|  | 38 |  |  | -97.1 | -94.4 | -92.8 | -91.7 | TDD | 3 |
|  | 119 |  |  | -98.1 | -95.1 | -93.3 | -92.1 | FDD | 38 |
|  | 319 |  |  | -95.1 | -92.1 | -90.3 | -89.1 |  |  |
| CA_1A-3C-8A-38AY | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 1 |
|  | 38 |  |  | -97.1 | -94.4 | -92.8 | -91.7 | TDD |  |
|  | 38 |  |  | -97.1 | -94.4 | -92.8 | -91.7 | TDD | 3 |
|  | 119 |  |  | -98.1 | -95.1 | -93.3 | -92.1 | FDD | 38 |
|  | 319 |  |  | -95.1 | -92.1 | -90.3 | -89.1 |  |  |
| NOTE X: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in Band 3 and Band 38, the requirement applies regardless of channel bandwidth in Band 1. NOTE Y: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in Band 3 and Band 38, the requirement applies regardless of channel bandwidth in Band 1. NOTE 19: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |  |

Table 5.8.3-6: Uplink configuration for reference sensitivity (exceptions due to cross band isolation issues of TDD and FDD bands)

| E-UTRA Band / Channel bandwidth of the affected DL band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| CA_1A-3C-8A-38A | 11,3 |  |  | 25 | 25 | 25 | 25 | FDD |
|  | 11,4 |  |  | 25 | 45 | 45 | 45 | FDD |
|  | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 38 |  |  | 25 | 50 | 75 | 100 | TDD |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1). NOTE 2: the UL configuration applies regardless of the channel bandwidth of the low band unless the UL resource blocks exceed that specified in Table 7.3.1-2 for the uplink bandwidth in which case the allocation according to Table 7.3.1-2 applies. NOTE 3: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. NOTE 4: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. |  |  |  |  |  |  |  |  |

## 5.9	CA_1-3-8-20

### 5.9.1	Channel bandwidths per operating band for CA

Table 5.9.1-1: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3C-8A-20A | CA_3C | 1 |  |  | Yes | Yes | Yes | Yes | 90 | 0 |
|  |  | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.9.2	∆TIB and ∆RIB values

Table 5.9.2-1: IB,c

| CA_1-3-8-20 | 1 | 0.3 |
| --- | --- | --- |
|  | 3 | 0.3 |
|  | 8 | 0.4 |
|  | 20 | 0.4 |

Table 5.9.2-2: R IB,c

| CA_1-3-8-20 | 1 | 0 |
| --- | --- | --- |
|  | 3 | 0 |
|  | 8 | 0 |
|  | 20 | 0 |

### 5.9.3 	REFSENS requirements

Table 5.9.3-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions for four bands due to close proximity of UL to DL channel)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3C-8A-20A | 34,9 |  |  | -94 | -91.5 | -90 | -89 | FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: The signal power is specified per port NOTE 4: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in the bands other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 5: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in the bands other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 6: Void NOTE 7: The Band 41 requirements are modified by -0.5dB when carrier frequency of the assigned E-UTRA channel bandwidth is within 2545-2690MHz. NOTE 8: The Band 41 requirements also apply to the supported CA_1A-41A. NOTE 9: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

Table 5.9.3-2: Uplink configuration for the low band (exceptions for four bands due to close proximity of UL to DL channel)

| E-UTRA Band / Channel bandwidth of the affected DL band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-3C-8A-20A | 11,2 |  |  | 25 | 25 | 25 | 25 | FDD |
|  | 11,3 |  |  | 25 | 45 | 45 | 45 |  |
| NOTE 1: refers to the UL resource blocks shall be located as close as possible to the downlink channel in Band 3 but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1) in the uplink channel in Band 1. NOTE 2: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz NOTE 3: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. |  |  |  |  |  |  |  |  |

Table 5.9.3-3: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issues in the combinations of intra-band and inter-band CA)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3C-8A-20A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| NOTE 4: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity for all active downlink component carriers is only verified when this is not the case (the requirements specified in clause 7.3.1 apply unless otherwise specified). |  |  |  |  |  |  |  |  |

Table 5.9.3-4: Uplink configuration for the low band (exceptions due to harmonic issues in the combinations of intra-band and inter-band CA)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-3C-8A-20A | 8 |  |  | 8 | 16 | 25 | 25 | FDD |

## 5.10	CA_1-3-7-20

### 5.10.1	Channel bandwidths per operating band for CA

Table 5.10.1-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_1A-3A-7C-20A | CA_7C | 1 |  |  | Yes | Yes | Yes | Yes | 100 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 7 | See CA_7C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.10.2	∆TIB and ∆RIB values

For CA_1-3-7-20, the TIB,c and RIB,c values are shown in table 5.10.2-1 and table 5.10.2-2 respectively.

Table 5.10.2-1: ΔTIB,c for 4DL aggregation

| E-UTRA operating band combination | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1-3-7-20 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 20 | 0.3 |

Table 5.10.2-2: ΔRIB,c for 4DL aggregation

| E-UTRA operating band combination | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1-3-7-20 | 1 | 0 |
|  | 3 | 0 |
|  | 7 | 0 |
|  | 20 | 0 |

### 5.10.3	REFSENS requirements

Table 5.10.3-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions for four bands due to close proximity of UL to DL channel)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A-7C-20A | 34,9 |  |  | -94 | -91.5 | -90 | -89 | FDD |
|  | 35 |  |  | -97 | -94 | -92.2 | -91 |  |
| NOTE 4: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in the bands other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 5: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in the bands other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 9: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

Table 5.10.3-2: Uplink configuration for the low band (exceptions for four bands due to close proximity of UL to DL channel)

| E-UTRA Band / Channel bandwidth of the affected DL band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-3A-7C-20A | 11,2 |  |  | 25 | 25 | 25 | 25 | FDD |
|  | 11,3 |  |  | 25 | 45 | 45 | 45 |  |
| NOTE 1: refers to the UL resource blocks shall be located as close as possible to the downlink channel in Band 3 but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1) in the uplink channel in Band 1. NOTE 2: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz NOTE 3: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. |  |  |  |  |  |  |  |  |

## 5.11	CA_1-3-40-41

### 5.11.1	Channel bandwidths per operating band for CA

Table 5.11.1-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_1A-3A-40A-41A | - | 1 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 3 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | 40 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 41 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.11.2	∆TIB and ∆RIB values

For CA_1A-3A-40A-41A, the TIB,c and RIB,c values are shown in table 5.11.2-1 and table 5.11.2-2, respectively.

Table 5.11.2-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1A-3A-40A-41A | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 40 | 0.5 |
|  | 41 | 0.35 |
|  |  | 0.86 |
| NOTE 5: The requirement is specified for the frequency range of 2545-2690MHz. NOTE 6: The requirement is specified for the frequency range of 2496-2545MHz. |  |  |

Table 5.11.2-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1A-3A-40A-41A | 1 | 0 |
|  | 3 | 0 |
|  | 40 | 0 |
|  | 41 | 05 |
|  |  | 0.56 |
| NOTE 5: The requirement is specified for the frequency range of 2545-2690MHz. NOTE 6: The requirement is specified for the frequency range of 2496-2545MHz. |  |  |

### 5.11.3	REFSENS requirements

Table 5.11.3-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions for four bands due to close proximity of UL to DL channel)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A-40A-41A | 34,12 |  |  | -94 | -91.5 | -90 | -89 | FDD |
|  | 35 |  |  | -97 | -94 | -92.2 | -91 |  |
|  | 40 |  |  | [-93.4] | -91.3 | -90 | -88.9 | TDD |
| NOTE 4: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in the Bands other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 5: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in the Bands other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 12: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

Table 5.11.3-2: Uplink configuration for the low band (exceptions for four bands due to close proximity of UL to DL channel)

| E-UTRA Band / Channel bandwidth of the affected DL band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-3A-40A-41A | 11,2 |  |  | 25 | 25 | 25 | 25 | FDD |
|  | 11,3 |  |  | 25 | 45 | 45 | 45 | FDD |
| NOTE 1: refers to the UL resource blocks shall be located as close as possible to the downlink channel in Band 3 but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1) in the uplink channel in Band 1. NOTE 2: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz NOTE 3: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. |  |  |  |  |  |  |  |  |

Table 5.11.3-3: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to cross band isolation issues of TDD and FDD bands)

| EUTRA CA Configuration | EUTRA band | Channel bandwidth |  |  |  |  |  | Duplex mode | Applicable active UL band |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |  |  |
| CA_1A-3A-40A-41A5,14 | 119 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |
|  | 312,19 | [-97.4] | [-95.3] | -94.2 | -91.2 | -89.5 | -88.3 |  |  |
|  |  |  |  | -94 | -91.5 | -90 | -89 |  | 1 |
|  | 313 |  |  | -97 | -94 | -92.2 | -91 |  |  |
|  | 4019 |  |  | [-93.4] | -91.9 | -90.4 | -89.4 | TDD |  |
|  | 4119 |  |  | -93.3 | -90.7 | -89.2 | -88.1 |  |  |
|  | 4019 |  |  | -95.4 | -92.9 | -91.3 | -90.2 |  | 3 |
|  | 4119 |  |  | -93.3 | -90.7 | -89.2 | -88.1 |  |  |
| NOTE 5: The B41 requirements are modified by -0.5dB when carrier frequency of the assigned E-UTRA channel bandwidth is within 2545-2690 MHz. NOTE 12: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in Band 3 and Band 41, the requirement applies regardless of channel bandwidth in Band 1. NOTE 13: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in Band 3 and Band 41, the requirement applies regardless of channel bandwidth in Band 1. NOTE 14: The B41 requirements also apply to the supported CA_1A-41A. NOTE 19: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |  |

Table 5.11.3-4: Uplink configuration for reference sensitivity (exceptions due to cross band isolation issues of TDD and FDD bands)

| E-UTRA Band / Channel bandwidth of the affected DL band / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| CA_1A-3A-40A-41A | 1 |  |  | 253 | 251,3 | 251,3 | 251,3 | FDD |
|  |  |  |  | 254 | 451,4 | 451,4 | 451,4 |  |
|  | 3 | 6 | 15 | 25 | 50 | 501 | 501 |  |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
|  | 41 |  |  | 25 | 50 | 75 | 100 |  |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1). NOTE 3: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. NOTE 4: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. |  |  |  |  |  |  |  |  |

## 5.12	CA_1A-7A-8A-28A

### 5.12.1	Channel bandwidths per operating band for CA

Table 5.12.1-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_1A-7A-8A-28A | - | 1 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 8 | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.12.2	∆TIB and ∆RIB values

For CA_1A-7A-8A-28A, the TIB,c and RIB,c values are shown in table 5.12.2-1 and table 5.12.2-2, respectively.

Table 5.12.2-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1A-7A-8A-28A | 1 | 0.5 |
|  | 7 | 0.6 |
|  | 8 | 0.6 |
|  | 28 | 0.6 |

Table 5.12.2-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1A-7A-8A-28A | 1 | 0 |
|  | 7 | 0 |
|  | 8 | 0.2 |
|  | 28 | 0.2 |

### 5.12.3	REFSENS requirements

Table 5.12.3-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-7A-8A-28A5,6 | 133 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
|  | 733 |  |  | -88 | -87.4 | -87 | -86.7 |  |
| NOTE 5: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 6: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 33: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |  |

Table 5.12.3-2: Uplink configuration for the low band (exceptions due to harmonic issue)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-7A-8A-28A | 8 |  |  | 8 | 16 | 25 | 25 | FDD |
|  | 28 |  |  | 8 | 16 | 25 | 25 |  |

## 5.13	CA_1-7-8-32

### 5.13.1	Channel bandwidths per operating band for CA

Table 5.13.1-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_1A-7A-8A-32A | - | 1 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 8 | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 32 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.13.2	∆TIB and ∆RIB values

For CA_1A-7A-8A-32A, the TIB,c and RIB,c values are shown in table 5.13.2-1 and table 5.13.2-2, respectively.

Table 5.13.2-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1A-7A-8A-32A | 1 | 0.7 |
|  | 7 | 0.7 |
|  | 8 | 0.6 |
|  | 32 | N/A |

Table 5.13.2-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1A-7A-8A-32A | 1 | 0 |
|  | 7 | 0 |
|  | 8 | 0.2 |
|  | 32 | 0 |

### 5.13.3	REFSENS requirements

Table 5.13.3-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-7A-8A-32A5,6 | 733 |  |  | -88 | -87.4 | -87 | -86.7 | FDD |
| NOTE 5: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 6: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 33: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |  |

Table 5.13.3-2: Uplink configuration for the low band (exceptions due to harmonic issue)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-7A-8A-32A | 8 |  |  | 8 | 16 | 25 | 25 | FDD |

Table 5.13.3-3: Reference sensitivity QPSK PREFSENS (CA with a SDL band)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-7A-8A-32A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 7 |  |  |  | -95 | -93.2 | -92 |  |
|  | 32 |  |  | -100 | -97 | -95.2 | -94 |  |

## 5.14	CA_1-7-28-32

### 5.14.1	Channel bandwidths per operating band for CA

Table 5.14.1-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_1A-7A-28A-32A | - | 1 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 28 |  | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | 32 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.14.2	∆TIB and ∆RIB values

For CA_1A-7A-28A-32A, the TIB,c and RIB,c values are shown in table 5.14.2-1 and table 5.14.2-2, respectively.

Table 5.14.2-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1A-7A-28A-32A | 1 | 0.7 |
|  | 7 | 0.7 |
|  | 28 | 0.6 |
|  | 32 | N/A |

Table 5.14.2-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1A-7A-28A-32A | 1 | 0 |
|  | 7 | 0 |
|  | 28 | 0.2 |
|  | 32 | 0 |

### 5.14.3	REFSENS requirements

Table 5.14.3-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-7A-28A-32A5,6 | 133 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
|  | 329,10 |  |  | -72.2 | -72.2 | -72.2 | -72.2 |  |
|  | 3211 |  |  | -97.6 | -95.2 | -93.7 | -93.0 |  |
| NOTE 5: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 6: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 9: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_3A-42A, CA_3A-3A-42A, CA_3A-42A-42A, CA_1A-3A-20A-32A-42A, CA_3A-42A-43A, CA_3A-32A-42A-43A, CA_1A-3A-42A, CA_2A-13A-48A-66A, CA_2A-48A, CA_2A-48C, CA_2A-48D, CA_48A-66A, CA_3A-7A-42A, CA_3A-19A-42A, CA_3A-20A-42A, CA_3A-28A-42A, CA_1A-3A-7A-42A, CA_5A-48A-66A, CA_5A-48A-66A-66A, CA_13A-48A-66A, CA_13A-48A-66A-66A, CA_13A-48A-66B, CA_13A-48A-66C, CA_13A-48A-48A-66A, CA_13A-48C-66A, CA_13A-48D-66A, CA_13A-48A-48C-66A, CA_28A-32A, CA_48A-66A-66A, CA_48A-66B , CA_48A-66C, CA_48A-48A-66A, CA_48C-66A, CA_48A-48A-66A-66A, CA_48A-48A-66B, CA_48A-48A-66C, CA_48C-66B, CA_48C-66C, CA_48E-66A, CA_1A-3A-18A-42A, CA_1A-3A-19A-42A, CA_1A-3A-32A-42A, CA_1A-3A-41A-42A, CA_3A-7A-20A-42A, CA_3A-20A-32A-42A, CA_3A-28A-41A-42A, CA_3A-18A-42A, CA_3A-18A-42C, CA_3A-8A-42A and CA_3A-8A-42C. FHD = 0MHz for CA_11A-28A, CA_1A-11A-28A and CA_3A-11A-28A. NOTE 10: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 11: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 33: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |  |

Table 5.14.3-2: Uplink configuration for the low band (exceptions due to harmonic issue)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-7A-28A-32A | 28 |  |  | 8 | 16 | 25 | 25 | FDD |

Table 5.14.3-3: Reference sensitivity QPSK PREFSENS (CA with a SDL band)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-7A-28A-32A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 7 |  |  |  | -95 | -93.2 | -92 |  |
|  | 32 |  |  | -100 | -97 | -95.2 | -94 |  |

## 5.15	CA_1-8-20-32

### 5.15.1	Channel bandwidths per operating band for CA

Table 5.15.1-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_1A-8A-20A-32A | - | 1 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 8 | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 32 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.15.2	∆TIB and ∆RIB values

For CA_1A-8A-20A-32A, the TIB,c and RIB,c values are shown in table 5.15.2-1 and table 5.15.2-2, respectively.

Table 5.15.2-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1A-8A-20A-32A | 1 | 0.5 |
|  | 8 | 0.4 |
|  | 20 | 0.4 |
|  | 32 | N/A |

Table 5.15.2-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1A-8A-20A-32A | 1 | 0 |
|  | 8 | 0 |
|  | 20 | 0 |
|  | 32 | 0 |

### 5.15.3	REFSENS requirements

Table 5.15.3-1: Reference sensitivity QPSK PREFSENS (CA with a SDL band)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-8A-20A-32A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 20 |  |  | -97 | -94 | -91.2 | -90 |  |
|  | 32 |  |  | -100 | -97 | -95.2 | -94 |  |

## 5.16	CA_1-8-28-32

### 5.16.1	Channel bandwidths per operating band for CA

Table 5.16.1-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_1A-8A-28A-32A | - | 1 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 8 | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 28 |  | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | 32 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.16.2	∆TIB and ∆RIB values

For CA_1A-8A-28A-32A, the TIB,c and RIB,c values are shown in table 5.16.2-1 and table 5.16.2-2, respectively.

Table 5.16.2-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1A-8A-28A-32A | 1 | 0.5 |
|  | 8 | 0.6 |
|  | 28 | 0.6 |
|  | 32 | N/A |

Table 5.16.2-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1A-8A-28A-32A | 1 | 0 |
|  | 8 | 0.2 |
|  | 28 | 0.2 |
|  | 32 | 0 |

### 5.16.3	REFSENS requirements

Table 5.16.3-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-8A-28A-32A5,6,14 | 133 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
|  | 32 |  |  | -72.2 | -72.2 | -72.2 | -72.2 | FDD |
| NOTE 5: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 6: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band.NOTE 33: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. NOTE 14: For the UE that supports CA_1A-18A-28A or CA_1A-19A-28A, no requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity should only be verified when this is not the case (the requirements specified in clause 7.3.1 apply). NOTE 33: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |  |

Table 5.16.3-2: Uplink configuration for the low band (exceptions due to harmonic issue)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-8A-28A-32A | 28 |  |  | 8 | 16 | 25 | 25 | FDD |

Table 5.16.3-3: Reference sensitivity QPSK PREFSENS (CA with a SDL band)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-8A-28A-32A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 32 |  |  | -100 | -97 | -95.2 | -94 |  |

## 5.17	CA_1-20-28-32

### 5.17.1	Channel bandwidths per operating band for CA

Table 5.17.1-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_1A-20A-28A-32A | - | 1 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 28 |  | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | 32 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.17.2	∆TIB and ∆RIB values

For CA_1A-20A-28A-32A, the TIB,c and RIB,c values are shown in table 5.17.2-1 and table 5.17.2-2, respectively.

Table 5.17.2-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1A-20A-28A-32A | 1 | 0.5 |
|  | 20 | 0.6 |
|  | 28 | 0.6 |
|  | 32 | N/A |

Table 5.17.2-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1A-20A-28A-32A | 1 | 0 |
|  | 20 | 0.2 |
|  | 28 | 0.2 |
|  | 32 | 0 |

### 5.17.3	REFSENS requirements

Table 5.17.3-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-20A-28A-32A9,10 | 133 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
|  | 32 |  |  | -72.2 | -72.2 | -72.2 | -72.2 | FDD |
| CA_1A-20A-28A-32A11 | 133 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
|  | 32 |  |  | -97.6 | -95.2 | -93.7 | -93.0 | FDD |
| NOTE 9: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_3A-42A, CA_3A-3A-42A, CA_3A-42A-42A, CA_1A-3A-20A-32A-42A, CA_3A-42A-43A, CA_3A-32A-42A-43A, CA_1A-3A-42A, CA_2A-13A-48A-66A, CA_2A-48A, CA_2A-48C, CA_2A-48D, CA_48A-66A, CA_3A-7A-42A, CA_3A-19A-42A, CA_3A-20A-42A, CA_3A-28A-42A, CA_1A-3A-7A-42A, CA_5A-48A-66A, CA_5A-48A-66A-66A, CA_13A-48A-66A, CA_13A-48A-66A-66A, CA_13A-48A-66B, CA_13A-48A-66C, CA_13A-48A-48A-66A, CA_13A-48C-66A, CA_13A-48D-66A, CA_13A-48A-48C-66A, CA_28A-32A, CA_48A-66A-66A, CA_48A-66B , CA_48A-66C, CA_48A-48A-66A, CA_48C-66A, CA_48A-48A-66A-66A, CA_48A-48A-66B, CA_48A-48A-66C, CA_48C-66B, CA_48C-66C, CA_48E-66A, CA_1A-3A-18A-42A, CA_1A-3A-19A-42A, CA_1A-3A-32A-42A, CA_1A-3A-41A-42A, CA_3A-7A-20A-42A, CA_3A-20A-32A-42A, CA_3A-28A-41A-42A, CA_3A-18A-42A, CA_3A-18A-42C, CA_3A-8A-42A and CA_3A-8A-42C. FHD = 0MHz for CA_11A-28A, CA_1A-11A-28A and CA_3A-11A-28A. NOTE 10: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 11: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 33: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |  |

Table 5.17.3-2: Uplink configuration for the low band (exceptions due to harmonic issue)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-20A-28A-32A | 28 |  |  | 8 | 16 | 25 | 25 | FDD |

Table 5.17.3-3: Reference sensitivity QPSK PREFSENS (CA with a SDL band)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-20A-28A-32A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 20 |  |  | -97 | -94 | -91.2 | -90 |  |
|  | 32 |  |  | -100 | -97 | -95.2 | -94 |  |

## 5.18	CA_3A-7A-8A-28A

### 5.18.1	Channel bandwidths per operating band for CA

Table 5.18.1-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_3A-7A-8A-28A | - | 3 | Yes | Yes | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 8 | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.18.2	∆TIB and ∆RIB values

For CA_3A-7A-8A-28A, the TIB,c and RIB,c values are shown in table 5.18.2-1 and table 5.18.2-2, respectively.

Table 5.18.2-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_3A-7A-8A-28A | 3 | 0.5 |
|  | 7 | 0.5 |
|  | 8 | 0.6 |
|  | 28 | 0.3 |

Table 5.18.2-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_3A-7A-8A-28A | 3 | 0 |
|  | 7 | 0 |
|  | 8 | 0.2 |
|  | 28 | 0.1 |

### 5.18.3	REFSENS requirements

Table 5.18.3-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_3A-7A-8A-28A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_3A-7A-8A-28A5,6 | 733 |  |  | -88 | -87.4 | -87 | -86.7 |  |
| NOTE 4: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity for all active downlink component carriers is only verified when this is not the case (the requirements specified in clause 7.3.1 apply unless otherwise specified). NOTE 5: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 6: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 33: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |  |

Table 5.18.3-2: Uplink configuration for the low band (exceptions due to harmonic issue)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_3A-7A-8A-28A | 8 |  |  | 8 | 16 | 25 | 25 | FDD |

## 5.19	CA_3-8-40-41

### 5.19.1	Channel bandwidths per operating band for CA

Table 5.19.1-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_3A-8A-40A-41A | - | 3 | Yes | Yes | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 8 | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 40 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 41 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.19.2	∆TIB and ∆RIB values

For CA_3A-8A-40A-41A, the TIB,c and RIB,c values are shown in table 5.19.2-1 and table 5.19.2-2, respectively.

Table 5.19.2-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_3A-8A-40A-41A | 3 | 0.5 |
|  | 8 | 0.3 |
|  | 40 | 0.5 |
|  | 41 | 0.35 |
|  |  | 0.86 |
| NOTE 5: The requirement is specified for the frequency range of 2545-2690MHz. NOTE 6: The requirement is specified for the frequency range of 2496-2545MHz. |  |  |

Table 5.19.2-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_3A-8A-40A-41A | 3 | 0 |
|  | 8 | 0 |
|  | 40 | 0 |
|  | 41 | 05 |
|  |  | 0.56 |
| NOTE 5: The requirement is specified for the frequency range of 2545-2690MHz. NOTE 6: The requirement is specified for the frequency range of 2496-2545MHz. |  |  |

### 5.19.3	REFSENS requirements

Table 5.19.3-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions for four bands due to close proximity of UL to DL channel)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_3A-8A-40A-41A4,8 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
|  | 41 |  |  | N/A | N/A | N/A | N/A | TDD |
| NOTE 4: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in the Bands other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 8: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity is only verified when this is not the case (the requirements specified in clause 7.3.1 apply). |  |  |  |  |  |  |  |  |

Table 5.19.3-2: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to cross band isolation issues of TDD and FDD bands)

| EUTRA CA Configuration | EUTRA band | Channel bandwidth |  |  |  |  |  | Duplex mode | Applicable active UL band |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |  |  |
| CA_3A-8A-40A-41A5,19 | 3 |  | [-95.3] | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |
|  | 40 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD | 3 |
|  | 41 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD | 3 |
| NOTE 5: The B41 requirements are modified by -0.5dB when carrier frequency of the assigned E-UTRA channel bandwidth is within 2545-2690 MHz. NOTE 19: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |  |

Table 5.19.3-4: Uplink configuration for reference sensitivity (exceptions due to cross band isolation issues of TDD and FDD bands)

| E-UTRA Band / Channel bandwidth of the affected DL band / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| CA_3A-8A-40A-41A | 3 | 6 | 15 | 25 | 50 | 501 | 501 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
|  | 41 |  |  | 25 | 50 | 75 | 100 |  |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth |  |  |  |  |  |  |  |  |

## 5.20	CA_7A-8A-20A-28A

### 5.20.1	Channel bandwidths per operating band for CA

Table 5.20.1-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_7A-8A-20A-28A | - | 7 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 8 | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.20.2	∆TIB and ∆RIB values

For CA_7A-8A-20A-28A, the TIB,c and RIB,c values are shown in table 5.20.2-1 and table 5.20.2-2, respectively.

Table 5.20.2-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_7A-8A-20A-28A | 7 | 0.3 |
|  | 8 | 0.6 |
|  | 20 | 0.6 |
|  | 28 | 0.6 |

Table 5.20.2-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_7A-8A-20A-28A | 7 | 0 |
|  | 8 | 0.2 |
|  | 20 | 0.2 |
|  | 28 | 0.2 |

### 5.20.3	REFSENS requirements

Table 5.20.3-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_7A-8A-20A-28A5,6 | 733 |  |  |  | -87.4 | -87 | -86.7 | FDD |
| NOTE 5: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 6: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 33: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |  |

Table 5.20.3-2: Uplink configuration for the low band (exceptions due to harmonic issue)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_7A-8A-20A-28A | 8 |  |  |  | 16 | 25 | 25 | FDD |

## 5.21	CA_7-8-20-32

### 5.21.1	Channel bandwidths per operating band for CA

Table 5.21.1-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_7A-8A-20A-32A | - | 7 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 8 | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 32 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.21.2	∆TIB and ∆RIB values

For CA_7A-8A-20A-32A, the TIB,c and RIB,c values are shown in table 5.21.2-1 and table 5.21.2-2, respectively.

Table 5.21.2-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_7A-8A-20A-32A | 7 | 0.7 |
|  | 8 | 0.6 |
|  | 20 | 0.6 |
|  | 32 | N/A |

Table 5.21.2-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_7A-8A-20A-32A | 7 | 0 |
|  | 8 | 0.2 |
|  | 20 | 0.2 |
|  | 32 | 0 |

### 5.21.3	REFSENS requirements

Table 5.21.3-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_7A-8A-20A-32A5,6 | 733 |  |  |  | -87.4 | -87 | -86.7 | FDD |
| NOTE 5: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 6: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 33: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |  |

Table 5.21.3-2: Uplink configuration for the low band (exceptions due to harmonic issue)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_7A-8A-20A-32A | 8 |  |  |  | 16 | 25 | 25 | FDD |

Table 5.21.3-3: Reference sensitivity QPSK PREFSENS (CA with a SDL band)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_7A-8A-20A-32A | 7 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 20 |  |  | -97 | -94 | -91.2 | -90 |  |
|  | 32 |  |  | -100 | -97 | -95.2 | -94 |  |

## 5.22	CA_7-8-28-32

### 5.22.1	Channel bandwidths per operating band for CA

Table 5.22.1-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_7A-8A-28A-32A | - | 7 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 8 | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 28 |  | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | 32 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.22.2	∆TIB and ∆RIB values

For CA_7A-8A-28A-32A, the TIB,c and RIB,c values are shown in table 5.22.2-1 and table 5.22.2-2, respectively.

Table 5.22.2-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_7A-8A-28A-32A | 7 | 0.7 |
|  | 8 | 0.6 |
|  | 28 | 0.3 |
|  |  | 0.514 |
|  | 32 | N/A |
| NOTE 14: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 8. |  |  |

Table 5.22.2-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_7A-8A-28A-32A | 7 | 0 |
|  | 8 | 0.2 |
|  | 28 | 0 |
|  |  | 0.113 |
|  | 32 | 0 |
| NOTE 13: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 8. |  |  |

### 5.22.3	REFSENS requirements

Table 5.22.3-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_7A-8A-28A-32A | 75,6,33 |  |  | -88 | -87.4 | -87 | -86.7 | FDD |
|  | 329,10 |  |  | -72.2 | -72.2 | -72.2 | -72.2 |  |
|  | 3211 |  |  | -97.6 | -95.2 | -93.7 | -93.0 |  |
| NOTE 5: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 6: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 9: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_3A-42A, CA_3A-3A-42A, CA_3A-42A-42A, CA_1A-3A-20A-32A-42A, CA_3A-42A-43A, CA_3A-32A-42A-43A, CA_1A-3A-42A, CA_2A-13A-48A-66A, CA_2A-48A, CA_2A-48C, CA_2A-48D, CA_48A-66A, CA_3A-7A-42A, CA_3A-19A-42A, CA_3A-20A-42A, CA_3A-28A-42A, CA_1A-3A-7A-42A, CA_5A-48A-66A, CA_5A-48A-66A-66A, CA_13A-48A-66A, CA_13A-48A-66A-66A, CA_13A-48A-66B, CA_13A-48A-66C, CA_13A-48A-48A-66A, CA_13A-48C-66A, CA_13A-48D-66A, CA_13A-48A-48C-66A, CA_28A-32A, CA_48A-66A-66A, CA_48A-66B , CA_48A-66C, CA_48A-48A-66A, CA_48C-66A, CA_48A-48A-66A-66A, CA_48A-48A-66B, CA_48A-48A-66C, CA_48C-66B, CA_48C-66C, CA_48E-66A, CA_1A-3A-18A-42A, CA_1A-3A-19A-42A, CA_1A-3A-32A-42A, CA_1A-3A-41A-42A, CA_3A-7A-20A-42A, CA_3A-20A-32A-42A, CA_3A-28A-41A-42A, CA_3A-18A-42A, CA_3A-18A-42C, CA_3A-8A-42A and CA_3A-8A-42C. FHD = 0MHz for CA_11A-28A, CA_1A-11A-28A and CA_3A-11A-28A. NOTE 10: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 11: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 33: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |  |

Table 5.22.3-2: Uplink configuration for the low band (exceptions due to harmonic issue)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_7A-8A-28A-32A | 8 |  |  | 8 | 16 | 25 | 25 | FDD |
|  | 28 |  |  | 12 | 25 | 36 | 50 |  |

Table 5.22.3-3: Reference sensitivity QPSK PREFSENS (CA with a SDL band)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_7A-8A-28A-32A | 7 |  |  |  | -95 | -93.2 | -92 | FDD |
|  | 32 |  |  | -100 | -97 | -95.2 | -94 |  |

## 5.23	CA_7-20-28-32

### 5.23.1	Channel bandwidths per operating band for CA

Table 5.23.1-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_7A-20A-28A-32A | - | 7 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 28 |  | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | 32 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.23.2	∆TIB and ∆RIB values

For CA_7A-20A-28A-32A, the TIB,c and RIB,c values are shown in table 5.23.2-1 and table 5.23.2-2, respectively.

Table 5.23.2-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_7A-20A-28A-32A | 7 | 0.7 |
|  | 20 | 0.6 |
|  | 28 | 0.6 |
|  | 32 | N/A |

Table 5.23.2-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_7A-20A-28A-32A | 7 | 0 |
|  | 20 | 0.2 |
|  | 28 | 0.2 |
|  | 32 | 0 |

### 5.23.3	REFSENS requirements

Table 5.23.3-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_7A-20A-28A-32A9,10 | 32 |  |  | -72.2 | -72.2 | -72.2 | -72.2 | FDD |
| CA_7A-20A-28A-32A11 | 32 |  |  | -97.6 | -95.2 | -93.7 | -93.0 | FDD |
| NOTE 9: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_3A-42A, CA_3A-3A-42A, CA_3A-42A-42A, CA_1A-3A-20A-32A-42A, CA_3A-42A-43A, CA_3A-32A-42A-43A, CA_1A-3A-42A, CA_2A-13A-48A-66A, CA_2A-48A, CA_2A-48C, CA_2A-48D, CA_48A-66A, CA_3A-7A-42A, CA_3A-19A-42A, CA_3A-20A-42A, CA_3A-28A-42A, CA_1A-3A-7A-42A, CA_5A-48A-66A, CA_5A-48A-66A-66A, CA_13A-48A-66A, CA_13A-48A-66A-66A, CA_13A-48A-66B, CA_13A-48A-66C, CA_13A-48A-48A-66A, CA_13A-48C-66A, CA_13A-48D-66A, CA_13A-48A-48C-66A, CA_28A-32A, CA_48A-66A-66A, CA_48A-66B , CA_48A-66C, CA_48A-48A-66A, CA_48C-66A, CA_48A-48A-66A-66A, CA_48A-48A-66B, CA_48A-48A-66C, CA_48C-66B, CA_48C-66C, CA_48E-66A, CA_1A-3A-18A-42A, CA_1A-3A-19A-42A, CA_1A-3A-32A-42A, CA_1A-3A-41A-42A, CA_3A-7A-20A-42A, CA_3A-20A-32A-42A, CA_3A-28A-41A-42A, CA_3A-18A-42A, CA_3A-18A-42C, CA_3A-8A-42A and CA_3A-8A-42C. FHD = 0MHz for CA_11A-28A, CA_1A-11A-28A and CA_3A-11A-28A. NOTE 10: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 11: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. |  |  |  |  |  |  |  |  |  |

Table 5.23.3-2: Uplink configuration for the low band (exceptions due to harmonic issue)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_7A-20A-28A-32A | 28 |  |  | 12 | 25 | 36 | 50 | FDD |

Table 5.23.3-3: Reference sensitivity QPSK PREFSENS (CA with a SDL band)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_7A-20A-28A-32A | 7 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 20 |  |  | -97 | -94 | -91.2 | -90 |  |
|  | 32 |  |  | -100 | -97 | -95.2 | -94 |  |

## 5.24	CA_8-20-28-32

### 5.24.1	Channel bandwidths per operating band for CA

Table 5.24.1-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_8A-20A-28A-32A | - | 8 | Yes | Yes | Yes | Yes |  |  | 70 | 0 |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 28 |  | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | 32 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.24.2	∆TIB and ∆RIB values

For CA_8A-20A-28A-32A, the TIB,c and RIB,c values are shown in table 5.24.2-1 and table 5.24.2-2, respectively.

Table 5.24.2-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_8A-20A-28A-32A | 8 | 0.6 |
|  | 20 | 0.5 |
|  | 28 | 0.5 |
|  | 32 | N/A |

Table 5.24.2-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_8A-20A-28A-32A | 8 | 0 |
|  | 20 | 0 |
|  | 28 | 0 |
|  | 32 | 0 |

### 5.24.3	REFSENS requirements

Table 5.24.3-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_8A-20A-28A-32A9,10 | 32 |  |  | -72.2 | -72.2 | -72.2 | -72.2 | FDD |
| CA_8A-20A-28A-32A11 | 32 |  |  | -97.6 | -95.2 | -93.7 | -93.0 | FDD |
| NOTE 9: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_3A-42A, CA_3A-3A-42A, CA_3A-42A-42A, CA_1A-3A-20A-32A-42A, CA_3A-42A-43A, CA_3A-32A-42A-43A, CA_1A-3A-42A, CA_2A-13A-48A-66A, CA_2A-48A, CA_2A-48C, CA_2A-48D, CA_48A-66A, CA_3A-7A-42A, CA_3A-19A-42A, CA_3A-20A-42A, CA_3A-28A-42A, CA_1A-3A-7A-42A, CA_5A-48A-66A, CA_5A-48A-66A-66A, CA_13A-48A-66A, CA_13A-48A-66A-66A, CA_13A-48A-66B, CA_13A-48A-66C, CA_13A-48A-48A-66A, CA_13A-48C-66A, CA_13A-48D-66A, CA_13A-48A-48C-66A, CA_28A-32A, CA_48A-66A-66A, CA_48A-66B , CA_48A-66C, CA_48A-48A-66A, CA_48C-66A, CA_48A-48A-66A-66A, CA_48A-48A-66B, CA_48A-48A-66C, CA_48C-66B, CA_48C-66C, CA_48E-66A, CA_1A-3A-18A-42A, CA_1A-3A-19A-42A, CA_1A-3A-32A-42A, CA_1A-3A-41A-42A, CA_3A-7A-20A-42A, CA_3A-20A-32A-42A, CA_3A-28A-41A-42A, CA_3A-18A-42A, CA_3A-18A-42C, CA_3A-8A-42A and CA_3A-8A-42C. FHD = 0MHz for CA_11A-28A, CA_1A-11A-28A and CA_3A-11A-28A. NOTE 10: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 11: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. |  |  |  |  |  |  |  |  |  |

Table 5.24.3-2: Uplink configuration for the low band (exceptions due to harmonic issue)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_8A-20A-28A-32A | 28 |  |  | 12 | 25 | 36 | 50 | FDD |

Table 5.24.3-3: Reference sensitivity QPSK PREFSENS (CA with a SDL band)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_8A-20A-28A-32A | 20 |  |  | -97 | -94 | -91.2 | -90 | FDD |
|  | 32 |  |  | -100 | -97 | -95.2 | -94 |  |

## 5.25		CA_1-7-20-38

### 5.25.1	Channel bandwidths per operating band for CA

Table 5.25.1-1: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-7A-20A-38A | - | 1 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.25.2	∆TIB and ∆RIB values

Table 5.25.2-1: IB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1-7-20-38 | 1 | 0.6 |
|  | 7 | 0.5 |
|  | 20 | 0.5 |
|  | 38 | 0.5 |

Table 5.25.2-2: R IB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1-7-20-38 | 1 | 0 |
|  | 7 | 0 |
|  | 20 | 0 |
|  | 38 | 0.2 |

### 5.25.3 	REFSENS requirements

No additional REFSENS / MSD are needed since it is already covered in fallback band combination.

## 5.26	CA_1A-3A-3A-8A-38A

### 5.26.1	Channel bandwidths per operating band for CA

Table 5.26.1-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_1A-3A-3A-8A-38A1 | - | 1 |  |  | Yes | Yes | Yes | Yes | 90 | 0 |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.26.2	∆TIB and ∆RIB values

For CA_1-3-3-8-38, the TIB,c and RIB,c values are shown in table 5.26.2-1 and table 5.26.2-2, respectively.

Table 5.26.2-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1-3-3-8-38 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 8 | 0.3 |
|  | 38 | 0.5 |

Table 5.26.2-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1-3-3-8-38 | 1 | 0 |
|  | 3 | 0 |
|  | 8 | 0 |
|  | 38 | 0.2 |

### 5.26.3	REFSENS requirements

No additional MSD required compared to fallbacks.

## 5.27	CA_1A-3A-28A-38A

### 5.27.1	Channel bandwidths per operating band for CA

Table 5.27.1-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_1A-3A-28A-38A | - | 1 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.27.2	∆TIB and ∆RIB values

For CA_1-3-28-38, the TIB,c and RIB,c values are shown in table 5.27.2-1 and table 5.27.2-2, respectively.

Table 5.27.2-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1-7-20-38 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 28 | 0.6 |
|  | 38 | 0.5 |

Table 5.27.2-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1-3-28-38 | 1 | 0 |
|  | 3 | 0 |
|  | 28 | 0.2 |
|  | 38 | 0.2 |

### 5.27.3	REFSENS requirements

No additional MSD required compared to fallbacks.

## 5.28	CA_1A-7A-20A-38A

### 5.28.1	Channel bandwidths per operating band for CA

Table 5.28.1-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_1A-7A-20A-38A1 | - | 1 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |
| NOTE 1: UL carrier shall be supported in Bands 1 and 20 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within [6dB] |  |  |  |  |  |  |  |  |  |  |

### 5.28.2	∆TIB and ∆RIB values

For CA_1-7-20-38, the TIB,c and RIB,c values are shown in table 5.28.2-1 and table 5.28.2-2, respectively.

Table 5.28.2-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1-7-20-38 | 1 | 0.5 |
|  | 20 | 0.3 |

Table 5.28.2-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1-7-20-38 | 1 | 0 |
|  | 7 | 0 |
|  | 20 | 0 |
|  | 38 | 0.2 |

### 5.28.3	REFSENS requirements

No additional MSD required compared to fallbacks.

## 5.29	CA_1A-7A-28A-38A

### 5.29.1	Channel bandwidths per operating band for CA

Table 5.29.1-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_1A-7A-28A-38A1 | - | 1 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |
| NOTE 1: UL carrier shall be supported in Bands 1 and 28 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within [6dB] |  |  |  |  |  |  |  |  |  |  |

### 5.29.2	∆TIB and ∆RIB values

For CA_1-7-28-38, the TIB,c and RIB,c values are shown in table 5.29.2-1 and table 5.29.2-2, respectively.

Table 5.29.2-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1-7-28-38 | 1 | 0.5 |
|  | 28 | 0.6 |

Table 5.29.2-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1-7-28-38 | 1 | 0 |
|  | 7 | 0 |
|  | 28 | 0.2 |
|  | 38 | 0.2 |

### 5.29.3	REFSENS requirements

No additional MSD required compared to fallbacks.

## 5.30	CA_1A-7A-32A-38A

### 5.30.1	Channel bandwidths per operating band for CA

Table 5.30.1-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_1A-7A-32A-38A1 | - | 1 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 32 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |
| NOTE 1: UL carrier shall be supported in Band 1 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within [6dB] |  |  |  |  |  |  |  |  |  |  |

### 5.30.2	∆TIB and ∆RIB values

For CA_1-7-32-38, the TIB,c and RIB,c values are shown in table 5.30.2-1 and table 5.30.2-2, respectively.

Table 5.30.2-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1-7-32-38 | 1 | 0.5 |

Table 5.30.2-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1-7-32-38 | 1 | 0 |
|  | 7 | 0 |
|  | 32 | 0 |
|  | 38 | 0.2 |

### 5.30.3	REFSENS requirements

No additional MSD required compared to fallbacks.

## 5.31	CA_1A-8A-32A-38A

### 5.31.1	Channel bandwidths per operating band for CA

Table 5.31.1-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_1A-8A-32A-38A | - | 1 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 32 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.31.2	∆TIB and ∆RIB values

For CA_1-8-32-38, the TIB,c and RIB,c values are shown in table 5.31.2-1 and table 5.31.2-2, respectively.

Table 5.31.2-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1-8-32-38 | 1 | 0.5 |
|  | 8 | 0.3 |
|  | 38 | 0.5 |

Table 5.31.2-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1-8-32-38 | 1 | 0 |
|  | 8 | 0 |
|  | 32 | 0 |
|  | 38 | 0 |

### 5.31.3	REFSENS requirements

No additional MSD required compared to fallbacks.

## 5.32	CA_1A-20A-28A-38A

### 5.32.1	Channel bandwidths per operating band for CA

Table 5.32.1-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_1A-20A-28A-38A12 | - | 1 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 20 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |
| NOTE 12: Power imbalance between downlink carriers on Band 20 and Band 28 is assumed to be within [6dB]. |  |  |  |  |  |  |  |  |  |  |

### 5.32.2	∆TIB and ∆RIB values

For CA_1-20-28-38, the TIB,c and RIB,c values are shown in table 5.32.2-1 and table 5.32.2-2, respectively.

Table 5.32.2-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1-20-28-38 | 1 | 0.5 |
|  | 20 | 0.6 |
|  | 28 | 0.6 |
|  | 38 | 0.5 |

Table 5.32.2-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1-20-28-38 | 1 | 0 |
|  | 20 | 0.2 |
|  | 28 | 0.2 |
|  | 38 | 0 |

### 5.32.3	REFSENS requirements

No additional MSD required compared to fallbacks.

## 5.33	CA_1A-20A-32A-38A

### 5.33.1	Channel bandwidths per operating band for CA

Table 5.33.1-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_1A-20A-32A-38A | - | 1 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 20 |  |  | Yes | Yes |  |  |  |  |
|  |  | 32 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.33.2	∆TIB and ∆RIB values

For CA_1-20-32-38, the TIB,c and RIB,c values are shown in table 5.33.2-1 and table 5.33.2-2, respectively.

Table 5.33.2-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1-20-32-38 | 1 | 0.5 |
|  | 20 | 0.3 |
|  | 38 | 0.5 |

Table 5.33.2-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1-20-32-38 | 1 | 0 |
|  | 20 | 0 |
|  | 32 | 0 |
|  | 38 | 0 |

### 5.33.3	REFSENS requirements

No additional MSD required compared to fallbacks.

## 5.34	CA_3A-7A-20A-38A

### 5.34.1	Channel bandwidths per operating band for CA

Table 5.34.1-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_3A-7A-20A-38A1 | - | 3 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |
| NOTE 1: UL carrier shall be supported in Bands 3 and 20 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within [6dB] |  |  |  |  |  |  |  |  |  |  |

### 5.34.2	∆TIB and ∆RIB values

For CA_3-7-20-38, the TIB,c and RIB,c values are shown in table 5.34.2-1 and table 5.34.2-2, respectively.

Table 5.34.2-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_3-7-20-38 | 3 | 0.5 |
|  | 20 | 0.3 |

Table 5.34.2-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_3-7-20-38 | 3 | 0 |
|  | 7 | 0 |
|  | 20 | 0 |
|  | 38 | 0.2 |

### 5.34.3	REFSENS requirements

No additional MSD required compared to fallbacks.

## 5.35	CA_3A-20A-28A-38A

### 5.35.1	Channel bandwidths per operating band for CA

Table 5.35.1-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_3A-20A-28A-38A12 | - | 3 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 20 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |
| NOTE 12: Power imbalance between downlink carriers on Band 20 and Band 28 is assumed to be within [6dB]. |  |  |  |  |  |  |  |  |  |  |

### 5.35.2	∆TIB and ∆RIB values

For CA_3-20-28-38, the TIB,c and RIB,c values are shown in table 5.35.2-1 and table 5.35.2-2, respectively.

Table 5.35.2-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_3-20-28-38 | 3 | 0.5 |
|  | 20 | 0.5 |
|  | 28 | 0.5 |
|  | 38 | 0.5 |

Table 5.35.2-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_3-20-28-38 | 3 | 0 |
|  | 20 | 0.1 |
|  | 28 | 0.2 |
|  | 38 | 0.2 |

### 5.35.3	REFSENS requirements

No additional MSD required compared to fallbacks.

## 5.36	CA_7A-8A-20A-38A

### 5.36.1	Channel bandwidths per operating band for CA

Table 5.36.1-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_7A-8A-20A-38A1 | - | 7 |  |  |  | Yes | Yes | Yes | 60 | 0 |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 20 |  |  | Yes | Yes |  |  |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |
| NOTE 1: UL carrier shall be supported in Bands 8 and 20 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within [6dB] |  |  |  |  |  |  |  |  |  |  |

### 5.36.2	∆TIB and ∆RIB values

For CA_7-8-20-38, the TIB,c and RIB,c values are shown in table 5.36.2-1 and table 5.36.2-2, respectively.

Table 5.36.2-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_7-8-20-38 | 8 | 0.6 |
|  | 20 | [0.6] |

Table 5.36.2-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_7-8-20-38 | 7 | 0 |
|  | 8 | 0.2 |
|  | 20 | [0.2] |
|  | 38 | 0.2 |

### 5.36.3	REFSENS requirements

No additional MSD required compared to fallbacks.

## 5.37	CA_7A-8A-32A-38A

### 5.37.1	Channel bandwidths per operating band for CA

Table 5.37.1-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_7A-8A-32A-38A13 | - | 7 |  |  |  | Yes | Yes | Yes | 70 | 0 |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 32 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |
| NOTE 13: UL carrier shall be supported in Band 8 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within [6dB]. |  |  |  |  |  |  |  |  |  |  |

### 5.37.2	∆TIB and ∆RIB values

For CA_7-8-32-38, the TIB,c and RIB,c values are shown in table 5.37.2-1 and table 5.37.2-2, respectively.

Table 5.37.2-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_7-8-32-38 | 7 | 0.7 |
|  | 8 | 0.6 |
|  | 38 | 0.5 |

Table 5.37.2-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_7-8-32-38 | 7 | 0 |
|  | 8 | 0.2 |
|  | 32 | 0 |
|  | 38 | 0.2 |

### 5.37.3	REFSENS requirements

No additional MSD required compared to fallbacks.

## 5.38	CA_7A-20A-28A-38A

### 5.38.1	Channel bandwidths per operating band for CA

Table 5.38.1-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_7A-20A-28A-38A8,12 | - | 7 |  |  |  | Yes | Yes | Yes | 80 | 0 |
|  |  | 20 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |
| NOTE 8: UL carrier shall be supported in Bands 20 and 28 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within [6dB] NOTE 12: Power imbalance between downlink carriers on Band 20 and Band 28 is assumed to be within [6dB]. |  |  |  |  |  |  |  |  |  |  |

### 5.38.2	∆TIB and ∆RIB values

For CA_7-20-28-38, the TIB,c and RIB,c values are shown in table 5.38.2-1 and table 5.38.2-2, respectively.

Table 5.38.2-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_7-20-28-38 | 7 | 0.3 |
|  | 20 | 0.6 |
|  | 28 | 0.6 |
|  | 38 | 0.3 |

Table 5.38.2-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_7-20-28-38 | 7 | 0 |
|  | 20 | 0.2 |
|  | 28 | 0.2 |
|  | 38 | 0.2 |

### 5.38.3	REFSENS requirements

No additional MSD required compared to fallbacks.

## 5.39	CA_7A-20A-32A-38A

### 5.39.1	Channel bandwidths per operating band for CA

Table 5.39.1-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_7A-20A-32A-38A8 | - | 7 |  |  |  | Yes | Yes | Yes | 80 | 0 |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 32 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |
| NOTE 8: UL carrier shall be supported in Band 20 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within [6dB] |  |  |  |  |  |  |  |  |  |  |

### 5.39.2	∆TIB and ∆RIB values

For CA_7-20-32-38, the TIB,c and RIB,c values are shown in table 5.39.2-1 and table 5.39.2-2, respectively.

Table 5.39.2-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_7-20-32-38 | 20 | 0.3 |

Table 5.39.2-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_7-20-32-38 | 7 | 0 |
|  | 20 | 0 |
|  | 32 | 0 |
|  | 38 | 0.2 |

### 5.39.3	REFSENS requirements

No additional MSD required compared to fallbacks.

## 5.40	CA_8A-20A-32A-38A

### 5.40.1	Channel bandwidths per operating band for CA

Table 5.40.1-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_8A-20A-32A-38A | - | 8 |  |  | Yes | Yes |  |  | 70 | 0 |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 32 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.40.2	∆TIB and ∆RIB values

For CA_8-20-32-38, the TIB,c and RIB,c values are shown in table 5.40.2-1 and table 5.40.2-2, respectively.

Table 5.40.2-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_8-20-32-38 | 8 | 0.4 |
|  | 20 | 0.4 |
|  | 38 | 0.3 |

Table 5.40.2-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_8-20-32-38 | 8 | 0 |
|  | 20 | 0 |
|  | 32 | 0 |
|  | 38 | 0 |

### 5.40.3	REFSENS requirements

No additional MSD required compared to fallbacks.

## 5.41	CA_1-3-28-32

### 5.41.1	Channel bandwidths per operating band for CA

Table 5.41.1-1: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A-28A-32A | - | 1 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 32 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.41.2	∆TIB and ∆RIB values

Table 5.41.2-1: DTIB,c

| CA_1-3-28-32 | 1 | 0.3 |
| --- | --- | --- |
|  | 3 | 0.3 |
|  | 28 | 0.7 |

Table 5.41.2-2: DR IB,c

| CA_1-3-28-32 | 1 | 0 |
| --- | --- | --- |
|  | 3 | 0 |
|  | 28 | 0.2 |
|  | 32 | 0 |

### 5.41.3 	REFSENS requirements

There is no MSD requirements for CA_1A-3A-28A-32A.

# 6	5 Band Carrier Aggregation with Single UL: Specific Band Combination Part

## 6.1	CA_1-3-7-8-40

### 6.1.1	Channel bandwidths per operating band for CA

Table 6.1.1-1: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A-7A-8A-40A CA_1A-3A-7A-8A-40C | - | 1 |  |  | Yes | Yes | Yes | Yes | 90 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 40 |  |  | Yes | Yes | Yes | Yes |  |  |

### 6.1.2	∆TIB and ∆RIB values

Table 6.1.2-1: IB,c

| CA_1-3-7-8-40 | 1 | 0.6 |
| --- | --- | --- |
|  | 3 | 0.6 |
|  | 7 | 0.8 |
|  | 8 | 0.3 |
|  | 40 | 0.9 |

Table 6.1.2-2: R IB,c

| CA_1-3-7-8-40 | 1 | 0 |
| --- | --- | --- |
|  | 3 | 0 |
|  | 7 | 0.3 |
|  | 8 | 0 |
|  | 40 | 0.8 |

### 6.1.3 	REFSENS requirements

Table 6.1.3-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to cross band isolation issues of TDD and FDD bands)

| EUTRA CA Configuration | EUTRA band | Channel bandwidth |  |  |  |  |  | Duplex mode | Applicable active UL band |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |  |  |
| CA_1A-3A-7A-8A-40A 15 CA_1A-3A-7A-8A-40C | 319 |  |  | -94 | -91.5 | -90 | -89 | FDD | 1 |
|  | 4019 |  |  | -92.6 | -90.5 | -89.2 | -88.1 | TDD |  |
| CA_1A-3A-7A-8A-40A 16 CA_1A-3A-7A-8A-40C | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 1 |
|  | 4019 |  |  | -92.6 | -90.5 | -89.2 | -88.1 | TDD |  |
| CA_1A-3A-7A-8A-40A CA_1A-3A-7A-8A-40C | 4019 |  |  | -94.6 | -92.1 | -90.5 | -89.4 | TDD | 3 |
| CA_1A-3A-7A-8A-40A CA_1A-3A-7A-8A-40C | 4019 |  |  | -96 | -93.3 | -91.7 | -90.6 | TDD | 7 |
| CA_1A-3A-7A-8A-40A CA_1A-3A-7A-8A-40C | 119 |  |  | -91.7 | -89.5 | -87.9 | -86.9 | FDD | 40 |
|  | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 |  |  |
|  | 719 |  |  |  | -94 | -92.4 | -91.2 |  |  |
| NOTE 15: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth other than Band 1, the requirement applies regardless of channel bandwidth in Band 1 NOTE 16: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 19: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |  |

Table 6.1.3-2: Uplink configuration for reference sensitivity (exceptions due to cross band isolation issues of TDD and FDD bands)

| E-UTRA Band / Channel bandwidth of the affected DL band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| CA_1A-3A-7A-8A-40A CA_1A-3A-7A-8A-40C | 11,3 |  |  | 25 | 25 | 25 | 25 | FDD |
|  | 11,4 |  |  | 25 | 45 | 45 | 45 | FDD |
|  | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1). NOTE 3: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. NOTE 4: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. |  |  |  |  |  |  |  |  |

Table 6.1.3-3: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions for four bands due to close proximity of UL to DL channel)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A-7A-8A-40A CA_1A-3A-7A-8A-40C | 34,9 |  |  | -94 | -91.5 | -90 | -89 | FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: The signal power is specified per port NOTE 4: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in the bands other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 5: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in the bands other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 6: Void NOTE 7: The Band 41 requirements are modified by -0.5dB when carrier frequency of the assigned E-UTRA channel bandwidth is within 2545-2690MHz. NOTE 8: The Band 41 requirements also apply to the supported CA_1A-41A. NOTE 9: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

Table 6.1.3-4: Uplink configuration for the low band (exceptions for four bands due to close proximity of UL to DL channel)

| E-UTRA Band / Channel bandwidth of the affected DL band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-3A-7A-8A-40A CA_1A-3A-7A-8A-40C | 11,2 |  |  | 25 | 25 | 25 | 25 | FDD |
|  | 11,3 |  |  | 25 | 45 | 45 | 45 |  |
| NOTE 1: refers to the UL resource blocks shall be located as close as possible to the downlink channel in Band 3 but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1) in the uplink channel in Band 1. NOTE 2: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz NOTE 3: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. |  |  |  |  |  |  |  |  |

Table 6.1.3-5: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issues in the combinations of intra-band and inter-band CA)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A-7A-8A-40A4 CA_1A-3A-7A-8A-40C4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_1A-3A-7A-8A-40A 4,5,6 CA_1A-3A-7A-8A-40C 4,5,6 | 733 |  |  | -88 | -87.4 | -87 | -86.7 | FDD |
| NOTE 4: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity for all active downlink component carriers is only verified when this is not the case (the requirements specified in clause 7.3.1 apply unless otherwise specified). NOTE 5: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 6: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 33: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

Table 6.1.3-6: Uplink configuration for the low band (exceptions due to harmonic issues in the combinations of intra-band and inter-band CA)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-3A-7A-8A-40A CA_1A-3A-7A-8A-40C | 8 |  |  | 8 | 16 | 25 | 25 | FDD |

## 6.2	CA_1A-3A-7A-8A-28A

### 6.2.1	Channel bandwidths per operating band for CA

Table 6.2.1-1: Supported channel bandwidths per CA configuration for 5DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_1A-3A-7A-8A-28A | - | 1 |  |  | Yes | Yes | Yes | Yes | 90 | 0 |
|  |  | 3 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 8 | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |

### 6.2.2	∆TIB and ∆RIB values

For CA_1A-3A-7A-8A-28A, the TIB,c and RIB,c values are shown in table 6.2.2-1 and table 6.2.2-2, respectively.

Table 6.2.2-1: ΔTIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1A-3A-7A-8A-28A | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 8 | 0.6 |
|  | 28 | 0.6 |

Table 6.2.2-2: ΔRIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1A-3A-7A-8A-28A | 1 | 0 |
|  | 3 | 0 |
|  | 7 | 0 |
|  | 8 | 0.2 |
|  | 28 | 0.2 |

### 6.2.3	REFSENS requirements

Table 6.2.3-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A-7A-8A-28A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_1A-3A-7A-8A-28A5,6 | 133 |  |  | -89.8 | -89.4 | -89 | -88.7 |  |
|  | 733 |  |  | -88 | -87.4 | -87 | -86.7 |  |
| NOTE 4: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity for all active downlink component carriers is only verified when this is not the case (the requirements specified in clause 7.3.1 apply unless otherwise specified). NOTE 5: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 6: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 33: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |  |

Table 6.2.3-2: Uplink configuration for the low band (exceptions due to harmonic issue)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-3A-7A-8A-28A | 8 |  |  | 8 | 16 | 25 | 25 | FDD |
|  | 28 |  |  |  | 16 | 25 | 25 |  |

Table 6.2.3-3: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions for five bands due to close proximity of UL to DL channel)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A-7A-8A-28A | 34,9 |  |  | -94 | -91.5 | -90 | -89 | FDD |
|  | 35 |  |  | -97 | -94 | -92.2 | -91 |  |
| NOTE 4: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in the bands other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 5: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in the bands other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 9: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

Table 6.2.3-4: Uplink configuration for the low band (exceptions for five bands due to close proximity of UL to DL channel)

| E-UTRA Band / Channel bandwidth of the affected DL band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-3A-7A-8A-28A | 11,2 |  |  | 25 | 25 | 25 | 25 | FDD |
|  | 11,3 |  |  | 25 | 45 | 45 | 45 |  |
| NOTE 1: refers to the UL resource blocks shall be located as close as possible to the downlink channel in Band 3 but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1) in the uplink channel in Band 1. NOTE 2: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz NOTE 3: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. |  |  |  |  |  |  |  |  |

## 6.3	CA_1A-3A-8A-20A-28A

### 6.3.1	Channel bandwidths per operating band for CA

Table 6.3.1-1: Supported channel bandwidths per CA configuration for 5DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_1A-3A-8A-20A-28A | - | 1 |  |  | Yes | Yes | Yes | Yes | 90 | 0 |
|  |  | 3 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | 8 | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |

### 6.3.2	∆TIB and ∆RIB values

For CA_1A-3A-8A-20A-28A, the TIB,c and RIB,c values are shown in table 6.3.2-1 and table 6.3.2-2, respectively.

Table 6.3.2-1: ΔTIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1A-3A-8A-20A-28A | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 8 | 0.6 |
|  | 20 | 0.6 |
|  | 28 | 0.6 |

Table 6.3.2-2: ΔRIB,c for 6DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1A-3A-8A-20A-28A | 1 | 0 |
|  | 3 | 0 |
|  | 8 | 0.2 |
|  | 20 | 0.2 |
|  | 28 | 0.2 |

### 6.3.3	REFSENS requirements

Table 6.3.3-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A-8A-20A-28A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_1A-3A-8A-20A-28A5,6 | 133 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
| NOTE 4: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity for all active downlink component carriers is only verified when this is not the case (the requirements specified in clause 7.3.1 apply unless otherwise specified). NOTE 5: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 6: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 33: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |  |

Table 6.3.3-2: Uplink configuration for the low band (exceptions due to harmonic issue)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-3A-8A-20A-28A | 28 |  |  | 8 | 16 | 25 | 25 | FDD |

Table 6.3.3-3: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions for five bands due to close proximity of UL to DL channel)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A-8A-20A-28A7,8 | 34,9 |  |  | -94 | -91.5 | -90 | -89 | FDD |
|  | 35 |  |  | -97 | -94 | -92.2 | -91 |  |
| NOTE 4: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in the bands other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 5: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in the bands other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 7: The Band 41 requirements are modified by -0.5dB when carrier frequency of the assigned E-UTRA channel bandwidth is within 2545-2690MHz. NOTE 8: The Band 41 requirements also apply to the supported CA_1A-41A. NOTE 9: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

Table 6.3.3-4: Uplink configuration for the low band (exceptions for five bands due to close proximity of UL to DL channel)

| E-UTRA Band / Channel bandwidth of the affected DL band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-3A-8A-20A-28A | 11,2 |  |  | 25 | 25 | 25 | 25 | FDD |
|  | 11,3 |  |  | 25 | 45 | 45 | 45 |  |
| NOTE 1: refers to the UL resource blocks shall be located as close as possible to the downlink channel in Band 3 but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1) in the uplink channel in Band 1. NOTE 2: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz NOTE 3: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. |  |  |  |  |  |  |  |  |

## 6.4	CA_1A-7A-8A-20A-28A

### 6.4.1	Channel bandwidths per operating band for CA

Table 6.4.1-1: Supported channel bandwidths per CA configuration for 5DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_1A-7A-8A-20A-28A | - | 1 |  |  | Yes | Yes | Yes | Yes | 90 | 0 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 8 | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |

### 6.4.2	∆TIB and ∆RIB values

For CA_1A-7A-8A-20A-28A, the TIB,c and RIB,c values are shown in table 6.4.2-1 and table 6.4.2-2, respectively.

Table 6.4.2-1: ΔTIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1A-7A-8A-20A-28A | 1 | 0.5 |
|  | 7 | 0.6 |
|  | 8 | 0.6 |
|  | 20 | 0.6 |
|  | 28 | 0.6 |

Table 6.4.2-2: ΔRIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1A-7A-8A-20A-28A | 1 | 0 |
|  | 7 | 0 |
|  | 8 | 0.2 |
|  | 20 | 0.2 |
|  | 28 | 0.2 |

### 6.4.3	REFSENS requirements

Table 6.4.3-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-7A-8A-20A-28A5,6 | 133 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
|  | 733 |  |  | -88 | -87.4 | -87 | -86.7 |  |
| NOTE 5: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 6: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 33: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |  |

Table 6.4.3-2: Uplink configuration for the low band (exceptions due to harmonic issue)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-7A-8A-20A-28A | 8 |  |  | 8 | 16 |  |  | FDD |
|  | 28 |  |  | 8 | 16 | 25 | 25 |  |

## 6.5	CA_1-7-8-20-32

### 6.5.1	Channel bandwidths per operating band for CA

Table 6.5.1-1: Supported channel bandwidths per CA configuration for 5DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_1A-7A-8A-20A-32A | - | 1 |  |  | Yes | Yes | Yes | Yes | 90 | 0 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 8 | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 32 |  |  | Yes | Yes | Yes | Yes |  |  |

### 6.5.2	∆TIB and ∆RIB values

For CA_1A-7A-8A-20A-32A, the TIB,c and RIB,c values are shown in table 6.5.2-1 and table 6.5.2-2, respectively.

Table 6.5.2-1: ΔTIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1A-7A-8A-20A-32A | 1 | 0.7 |
|  | 7 | 0.7 |
|  | 8 | 0.6 |
|  | 20 | 0.6 |
|  | 32 | N/A |

Table 6.5.2-2: ΔRIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1A-7A-8A-20A-32A | 1 | 0 |
|  | 7 | 0 |
|  | 8 | 0.2 |
|  | 20 | 0.2 |
|  | 32 | 0 |

### 6.5.3	REFSENS requirements

Table 6.5.3-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-7A-8A-20A-32A5,6 | 733 |  |  |  | -87.4 | -87 | -86.7 | FDD |
| CA_1A-7A-8A-20A-32A | 133 |  |  | -89.8 | -89.4 | -89 | -88.7 |  |
| NOTE 5: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 6: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 33: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |  |

Table 6.5.3-2: Uplink configuration for the low band (exceptions due to harmonic issue)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-7A-8A-20A-32A | 8 |  |  | 8 | 16 |  |  | FDD |

Table 6.5.3-3: Reference sensitivity QPSK PREFSENS (CA with a SDL band)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-7A-8A-20A-32A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 7 |  |  | -98 | -95 | -93.2 | -92 |  |
|  | 20 |  |  | -97 | -94 | -91.2 | -90 |  |
|  | 32 |  |  | -100 | -97 | -95.2 | -94 |  |

## 6.6	CA_1-7-8-28-32

### 6.6.1	Channel bandwidths per operating band for CA

Table 6.6.1-1: Supported channel bandwidths per CA configuration for 5DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_1A-7A-8A-28A-32A | - | 1 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 8 | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 28 |  | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | 32 |  |  | Yes | Yes | Yes | Yes |  |  |

### 6.6.2	∆TIB and ∆RIB values

For CA_1A-7A-8A-28A-32A, the TIB,c and RIB,c values are shown in table 6.6.2-1 and table 6.6.2-2, respectively.

Table 6.6.2-1: ΔTIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1A-7A-8A-28A-32A | 1 | 0.5 |
|  | 7 | 0.7 |
|  | 8 | 0.6 |
|  | 28 | 0.3 |
|  |  | 0.514 |
|  | 32 | N/A |
| NOTE 14: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 8. |  |  |

Table 6.6.2-2: ΔRIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1A-7A-8A-28A-32A | 1 | 0 |
|  | 7 | 0 |
|  | 8 | 0.2 |
|  | 28 | 0 |
|  |  | 0.113 |
|  | 32 | 0 |
| NOTE 13: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 8. |  |  |

### 6.6.3	REFSENS requirements

Table 6.6.3-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-7A-8A-28A-32A | 133 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
|  | 75,6,33 |  |  | -88 | -87.4 | -87 | -86.7 |  |
|  | 329,10 |  |  | -72.2 | -72.2 | -72.2 | -72.2 |  |
|  | 3211 |  |  | -97.6 | -95.2 | -93.7 | -93.0 |  |
| NOTE 5: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 6: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 9: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_3A-42A, CA_3A-3A-42A, CA_3A-42A-42A, CA_1A-3A-20A-32A-42A, CA_3A-42A-43A, CA_3A-32A-42A-43A, CA_1A-3A-42A, CA_2A-13A-48A-66A, CA_2A-48A, CA_2A-48C, CA_2A-48D, CA_48A-66A, CA_3A-7A-42A, CA_3A-19A-42A, CA_3A-20A-42A, CA_3A-28A-42A, CA_1A-3A-7A-42A, CA_5A-48A-66A, CA_5A-48A-66A-66A, CA_13A-48A-66A, CA_13A-48A-66A-66A, CA_13A-48A-66B, CA_13A-48A-66C, CA_13A-48A-48A-66A, CA_13A-48C-66A, CA_13A-48D-66A, CA_13A-48A-48C-66A, CA_28A-32A, CA_48A-66A-66A, CA_48A-66B , CA_48A-66C, CA_48A-48A-66A, CA_48C-66A, CA_48A-48A-66A-66A, CA_48A-48A-66B, CA_48A-48A-66C, CA_48C-66B, CA_48C-66C, CA_48E-66A, CA_1A-3A-18A-42A, CA_1A-3A-19A-42A, CA_1A-3A-32A-42A, CA_1A-3A-41A-42A, CA_3A-7A-20A-42A, CA_3A-20A-32A-42A, CA_3A-28A-41A-42A, CA_3A-18A-42A, CA_3A-18A-42C, CA_3A-8A-42A and CA_3A-8A-42C. FHD = 0MHz for CA_11A-28A, CA_1A-11A-28A and CA_3A-11A-28A. NOTE 10: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 11: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 33: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |  |

Table 6.6.3-2: Uplink configuration for the low band (exceptions due to harmonic issue)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-7A-8A-28A-32A | 8 |  |  | 8 | 16 | 25 | 25 | FDD |
|  | 28 |  |  | 12 | 25 | 36 | 50 |  |

Table 6.6.3-3: Reference sensitivity QPSK PREFSENS (CA with a SDL band)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-7A-8A-28A-32A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 7 |  |  |  | -95 | -93.2 | -92 |  |
|  | 32 |  |  | -100 | -97 | -95.2 | -94 |  |

## 6.7	CA_1-7-20-28-32

### 6.7.1	Channel bandwidths per operating band for CA

Table 6.7.1-1: Supported channel bandwidths per CA configuration for 5DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_1A-7A-20A-28A-32A | - | 1 |  |  | Yes | Yes | Yes | Yes | 100 | 0 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 28 |  | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | 32 |  |  | Yes | Yes | Yes | Yes |  |  |

### 6.7.2	∆TIB and ∆RIB values

For CA_1A-7A-20A-28A-32A, the TIB,c and RIB,c values are shown in table 6.7.2-1 and table 6.7.2-2, respectively.

Table 6.7.2-1: ΔTIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1A-7A-20A-28A-32A | 1 | 0.7 |
|  | 7 | 0.7 |
|  | 20 | 0.6 |
|  | 28 | 0.6 |
|  | 32 | N/A |

Table 6.7.2-2: ΔRIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1A-7A-20A-28A-32A | 1 | 0 |
|  | 7 | 0 |
|  | 20 | 0.2 |
|  | 28 | 0.2 |
|  | 32 | 0 |

### 6.7.3	REFSENS requirements

Table 6.7.3-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-7A-20A-28A-32A9,10 | 32 |  |  | -72.2 | -72.2 | -72.2 | -72.2 | FDD |
|  | 133 |  |  | -89.8 | -89.4 | -89 | -88.7 |  |
| CA_1A-7A-20A-28A-32A11 | 32 |  |  | -97.6 | -95.2 | -93.7 | -93.0 |  |
|  | 133 |  |  | -89.8 | -89.4 | -89 | -88.7 |  |
| NOTE 9: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_3A-42A, CA_3A-3A-42A, CA_3A-42A-42A, CA_1A-3A-20A-32A-42A, CA_3A-42A-43A, CA_3A-32A-42A-43A, CA_1A-3A-42A, CA_2A-13A-48A-66A, CA_2A-48A, CA_2A-48C, CA_2A-48D, CA_48A-66A, CA_3A-7A-42A, CA_3A-19A-42A, CA_3A-20A-42A, CA_3A-28A-42A, CA_1A-3A-7A-42A, CA_5A-48A-66A, CA_5A-48A-66A-66A, CA_13A-48A-66A, CA_13A-48A-66A-66A, CA_13A-48A-66B, CA_13A-48A-66C, CA_13A-48A-48A-66A, CA_13A-48C-66A, CA_13A-48D-66A, CA_13A-48A-48C-66A, CA_28A-32A, CA_48A-66A-66A, CA_48A-66B , CA_48A-66C, CA_48A-48A-66A, CA_48C-66A, CA_48A-48A-66A-66A, CA_48A-48A-66B, CA_48A-48A-66C, CA_48C-66B, CA_48C-66C, CA_48E-66A, CA_1A-3A-18A-42A, CA_1A-3A-19A-42A, CA_1A-3A-32A-42A, CA_1A-3A-41A-42A, CA_3A-7A-20A-42A, CA_3A-20A-32A-42A, CA_3A-28A-41A-42A, CA_3A-18A-42A, CA_3A-18A-42C, CA_3A-8A-42A and CA_3A-8A-42C. FHD = 0MHz for CA_11A-28A, CA_1A-11A-28A and CA_3A-11A-28A. NOTE 10: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 11: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 33: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |  |

Table 6.7.3-2: Uplink configuration for the low band (exceptions due to harmonic issue)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-7A-20A-28A-32A | 28 |  |  | 12 | 25 | 36 | 50 | FDD |

Table 6.7.3-3: Reference sensitivity QPSK PREFSENS (CA with a SDL band)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-7A-20A-28A-32A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 7 |  |  | -98 | -95 | -93.2 | -92 |  |
|  | 20 |  |  | -97 | -94 | -91.2 | -90 |  |
|  | 32 |  |  | -100 | -97 | -95.2 | -94 |  |

## 6.8	CA_7-8-20-28-32

### 6.8.1	Channel bandwidths per operating band for CA

Table 6.8.1-1: Supported channel bandwidths per CA configuration for 5DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_7A-8A-20A-28A-32A | - | 7 |  |  | Yes | Yes | Yes | Yes | 90 | 0 |
|  |  | 8 | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 28 |  | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | 32 |  |  | Yes | Yes | Yes | Yes |  |  |

### 6.8.2	∆TIB and ∆RIB values

For CA_7A-8A-20A-28A-32A, the TIB,c and RIB,c values are shown in table 6.8.2-1 and table 6.8.2-2, respectively.

Table 6.8.2-1: ΔTIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_7A-8A-20A-28A-32A | 7 | 0.7 |
|  | 8 | 0.6 |
|  | 20 | 0.6 |
|  | 28 | 0.5 |
|  | 32 | N/A |

Table 6.8.2-2: ΔRIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_7A-8A-20A-28A-32A | 7 | 0 |
|  | 8 | 0 |
|  | 20 | 0.2 |
|  | 28 | 0.2 |
|  | 32 | 0 |

### 6.8.3	REFSENS requirements

Table 6.8.3-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_7A-8A-20A-28A-32A9,10 | 32 |  |  | -72.2 | -72.2 | -72.2 | -72.2 | FDD |
| CA_7A-8A-20A-28A-32A11 | 32 |  |  | -97.6 | -95.2 | -93.7 | -93.0 |  |
| CA_7A-8A-20A-28A-32A5,6 | 733 |  |  |  | -87.4 | -87 | -86.7 |  |
| NOTE 5: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 6: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 9: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_3A-42A, CA_3A-3A-42A, CA_3A-42A-42A, CA_1A-3A-20A-32A-42A, CA_3A-42A-43A, CA_3A-32A-42A-43A, CA_1A-3A-42A, CA_2A-13A-48A-66A, CA_2A-48A, CA_2A-48C, CA_2A-48D, CA_48A-66A, CA_3A-7A-42A, CA_3A-19A-42A, CA_3A-20A-42A, CA_3A-28A-42A, CA_1A-3A-7A-42A, CA_5A-48A-66A, CA_5A-48A-66A-66A, CA_13A-48A-66A, CA_13A-48A-66A-66A, CA_13A-48A-66B, CA_13A-48A-66C, CA_13A-48A-48A-66A, CA_13A-48C-66A, CA_13A-48D-66A, CA_13A-48A-48C-66A, CA_28A-32A, CA_48A-66A-66A, CA_48A-66B , CA_48A-66C, CA_48A-48A-66A, CA_48C-66A, CA_48A-48A-66A-66A, CA_48A-48A-66B, CA_48A-48A-66C, CA_48C-66B, CA_48C-66C, CA_48E-66A, CA_1A-3A-18A-42A, CA_1A-3A-19A-42A, CA_1A-3A-32A-42A, CA_1A-3A-41A-42A, CA_3A-7A-20A-42A, CA_3A-20A-32A-42A, CA_3A-28A-41A-42A, CA_3A-18A-42A, CA_3A-18A-42C, CA_3A-8A-42A and CA_3A-8A-42C. FHD = 0MHz for CA_11A-28A, CA_1A-11A-28A and CA_3A-11A-28A. NOTE 10: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 11: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 33: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |  |

Table 6.8.3-2: Uplink configuration for the low band (exceptions due to harmonic issue)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_7A-8A-20A-28A-32A | 28 |  |  | 12 | 25 | 36 | 50 | FDD |
|  | 8 |  |  |  | 16 | 25 | 25 |  |

Table 6.8.3-3: Reference sensitivity QPSK PREFSENS (CA with a SDL band)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_7A-8A-20A-28A-32A | 7 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 20 |  |  | -97 | -94 | -91.2 | -90 |  |
|  | 32 |  |  | -100 | -97 | -95.2 | -94 |  |

## 6.9	CA_1-3-8-20-38

### 6.9.1	Channel bandwidths per operating band for CA

Table 6.9.1-1: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A-8A-20A-38A | - | 1 |  |  | Yes | Yes | Yes | Yes | 90 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |

### 6.9.2	∆TIB and ∆RIB values

Table 6.9.2-1: IB,c

| CA_1-3-8-20-38 | 1 | 0.5 |
| --- | --- | --- |
|  | 3 | 0.5 |
|  | 8 | 0.4 |
|  | 20 | 0.4 |
|  | 38 | 0.5 |

Table 6.9.2-2: R IB,c

| CA_1-3-8-20-38 | 1 | 0 |
| --- | --- | --- |
|  | 3 | 0 |
|  | 8 | 0 |
|  | 20 | 0 |
|  | 38 | 0 |

### 6.9.3 	REFSENS requirements

Based on the approved WF R4-2016940, higher-order MSD test points are no longer needed since constituent lower-order fall-back MSD are agreed and specified.

## 6.10	CA_1-3-7-8-38

### 6.10.1	Channel bandwidths per operating band for CA

Table 6.10.1-1: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A-7A-8A-38AX | - | 1 |  |  | Yes | Yes | Yes | Yes | 90 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |
| NOTE X: UL carrier shall be supported in Band 1, 3 or 8 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within 6dB. |  |  |  |  |  |  |  |  |  |  |

### 6.10.2	∆TIB and ∆RIB values

Table 6.10.2-1: IB,c

| CA_1-3-7-8-38 | 1 | 0.6 |
| --- | --- | --- |
|  | 3 | 0.6 |
|  | 7 | 0 |
|  | 8 | 0.6 |
|  | 38 | 0 |

Table 6.10.2-2: R IB,c

| CA_1-3-7-8-38 | 1 | 0 |
| --- | --- | --- |
|  | 3 | 0 |
|  | 7 | 0 |
|  | 8 | 0 |
|  | 38 | 0 |

### 6.10.3 	REFSENS requirements

Based on the approved WF R4-2016940, higher-order MSD test points are no longer needed since constituent lower-order fall-back MSD are agreed and specified.

## 6.11	CA_3A-7A-8A-20A-28A

### 6.11.1	Channel bandwidths per operating band for CA

Table 6.11.1-1: Supported channel bandwidths per CA configuration for 5DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_3A-7A-8A-20A-28A | - | 3 | Yes | Yes | Yes | Yes | Yes | Yes | 90 | 0 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 8 | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |

### 6.11.2	∆TIB and ∆RIB values

For CA_3A-7A-8A-20A-28A, the TIB,c and RIB,c values are shown in table 6.11.2-1 and table 6.11.2-2, respectively.

Table 6.11.2-1: ΔTIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_3A-7A-8A-20A-28A | 3 | 0.5 |
|  | 7 | 0.5 |
|  | 8 | 0.6 |
|  | 20 | 0.6 |
|  | 28 | 0.5 |

Table 6.11.2-2: ΔRIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_3A-7A-8A-20A-28A | 3 | 0 |
|  | 7 | 0 |
|  | 8 | 0.2 |
|  | 20 | 0.2 |
|  | 28 | 0.1 |

### 6.11.3	REFSENS requirements

Table 6.11.3-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_3A-7A-8A-20A-28A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_3A-7A-8A-20A-28A5,6 | 733 |  |  | -88 | -87.4 | -87 | -86.7 |  |
| NOTE 4: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity for all active downlink component carriers is only verified when this is not the case (the requirements specified in clause 7.3.1 apply unless otherwise specified). NOTE 5: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 6: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 33: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |  |

Table 6.11.3-2: Uplink configuration for the low band (exceptions due to harmonic issue)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_3A-7A-8A-20A-28A | 8 |  |  | 8 | 16 | 25 | 25 | FDD |

## 6.12	CA_1A-3A-7A-20A-38A

### 6.12.1	Channel bandwidths per operating band for CA

Table 6.12.1-1: Supported channel bandwidths per CA configuration for 5DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_1A-3A-7A-20A-38A1 | - | 1 |  |  | Yes | Yes | Yes | Yes | 100 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |
| NOTE 1: UL carrier shall be supported in Band 1, 3 or 20 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within [6dB]. |  |  |  |  |  |  |  |  |  |  |

### 6.12.2	∆TIB and ∆RIB values

For CA_1-3-7-20-38, the TIB,c and RIB,c values are shown in table 6.12.2-1 and table 6.12.2-2, respectively.

Table 6.12.2-1: ΔTIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1-3-7-20-38 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 20 | 0.3 |

Table 6.12.2-2: ΔRIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1-3-7-20-38 | 1 | 0 |
|  | 3 | 0 |
|  | 7 | 0 |
|  | 20 | 0 |
|  | 38 | 0 |

### 6.12.3	REFSENS requirements

No additional MSD required compared to fallbacks.

## 6.13	CA_1A-3A-7A-28A-38A

### 6.13.1	Channel bandwidths per operating band for CA

Table 6.13.1-1: Supported channel bandwidths per CA configuration for 5DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_1A-3A-7A-28A-38A9 | - | 1 |  |  | Yes | Yes | Yes | Yes | 100 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |
| NOTE 9: UL carrier shall be supported in Band 1, 3, 8 or 28 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within [6dB]. |  |  |  |  |  |  |  |  |  |  |

### 6.13.2	∆TIB and ∆RIB values

For CA_1-3-7-28-38, the TIB,c and RIB,c values are shown in table 6.13.2-1 and table 6.13.2-2, respectively.

Table 6.13.2-1: ΔTIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1-3-7-28-38 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 28 | 0.6 |

Table 6.13.2-2: ΔRIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1-3-7-28-38 | 1 | 0 |
|  | 3 | 0 |
|  | 7 | 0 |
|  | 28 | 0.2 |
|  | 38 | 0 |

### 6.13.3	REFSENS requirements

No additional MSD required compared to fallbacks.

## 6.14	CA_1A-3A-20A-28A-38A

### 6.14.1	Channel bandwidths per operating band for CA

Table 6.14.1-1: Supported channel bandwidths per CA configuration for 5DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_1A-3A-20A-28A-38A7 | - | 1 |  |  | Yes | Yes | Yes | Yes | 100 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 20 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |
| NOTE 7: Power imbalance between downlink carriers on Band 20 and Band 28 is assumed to be within [6dB]. |  |  |  |  |  |  |  |  |  |  |

### 6.14.2	∆TIB and ∆RIB values

For CA_1-3-20-28-38, the TIB,c and RIB,c values are shown in table 6.14.2-1 and table 6.14.2-2, respectively.

Table 6.14.2-1: ΔTIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1-3-20-28-38 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 20 | 0.6 |
|  | 28 | 0.6 |
|  | 38 | 0.3 |

Table 6.14.2-2: ΔRIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1-3-20-28-38 | 1 | 0 |
|  | 3 | 0 |
|  | 20 | 0.2 |
|  | 28 | 0.2 |
|  | 38 | 0 |

### 6.14.3	REFSENS requirements

No additional MSD required compared to fallbacks.

## 6.15	CA_1A-7A-8A-20A-38A

### 6.15.1	Channel bandwidths per operating band for CA

Table 6.15.1-1: Supported channel bandwidths per CA configuration for 5DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_1A-7A-8A-20A-38A1 | - | 1 |  |  | Yes | Yes | Yes | Yes | 90 | 0 |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 20 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |
| NOTE 1: UL carrier shall be supported in Bands 1, 8 and 20 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within [6dB] |  |  |  |  |  |  |  |  |  |  |

### 6.15.2	∆TIB and ∆RIB values

For CA_1-7-8-20-38, the TIB,c and RIB,c values are shown in table 6.15.2-1 and table 6.15.2-2, respectively.

Table 6.15.2-1: ΔTIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1-7-8-20-38 | 1 | 0.5 |
|  | 8 | 0.6 |
|  | 20 | 0.6 |

Table 6.15.2-2: ΔRIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1-7-8-20-38 | 1 | 0 |
|  | 7 | 0 |
|  | 8 | 0.2 |
|  | 20 | 0.2 |
|  | 38 | 0.2 |

### 6.15.3	REFSENS requirements

No additional MSD required compared to fallbacks.

## 6.16	CA_1A-7A-8A-32A-38A

### 6.16.1	Channel bandwidths per operating band for CA

Table 6.16.1-1: Supported channel bandwidths per CA configuration for 5DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_1A-7A-8A-32A-38A10 | - | 1 |  |  | Yes | Yes | Yes | Yes | 90 | 0 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 8 | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 32 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |
| NOTE 10: UL carrier shall be supported in Band 1 or 8 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within [6dB]. |  |  |  |  |  |  |  |  |  |  |

### 6.16.2	∆TIB and ∆RIB values

For CA_1-7-8-32-38, the TIB,c and RIB,c values are shown in table 6.16.2-1 and table 6.16.2-2, respectively.

Table 6.16.2-1: ΔTIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1-7-8-32-38 | 1 | 0.7 |
|  | 8 | 0.6 |

Table 6.16.2-2: ΔRIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1-7-8-32-38 | 1 | 0 |
|  | 7 | 0 |
|  | 8 | 0.2 |
|  | 32 | 0 |
|  | 38 | 0.2 |

### 6.16.3	REFSENS requirements

No additional MSD required compared to fallbacks.

## 6.17	CA_1A-7A-20A-28A-38A

### 6.17.1	Channel bandwidths per operating band for CA

Table 6.17.1-1: Supported channel bandwidths per CA configuration for 5DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_1A-7A-20A-28A-38A1,7 | - | 1 |  |  | Yes | Yes | Yes | Yes | 100 | 0 |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 20 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |
| NOTE 1: UL carrier shall be supported in Bands 1, 20 and 28 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within [6dB] NOTE 7: Power imbalance between downlink carriers on Band 20 and Band 28 is assumed to be within [6dB]. |  |  |  |  |  |  |  |  |  |  |

### 6.17.2	∆TIB and ∆RIB values

For CA_1-7-20-28-38, the TIB,c and RIB,c values are shown in table 6.17.2-1 and table 6.17.2-2, respectively.

Table 6.17.2-1: ΔTIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1-7-20-28-38 | 1 | 0.5 |
|  | 20 | 0.6 |
|  | 28 | 0.6 |

Table 6.17.2-2: ΔRIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1-7-20-28-38 | 1 | 0 |
|  | 7 | 0 |
|  | 20 | 0.2 |
|  | 28 | 0.2 |
|  | 38 | 0.2 |

### 6.17.3	REFSENS requirements

No additional MSD required compared to fallbacks.

## 6.18	CA_1A-7A-20A-32A-38A

### 6.18.1	Channel bandwidths per operating band for CA

Table 6.18.1-1: Supported channel bandwidths per CA configuration for 5DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_1A-7A-20A-32A-38A1 | - | 1 |  |  | Yes | Yes | Yes | Yes | 90 | 0 |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 20 |  |  | Yes | Yes |  |  |  |  |
|  |  | 32 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |
| NOTE 1: UL carrier shall be supported in Band 1 or 20 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within [6dB]. |  |  |  |  |  |  |  |  |  |  |

### 6.18.2	∆TIB and ∆RIB values

For CA_1-7-20-32-38, the TIB,c and RIB,c values are shown in table 6.18.2-1 and table 6.18.2-2, respectively.

Table 6.18.2-1: ΔTIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1-7-20-32-38 | 1 | 0.7 |
|  | 20 | 0.3 |

Table 6.18.2-2: ΔRIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1-7-20-32-38 | 1 | 0 |
|  | 7 | 0 |
|  | 20 | 0 |
|  | 32 | 0 |
|  | 38 | 0 |

### 6.18.3	REFSENS requirements

No additional MSD required compared to fallbacks.

## 6.19	CA_1A-8A-20A-32A-38A

### 6.19.1	Channel bandwidths per operating band for CA

Table 6.19.1-1: Supported channel bandwidths per CA configuration for 5DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_1A-8A-20A-32A-38A | - | 1 |  |  | Yes | Yes | Yes | Yes | 90 | 0 |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 32 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |

### 6.19.2	∆TIB and ∆RIB values

For CA_1-8-20-32-38, the TIB,c and RIB,c values are shown in table 6.19.2-1 and table 6.19.2-2, respectively.

Table 6.19.2-1: ΔTIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1-8-20-32-38 | 1 | 0.5 |
|  | 8 | 0.6 |
|  | 20 | 0.5 |
|  | 38 | 0.5 |

Table 6.19.2-2: ΔRIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1-8-20-32-38 | 1 | 0 |
|  | 8 | 0 |
|  | 20 | 0 |
|  | 32 | 0 |
|  | 38 | 0 |

### 6.19.3	REFSENS requirements

No additional MSD required compared to fallbacks.

## 6.20	CA_3A-7A-8A-20A-38A

### 6.20.1	Channel bandwidths per operating band for CA

Table 6.20.1-1: Supported channel bandwidths per CA configuration for 5DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_3A-7A-8A-20A-38A1 | - | 3 |  |  | Yes | Yes | Yes | Yes | 90 | 0 |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |
| NOTE 1: UL carrier shall be supported in Bands 3, 8 and 20 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within [6dB] |  |  |  |  |  |  |  |  |  |  |

### 6.20.2	∆TIB and ∆RIB values

For CA_3-7-8-20-38, the TIB,c and RIB,c values are shown in table 6.20.2-1 and table 6.20.2-2, respectively.

Table 6.20.2-1: ΔTIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_3-7-8-20-38 | 3 | 0.5 |
|  | 8 | 0.6 |
|  | 20 | 0.5 |

Table 6.20.2-2: ΔRIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_3-7-8-20-38 | 3 | 0 |
|  | 7 | 0 |
|  | 8 | 0 |
|  | 20 | 0 |
|  | 38 | 0.2 |

### 6.20.3	REFSENS requirements

No additional MSD required compared to fallbacks.

## 6.21	CA_3A-7A-20A-28A-38A

### 6.21.1	Channel bandwidths per operating band for CA

Table 6.21.1-1: Supported channel bandwidths per CA configuration for 5DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_3A-7A-20A-28A-38A1,7 | - | 3 |  |  | Yes | Yes | Yes | Yes | 100 | 0 |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 20 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |
| NOTE 1: UL carrier shall be supported in Bands 3, 20 and 28 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within [6dB] NOTE 7: Power imbalance between downlink carriers on Band 20 and Band 28 is assumed to be within [6dB]. |  |  |  |  |  |  |  |  |  |  |

### 6.21.2	∆TIB and ∆RIB values

For CA_3-7-20-28-38, the TIB,c and RIB,c values are shown in table 6.21.2-1 and table 6.21.2-2, respectively.

Table 6.21.2-1: ΔTIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_3-7-20-28-38 | 3 | 0.5 |
|  | 20 | 0.6 |
|  | 28 | 0.5 |

Table 6.21.2-2: ΔRIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_3-7-20-28-38 | 3 | 0 |
|  | 7 | 0 |
|  | 20 | 0.2 |
|  | 28 | 0.1 |
|  | 38 | 0.2 |

### 6.21.3	REFSENS requirements

No additional MSD required compared to fallbacks.

## 6.22	CA_7A-8A-20A-32A-38A

### 6.22.1	Channel bandwidths per operating band for CA

Table 6.22.1-1: Supported channel bandwidths per CA configuration for 5DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_7A-8A-20A-32A-38A10 | - | 7 |  |  | Yes | Yes | Yes | Yes | 90 | 0 |
|  |  | 8 | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 32 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |
| NOTE 10: UL carrier shall be supported in Band 8 or 20 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within [6dB]. |  |  |  |  |  |  |  |  |  |  |

### 6.22.2	∆TIB and ∆RIB values

For CA_7-8-20-32-38, the TIB,c and RIB,c values are shown in table 6.22.2-1 and table 6.22.2-2, respectively.

Table 6.22.2-1: ΔTIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
|  | 8 | 0.6 |
|  | 20 | 0.6 |

Table 6.22.2-2: ΔRIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_7-8-20-32-38 | 7 | 0 |
|  | 8 | 0.2 |
|  | 20 | 0.2 |
|  | 32 | 0 |
|  | 38 | 0 |

### 6.22.3	REFSENS requirements

No additional MSD required compared to fallbacks.

# 7	6 Band Carrier Aggregation with Single UL: Specific Band Combination Part

## 7.1	CA_1A-3A-7A-8A-20A-28A

### 7.1.1	Channel bandwidths per operating band for CA

Table 7.1.1-1: Supported channel bandwidths per CA configuration for 6DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_1A-3A-7A-8A-20A-28A | - | 1 |  |  | Yes | Yes | Yes | Yes | 110 | 0 |
|  |  | 3 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 8 | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |

### 7.1.2	∆TIB and ∆RIB values

For CA_1A-3A-7A-8A-20A-28A, the TIB,c and RIB,c values are shown in table 7.1.2-1 and table 7.1.2-2, respectively.

Table 7.1.2-1: ΔTIB,c for 6DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1A-3A-7A-8A-20A-28A | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 8 | 0.6 |
|  | 20 | 0.6 |
|  | 28 | 0.6 |

Table 7.1.2-2: ΔRIB,c for 6DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1A-3A-7A-8A-20A-28A | 1 | 0 |
|  | 3 | 0 |
|  | 7 | 0 |
|  | 8 | 0.2 |
|  | 20 | 0.2 |
|  | 28 | 0.2 |

### 7.1.3	REFSENS requirements

Table 7.1.3-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A-7A-8A-20A-28A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_1A-3A-7A-8A-20A-28A5,6 | 133 |  |  | -89.8 | -89.4 | -89 | -88.7 |  |
|  | 733 |  |  | -88 | -87.4 | -87 | -86.7 |  |
| NOTE 4: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity for all active downlink component carriers is only verified when this is not the case (the requirements specified in clause 7.3.1 apply unless otherwise specified). NOTE 5: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 6: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 33: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |  |

Table 7.1.3-2: Uplink configuration for the low band (exceptions due to harmonic issue)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-3A-7A-8A-20A-28A | 8 |  |  | 8 | 16 | 25 | 25 | FDD |
|  | 28 |  |  | 8 | 16 | 25 | 25 |  |

Table 7.1.3-3: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions for six bands due to close proximity of UL to DL channel)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A-7A-8A-20A-28A | 34,7 |  |  | -94 | -91.5 | -90 | -89 | FDD |
|  | 35 |  |  | -97 | -94 | -92.2 | -91 |  |
| NOTE 4: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in the bands other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 5: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in the bands other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 7: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

Table 7.1.3-4: Uplink configuration for the low band (exceptions for six bands due to close proximity of UL to DL channel)

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-3A-7A-8A-20A-28A | 11,2 |  |  | 25 | 25 | 25 | 25 | FDD |
|  | 11,3 |  |  | 25 | 45 | 45 | 45 |  |
| NOTE 1: refers to the UL resource blocks shall be located as close as possible to the downlink channel in Band 3 but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1) in the uplink channel in Band 1. NOTE 2: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. NOTE 3: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. |  |  |  |  |  |  |  |  |

## 7.2	CA_1-7-8-20-28-32

### 7.2.1	Channel bandwidths per operating band for CA

Table 7.2.1-1: Supported channel bandwidths per CA configuration for 6DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_1A-7A-8A-20A-28A-32A | - | 1 |  |  | Yes | Yes | Yes | Yes | 110 | 0 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 8 | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 28 |  | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | 32 |  |  | Yes | Yes | Yes | Yes |  |  |

### 7.2.2	∆TIB and ∆RIB values

For CA_1A-7A-8A-20A-28A-32A, the TIB,c and RIB,c values are shown in table 7.2.2-1 and table 7.2.2-2, respectively.

Table 7.2.2-1: ΔTIB,c for 6DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1A-7A-8A-20A-28A-32A | 1 | 0.7 |
|  | 7 | 0.7 |
|  | 8 | 0.6 |
|  | 20 | 0.6 |
|  | 28 | 0.6 |
|  | 32 | N/A |

Table 7.2.2-2: ΔRIB,c for 6DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1A-7A-8A-20A-28A-32A | 1 | 0 |
|  | 7 | 0 |
|  | 8 | 0 |
|  | 20 | 0.2 |
|  | 28 | 0.2 |
|  | 32 | 0 |

### 7.2.3	REFSENS requirements

Table 7.2.3-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-7A-8A-20A-28A-32A | 329,10 |  |  | -72.2 | -72.2 | -72.2 | -72.2 | FDD |
|  | 3211 |  |  | -97.6 | -95.2 | -93.7 | -93.0 |  |
|  | 75,6,33 |  |  |  | -87.4 | -87 | -86.7 |  |
|  | 133 |  |  | -89.8 | -89.4 | -89 | -88.7 |  |
| NOTE 5: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 6: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 9: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_3A-42A, CA_3A-3A-42A, CA_3A-42A-42A, CA_1A-3A-20A-32A-42A, CA_3A-42A-43A, CA_3A-32A-42A-43A, CA_1A-3A-42A, CA_2A-13A-48A-66A, CA_2A-48A, CA_2A-48C, CA_2A-48D, CA_48A-66A, CA_3A-7A-42A, CA_3A-19A-42A, CA_3A-20A-42A, CA_3A-28A-42A, CA_1A-3A-7A-42A, CA_5A-48A-66A, CA_5A-48A-66A-66A, CA_13A-48A-66A, CA_13A-48A-66A-66A, CA_13A-48A-66B, CA_13A-48A-66C, CA_13A-48A-48A-66A, CA_13A-48C-66A, CA_13A-48D-66A, CA_13A-48A-48C-66A, CA_28A-32A, CA_48A-66A-66A, CA_48A-66B , CA_48A-66C, CA_48A-48A-66A, CA_48C-66A, CA_48A-48A-66A-66A, CA_48A-48A-66B, CA_48A-48A-66C, CA_48C-66B, CA_48C-66C, CA_48E-66A, CA_1A-3A-18A-42A, CA_1A-3A-19A-42A, CA_1A-3A-32A-42A, CA_1A-3A-41A-42A, CA_3A-7A-20A-42A, CA_3A-20A-32A-42A, CA_3A-28A-41A-42A, CA_3A-18A-42A, CA_3A-18A-42C, CA_3A-8A-42A and CA_3A-8A-42C. FHD = 0MHz for CA_11A-28A, CA_1A-11A-28A and CA_3A-11A-28A. NOTE 10: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 11: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 33: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |  |

Table 7.2.3-2: Uplink configuration for the low band (exceptions due to harmonic issue)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-7A-8A-20A-28A-32A | 28 |  |  | 12 | 25 | 36 | 50 | FDD |
|  | 8 |  |  | 8 | 16 | 25 | 25 |  |

Table 7.2.3-3: Reference sensitivity QPSK PREFSENS (CA with a SDL band)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-7A-8A-20A-28A-32A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 7 |  |  | -98 | -95 | -93.2 | -92 |  |
|  | 20 |  |  | -97 | -94 | -91.2 | -90 |  |
|  | 32 |  |  | -100 | -97 | -95.2 | -94 |  |

## 7.3	CA_1A-3A-7A-20A-28A-38A

### 7.3.1	Channel bandwidths per operating band for CA

Table 7.3.1-1: Supported channel bandwidths per CA configuration for 6DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_1A-3A-7A-20A-28A-38A1,7 | - | 1 |  |  | Yes | Yes | Yes | Yes | 120 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 20 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |
| NOTE 1: UL carrier shall be supported in Bands 1, 3, 20 and 28 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within [6dB] NOTE 7: Power imbalance between downlink carriers on Band 20 and Band 28 is assumed to be within [6dB]. |  |  |  |  |  |  |  |  |  |  |

### 7.3.2	∆TIB and ∆RIB values

For CA_1-3-7-20-28-38, the TIB,c and RIB,c values are shown in table 7.3.2-1 and table 7.3.2-2, respectively.

Table 7.3.2-1: ΔTIB,c for 6DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1-3-7-20-28-38 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 20 | 0.6 |
|  | 28 | 0.6 |

Table 7.3.2-2: ΔRIB,c for 6DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1-3-7-20-28-38 | 1 | 0 |
|  | 3 | 0 |
|  | 7 | 0 |
|  | 20 | 0.2 |
|  | 28 | 0.2 |
|  | 38 | 0 |

### 7.3.3	REFSENS requirements

No additional MSD required compared to fallbacks.

## 7.4	CA_1A-3A-7C-20A-28A-38A

### 7.4.1	Channel bandwidths per operating band for CA

Table 7.4.1-1: Supported channel bandwidths per CA configuration for 6DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_1A-3A-7C-20A-28A-38A1,7 | - | 1 |  |  | Yes | Yes | Yes | Yes | 140 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 7 | See CA_7C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 20 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |
| NOTE 1: UL carrier shall be supported in Bands 1, 3, 20 and 28 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within [6dB] NOTE 7: Power imbalance between downlink carriers on Band 20 and Band 28 is assumed to be within [6dB]. |  |  |  |  |  |  |  |  |  |  |

### 7.4.2	∆TIB and ∆RIB values

For CA_1-3-7-20-28-38, the TIB,c and RIB,c values are shown in table 7.4.2-1 and table 7.4.2-2, respectively.

Table 7.4.2-1: ΔTIB,c for 6DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1-3-7-20-28-38 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 20 | 0.6 |
|  | 28 | 0.6 |

Table 7.4.2-2: ΔRIB,c for 6DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1-3-7-20-28-38 | 1 | 0 |
|  | 3 | 0 |
|  | 7 | 0 |
|  | 20 | 0.2 |
|  | 28 | 0.2 |
|  | 38 | 0 |

### 7.4.3	REFSENS requirements

No additional MSD required compared to fallbacks.

## 7.5	CA_1A-7A-8A-20A-32A-38A

### 7.5.1	Channel bandwidths per operating band for CA

Table 7.5.1-1: Supported channel bandwidths per CA configuration for 6DL inter-band CA

| E-UTRA CA Configuration | UL CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_1A-7A-8A-20A-32A-38A1 | - | 1 |  |  | Yes | Yes | Yes | Yes | 100 | 0 |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 20 |  |  |  | Yes |  |  |  |  |
|  |  | 32 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |
| NOTE 1: UL carrier shall be supported in Band 1, 8 or 20 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within [6dB]. |  |  |  |  |  |  |  |  |  |  |

### 7.5.2	∆TIB and ∆RIB values

For CA_1-7-8-20-32-38, the TIB,c and RIB,c values are shown in table 7.5.2-1 and table 7.5.2-2, respectively.

Table 7.5.2-1: ΔTIB,c for 6DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1-7-8-20-32-38 | 1 | 0.7 |
|  | 8 | 0.6 |
|  | 20 | 0.6 |

Table 7.5.2-2: ΔRIB,c for 6DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1-7-8-20-32-38 | 1 | 0 |
|  | 7 | 0 |
|  | 8 | 0.2 |
|  | 20 | 0.2 |
|  | 32 | 0 |
|  | 38 | 0 |

### 7.5.3	REFSENS requirements

No additional MSD required compared to fallbacks.

# Annex A: Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2020-08 | 3GPP RAN4#96e |  |  |  |  | Initial TR skeleton | 0.0.1 |
| 2020-08 | 3GPP RAN4#96e |  |  |  |  | The following agreed text proposals have been included: R4-2011405; Updated scope of TR: LTE inter-band CA for 4/5 bands DL with 1 band UL; Nokia, Nokia Shanghai Bell R4-2011582 TP for TR 36.717-04-01: CA_1-3-20-38; Vodafone R4-2010875 TP for TR 36.717-04-01: CA_2A-7A-28A-66A / CA_2A-7C-28A-66A; Huawei R4-2010876 TP for TR 36.717-04-01: CA_2A-5A-7A-66A / CA_2A-5A-7C-66A; Huawei | 0.1.0 |
| 2020-11 | 3GPP RAN4#97e |  |  |  |  | The following agreed text proposals have been included: R4-2016182; Updated scope of TR: LTE inter-band CA for 4/5 bands DL with 1 band UL; Nokia, Nokia Shanghai Bell R4-2016767 TP for TR 36.717-04-01: CA_1-3-8-41; Vodafone R4-2016770 TP for TR 36.717-04-01: CA_1A-7A-8A-38A; Huawei R4-2016771 TP for TR 36.717-04-01: CA_1A-8A-20A-38A; Huawei R4-2016772 TP for TR 36.717-04-01: CA_3A-8A-20A-38A; Huawei R4-2016773 TP for TR 36.717-04-01: CA_1A-3C-8A-38A with UL CA_3C; Huawei R4-2016774 TP for TR 36.717-04-01: CA_1A-3C-8A-20A with UL CA_3C; Huawei R4-2016775 TP for TR 36.717-04-01: CA_1A-3C-20A-38A with UL CA_3C; Huawei R4-2015402 Updated TP for TR 36.717-04-01: CA_2A-5A-7A-66A-66A; Huawei R4-2016776 TP for TR 36.717-04-01: CA_1A-3A-7A-8A-40A / CA_1A-3A-7A-8A-40C; Huawei | 0.2.0 |
| 2021-02 | 3GPP RAN4#98e |  |  |  |  | The following agreed text proposals have been included: R4-2102439; Updated scope of TR: LTE inter-band CA for 4/5 bands DL with 1 band UL; Nokia, Nokia Shanghai Bell R4-2101406 TP for TR 36.717-04-01: CA_1A-3A-7C-20A with UL CA_7C; Vodafone R4-2101468 TP for TR 36.717-04-01: CA_1-3-40-41; Vodafone R4-2101469 TP for TR 36.717-04-01: CA_1-7-8-28; Vodafone R4-2101470 TP for TR 36.717-04-01: CA_1-7-8-32; Vodafone R4-2101471 TP for TR 36.717-04-01: CA_1-7-28-32; Vodafone R4-2101472 TP for TR 36.717-04-01: CA_1-8-20-32; Vodafone R4-2101473 TP for TR 36.717-04-01: CA_1-8-28-32; Vodafone R4-2101474 TP for TR 36.717-04-01: CA_1-20-28-32; Vodafone R4-2101475 TP for TR 36.717-04-01: CA_3-7-8-28; Vodafone R4-2101476 TP for TR 36.717-04-01: CA_3-8-40-41; Vodafone R4-2101477 TP for TR 36.717-04-01: CA_7-8-20-28; Vodafone R4-2101478 TP for TR 36.717-04-01: CA_7-8-20-32; Vodafone R4-2101479 TP for TR 36.717-04-01: CA_7-8-28-32; Vodafone R4-2101480 TP for TR 36.717-04-01: CA_7-20-28-32; Vodafone R4-2101481 TP for TR 36.717-04-01: CA_8-20-28-32; Vodafone R4-2102624; TP to TR 36.717-04-01 Correction of CA_2-5-7-66-66; Nokia, Nokia Shanghai Bell R4-2101488 TP for TR 36.717-04-01: CA_1-3-7-8-28; Vodafone R4-2101489 TP for TR 36.717-04-01: CA_1-3-8-20-28; Vodafone R4-2101490 TP for TR 36.717-04-01: CA_1-7-8-20-28; Vodafone R4-2101491 TP for TR 36.717-04-01: CA_1-7-8-20-32; Vodafone R4-2101492 TP for TR 36.717-04-01: CA_1-7-8-28-32; Vodafone R4-2101493 TP for TR 36.717-04-01: CA_1-7-20-28-32; Vodafone R4-2101524 TP for TR 36.717-04-01: CA_7-8-20-28-32; Vodafone R4-2101582 TP for TR 36.717-04-01: CA_1-3-8-20-38; Huawei, HiSilicon R4-2101583 TP for TR 36.717-04-01: CA_1-3-7-8-38; Huawei, HiSilicon | 0.3.0 |
| 2021-04 | 3GPP RAN4#98-bis-e |  |  |  |  | The following agreed text proposals have been included: R4-2101522 TP for TR 36.717-04-01: CA_3-7-8-20-28; Vodafone R4-2101525 TP for TR 36.717-04-01: CA_1-3-7-8-20-28; Vodafone R4-2101544 TP for TR 36.717-04-01: CA_1-7-8-20-28-32; Vodafone | 0.4.0 |
| 2021-06 | RAN4#99-e | R4-2111282 |  |  |  | Updated to include all TPs | 0.5.0 |
| 2021-08 | RAN4#100-e |  |  |  |  | The following agreed text proposals have been included: R4-2114364; Updated scope of TR: LTE inter-band CA for 4/5 bands DL with 1 band UL; Nokia, Nokia Shanghai Bell R4-2114758 TP for TR 36.717-04-01: CA_1A-7A-20A-38A; ZTE R4-2114765 TP for TR 36.717-04-01: CA_1A-3A-3A-8A-38A; Vodafone R4-2114766 TP for TR 36.717-04-01: CA_1A-3A-28A-38A; Vodafone R4-2114767 TP for TR 36.717-04-01: CA_1A-7A-20A-38A; Vodafone R4-2114768 TP for TR 36.717-04-01: CA_1A-7A-28A-38A; Vodafone R4-2114769 TP for TR 36.717-04-01: CA_1A-7A-32A-38A; Vodafone R4-2114770 TP for TR 36.717-04-01: CA_1A-8A-32A-38A; Vodafone R4-2114771 TP for TR 36.717-04-01: CA_1A-20A-28A-38A; Vodafone R4-2114772 TP for TR 36.717-04-01: CA_1A-20A-32A-38A; Vodafone R4-2114773 TP for TR 36.717-04-01: CA_3A-7A-20A-38A; Vodafone R4-2114774 TP for TR 36.717-04-01: CA_3A-20A-28A-38A; Vodafone R4-2114775 TP for TR 36.717-04-01: CA_7A-8A-20A-38A; Vodafone R4-2114776 TP for TR 36.717-04-01: CA_7A-8A-32A-38A; Vodafone R4-2114777 TP for TR 36.717-04-01: CA_7A-20A-28A-38A; Vodafone R4-2114778 TP for TR 36.717-04-01: CA_7A-20A-32A-38A; Vodafone R4-2114779 TP for TR 36.717-04-01: CA_8A-20A-32A-38A; Vodafone R4-2114780 TP for TR 36.717-04-01: CA_1A-3A-7A-20A-38A; Vodafone R4-2114781 TP for TR 36.717-04-01: CA_1A-3A-7A-28A-38A; Vodafone R4-2114782 TP for TR 36.717-04-01: CA_1A-3A-20A-28A-38A; Vodafone R4-2114783 TP for TR 36.717-04-01: CA_1A-7A-8A-20A-38A; Vodafone R4-2114784 TP for TR 36.717-04-01: CA_1A-7A-8A-32A-38A; Vodafone R4-2114785 TP for TR 36.717-04-01: CA_1A-7A-20A-28A-38A; Vodafone R4-2114786 TP for TR 36.717-04-01: CA_1A-7A-20A-32A-38A; Vodafone R4-2114787 TP for TR 36.717-04-01: CA_1A-8A-20A-32A-38A; Vodafone R4-2114788 TP for TR 36.717-04-01: CA_3A-7A-8A-20A-38A; Vodafone R4-2114789 TP for TR 36.717-04-01: CA_3A-7A-20A-28A-38A; Vodafone R4-2114790 TP for TR 36.717-04-01: CA_7A-8A-20A-32A-38A; Vodafone R4-2114791 TP for TR 36.717-04-01: CA_1A-3A-7A-20A-28A-38A; Vodafone R4-2114792 TP for TR 36.717-04-01: CA_1A-3A-7C-20A-28A-38A; Vodafone R4-2114793 TP for TR 36.717-04-01: CA_1A-7A-8A-20A-32A-38A; Vodafone |  |
| 2021-11 | RAN4#101-e |  |  |  |  | The following agreed text proposals have been included: R4-2118560; TP for TR 36.717-04-01: CA_1A-3A-28A-32A; Huawei |  |