3GPP TSG-RAN4 Meeting #101-e	R4-2119340

# Electronic Meeting, 1 – 12 November, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to CGI reading requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_enh-Core |  |  |  |  |  | Date: |  |  | 2021-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In CGI reading requirements, the time for SIB1 reading is defined based on periodicity with which the SIB1 is actually transmitted by the NR target cell. For SSB and RMSI CORESET multiplexing pattern 2 and 3, this periodicity can be same as SSB periodicity, e.g. 20ms. However, UE does not know the SSB periodicity of the tartget cells before obtaining the SIB1 of the target cell, so for CGI reading it can only attempt to decode SIB1 of target cell based on SMTC periodicity, which may be larger than SSB periodicity, e.g. 40ms. This means the current delay requirements cannot be met by a correct UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that for SSB and RMSI CORESET multiplexing pattern 2 and 3, TRMSI-scheduling is the maximum between the actual scheduled periodicity and the SMTC periodicity. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CGI reading delay requirements cannot be met by a correct UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.11.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

9.11.2	CGI identification of an NR cell with autonomous gaps

The UE shall identify and report the CGI of a known NR target cell when requested by the network for the purpose of reportCGI. Only one cell is provided to the UE with cellForWhichToReportCGI for identifying the CGI.The UE may make autonomous gaps in both downlink reception and uplink transmission for receiving MIB and SIB1 message according to clause 5.5.3 of TS 38.331 [2]. Note that a UE is not required to use autonomous gap if useAutonomousGaps is set to false. If autonomous gaps are used for measurement with the purpose of reportCGI, regardless of whether DRX is used or not, or whether SCell(s) are configured or not, the UE shall be able to identify a new CGI of NR cell within:

Tidentify_CGI = (TMIB + T SIB1) ms

Where:

TMIB is the time period used to acquire MIB message. TMIB = 6 * TSMTC ms for target cell carrier frequency on FR1 and TMIB = 25 * TSMTC ms for target cell carrier frequency on FR2.

TSIB1 is the time period used to acquire SIB1 message. TSIB1 = 6 * TRMSI-scheduling ms.

Where TSMTC is the SMTC periodicity configured for the target cell measurement, and TRMSI-scheduling is

- the periodicity with which the SIB1 is actually transmitted by the NR target cell, for SSB and RMSI CORESET multiplexing pattern 1, and

- the maximum between the periodicity with which the SIB1 is actually transmitted by the NR target cell and TSMTC, for SSB and RMSI CORESET multiplexing pattern 2 and 3.

The requirement for identifying the CGI of an NR cell within Tidentify_CGI is applicable when no DRX is used as well as when any of the DRX cycles specified in TS 38.331 [2] is used.

Within the time Tidentify_CGI, over which the UE identifies the CGI of an NR cell, the UE shall fulfil interruption requirements specified in,

-	Clause 8.2.1.2.16 for NR serving cells and Clause 7.32.2.15 in TS36.133 [15] for E-UTRA serving cells if the UE is configured with EN-DC operation mode,

-	Clause 8.2.2.2.14 if the UE is configured with SA operation mode,

-	Clause 8.2.3.2.14 for NR serving cells and Clause 7.36.2.14 in TS36.133 [15] for E-UTRA serving cells if the UE is configured with NE-DC operation mode,

-	Clause 8.2.4.2.11 if the UE is configured with NR-DC operation mode.

In the requirement a cell is known if,

-	During the last 5 seconds for FR1 or 3 seconds for FR2 before the reception of the report CGI command:

-	The UE has sent a valid L3-RSRP measurement report with SSB index for the target cell and

-	During MIB decoding at least reported SSBs remains detectable according to the cell identification conditions specified in clauses 9.2 or 9.3 of TS 38.133, and

-	During SIB1 decoding the SSB used for MIB decoding remains detectable according to the cell identification conditions specified in clauses 9.2 or 9.3 of TS 38.133, and

-	During MIB decoding, the SSB for MIB decoding remains detectable with SNR ≥-3dB

-	During SIB1 decoding, the PDSCH for SIB1 decoding remains detectable with SNR ≥-3dB

<End of Change 1>