3GPP TSG-RAN WG4 Meeting #101-e	R4-2119362

Electronic Meeting, 1 – 12 November, 2021

Title: 	On reference point for Te requirements

Source: 	Huawei, HiSilicon

Agenda item:	8.23.2.2

Document for:	Approval

Introduction

RRM requirements for Rel-17 URLLC/IIOT were discussed in RAN4#100-e, and the outcomes are captured in the WF [1]. In particular, one topic that was extensively discussed but not directly related to the WI is the definition of the reference point for Te requirements. Based on [1] we understand the following open issues are to be further discussed:

- Detected time v.s. arrival time

- ‘Received’ v.s. ‘arrives’

- Side condition of Te requirements

In this paper, we will provide our views on the definition of the reference point in Te requirements.

Discussion

Detected time v.s. arrival time

The outcome of the 2nd discussion was not conclusive as such and distribution was as follows: - Option 1: 3 companies (vivo, Apple, Nokia,) - Option 3: 8 companies (CMCC, Ericsson, Intel, Qualcomm, ZTE, OPPO, Nokia, Huawei) There are some companies who have concern changing the existing wording and some prefer to keep ‘detected’. A number of companies can support not to mention ‘detected’. WF1: Continue the discussion whether to use ‘detected’ or not use ‘detected’ in the 38.133.

In RAN1#104-e, an LS was sent to RAN4 [2] seeking for clarification on the UE transmit timing error (i.e. Te) defined in RAN4 spec 38.133. In RAN4#98-bis-e, a reply LS [3] was sent informing that the DL detection error is already included in Te, but during the discussion, companies had different understandings on the “reference point” in 38.133.

In [2] RAN1 listed two options of different interpretations of the “reference point” as below:

- Option 1: downlink frame timing detection error is already included in UE transmit timing error (i.e. Te); - Interpretation: “the reference point” defined in section 7.1.2 in TS 38.133 for UE transmission timing is ahead of “True arrival timing at UE”, and the timing error limit value Te is given taking downlink frame timing detection error into account, as shown in figure 1 as an example. - Option 2: downlink frame timing detection error is not included in UE transmit timing error (i.e. Te); - Interpretation: “the reference point” defined in section 7.1.2 in TS 38.133 for UE transmission time is ahead of the first path detected by the UE, and the timing error limit value Te is given without consideration of downlink frame timing detection error, as shown in figure 2 as an example. Note that “arrival time detected by UE” in Figure 1 and Figure 2 refers to the first detected path (in time) of the corresponding downlink frame from the reference cell, as described in Section 7.1 of TS 38.133 V17.0.0. Figure 1 Figure 2

The difference between the two options is whether the reference point is based on the arrival timing or the detected timing. In our view, it should be the arrival timing i.e. option 1.

First, we should clarify whether the reference point in clause 7.1.2 of 38.133 is defined for the ideal UL timing, or the actual UL timing.

The UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available at the UE during the last 160 ms. The reference point for the UE initial transmit timing control requirement shall be the downlink timing of the reference cell minus .

It can be seen that the reference point is defined for the UE initial transmit timing control requirement, or in other words, for the Te requirements. In our understanding, it should be for the ideal UL timing.

For comparison, the determination of actual UL timing is described in clause 7.1.1, and it can be seen that the actual UL timing is based on the reception of the first detected path.

The uplink frame transmission takes place before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell.

It is clear that the reference point for the ideal UL timing and the actual UL timing should be different. If the ideal UL timing is also based on the first detected path, then the DL timing detection error is is not included in Te, as shown in Figure 2 in [1].

Besides, during RAN4#98-bis-e discussion, it was also confirmed by TE vendors that in RRM testing, the actual transmitted UL timing is compared against the transmit timing of the TE. It is noted that in timing accuracy tests AWGN is used, and the propagation time from TE to the UE is negligible, so the ideal UL timing is effectively the arrival time of the first path. This is well aligned with option 1, and also with the intention of the Te requirements.

In RAN4#100-e discussion, some companies commented that the DL timing detection error may not be fully accounted in Te requirements. The argument was that DL timing detection error includes both the baseband estimation error of the first path and also the timing uncertainty due to limitation of the sampling interval, and Te only accounts for the latter one. While we agree with the observation but we think both were included in Te requirements.

It is noted that baseband estimation error can become large for propagation channels with large delay spread or with low Es/Iot which was already observed in the Rel-16 positioning simulation, and we understand that although the side conditions for Te requirements are not defined, the Te requirements only apply under certain conditions. As far as normal condition is concerned, we think all the DL timing detection error have been accounted in Te.

Observation 1: The “reference point” in clause 7.1.2 of 38.133 is for “ideal UL timing” instead of “actual UL timing”. If the “reference point” is defined based on “detected path”, then the DL timing detection error is not included in Te.

Observation 2: For testing, the “ideal UL timing” is derived based on transmit timing of TE, which is effectively the arrival time of the first path.

Observation 3: Both the baseband estimation error of the first path and also the timing uncertainty due to limitation of the sampling interval are accounted in Te requirements.

Proposal 1a: Do not mention ‘detected’ nor ‘detectable’ in the definition of the “reference point” for Te requirements in clause 7.1.2 of 38.133.

Proposal 1b: Add a note in the requirements that the requirements may not apply in all conditions but shall apply under conditions used in the test cases.

‘Received’ v.s. ‘arrives’

The outcome of the 2nd discussion was not fully conclusive: - Option 1: 1 company (vivo) - Option 2: 8 companies (Ericsson, Intel, Qualcomm, Apple, ZTE, OPPO, Nokia, Huawei) Based on the discussion outcome: Tentative agreement: - Use ‘arrives’ in definition

Among “arrives and “is received” we think it is more clear to use “when the first detectable path … arrives”, because “received” is still from UE perspective, and there could still be a difference between when a path arrives and when a path is received, and the latter may still be interpreted as “detected”.

Proposal 2: Use ‘arrives’ in the definition of the “reference point” for Te requirements in clause 7.1.2 of 38.133.

Side condition of Te reqirements

Outcome of the 2nd round discussion was not full consensus, as one company prefer option 1. No company prefer option 2: - Option 1: 1 company (MediaTek) - Option 2: 0 companies () - Option 3: 8 companies (CMCC; Ericsson, Intel, Qualcomm, vivo, Apple, OPPO, Huawei) Based on the email discussion outcome: Tentative agreement: - Option 3: Do not change current and rely on the test case side conditions. - note: No side conditions (Ês/Iot) will be defined in the core requirements Outcome of 2nd discussion: - Option 1: 8 companies (CMCC, Ericsson, Intel, Qualcomm, vivo, Apple, OPPO, Huawei) - Option 2: 1 company (MediaTek) Tentative agreement: - Option 1: RAN4 does not capture any UE timing error requirements side conditions in the core requirements (section 7.1).

We prefer to not define the side condition for Te requirements.

It may be difficult to specify the side condition, because besides the Es/Iot, the DL timing detection error also depends on the propagation model, e.g. the delay spread of the channel. It would be a lengthy discussion if RAN4 is to define exact side condition for detectable path. We also do not see clear necessity since we have not seen conformance issues from LTE or NR. It is sufficient that the side condition is defined in the test cases.

Proposal 3: RAN4 not to define side conditions for Te requirements in clause 7.1.2.

Text proposal

Based on above discussions, we think the current wording for the reference point in 38.133 is a bit confusing for people outside RAN4 because it is aligned with option 2 in [2], and it is the reason why RAN1 would send LS to clarify the interpretation. To void potential ambiguity, we suggest to update the wording in 38.133.

Proposal 4: Update the definition of the “reference point” in clause 7.1.2 of 38.133 from Rel-15:

“The downlink timing is defined as the time when the first path (in time) of the corresponding downlink frame from the reference cell arrives at the UE antenna.”

Once the updated wording for 38.133 is agreed, RAN4 should inform it to RAN1.

Proposal 5: Send LS to inform RAN1 about the updated definition of the “reference point”.

A draft LS to RAN1 is provided in the Annex.

Conclusions

In this paper we provided our views on the definition of the reference point in Te requirements.

Proposal 1a: Do not mention ‘detected’ nor ‘detectable’ in the definition of the “reference point” for Te requirements in clause 7.1.2 of 38.133.

Proposal 1b: Add a note in the requirements that the requirements may not apply in all conditions but shall apply under conditions used in the test cases.

Proposal 2: Use ‘arrives’ in the definition of the “reference point” for Te requirements in clause 7.1.2 of 38.133.

Proposal 3: RAN4 not to define side conditions for Te requirements in clause 7.1.2.

Proposal 4: Update the definition of the “reference point” in clause 7.1.2 of 38.133 from Rel-15:

“The downlink timing is defined as the time when the first path (in time) of the corresponding downlink frame from the reference cell arrives at the UE antenna.”

Proposal 5: Send LS to inform RAN1 about the updated definition of the “reference point”.

Reference

- R4-2115371, WF on NR_IIOT_URLLC_enh_RRM, Nokia, Nokia Shanghai Bell

- R1-2102245, LS on UE transmit timing error, RAN1

- R4-2105850, Reply LS on UE transmit timing error, RAN4

Annex: draft reply LS on UE transmit timing error

3GPP TSG-RAN WG4 Meeting #101-e R4-210xxxx Electronic Meeting, 1 – 12 November, 2021 Title: reply LS on UE transmit timing error Response to: R1-2102245 Release: Release 17 Work Item: NR_IIOT_URLLC_enh-Core Source: RAN4 To: RAN1 Cc: Contact Person: - Name: Li Zhang - E-mail Address: zhangli164@huawei.com Attachments: - 1. Overall Description: In R4-2105850, RAN4 indicated that RAN4 will further discuss the reference point definition in the future to clarify the term. Based on discussions in RAN4#99-e, RAN4 reached consensus to update the reference point definition in clause 7.1.2 of 38.133 as follows: “The downlink timing is defined as the time when the first path (in time) of the corresponding downlink frame from the reference cell arrives at the UE antenna.” 2. Actions: To RAN1: RAN4 respectfully asks RAN1 to take the above information into account. 3. Date of Next TSG-RAN4 Meetings: TBA