3GPP TSG-RAN WG4 Meeting # 101-e	R4-2119379

# Electronic Meeting, 1 – 12 November 2021

Agenda item:	8.1.3.3

Source:	Keysight Technologies, Spirent Communications, CMCC, CAICT

Title:	Beam Specific Reference Channel Emulation Curves for Validation Purposes for FR1 CDL-C UMa

Document for:	Approval

# Introduction

This contribution presents the FR1 CDL-C UMa beam specific reference channel emulation curves for validation purposes.

# Discussion

Three companies (Keysight, Spirent, CMCC) presented reference channel emulation curves in R4-2115759 [1] for FR1 MIMO OTA CDL-C UMa combined beams. This contribution replaces the UMa combined beam reference curves with beam specific curves for PDP and temporal autocorrelation (TCF).

The approach agreed by the contributors to determine the reference curves is as follows:

- Power Delay Profile (PDP): Take average of KS, Spirent, CMCC reference curves and round the power to 0.1 dB accuracy.

- Temporal Correlation Function (TCF): Take average of KS, Spirent, CMCC reference curves and keep 0.001 accuracy.

The averaged reference CDL-C UMa results for

- PDP are tabulated in Table 1, Table 3, Table 5 and Table 7 and shown in Figure 1, Figure 2, Figure 5, Figure 6, Figure 9, Figure 10, Figure 13 and Figure 14.

- Temporal autocorrelation are tabulated in Table 2, Table 4, Table 6 and Table 8 and shown in Figure 3, Figure 4, Figure 7, Figure 8, Figure 11, Figure 12, Figure 15 and Figure 16.

It is proposed to adopt the averaged CDL-C UMa curves as reference channel emulation curves for validation purposes.

Proposal 1: Adopt the averaged beam specific CDL-C UMa PDP and TCF curves as reference channel emulation curves for validation purposes.

Proposal 2: Omit the combined beam curves for PDP and TCF in R4-2115759 [1].

The annex presents the individual validation curves presented in [2-4] as well as the resulting averages.

# UMa beam 1 at fc ≤ 2.5 GHz

Table 1. Reference PDP table of CDL-C UMa beam 1 at ≤ 2.5 GHz

| Delay [ns] | Power [dB] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | X2V | H11 | H21 | H12 | H22 |
| 0.0 | -31.4 | -32.8 | -30.6 | -30.7 | -33.0 |
| 76.6 | -28.1 | -28.1 | -28.3 | -28.6 | -28.4 |
| 79.4 | -20.7 | -21.3 | -20.3 | -20.5 | -21.5 |
| 81.0 | -20.5 | -23.2 | -19.1 | -19.1 | -23.2 |
| 85.0 | -25.9 | -25.9 | -24.8 | -26.2 | -27.8 |
| 232.4 | 0.0 | -0.1 | 0.0 | -0.3 | -0.4 |
| 235.4 | -2.8 | -3.6 | -2.1 | -2.4 | -4.1 |
| 239.4 | -3.9 | -4.0 | -3.7 | -4.3 | -4.6 |
| 289.6 | -30.8 | -31.1 | -30.5 | -30.8 | -31.5 |
| 299.8 | -38.7 | -39.3 | -38.4 | -38.5 | -39.4 |
| 448.4 | -32.9 | -32.8 | -32.8 | -33.4 | -33.5 |
| 477.5 | -31.1 | -31.4 | -30.8 | -31.2 | -31.8 |

Figure 1. Reference PDP of CDL-C UMa beam 1 ≤ 2.5 GHz

Figure 2. Reference X2V PDP of CDL-C UMa beam 1 at ≤ 2.5 GHz

Table 2. Reference TCF table of CDL-C UMa beam 1 ≤ 2.5 GHz

| lambdas | X2V | H11 | H21 | H12 | H22 |
| --- | --- | --- | --- | --- | --- |
| 0.0 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 |
| 0.1 | 0.986 | 0.990 | 0.983 | 0.982 | 0.990 |
| 0.2 | 0.945 | 0.962 | 0.932 | 0.929 | 0.962 |
| 0.3 | 0.882 | 0.918 | 0.853 | 0.847 | 0.918 |
| 0.4 | 0.801 | 0.861 | 0.753 | 0.743 | 0.860 |
| 0.5 | 0.709 | 0.795 | 0.641 | 0.626 | 0.793 |
| 0.6 | 0.613 | 0.724 | 0.526 | 0.506 | 0.719 |
| 0.7 | 0.518 | 0.651 | 0.417 | 0.390 | 0.642 |
| 0.8 | 0.430 | 0.580 | 0.322 | 0.289 | 0.563 |
| 0.9 | 0.353 | 0.512 | 0.246 | 0.206 | 0.485 |
| 1.0 | 0.289 | 0.449 | 0.194 | 0.146 | 0.410 |
| 1.1 | 0.240 | 0.395 | 0.166 | 0.106 | 0.339 |
| 1.2 | 0.204 | 0.349 | 0.157 | 0.079 | 0.273 |
| 1.3 | 0.181 | 0.313 | 0.163 | 0.059 | 0.213 |
| 1.4 | 0.167 | 0.287 | 0.178 | 0.050 | 0.160 |
| 1.5 | 0.159 | 0.268 | 0.200 | 0.065 | 0.113 |
| 1.6 | 0.155 | 0.256 | 0.225 | 0.094 | 0.073 |
| 1.7 | 0.153 | 0.248 | 0.248 | 0.124 | 0.043 |
| 1.8 | 0.150 | 0.242 | 0.266 | 0.149 | 0.035 |
| 1.9 | 0.144 | 0.236 | 0.275 | 0.163 | 0.048 |
| 2.0 | 0.135 | 0.230 | 0.273 | 0.165 | 0.066 |
| 2.1 | 0.121 | 0.221 | 0.260 | 0.154 | 0.081 |
| 2.2 | 0.105 | 0.210 | 0.238 | 0.132 | 0.092 |
| 2.3 | 0.085 | 0.197 | 0.211 | 0.104 | 0.099 |
| 2.4 | 0.065 | 0.181 | 0.182 | 0.077 | 0.100 |
| 2.5 | 0.048 | 0.164 | 0.157 | 0.066 | 0.097 |
| 2.6 | 0.039 | 0.147 | 0.141 | 0.080 | 0.090 |
| 2.7 | 0.038 | 0.132 | 0.131 | 0.104 | 0.078 |
| 2.8 | 0.042 | 0.119 | 0.125 | 0.126 | 0.064 |
| 2.9 | 0.043 | 0.112 | 0.115 | 0.139 | 0.048 |
| 3.0 | 0.041 | 0.111 | 0.100 | 0.141 | 0.032 |
| 3.1 | 0.037 | 0.115 | 0.078 | 0.133 | 0.022 |
| 3.2 | 0.036 | 0.122 | 0.056 | 0.119 | 0.024 |
| 3.3 | 0.044 | 0.131 | 0.051 | 0.103 | 0.034 |
| 3.4 | 0.056 | 0.138 | 0.071 | 0.089 | 0.045 |
| 3.5 | 0.068 | 0.141 | 0.098 | 0.081 | 0.055 |
| 3.6 | 0.075 | 0.140 | 0.121 | 0.078 | 0.064 |
| 3.7 | 0.076 | 0.133 | 0.134 | 0.074 | 0.071 |
| 3.8 | 0.068 | 0.121 | 0.135 | 0.064 | 0.078 |
| 3.9 | 0.051 | 0.104 | 0.125 | 0.050 | 0.085 |
| 4.0 | 0.027 | 0.083 | 0.106 | 0.041 | 0.092 |
| 4.1 | 0.007 | 0.062 | 0.087 | 0.058 | 0.100 |
| 4.2 | 0.036 | 0.045 | 0.081 | 0.092 | 0.109 |
| 4.3 | 0.067 | 0.040 | 0.093 | 0.129 | 0.117 |
| 4.4 | 0.093 | 0.047 | 0.113 | 0.160 | 0.124 |
| 4.5 | 0.111 | 0.059 | 0.129 | 0.182 | 0.128 |
| 4.6 | 0.119 | 0.069 | 0.135 | 0.190 | 0.126 |
| 4.7 | 0.116 | 0.075 | 0.127 | 0.183 | 0.119 |
| 4.8 | 0.101 | 0.077 | 0.105 | 0.166 | 0.107 |
| 4.9 | 0.078 | 0.077 | 0.071 | 0.144 | 0.092 |
| 5.0 | 0.051 | 0.076 | 0.034 | 0.132 | 0.073 |

Figure 3. Reference TCF of CDL-C UMa beam 1 ≤ 2.5 GHz

Figure 4. Reference X2V TCF of CDL-C UMa beam 1 at ≤ 2.5 GHz

# UMa beam 2 at fc ≤ 2.5 GHz

Table 3. Reference PDP table of CDL-C UMa beam 2 at ≤ 2.5 GHz

| Delay [ns] | Power [dB] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | X2V | H11 | H21 | H12 | H22 |
| 0.0 | -24.8 | -26.6 | -23.9 | -23.9 | -26.7 |
| 76.6 | 0.0 | -0.1 | 0.0 | -0.4 | -0.5 |
| 79.4 | -11.3 | -12.2 | -10.9 | -11.0 | -12.4 |
| 81.0 | -2.7 | -3.7 | -2.0 | -2.2 | -4.0 |
| 85.0 | -3.8 | -3.8 | -3.5 | -4.2 | -4.5 |
| 232.4 | -23.2 | -23.3 | -24.3 | -23.5 | -22.7 |
| 235.4 | -16.9 | -17.1 | -15.5 | -17.1 | -19.7 |
| 239.4 | -23.8 | -24.0 | -22.7 | -24.1 | -25.9 |
| 289.6 | -25.0 | -25.5 | -24.7 | -25.0 | -25.8 |
| 299.8 | -37.4 | -38.2 | -37.1 | -37.2 | -38.2 |
| 448.4 | -24.8 | -24.9 | -24.7 | -25.2 | -25.4 |
| 477.5 | -24.9 | -25.3 | -24.6 | -24.9 | -25.6 |
| 1679.1 | -38.1 | -38.2 | -38.0 | -38.4 | -38.7 |

Figure 5. Reference PDP of CDL-C UMa beam 2 ≤ 2.5 GHz

Figure 6. Reference X2V PDP of CDL-C UMa beam 2 at ≤ 2.5 GHz

Table 4. Reference TCF table of CDL-C UMa beam 2 at ≤ 2.5 GHz

| lambdas | X2V | H11 | H21 | H12 | H22 |
| --- | --- | --- | --- | --- | --- |
| 0.0 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 |
| 0.1 | 0.974 | 0.978 | 0.970 | 0.969 | 0.978 |
| 0.2 | 0.907 | 0.924 | 0.894 | 0.891 | 0.923 |
| 0.3 | 0.832 | 0.868 | 0.805 | 0.799 | 0.865 |
| 0.4 | 0.776 | 0.830 | 0.735 | 0.724 | 0.826 |
| 0.5 | 0.738 | 0.810 | 0.685 | 0.670 | 0.805 |
| 0.6 | 0.695 | 0.783 | 0.630 | 0.610 | 0.775 |
| 0.7 | 0.623 | 0.730 | 0.549 | 0.521 | 0.716 |
| 0.8 | 0.525 | 0.653 | 0.442 | 0.402 | 0.629 |
| 0.9 | 0.426 | 0.576 | 0.341 | 0.282 | 0.537 |
| 1.0 | 0.360 | 0.523 | 0.285 | 0.208 | 0.467 |
| 1.1 | 0.335 | 0.497 | 0.277 | 0.198 | 0.428 |
| 1.2 | 0.320 | 0.475 | 0.275 | 0.201 | 0.398 |
| 1.3 | 0.287 | 0.435 | 0.251 | 0.180 | 0.349 |
| 1.4 | 0.233 | 0.373 | 0.205 | 0.140 | 0.277 |
| 1.5 | 0.176 | 0.304 | 0.159 | 0.114 | 0.199 |
| 1.6 | 0.141 | 0.249 | 0.132 | 0.118 | 0.149 |
| 1.7 | 0.135 | 0.222 | 0.131 | 0.127 | 0.144 |
| 1.8 | 0.137 | 0.213 | 0.147 | 0.118 | 0.147 |
| 1.9 | 0.132 | 0.204 | 0.172 | 0.092 | 0.130 |
| 2.0 | 0.117 | 0.186 | 0.196 | 0.062 | 0.096 |
| 2.1 | 0.097 | 0.159 | 0.209 | 0.039 | 0.079 |
| 2.2 | 0.076 | 0.130 | 0.209 | 0.023 | 0.103 |
| 2.3 | 0.062 | 0.113 | 0.206 | 0.014 | 0.132 |
| 2.4 | 0.071 | 0.117 | 0.213 | 0.034 | 0.141 |
| 2.5 | 0.090 | 0.133 | 0.230 | 0.054 | 0.125 |
| 2.6 | 0.099 | 0.144 | 0.238 | 0.060 | 0.098 |
| 2.7 | 0.088 | 0.137 | 0.222 | 0.043 | 0.086 |
| 2.8 | 0.058 | 0.112 | 0.183 | 0.014 | 0.102 |
| 2.9 | 0.037 | 0.086 | 0.144 | 0.050 | 0.121 |
| 3.0 | 0.067 | 0.089 | 0.142 | 0.094 | 0.124 |
| 3.1 | 0.103 | 0.117 | 0.172 | 0.120 | 0.107 |
| 3.2 | 0.120 | 0.140 | 0.195 | 0.120 | 0.075 |
| 3.3 | 0.115 | 0.145 | 0.193 | 0.096 | 0.040 |
| 3.4 | 0.097 | 0.139 | 0.173 | 0.060 | 0.018 |
| 3.5 | 0.082 | 0.138 | 0.151 | 0.030 | 0.020 |
| 3.6 | 0.083 | 0.154 | 0.142 | 0.019 | 0.020 |
| 3.7 | 0.090 | 0.173 | 0.138 | 0.010 | 0.027 |
| 3.8 | 0.089 | 0.179 | 0.122 | 0.043 | 0.051 |
| 3.9 | 0.079 | 0.163 | 0.088 | 0.098 | 0.077 |
| 4.0 | 0.068 | 0.133 | 0.050 | 0.154 | 0.094 |
| 4.1 | 0.063 | 0.104 | 0.046 | 0.196 | 0.096 |
| 4.2 | 0.062 | 0.096 | 0.075 | 0.212 | 0.086 |
| 4.3 | 0.057 | 0.100 | 0.096 | 0.207 | 0.079 |
| 4.4 | 0.052 | 0.098 | 0.106 | 0.192 | 0.090 |
| 4.5 | 0.055 | 0.083 | 0.113 | 0.182 | 0.109 |
| 4.6 | 0.063 | 0.071 | 0.123 | 0.175 | 0.119 |
| 4.7 | 0.066 | 0.082 | 0.134 | 0.156 | 0.112 |
| 4.8 | 0.058 | 0.102 | 0.138 | 0.111 | 0.090 |
| 4.9 | 0.047 | 0.111 | 0.135 | 0.044 | 0.068 |
| 5.0 | 0.048 | 0.100 | 0.127 | 0.041 | 0.072 |

Figure 7. Reference TCF of CDL-C UMa beam 2 at ≤ 2.5 GHz

Figure 8. Reference X2V TCF of CDL-C UMa beam 2 at ≤ 2.5 GHz

# UMa beam 1 at fc > 2.5 GHz

Table 5. Reference PDP table of CDL-C UMa beam 1 at > 2.5 GHz

| Delay [ns] | Power [dB] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | X2V | H11 | H21 | H12 | H22 |
| 0.0 | -31.5 | -33.0 | -30.7 | -30.8 | -33.2 |
| 76.6 | -28.2 | -28.3 | -28.4 | -28.5 | -28.5 |
| 79.4 | -20.8 | -21.4 | -20.3 | -20.6 | -21.8 |
| 81.0 | -20.3 | -23.3 | -18.9 | -18.9 | -23.2 |
| 85.0 | -25.7 | -25.6 | -24.7 | -26.1 | -27.5 |
| 232.4 | 0.0 | -0.3 | 0.0 | -0.2 | -0.4 |
| 235.4 | -3.4 | -4.0 | -2.4 | -3.2 | -5.3 |
| 239.4 | -3.8 | -3.7 | -3.5 | -4.3 | -4.5 |
| 289.6 | -32.0 | -32.2 | -31.7 | -32.1 | -32.7 |
| 448.4 | -33.1 | -33.1 | -33.0 | -33.6 | -33.7 |
| 477.5 | -32.0 | -32.5 | -31.7 | -32.0 | -32.8 |

Figure 9. Reference PDP of CDL-C UMa beam 1 > 2.5 GHz

Figure 10. Reference X2V PDP of CDL-C UMa beam 1 > 2.5 GHz

Table 6. Reference TCF table of CDL-C UMa beam 1 at > 2.5 GHz

| lambdas | X2V | H11 | H21 | H12 | H22 |
| --- | --- | --- | --- | --- | --- |
| 0.0 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 |
| 0.1 | 0.985 | 0.990 | 0.982 | 0.980 | 0.990 |
| 0.2 | 0.942 | 0.960 | 0.928 | 0.924 | 0.960 |
| 0.3 | 0.874 | 0.914 | 0.844 | 0.835 | 0.913 |
| 0.4 | 0.787 | 0.854 | 0.738 | 0.723 | 0.852 |
| 0.5 | 0.689 | 0.785 | 0.619 | 0.597 | 0.781 |
| 0.6 | 0.586 | 0.711 | 0.497 | 0.469 | 0.703 |
| 0.7 | 0.486 | 0.634 | 0.382 | 0.349 | 0.621 |
| 0.8 | 0.394 | 0.560 | 0.282 | 0.248 | 0.539 |
| 0.9 | 0.315 | 0.489 | 0.206 | 0.174 | 0.458 |
| 1.0 | 0.252 | 0.425 | 0.157 | 0.132 | 0.381 |
| 1.1 | 0.206 | 0.369 | 0.136 | 0.111 | 0.308 |
| 1.2 | 0.174 | 0.324 | 0.133 | 0.094 | 0.242 |
| 1.3 | 0.154 | 0.289 | 0.141 | 0.070 | 0.182 |
| 1.4 | 0.143 | 0.265 | 0.157 | 0.038 | 0.128 |
| 1.5 | 0.137 | 0.249 | 0.180 | 0.030 | 0.080 |
| 1.6 | 0.135 | 0.241 | 0.208 | 0.069 | 0.039 |
| 1.7 | 0.134 | 0.236 | 0.234 | 0.112 | 0.014 |
| 1.8 | 0.134 | 0.232 | 0.255 | 0.148 | 0.037 |
| 1.9 | 0.130 | 0.229 | 0.265 | 0.172 | 0.064 |
| 2.0 | 0.122 | 0.224 | 0.263 | 0.181 | 0.087 |
| 2.1 | 0.109 | 0.217 | 0.250 | 0.175 | 0.104 |
| 2.2 | 0.090 | 0.207 | 0.226 | 0.156 | 0.117 |
| 2.3 | 0.069 | 0.193 | 0.197 | 0.130 | 0.124 |
| 2.4 | 0.047 | 0.176 | 0.169 | 0.104 | 0.126 |
| 2.5 | 0.031 | 0.157 | 0.148 | 0.092 | 0.124 |
| 2.6 | 0.033 | 0.136 | 0.139 | 0.102 | 0.116 |
| 2.7 | 0.046 | 0.114 | 0.139 | 0.124 | 0.105 |
| 2.8 | 0.057 | 0.093 | 0.142 | 0.145 | 0.091 |
| 2.9 | 0.062 | 0.076 | 0.138 | 0.158 | 0.074 |
| 3.0 | 0.060 | 0.064 | 0.125 | 0.160 | 0.057 |
| 3.1 | 0.050 | 0.062 | 0.101 | 0.152 | 0.041 |
| 3.2 | 0.036 | 0.069 | 0.069 | 0.134 | 0.027 |
| 3.3 | 0.019 | 0.079 | 0.035 | 0.111 | 0.020 |
| 3.4 | 0.010 | 0.089 | 0.031 | 0.087 | 0.024 |
| 3.5 | 0.019 | 0.096 | 0.060 | 0.065 | 0.035 |
| 3.6 | 0.029 | 0.099 | 0.088 | 0.048 | 0.049 |
| 3.7 | 0.034 | 0.098 | 0.107 | 0.032 | 0.064 |
| 3.8 | 0.036 | 0.094 | 0.116 | 0.021 | 0.082 |
| 3.9 | 0.044 | 0.089 | 0.117 | 0.035 | 0.101 |
| 4.0 | 0.062 | 0.087 | 0.115 | 0.071 | 0.123 |
| 4.1 | 0.090 | 0.091 | 0.121 | 0.115 | 0.145 |
| 4.2 | 0.123 | 0.102 | 0.137 | 0.163 | 0.167 |
| 4.3 | 0.155 | 0.118 | 0.162 | 0.208 | 0.187 |
| 4.4 | 0.182 | 0.134 | 0.186 | 0.244 | 0.203 |
| 4.5 | 0.200 | 0.148 | 0.202 | 0.267 | 0.213 |
| 4.6 | 0.207 | 0.158 | 0.206 | 0.273 | 0.216 |
| 4.7 | 0.200 | 0.163 | 0.194 | 0.261 | 0.211 |
| 4.8 | 0.180 | 0.163 | 0.166 | 0.231 | 0.199 |
| 4.9 | 0.149 | 0.160 | 0.125 | 0.189 | 0.182 |
| 5.0 | 0.110 | 0.155 | 0.073 | 0.143 | 0.160 |

Figure 11. Reference TCF of CDL-C UMa beam 1 at > 2.5 GHz

Figure 12. Reference X2V TCF of CDL-C UMa beam 1 at fc > 2.5 GHz

# UMa beam 2 at fc > 2.5 GHz

Table 7. Reference PDP table of CDL-C UMa beam 2 at fc > 2.5 GHz

| Delay [ns] | Power [dB] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | X2V | H11 | H21 | H12 | H22 |
| 0.0 | -24.8 | -26.7 | -23.8 | -23.9 | -26.8 |
| 76.6 | 0.0 | -0.3 | 0.0 | -0.2 | -0.5 |
| 79.4 | -11.4 | -12.3 | -10.8 | -11.0 | -12.6 |
| 81.0 | -3.2 | -4.2 | -2.3 | -2.8 | -5.1 |
| 85.0 | -3.6 | -3.5 | -3.4 | -4.3 | -4.4 |
| 232.4 | -23.3 | -23.6 | -24.4 | -23.4 | -22.8 |
| 235.4 | -16.9 | -17.3 | -15.5 | -17.0 | -19.9 |
| 239.4 | -23.5 | -23.6 | -22.4 | -23.8 | -25.5 |
| 289.6 | -26.1 | -26.5 | -25.9 | -26.2 | -26.9 |
| 299.8 | -38.9 | -39.7 | -38.8 | -38.7 | -39.5 |
| 448.4 | -25.1 | -25.3 | -25.0 | -25.5 | -25.7 |
| 477.5 | -25.8 | -26.4 | -25.5 | -25.8 | -26.6 |

Figure 13. Reference PDP of CDL-C UMa beam 2 > 2.5 GHz

Figure 14. Reference X2V PDP of CDL-C UMa beam 2 > 2.5 GHz

Table 8. Reference TCF table of CDL-C UMa beam 2 at fc > 2.5 GHz

| lambdas | X2V | H11 | H21 | H12 | H22 |
| --- | --- | --- | --- | --- | --- |
| 0.0 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 |
| 0.1 | 0.973 | 0.978 | 0.969 | 0.968 | 0.977 |
| 0.2 | 0.904 | 0.923 | 0.890 | 0.886 | 0.921 |
| 0.3 | 0.825 | 0.865 | 0.798 | 0.789 | 0.860 |
| 0.4 | 0.765 | 0.826 | 0.724 | 0.708 | 0.818 |
| 0.5 | 0.723 | 0.803 | 0.671 | 0.648 | 0.792 |
| 0.6 | 0.675 | 0.774 | 0.612 | 0.582 | 0.759 |
| 0.7 | 0.599 | 0.718 | 0.526 | 0.487 | 0.696 |
| 0.8 | 0.496 | 0.638 | 0.415 | 0.361 | 0.605 |
| 0.9 | 0.391 | 0.557 | 0.308 | 0.231 | 0.507 |
| 1.0 | 0.319 | 0.501 | 0.248 | 0.147 | 0.431 |
| 1.1 | 0.290 | 0.471 | 0.242 | 0.141 | 0.386 |
| 1.2 | 0.273 | 0.447 | 0.240 | 0.152 | 0.351 |
| 1.3 | 0.239 | 0.406 | 0.215 | 0.135 | 0.301 |
| 1.4 | 0.185 | 0.344 | 0.168 | 0.105 | 0.227 |
| 1.5 | 0.129 | 0.274 | 0.121 | 0.100 | 0.147 |
| 1.6 | 0.096 | 0.217 | 0.095 | 0.118 | 0.096 |
| 1.7 | 0.092 | 0.188 | 0.094 | 0.123 | 0.098 |
| 1.8 | 0.095 | 0.179 | 0.112 | 0.102 | 0.106 |
| 1.9 | 0.093 | 0.174 | 0.145 | 0.063 | 0.092 |
| 2.0 | 0.089 | 0.164 | 0.182 | 0.026 | 0.068 |
| 2.1 | 0.086 | 0.149 | 0.207 | 0.026 | 0.077 |
| 2.2 | 0.076 | 0.131 | 0.215 | 0.029 | 0.114 |
| 2.3 | 0.064 | 0.116 | 0.215 | 0.015 | 0.140 |
| 2.4 | 0.067 | 0.118 | 0.223 | 0.025 | 0.141 |
| 2.5 | 0.088 | 0.136 | 0.241 | 0.051 | 0.120 |
| 2.6 | 0.103 | 0.154 | 0.251 | 0.063 | 0.095 |
| 2.7 | 0.099 | 0.156 | 0.238 | 0.049 | 0.092 |
| 2.8 | 0.073 | 0.138 | 0.200 | 0.017 | 0.110 |
| 2.9 | 0.038 | 0.108 | 0.157 | 0.043 | 0.123 |
| 3.0 | 0.045 | 0.090 | 0.143 | 0.088 | 0.116 |
| 3.1 | 0.080 | 0.103 | 0.163 | 0.114 | 0.088 |
| 3.2 | 0.100 | 0.124 | 0.183 | 0.114 | 0.046 |
| 3.3 | 0.099 | 0.132 | 0.181 | 0.088 | 0.011 |
| 3.4 | 0.081 | 0.123 | 0.157 | 0.048 | 0.031 |
| 3.5 | 0.061 | 0.112 | 0.128 | 0.011 | 0.043 |
| 3.6 | 0.053 | 0.116 | 0.110 | 0.011 | 0.035 |
| 3.7 | 0.060 | 0.131 | 0.102 | 0.029 | 0.036 |
| 3.8 | 0.073 | 0.140 | 0.091 | 0.075 | 0.073 |
| 3.9 | 0.091 | 0.131 | 0.079 | 0.140 | 0.117 |
| 4.0 | 0.111 | 0.104 | 0.084 | 0.207 | 0.149 |
| 4.1 | 0.127 | 0.069 | 0.108 | 0.258 | 0.162 |
| 4.2 | 0.133 | 0.041 | 0.132 | 0.281 | 0.159 |
| 4.3 | 0.129 | 0.031 | 0.145 | 0.280 | 0.153 |
| 4.4 | 0.126 | 0.023 | 0.148 | 0.269 | 0.161 |
| 4.5 | 0.131 | 0.004 | 0.152 | 0.262 | 0.183 |
| 4.6 | 0.139 | 0.026 | 0.157 | 0.257 | 0.200 |
| 4.7 | 0.138 | 0.057 | 0.157 | 0.238 | 0.201 |
| 4.8 | 0.117 | 0.076 | 0.144 | 0.192 | 0.182 |
| 4.9 | 0.079 | 0.079 | 0.117 | 0.123 | 0.156 |
| 5.0 | 0.034 | 0.065 | 0.085 | 0.065 | 0.143 |

Figure 15. Reference TCF of CDL-C UMa beam 2 at fc > 2.5 GHz

Figure 16. Reference X2V TCF of CDL-C UMa beam 2 at fc > 2.5 GHz

# Conclusion

The following observations and proposals were made in this contribution

Proposal 1: Adopt the averaged beam specific CDL-C UMa PDP and TCF curves as reference channel emulation curves for validation purposes.

Proposal 2: Omit the combined beam curves for PDP and TCF in R4-2115759 [1].

# References

- R4-2115759, Reference Channel Emulation Curves for Validation Purposes, Keysight Technologies UK Ltd, Spirent Communications, CMCC, CAICT, 3GPP TSG-RAN WG4 Meeting # 100-e, August 2021

- R4-2114382, Reference Channel Emulation Curves, Keysight Technologies UK Ltd, 3GPP TSG-RAN WG4 Meeting # 100-e, August 2021

- R4-2113858, Reference Channel Emulation Curves for FR1, CAICT, CMCC, 3GPP TSG-RAN WG4 Meeting # 100-e, August 2021

- R4-2113854, Channel Model Targets, Spirent Communications, 3GPP TSG-RAN WG4 Meeting # 100-e, August 2021

# Annex: Comparison between Keysight, Spirent, CMCC and the Resulting Average

This annex presents the reference curve comparisons between Keysight, Spirent, CMCC. Additionally, the resulting averages are shown as well.